LISr-ISP SoC

Software Manual

(For ES2)

株式会社ロジック・アンド・デザイン

Logic and Design Inc.

版权说明

本文档中所包含信息若有更改，恕不另行通知。

上海明波通信技术股份有限公司（以下简称：本单位）不对此文档做任何明确或暗含的担保，（包括但不限于）可用于商业化和适用于某个特殊目的。

本单位不对使用本文档时产生的与设备、性能有关的错误、副作用或者间接损坏后果负责。

本单位不对非本单位提供的设备的使用和互连设备的可靠性负责。

本文档内容受版权法保护。保留所有权力。未经本单位事先书面许可，不得对本文档的任何部分进行影印、复制或者翻译成其他语言。

本文档由本单位提供，仅限于本单位的员工、代理商和客户使用。未经本单位书面许可，不得对本文档的信息和/或所包含的观点向他人进行传播。

本文档中提及的其他厂商和开发者的商业产品名称，可能已注册商标或未注册商标。

Copyright©2022-2027上海明波通信技术股份有限公司版权所有。

上海明波通信技术股份有限公司

总部：中国 上海市 张江高科技园区科苑路399号12号楼 6层

邮编：201203

电话：021-50803833

传真：021-50803831

Revision history

| Version | Date | Author | Description |
| --- | --- | --- | --- |
| V1.0 | 2023-8-14 | Liu Tie, Zeng Yu | Initial version |
| V1.1 | 2024-1-19 | Tang Fei | Change Chinese to English |
| V1.2 | 2024-7-2 | Tang Fei | Add rcc/rtc/ddr program guide |
| V1.3 | 2024-7-2 | Zheng YongQi | Add APB_MIPI_TX 0x004 register to register table |
| V2.0 | 2025-4-17 | Tang Fei | Update interrupt vector table Update SYCCFG(SOC_CCON1,AF_CCON0,AF_CCON1,AF_COUNT)/ RCC(RCC_DPBLKRSTR,RCC_RTC32KCTRL,RCC_ANATEST) register for ES2 |
| V2.0.1 | 2025-4-18 | Tang Fei | Update NVC2_S2 bit width |
| V2.0.2 | 2025-4-18 | Zhou LongZhi | Add Chapter 23\24 |
| V2.0.2 | 2025-4-21 | Zhou LongZhi | Fix the description of AF register definition |

ssss

Contents

Chapter 1 LISoC Chip Overview	11

1 Chip Overview	11

1.1 Chip Overview	11

1.2 Main functions of the chip	11

1.3 Chip block diagram	13

2 chip Boot mode	14

3 Chip working mode (clock/reset)	14

4 Address allocation of each module of the chip	15

Chapter 2 LISoC_AHB_RCC	17

1  Function list	17

2  Register list	17

3  Register detailed definition	18

4  Program Guide	34

Chapter 3 LISoC_AHB_SYSCFG	37

1  Function list	37

2  Register list	37

3  Register detailed definition	38

4  Program Guide	45

Chapter 4 LISoC_AHB_NVIC	47

1 Function list	47

2 Register List	48

3 Detailed definition of registers	48

4 Program Guide	60

Chapter 5 LISoC_AHB_QSPI	61

1  Function list	61

2  Register list	62

3	Register detailed definition	64

3.1	CTRLR0	64

3.2	CTRLR1	72

3.3	SSIENR	73

3.4	MWCR	74

3.5	SER	75

3.6	BAUDR	76

3.7	TXFTLR	77

3.8	RXFTLR	78

3.9	TXFLR	79

3.10	RXFLR	80

3.11	SR	80

3.12	IMR	83

3.13	ISR	84

3.14	RISR	86

3.15	RXOICR	88

3.16	RXUICR	89

3.17	MSTICR	89

3.18	ICR	90

3.19	IDR	90

3.20	SSIC_VERSION_ID	91

3.21	DRx(for i=0; i <=31)	91

3.22	RX_SAMPLE_DELAY	92

3.23	SPI_CTRLR0	93

3.24	XIP_MODE_BITS	97

3.25	XIP_INCR_INST	98

3.26	XIP_WRAP_INST	99

4.	Program Guide	99

4.1	Programming Considerations	99

4.2	Host SPI and SSP serial transfer	100

4.3	Host Microwire serial transmission	102

4.4	XIP transport	104

Chapter 6 LISoC_AHB_FlashCache	106

1  Function list	106

2  Register list	106

3  Register detailed definition	107

4  Program Guide	110

Chapter 7 LISoC_AHB_BOOTROM	112

1 Function list	112

2 Register list	112

3 Detailed definition of registers	112

4 Program Guide	112

Chapter 8 LISoC_AHB_SramC	113

1 Function list	113

2 Register list	113

3 Detailed definition of registers	113

4 Program Guide	113

Chapter 9 LISoC_APB_Timers	114

1 Function list	114

2 Register list	114

3 Register detailed definition	114

3.1	Timer N LoadCount (for N = 0; N < 3)	115

3.2	TimerNCurrentValue(for N = 0; N < 3)	115

3.3	TimerNControlReg(for N = 0; N < 3)	115

3.4	TimerNEOI(for N = 0; N < 3)	117

3.5	TimerNIntStatus(for N = 0; N < 3)	117

3.6	TimeNRawIntStatus(for N = 0; N < 3)	117

3.7	TIMERS_N_COMP_VERSION(for N = 0; N < 3)	118

4 Program Guide	118

Chapter 10 LISoC_APB_WDT	121

1Function list	121

2 Register list	121

3 Register detailed definition	122

3.1	WDT_CR	122

3.2	WDT_TORR	123

3.3	WDT_CCVR	124

3.4	WDT_CRR	124

3.5	WDT_STAT	125

3.6	WDT_EOI	125

3.7	WDT_COMP_VERSION	125

4 Program Guide	126

Chapter 11 LISoC_APB_SSI Master	129

1 Function list	129

2Register List	129

3 Detailed definition of registers	131

3.1	CTRLR0	131

3.2	CTRLR1	134

3.3	SSIENR	135

3.4	MWCR	136

3.5	SER	137

3.6	BAUDR	137

3.7	TXFTLR	138

3.8	RXFTLR	139

3.9	TxFLR	139

3.10	RXFLR	140

3.11	SR	140

3.12	IMR	141

3.13	ISR	142

3.14	RISR	143

3.15	TxOICR	145

3.16	RXOICR	145

3.17	RXUICR	145

3.18	MSTICR	146

3.19	ICR	146

3.20	IDR	147

3.21	SSI_VERSION_ID	147

3.22	DR x(for x=0; x< 8)	148

4 Program Guide	148

4.1 Detailed description of some functions of SSI host	148

4.2 Host SPI and SSP serial transmission	156

4.3 Host Microwire serial transmission	158

Chapter 12 LISoC_APB_SSI Slave	162

1 Function list	162

2 Register list	162

3．	Register detailed definition	164

3.1	CTRLR0	164

3.2	SSIENR	167

3.3	MWCR	168

3.4	TXFTLR	169

3.5	RXFTLR	169

3.6	TxFLR	170

3.7	RXFLR	170

3.8	SR	171

3.9	IMR	172

3.10	ISR	173

3.11	RISR	174

3.12	TxOICR	175

3.13	RXOICR	175

3.14	RXUICR	176

3.15	MSTICR	176

3.16	ICR	177

3.17	IDR	177

3.18	SSI_VERSION_ID	178

3.19	DRx(for x=0; x< 8)	178

4 Program Guide	179

4.1 Detailed description of some functions of SSI slave	179

4.2 Slave SPI and SSP serial transmission	181

4.3 Slave Microwire Serial Transfers	182

Chapter 13 LISoC_ APB_UART	184

1 function list	184

2 Register list	184

3 Register detailed definition	185

4 Program Guide	202

Chapter 14 LISoC_APB_GPIO	205

1Function list	205

2 Register list	205

3 Detailed definition of registers	206

3.1	GPIOx_SWPORT_D R(x=A,B)	206

3.2	GPIOx_SWPORT_DDR (x=A,B)	206

3.3	GPIOx_INTEN (x=A,B)	207

3.4	GPIOx_INTMASK (x=A,B)	207

3.5	GPIOx_INTTYPE_LEVEL (x=A,B)	208

3.6	GPIOx_INT_POLARITY (x=A,B)	208

3.7	GPIOx_INTSTATUS (x=A,B)	209

3.8	GPIOx_RAW_INTSTATUS (x=A,B)	209

3.9	GPIOx_DEBOUNCE (x=A,B)	210

3.10	GPIOx_PORTA_EOI (x=A,B)	210

3.11	GPIOx_EXT_PORT (x=A,B)	211

3.12	GPIOx_LS_SYNC (x=A,B)	211

3.13	GPIOx_INT_BOTHEDGE(x=A,B)	212

3.14	GPIOx_VER_ID_CODE (x=A,B)	213

4 Program Guide	213

4.1 I/O port (portx,x=a,b)	213

4.2 Interrupts	214

4.3 Description	215

Chapter 15 LISoC_APB_I2C	217

1 Function list	217

2 Register list	217

3 Detailed definition of registers	220

3.1	ICx_CON (x=0,1,2)	220

3.2	ICx_TAR (x=0,1,2)	223

3.3	ICx_SAR (x=0,1,2)	224

3.4	ICx_HAS_MADDR (x=0,1,2)	225

3.5	ICx_DATA_CMD (x=0,1,2)	225

3.6	ICx_SS_SCL_HCNT (x=0,1,2)	227

3.7	ICx_SS_SCL_LCNT (x=0,1,2)	227

3.8	ICx_FS_SCL_HCNT(x=0,1,2)	228

3.9	ICx_FS_SCL_LCNT(x=0,1,2)	228

3.10	ICx_HS_SCL_HCNT(x=0,1,2)	229

3.11	ICx_HS_SCL_LCNT(x=0,1,2)	230

3.12	ICx_INTR_STAT (x=0,1,2)	230

3.13	ICx_INTR_MASK (x=0,1,2)	233

3.14	ICx_RAW_INTR_STAT (x=0,1,2)	237

3.15	IC x_RX_TL (x=0,1,2)	241

3.16	ICx_TX_TL (x=0,1,2)	241

3.17	IC x_CLR_INTR (x=0,1,2)	242

3.18	ICx_CLR_RX_UNDER (x=0,1,2)	242

3.19	ICx_CLR_RX_OVER (x=0,1,2)	243

3.20	ICx_CLR_TX_OVER (x=0,1,2)	243

3.21	ICx_CLR_RD_REQ (x=0,1,2)	244

3.22	ICx_CLR_TX_ABRT (x=0,1,2)	244

3.23	ICx_CLR_RX_DONE (x=0,1,2)	245

3.24	ICx_CLR_ACTIVITY (x=0,1,2)	245

3.25	ICx_CLR_STOP_DET (x=0,1,2)	245

3.26	ICx_CLR_START_DET (x=0,1,2)	246

3.27	ICx_CLR_GEN_CALL (x=0,1,2)	246

3.28	ICx_ENABLE (x=0,1,2)	247

3.29	ICx_STATUS (x=0,1,2)	249

3.30	ICx_TXFLR (x=0,1,2)	250

3.31	ICx_RXFLR (x=0,1,2)	251

3.32	ICx_SDA_HOLD(x=0,1,2)	251

3.33	ICx_TX_ABRT_SOURCE (x=0,1,2)	252

3.34	ICx_ACK_GENERAL_CALL(x=0,1,2)	256

3.35	ICx_ENABLE_STATUS (x=0,1,2)	256

3.36	ICx_FS_SPKLEN(x=0,1,2)	258

3.37	ICx_HS_SPKLEN(x=0,1,2)	259

3.38	ICx_COMP_VERSION (x=0,1,2)	260

4 Program Guide	260

4.1	a few terms	260

4.2	I2C/SMBus Definition of Bits in First Byte	261

4.3	operating mode	261

4.4	I2C/SMBus Definition of Bits in First Byte	264

Chapter 16 LISoC_APB_RTC	266

1 Function list	266

2 Register list	266

3 Detailed definition of registers	267

3.1	RTC_CCVR	267

3.2	RTC_CMR	267

3.3	RTC_CLR	268

3.4	RTC_CCR	268

3.5	RTC_STAT	270

3.6	RTC_RSTAT	270

3.7	RTC_EOI	271

3.8	RTC_COMP_VERSION	271

3.9	RTC_CPSR	272

3.10	RTC_CPCVR	273

4 Program Guide	273

Chapter 17 LISoC_APB_DDRC	274

1 Function list	274

2 Register list	274

2.1 DDR PHY register list	274

2.2 DDR Control register list	274

3 Detailed definition of registers	274

3.1 Detailed definition of DDR PHY register	274

3.2 Detailed definition of DDR Control register	304

4 Program Guide	328

Chapter 18 LISoC_APB_MIPI_RX	329

1 Function list	329

2 Register list	329

3 Detailed definition of registers	330

4  Program Guide	339

Chapter 19 LISoC_APB_MIPI_TX	340

1 Function list	340

2 Register list	340

3 Detailed definition of registers	340

4 Program Guide	347

Chapter 20 LISoC_APB_CCHG	348

1 Function list	348

2 Register list	348

3 Detailed definition of registers	348

4 Program Guide	350

Chapter 21 LISoC_APB_ISP	351

1 Function list	351

2 Register list	351

3 Detailed definition of registers	351

4 Program Guide	356

Chapter 22 LISoC_APB_LISr	357

1 Function list	357

2 Register list	357

3 Detailed definition of registers	358

4 Program Guide	371

Chapter 23 LISoC_APB_AF	372

1	Function list	372

2	Register list	372

3	Detailed definition of registers	372

4 Program Guide	374

Chapter 24 LISoC_APB_TPGEN	375

1	Function list	375

2	Register list	375

3 Detailed definition of registers	376

Chapter 1 LISoC Chip Overview

1 Chip Overview

1.1 Chip Overview

LISr-ISP SoC chip (abbreviated as LISOC chip) is a powerful video image processing chip that can be used in applications such as digital cameras, security monitoring, and machine vision.

The LISOC chip integrates Arm Cortex M3 32-bit CPU, various hardware algorithm modules such as ISP and LISr, as well as high-speed communication interfaces such as QSPI flash, DDR, MIPI, SPI, and general interfaces such as GPIO, UART, and i2c.

1.2 Main functions of the chip

    - System functions

- CPU:

  - 32-bit Arm Cortex M3

  - frequency: 133MHz

- on-chip memory

  - 32K program ROM for Boot

  - 3 x 128 K SRAM , supports single-cycle read and write

- interrupt controller

  - Provides 32 interrupt sources

  - Interrupt priority is configurable

- Timer/WatchDog/RTC

  - Provides three independent Timers for counting system clocks

  - Provide 32bit watchdog

  - Provide RTC

- Algorithm module

  - Provide hardware algorithm accelerators such as ISP and LISr

    - external interface

- DDR4 interface

  - Provides 2 32bit DDR4 interface channels and supports 800MHz dual rate access.

  - Supports 4 external 256MB or 512MB DDR RAM (total capacity 1GB or 2GB)

- QSPI flash interface

  - Maximum operating clock 33MHz (TBD)

  - Supports external flash

(SPI-flash chip with capacity up to 16MB can be used, but the addressable space size is 256KB)

  - Supports W25Q128, N25Q128 and other flash manufacturers models

  - The built-in flash cache controller provides 8KB instruction cache space.

- MIPI-Tx/Rx interface

  - MIPI-CSI-2 specifications

  - Max 4 lanes

  - Max pixel clock: 324MHz

- SPI master/slave interface

  - Support 4~16bit transmission

  - Provide RX, TX fifo

  - Max 4Mbps transmission

- I2C interface

  - Configurable baud rate

  - Support 400Kbps high-speed transmission

  - Supports hardware parity checking

- UART interface

  - Configurable baud rate

  - Support 8, 9bit and other data formats

  - Supports hardware parity checking

- GPIO interface

  - Provides 2 groups (16 in each group, 32 in total) GPIO,

  - 11 are available by default after power-on, and the other 21 can be used after modifying the default configuration.

  - Interrupt trigger mode configurable

- JTAG debugging interface, SWD debugging interface.

  - Both debug interfaces share the same set of pins

  - The chip automatically adapts and switches internally according to the external debugger signal.

1.3 Chip block diagram

Note:

HSE = High speed external clock input

RCC = Reset and clock control

CCHG = Clock change

AF = Auto focus

C/P regs = Controller peripheral registers

2 chip Boot mode

After the chip is powered on or reset, it enters BOOT ROM operation. The code in the BOOT ROM enters different Boot modes based on the UART1 signal input and the BOOT config pin level:

  - If several consecutive characters of 0x55 0xaa 0x66 0xbb are received on UART1 within 50ms, the UART command mode will be entered.

  - If the 0x55 0xaa 0x66 0xbb string is not received on UART1 within 50ms, the boot mode will be determined based on the levels of the two boot config pins BOOT1 and BOOT0.

The meaning of BOOT config pin level configuration is shown in the table below:

| BOOT config pin |  | Boot mode |
| --- | --- | --- |
| BOOT1 | BOOT0 |  |
| 0 | 0 | Copy QSPI-flash content to SRAM and run on SRAM |
| 0 | 1 | XIP (Execution in-place) from QSPI-flash |
| 1 | 0 | XIP (Execution in-place) from QSPI-flash, with L1 cache support |
| 1 | 1 | Reserved for debug purpose |

3 Chip working mode (clock/reset)

The chip provides a variety of clock and reset generation mechanisms. Users can set external pins for configuration. The specific setting methods are shown in the table below:

Reset mechanism selection:

| PIN input | Logic high | Logic low (default) |
| --- | --- | --- |
| DLCK_MASK | Allow signal DCXO_LOCK to affect system reset | Prevent the impact of signal DCXO_LOCK on system reset |

Recommend to pull this pin high through a 10K ohm resister on your application board. (ie change the default setting to “ Allow signal DCXO_LOCK to affect system reset” )

The clock structure of the chip is shown in the figure below:

4 Address allocation of each module of the chip

LiSOC memory map table

| name | AHB/APB | size | start addr | end addr |
| --- | --- | --- | --- | --- |
| BOOT ROM | AHB | 64K | 0x0000 0000 | 0x0000 FFFF |
| QSPI | AHB | 1024K | 0x1F00 0000 | 0x1F0F FFFF |
| SRAM0 | AHB | 128K | 0x1FF0 0000 | 0x1FF1 FFFF |
| SRAM1 | AHB | 128K | 0x1FF2 0000 | 0x1FF3 FFFF |
| SRAM2 | AHB | 128K | 0x1FF4 0000 | 0x1FF5 FFFF |
| SRAM0 (mirror) | AHB | 128K | 0x2000 0000 | 0x2001 FFFF |
| SRAM1 (mirror) | AHB | 128K | 0x2002 0000 | 0x2003 FFFF |
| SRAM2 (mirror) | AHB | 128K | 0x2004 0000 | 0x2005 FFFF |
| UART 0 | APB | 1K | 0x4000 0000 | 0x4000 03FF |
| UART 1 | APB | 1K | 0x4000 0400 | 0x4000 07FF |
| Timer 0 | APB | 1K | 0x4000 0800 | 0x4000 0BFF |
| Timer 1 | APB | 1K | 0x4000 0C00 | 0x4000 0FFF |
| Timer 2 | APB | 1K | 0x4000 1000 | 0x4000 13FF |
| I2C 0 | APB | 1K | 0x4000 1400 | 0x4000 17FF |
| I2C 1 | APB | 1K | 0x4000 1800 | 0x4000 1BFF |
| I2C 2 | APB | 1K | 0x4000 1C00 | 0x4000 1FFF |
| SPI slave | APB | 1K | 0x4000 2000 | 0x4000 23FF |
| SPI master | APB | 1K | 0x4000 2400 | 0x4000 27FF |
| RTC | APB | 1K | 0x4000 2C00 | 0x4000 2FFF |
| WDT | APB | 1K | 0x4000 3000 | 0x4000 33FF |
| GPIO-A | APB | 1K | 0x4000 3400 | 0x4000 37FF |
| GPIO-B | APB | 1K | 0x4000 3800 | 0x4000 3BFF |
| DDR0_PHY | APB | 64K | 0x4001 0000 | 0x4001 FFFF |
| DDR1_PHY | APB | 64K | 0x4002 0000 | 0x4002 FFFF |
| DDR0_Ctrl | APB | 64K | 0x4003 0000 | 0x4003 FFFF |
| DDR1_Ctrl | APB | 64K | 0x4004 0000 | 0x4004 FFFF |
| MIPI-RX | APB | 64K | 0x4005 0000 | 0x4005 FFFF |
| MIPI-TX | APB | 64K | 0x4006 0000 | 0x4006 FFFF |
| ISP | APB | 64K | 0x4007 0000 | 0x4007 FFFF |
| LISR | APB | 64K | 0x4008 0000 | 0x4008 FFFF |
| Cache | APB | 64K | 0x4009 0000 | 0x4009 FFFF |
| CCHG | APB | 64K | 0x400A 0000 | 0x400A FFFF |
| RCC | AHB | 4K | 0x400F 0000 | 0x400F 0FFF |
| SYSCFG | AHB | 4K | 0x400F 1000 | 0x400F 1FFF |
| ext DDR RAM0 | AHB | 512M | 0x6000 0000 | 0x7FFF FFFF |
| ext DDR RAM1 | AHB | 512M | 0x8000 0000 | 0x9FFF FFFF |

Chapter 2 LISoC_AHB_RCC

1  Function list

- CPU reset/remap control

- Provide JTAG disable Function

- Pflash write protection control

- USB/DMA/SPI/SDIO protocol mode control

- PLL bypass/powerdown/ratio control

- OSC bypass control

- Clock/reset control of each module on the bus

2  Register list

BASEADDR: 0x 400F 0000

| Offset | Name | Access | Reset | Width | Description |
| --- | --- | --- | --- | --- | --- |
| 0x00 | RCC_CR | R/W | 0xFD09 | 32 | Clock control register |
| 0x04 | RCC_DATPLLCFGR | R/W | 0x30 | 32 | Data PLL(PLL0) config register |
| 0x08 | RCC_SYSPLLCFGR | R/W | 0x1D | 32 | Sys PLL(PLL1) config register |
| 0x0c | RCC_SYSCFGR | R/W | 0x00 | 32 | Sys clock config register |
| 0x10 | RCC_SYSENR | R/W | 0x3F | 32 | MCU Sys bus module clock control |
| 0x14 | RCC_PERENR | R/W | 0x1FFF | 32 | MCU peripheral module clock control |
| 0x18 | RCC_DPENR | R/W | 0x1FF | 32 | Data path module clock control |
| 0x1c | RCC_SYSBLKRSTR | R/W | 0x00 | 32 | Sys block reset control |
| 0x20 | RCC_DPBLKRSTR | R/W | 0x00 | 32 | Data path block reset control |
| 0x24 | RCC_DCXO_CTRL0 | R/W | 0x882020 | 32 | DCXO control #0 |
| 0x28 | RCC_DCXO_CTRL1 | R/W | 0xE011010 | 32 | DCXO control #1 |
| 0x2c | RCC_DATPLLCTRL | R/W | 0x0142 4497 | 32 | Data PLL debug control |
| 0x30 | RCC_SYSPLLCTRL | R/W | 0x0142 4497 | 32 | Sys PLL debug control |
| 0x34 | RCC_RTC32KCTRL | R/W | 0x00 | 32 | RTC clock debug control |
| 0x38 | RCC_TRIMCTRL | R/W | 0x3333 | 32 | Analog trimming control |
| 0x3c | RCC_ANATEST | R/W | 0x00 | 32 | Analog test control |
| 0x40 | RCC_DEBUGRO | RO | 0x00 | 32 | RTC clock status monitoring |

3  Register detailed definition

RCC_CR

- Name: Clock control register

- Size：32bits

- Addr Offset：0x00

- Read/Write access：read/write

- Reset Value: 0xFD09

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [31] | DATPLL_LOCK | RO | 0x00 | DAT PLL lock status flag bit |
| [30] | SYSPLL_LOCK | RO | 0x00 | SYSPLL 's lock status flag bit |
| [29:20] | Reserved | RO | 0x00 | Reserved |
| [19] | APBDIV | R/W | 0x00 | 1: APB clock is AHB/2 0: APB and AHB clock frequencies are equal |
| [18:16] | MCOSEL | R/W | 0x00 | Microcontroller clock output clock will be divided by 16 000: Select CPU_CLK 001: Select AXI_CLK 010: Select MRX_BYTE_CLK 011: Select MRX_PIX_CLK 100: Select VIDEO_CLK1 101:Select VIDEO_CLK2 110-1111: Reserved |
| [ 15:10 ] | NVC2_S1 | R/W | 0x3f | First stage divide ratio of Video_clk2 |
| [ 9:8 ] | NVC1 | R/W | 0x01 | Divide ratio of Video_clk1 |
| [ 7 :6] | NMP | R/W | 0x00 | Divide ratio of MRX_PIX_CLK |
| [5:4] | Reserved | RO | 0x00 | Reserved |
| [ 3 :1] | NVC2_S2 | R/W | 0x04 | Second stage divide ratio of Video_clk2 |
| [ 0 ] | FRAC_EN | R/W | 0x01 | Enable fractional division |

RCC_DATPLLCFGR

- Name: Data PLL(PLL0) config register

- Size: 32bits

- Addr Offset: 0x04

- Read/Write access: read/write

- Reset Value: 0x30

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [31:12] | Reserved | RO | 0x00 | Reserved |
| [11] | DA_EN | R/W | 0x00 | PLL enable control 0: disable (power down) 1: enable |
| [10] | DA_OUT_EN | R/W | 0x00 | PLL output buffer enable control 0: the output clock is forced to low 1: the output clock send out |
| [9] | Reserved | RO | 0x00 | Reserved |
| [8] | RG_DIV_RST | R/W | 0x00 | Reset signal for divider. High active. |
| [7] | Reserved | RO | 0x00 | Reserved |
| [6] | RG_NPOST | R/W | 0x00 | Post divider ratio for PLL 0: divider ratio=1 1: divider ratio=2 |
| [5:0] | RG_NDIV | R/W | 0x30 | Feedback divider ratio for PLL |

RCC_SYSPLLCFGR

- Name: Sys PLL(PLL1) config register

- Size: 32bits

- Addr Offset: 0x08

- Read/Write access: read/write

- Reset Value: 0x1D

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [31:12] | Reserved | RO | 0x00 | Reserved |
| [11] | DA_EN | R/W | 0x00 | PLL enable control 0: disable (power down) 1: enable |
| [10] | DA_OUT_EN | R/W | 0x00 | PLL output buffer enable control 0: the output clock is forced to low 1: the output clock send out |
| [9] | Reserved | RO | 0x00 | Reserved |
| [8] | RG_DIV_RST | R/W | 0x00 | Reset signal for divider. High active. |
| [7] | Reserved | RO | 0x00 | Reserved |
| [6] | RG_NPOST | R/W | 0x00 | Post divider ratio for PLL 0: divider ratio=1 1: divider ratio=2 |
| [5:0] | RG_NDIV | R/W | 0x1D | Feedback divider ratio for PLL |

RCC_SYSCFGR

- Name: Sys clock config register

- Size: 32bits

- Addr Offset: 0x0c

- Read/Write access: read/write

- Reset Value: 0x0

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [31:6] | Reserved | RO | 0x00 | Reserved |
| [5] | VI2SW | R/W | 0x00 | VIDEO_CLK2 0: Select HSE as the source clock 1: Select PLL1 clock as the source clock |
| [4] | VI1SW | R/W | 0x00 | VIDEO_CLK1 0: Select HSE as the source clock 1: Select PLL1 clock as the source clock |
| [3] | PIXSW | R/W | 0x00 | MRX_PIX_CLK 0: Select HSE as the source clock 1: Select PLL1 clock as the source clock |
| [2] | BYTSW | R/W | 0x00 | MRX_BYTE_CLK 0: Select HSE as the source clock 1: Select 1/3 1/2 clock of PLL1 as the source clock |
| [1] | AXISW | R/W | 0x00 | LISR/DDR/AXI clock switching 0: Select HSE as the source clock the clock of PLL0 divided by 1/2 |
| [0] | SYSSW | R/W | 0x00 | System clock sys_ck switch (System clock switch) cleared by software and used to select the system clock source. Set by hardware when exiting shutdown or standby mode or when used directly or indirectly as the system clock's HSE oscillator. [TBD] 0: HSE oscillator is used as system clock 1: SYSPLL divides the clock by 1/3 and 1/2 as the system clock. |

RCC_SYSENR

- Name: MCU Sys bus module clock control

- Size: 32bits

- Addr Offset: 0x10

- Read/Write access: read/write

- Reset Value: 0x3F

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [31:18] | Reserved | RO | 0x00 | Reserved |
| [17] | SYSRST | R/W | 0x0 | Software control of system reset. 0: Writing 0 is invalid 1: Writing 1 resets the entire system |
| [16:6] | Reserved | RO | 0x00 | Reserved |
| [5] | SRAM2HENR | R/W | 0x1 | Internal SRAM 2 clock enable 0: disable clock 1: Turn on the clock |
| [4] | SRAM1HENR | R/W | 0x1 | Internal SRAM1 clock enable 0: disable clock 1: Turn on the clock |
| [3] | SRAM0HENR | R/W | 0x1 | Internal SRAM 0 clock enable 0: disable clock 1: Turn on the clock |
| [2] | ROMHENR | R/W | 0x1 | Internal boot ROM clock enable 0: disable clock 1: Turn on the clock |
| [1] | CACHEHENR | R/W | 0x1 | Flash cache clock is enabled. 0: disable clock 1: Turn on the clock |
| [0] | DDRHENR | R/W | 0x1 | DDR HCLK clock enable. 0: disable clock 1: Turn on the clock |

RCC_PERENR

- Name: MCU peripheral module clock control

- Size: 32bits

- Addr Offset: 0x14

- Read/Write access: read/write

- Reset Value: 0x1FFF

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [31:13] | Reserved | RO | 0x00 | Reserved |
| [12] | UART0ENR | R/W | 0x1 | 0: disable clock 1: Turn on the clock |
| [11] | UART1ENR | R/W | 0x1 | 0: disable clock 1: Turn on the clock |
| [10] | TIMER0ENR | R/W | 0x1 | 0: disable clock 1: Turn on the clock |
| [9] | TIMER1ENR | R/W | 0x1 | 0: disable clock 1: Turn on the clock |
| [8] | TIMER2ENR | R/W | 0x1 | 0: disable clock 1: Turn on the clock |
| [7] | I2C0ENR | R/W | 0x1 | 0: disable clock 1: Turn on the clock |
| [6] | I2C1ENR | R/W | 0x1 | 0: disable clock 1: Turn on the clock |
| [5] | I2C2ENR | R/W | 0x1 | 0: disable clock 1: Turn on the clock |
| [4] | SPIMSTENR | R/W | 0x1 | 0: disable clock 1: Turn on the clock |
| [3] | SPISLVENR | R/W | 0x1 | 0: disable clock 1: Turn on the clock |
| [2] | GPIOENR | R/W | 0x1 | 0: disable clock 1: Turn on the clock |
| [1] | RTCENR | R/W | 0x1 | 0: disable clock 1: Turn on the clock |
| [0] | WDTENR | R/W | 0x1 | 0: disable clock 1: Turn on the clock |

RCC_DP ENR

- Name: Data path module clock control

- Size: 32bits

- Addr Offset: 0x18

- Read/Write access: read/write

- Reset Value: 0x1FF

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [31:9] | Reserved | RO | 0x00 | Reserved |
| [8] | PLL1TDPENR | R/W | 0x1 | 0: disable clock 1: Turn on the clock |
| [7] | PLL0TAXIENR | R/W | 0x1 | 0: disable clock 1: Turn on the clock |
| [6] | DDR1COREENR | R/W | 0x1 | 0: disable clock 1: Turn on the clock |
| [5] | DDR0COREENR | R/W | 0x1 | 0: disable clock 1: Turn on the clock |
| [4] | AXIENR | R/W | 0x1 | 0: disable clock 1: Turn on the clock |
| [3] | BYTENR | R/W | 0x1 | 0: disable clock 1: Turn on the clock |
| [2] | PIXENR | R/W | 0x1 | 0: disable clock 1: Turn on the clock |
| [1] | VIDEO1ENR | R/W | 0x1 | 0: disable clock 1: Turn on the clock |
| [0] | VIDEO2ENR | R/W | 0x1 | 0: disable clock 1: Turn on the clock |

RCC_SYSBLKRSTR

- Name: Sys block reset control

- Size: 32bits

- Addr Offset: 0x1c

- Read/Write access: read/write

- Reset Value: 0x0

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [31:15] | Reserved | RO | 0x0 | Reserved |
| [14] | SPISLVRST | R/W | 0x0 | 0 : disable reset 1: Reset module |
| [13] | SPIMSTRST | R/W | 0x0 | 0 : disable reset 1: Reset module |
| [12] | WDTRST | R/W | 0x0 | 0 : disable reset 1 : Reset module |
| [11] | RTCRST | R/W | 0x0 | 0 : disable reset 1: Reset module |
| [10] | GPIOBRST | R/W | 0x0 | 0 : disable reset 1: Reset module |
| [9] | GPIOARST | R/W | 0x0 | 0 : disable reset 1: Reset module |
| [8] | I2C2RST | R/W | 0x0 | 0 : disable reset 1: Reset module |
| [7] | I2C1RST | R/W | 0x0 | 0 : disable reset 1: Reset module |
| [6] | I2C0RST | R/W | 0x0 | 0 : disable reset 1: Reset module |
| [5] | TIMER2RST | R/W | 0x0 | 0 : disable reset 1: Reset module |
| [4] | TIMER1RST | R/W | 0x0 | 0 : disable reset 1: Reset module |
| [3] | TIMER0RST | R/W | 0x0 | 0 : disable reset 1: Reset module |
| [2] | UART1RST | R/W | 0x0 | 0 : disable reset 1: Reset module |
| [1] | UART0RST | R/W | 0x0 | 0 : disable reset 1: Reset module |
| [0] | CACHERST | R/W | 0x0 | 0 : disable reset 1: Reset module |

RCC_DPBLKRSTR

- Name: Data path block reset control

- Size: 32bits

- Addr Offset: 0x20

- Read/Write access: read/write

- Reset Value: 0x0

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [31:11] | Reserved | RO | 0x0 | Reserved |
| [12] | AFRST | R/W | 0x0 | 0 : disable reset 1: Reset module |
| [11] | TPGENRST | R/W | 0x0 | 0 : disable reset 1: Reset module |
| [10] | DDR1PHYRST | R/W | 0x0 | 0 : disable reset 1: Reset module |
| [9] | DDR0PHYRST | R/W | 0x0 | 0 : disable reset 1: Reset module |
| [8] | DDR1CORRERST | R/W | 0x0 | 0 : disable reset 1: Reset module |
| [7] | DDR0CORRERST | R/W | 0x0 | 0 : disable reset 1: Reset module |
| [6] | MIPIRXRST | R/W | 0x0 | 0 : disable reset 1: Reset module |
| [5] | CCHGRST | R/W | 0x0 | 0 : disable reset 1: Reset module |
| [4] | ISPRST | R/W | 0x0 | 0 : disable reset 1: Reset module |
| [3] | MIPITXRST | R/W | 0x0 | 0 : disable reset 1: Reset module |
| [2] | LISRRST | R/W | 0x0 | 0 : disable reset 1: Reset module |
| [1] | DDR0RST | R/W | 0x0 | 0 : disable reset 1: Reset module |
| [0] | DDR1RST | R/W | 0x0 | 0 : disable reset 1: Reset module |

RCC_DCXO_CTRL0

- Name: DCXO control #0

- Size: 32bits

- Addr Offset: 0x24

- Read/Write access: read/write

- Reset Value: 0x882020

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [ 31:25 ] | Reserved | RO | 0x0 | - |
| [ 24] | DCXO_MODE | R/W | 0x0 | Work mode selection 0: XO mode 1: buffer mode |
| [ 23:20 ] | IBIAS_CTRL | R/W | 0x8 | Current bias control |
| [ 19:16 ] | GM_CTRL | R/W | 0x8 | GM control |
| [ 15 ] | Reserved | RO | 0x0 | - |
| [ 14:8 ] | CD_CTRL | R/W | 0x20 | Control G-CDAC |
| [ 7 ] | Reserved | RO | 0x0 | - |
| [ 6:0 ] | CG_CTRL | R/W | 0x20 | Control G-CDAC |

RCC_DCXO_CTRL1

- Name: DCXO control #1

- Size: 32bits

- Addr Offset: 0x28

- Read/Write access: read/write

- Reset Value: 0xE011010

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [31:27] | Reserved | RO | 0x1 | (Bit 27 is reserved and read as 1) |
| [26] | DREG_RSTN_PDT | R/W | 0x1 | Register input for option of peak detector output latch reset |
| [25] | DREG_RSTN_CLKDIG | R/W | 0x1 | Register input for option of digital clock output latch reset |
| [24] | RSTN_SEL | R/W | 0x0 | Digital clock and peak detector output latch reset selection |
| [23:17] | Reserved | RO | 0x0 | - |
| [16] | PKDET_EN | R/W | 0x1 | Peak detector enable |
| [15:13] | Reserved | RO | 0x0 | - |
| [12:8] | SW_VCTRL | R/W | 0x10 | Control the 0.9v for Cbank switch |
| [7:5] | Reserved | RO | 0x0 | - |
| [4:0] | TC_VCTRL | R/W | 0x10 | DCXO core temperature compensation control |

RCC_DATPLLCTRL

- Name: Data PLL debug control

- Size: 32bits

- Addr Offset: 0x2c

- Read/Write access: read/write

- Reset Value: 0x01424497

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [ 31:25 ] | Reserved | RO | 0x0 | - |
| [ 24:23 ] | DPLL_PFD_RSTDLY | R/W | 0x2 | PFD reset delay control signal: tt/50/1.55 ff/-25/1.6 ss/105/1.5 00: 305p 211p 430p 01: 362p 252p 511p 10: 305p 211p 430p 11: 362p 252p 511p |
| [ 22:21 ] | DPLL_PDF_MODE | R/W | 0x2 | PLL PFD function mode control signal: 00: Normal (PFD time offset to ensure linearity of PFD-CP) 01: improved PFDCP (CP offset current to ensure linearity of PFD-CP) 10: Normal (PFD time offset to ensure linearity of PFD-CP) 11: improved PFDCP (CP offset current to ensure linearity of PFD-CP) |
| [ 20 ] | DPLL_LOCK_CNTTUNE | R/W | 0 x 0 | PLL Lock detector counter control |
| [ 19 ] | DPLL_LOCK_DLYTUNE | R/W | 0 x 0 | PLL Lock detector delay control |
| [ 18 ] | DPLL_LOCK_RST | R/W | 0x0 | PLL Lock detector reset signal. High active. |
| [ 17:15 ] | DPLL_CP_CPI | R/W | 0x4 | CP current control signal, 6~20uA, step = 2uA, CP output current: Icp=6uA+CPI[2:0]*2uA |
| [ 14:11 ] | DPLL_LPF_R2 | R/W | 0x8 | PLL LPF R1 control signal: Fixed: 4k, 500ohm/step |
| [ 10:8 ] | DPLL_LPF_C2 | R/W | 0x4 | PLL LPF C2 control signal: Fixed: 12p, 3p/step |
| [ 7:5 ] | DPLL_LPF_C1 | R/W | 0x4 | PLL LPF C1 control signal: Fixed: 3.2p, 0.8p/step |
| [ 4 ] | DPLL_VCO_FC | R/W | 0x1 | BBPLL VCOfast start up. 1: fast charge. 0: normal mode when stable. |
| [ 3:0 ] | DPLL_VCO_ICTRL | R/W | 0x7 | control word for IBG of VCO |

RCC_SYSPLLCTRL

- Name: Sys PLL debug control

- Size : 32bits

- Addr Offset : 0x30

- Read/Write access : read/write

- Reset Value: 0x 01424497

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [31:30] | SPLL_DC_TEST_SEL | R/W | 0x0 | DC test signals selection. 00: VCO LDO VCO 1.1V LDO output 01: PLL LDO PLL 1.1V LDO output 10: VCTRL VCO control voltage 11: floating |
| [29:28] | Reserved | RO | 0x0 | - |
| [27] | SPLL_CLK_TEST_SEL | R/W | 0x0 | Clok test signals selection. 0: divider clock 1: reference clock |
| [26:25] | Reserved | RO | 0x0 | - |
| [24:23] | SPLL_PFD_RSTDLY | R/W | 0x2 | PFD reset delay control signal: tt/50/1.55 ff/-25/1.6 ss/105/1.5 00: 305p 211p 430p 01: 362p 252p 511p 10: 305p 211p 430p 11: 362p 252p 511p |
| [22:21] | SPLL_PDF_MODE | R/W | 0x2 | PLL PFD function mode control signal: 00: Normal (PFD time offset to ensure linearity of PFD-CP) 01: improved PFDCP (CP offset current to ensure linearity of PFD-CP) 10: Normal (PFD time offset to ensure linearity of PFD-CP) 11: improved PFDCP (CP offset current to ensure linearity of PFD-CP) |
| [20] | SPLL_LOCK_CNTTUNE | R/W | 0x0 | PLL Lock detector counter control |
| [19] | SPLL_LOCK_DLYTUNE | R/W | 0x0 | PLL Lock detector delay control |
| [18] | SPLL_LOCK_RST | R/W | 0x0 | PLL Lock detector reset signal. High active. |
| [17:15] | SPLL_CP_CPI | R/W | 0x4 | CP current control signal, 6~20uA, step = 2uA, CP output current: Icp=6uA+CPI[2:0]*2uA |
| [14:11] | SPLL_LPF_R2 | R/W | 0x8 | PLL LPF R1 control signal: Fixed: 4k, 500ohm/step |
| [10:8] | SPLL_LPF_C2 | R/W | 0x4 | PLL LPF C2 control signal: Fixed: 12p, 3p/step |
| [7:5] | SPLL_LPF_C1 | R/W | 0x4 | PLL LPF C1 control signal: Fixed: 3.2p, 0.8p/step |
| [4] | SPLL_VCO_FC | R/W | 0x1 | BBPLL VCOfast start up. 1: fast charge. 0: normal mode when stable. |
| [3:0] | SPLL_VCO_ICTRL | R/W | 0x7 | control word for IBG of VCO |

RCC_RTC32KCTRL

- Name: RTC clock debug control

- Size: 32bits

- Addr Offset: 0x34

- Read/Write access: read/write

- Reset Value: 0x0

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [31:23] | Reserved | RO | 0x0 | - |
| [22:19] | ANA_RES | R/W | 0x0 |  |
| [18:13] | Reserved | RO | 0x0 |  |
| [12] | CNT_SEL | R/W | 0x0 | RC32K reset time selector |
| [11:5] | EXT_CODE | R/W | 0x0 | RC32K external code value |
| [4] | EXT_CODE_EN | R/W | 0x0 | RC32K external code enable |
| [3:2] | CAL_DIV | R/W | 0x0 | RC32K calibration frequency divider 00: div 1/16, calibration time 4+8 ms 01: div 1/32, calibration time 8+8 ms 02: div 1/64, calibration time 16+8 ms 03: div 1/128, calibration time 32+8 ms |
| [1] | CAL_ALLOW | R/W | 0x0 | RC32K calibration allow signal, high active |
| [0] | CAL_EN | R/W | 0x0 | RC32K calibration enable signal, high active 0: disable 1: enable |

RCC_TRIMCTRL

- Name: Analog trimming control

- Size: 32bits

- Addr Offset: 0x38

- Read/Write access: read/write

- Reset Value: 0x3333

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [31:15] | Reserved | RO | 0x0 | - |
| [14:12] | LDO18_TRIM | R/W | 0x3 | BIAS_1P8V_TOP LDO trimming 000: 1.63V 001: 1.7V 010: 1.75V 011: 1.8V 100: 1.84V 101: 1.89V 110: 1.94V 111: 2V |
| [11] | Reserved | RO | 0x0 | - |
| [10:8] | LDO11_TRIM | R/W | 0x3 | BBPLL LDO PFD(PFD) output voltage control signal: 000: 1.025V, 001: 1.050V 010: 1.075V, 011: 1.100V 100: 1.125V, 101: 1.150V 110: 1.175V, 111: 1.200V |
| [7] | Reserved | RO | 0x0 | - |
| [6:4] | BIAS_TEMP_TRIM | R/W | 0x3 | it is used to trimming the temp cofficient deviation caused by the resistance mismatch , the tuning step is about 40ppm. Vref is also changed along with the code typical case: 000 : 1.283 001 : 1.257 010 : 1.231 011 : 1.205 100 : 1.184 101 : 1.163 110 : 1.142 111 : 1.122 ref: 《BG_1P8V_SIM》 |
| [3] | Reserved | RO | 0x0 | - |
| [2:0] | BIAS_VBG_TRIM | R/W | 0x3 | it is used to trimming the VREF 1P2V output |

RCC_ANATEST

- Name: Analog test control

- Size: 32bits

- Addr Offset: 0x3c

- Read/Write access: read/write

- Reset Value: 0x0

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [31] | DIV_RSTN | R/W | 0x0 |  |
| [30] | DIV_TEST_EN | R/W | 0x0 |  |
| [29:26] | DIV_TEST_SEL | R/W | 0x0 |  |
| [25:23] | RC32K_TEST_CLK_SEL | R/W | 0x0 | RC32K test signal selection 000: CAL_INPROGRESS RC32K Calibration process signal 001: CAL_PRECHARGE RC32K Calibration precharge signal 010: CAL_DONE_INT RC32K Calibration done signal 011: CAL_CODE_DOWN 100: CAL_STEP_DONE 101: RC32K_RDY RC32K ready signal (high for ready) 110: RC32K_OUT RC32K clock output 111: CLK_SYNC Clock synchronization for Calibration |
| [22] | SPLL_CLK_TEST_DIVSEL | R/W | 0x0 | Control the division ration of tested clock. 0: output frequency divided by 1/2 1: output frequency divided by 1/64 |
| [21:20] | SPLL_DC_TEST_SEL | R/W | 0x0 | DC test signals selection. 00: VCO LDO VCO 1.1V LDO output 01: PLL LDO PLL 1.1V LDO output 10: VCTRL VCO control voltage 11: floating |
| [19] | SPLL_CLK_TEST_SEL | R/W | 0x0 | Clock test signals selection. 0: divider clock 1: reference clock |
| [18] | DPLL_CLK_TEST_DIVSEL | R/W | 0x0 | Control the division ration of tested clock. 0: output frequency divided by 1/2 1: output frequency divided by 1/64 |
| [17:16] | DPLL_DC_TEST_SEL | R/W | 0x0 | DC test signals selection. 00: VCO LDO VCO 1.1V LDO output 01: PLL LDO PLL 1.1V LDO output 10: VCTRL VCO control voltage 11: floating |
| [15] | DPLL_CLK_TEST_SEL | R/W | 0x0 | Clock test signals selection. 0: divider clock 1: reference clock |
| [14:13] | DCXO_TEST_CLK_SEL | R/W | 0x0 | Clock test selection 00: CLK for RC32K 01: CLK for PLL 10: CLK for digital 11: PDT_OUT |
| [12:11] | DCXO_TEST_DC_SEL | R/W | 0x0 | DC test selection 00: AVDD18 01~11: AVDD11 |
| [10:9] | TEST_PLL_EXT_CH | R/W | 0x0 | PLL test or external input channel selection 00: SYSPLL output test 01: DATAPLL output test 10: external clock to SYSPLL 11: external clock to DATAPLL |
| [8] | TEST_PLL_EXT_EN | R/W | 0x0 | PLL test and external input control 0: disable test or external mode 1: enable test or external mode |
| [7:5] | TEST_CLK_CH | R/W | 0x0 | Clock test channel selection 000：DCXO 001：RC32K 010：DATAPLL 011：SYSPLL 100：BIAS 101~111：none |
| [4] | TEST_CLK_EN | R/W | 0x0 | Analog clock test enable control 0： disable clock test 1： enable clock test |
| [3:1] | TEST_DC_CH | R/W | 0x0 | DC test channel selection 000：DCXO 001：RC32K 010：DATAPLL 011：SYSPLL 100：DCXO buffer output for performance test 101~111：none |
| [0] | TEST_DC_EN | R/W | 0x0 | Analog DC test enable control 0： disable DC test 1： enable DC test |

RCC_DEBUGRO

- Name: RTC clock status monitoring

- Size: 32bits

- Addr Offset: 0x40

- Read/Write access: read/write

- Reset Value: 0x0

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [31:10] | Reserved | RO | 0x0 | - |
| [9] | RC32K_RDY | RO | 0x0 | RC32K clock ready indicator 0: not ready 1: ready |
| [8] | RC32K_CAL_INPROGRESS | RO | 0x0 | RC32K calibration inprocess state |
| [7] | RC32K_CAL_DONE | RO | 0x0 | RC32K Calibration done signal |
| [6:0] | RC32K_CODE_FR_CAL | RO | 0x0 | RC32K calibration output to SPI |

4  Program Guide

Now take changing the CPU clock as an example to describe how to configure the register to change the CPU clock:

- Calling rcc_init() for rcc initialization;

- Calling rccswitch() to perform rcc switching;

- The detailed code is as follows:

void rcc_init(void)

{

// 0. Parameters setting

uint32_t datpll_ndiv    = 0x30;

uint32_t datpll_npost   = 0;

uint32_t datpll_da_en   = 1;

uint32_t datpll_da_out_en   = 1;

uint32_t syspll_ndiv    = 0x1d;

uint32_t syspll_npost   = 0;

uint32_t syspll_da_en   = 1;

uint32_t syspll_da_out_en   = 1;

uint32_t nmp            = D_NMP;

uint32_t nvc1           = D_NVC1;

uint32_t nvc2_s1        = D_NVC2_S1;

uint32_t nvc2_s2        = D_NVC2_S2;

uint32_t frac_en        = 1;

uint32_t apbdiv         = 1;

// Here, LISOC_RCC_RTC32KCTRL setting just to verify the interface of BWLISOCANATN40.

uint32_t cnt_sel        = 0;

uint32_t ext_code       = 55;

uint32_t ext_code_en    = 1;

uint32_t cal_div        = 0;

uint32_t cal_allow      = 1;

uint32_t cal_en         = 1;

LISOC_RCC->RCC_SDMCFGR_W = 0x15A5A5; // for dp normal mode

// 1. Set LISOC_RCC_CR with 0x0008fd09

LISOC_RCC->RCC_CR.APBDIV      = apbdiv;  // [19]

LISOC_RCC->RCC_CR.NVC2_S1     = nvc2_s1; // [15:10]

LISOC_RCC->RCC_CR.NVC1        = nvc1;    // [9:8]

LISOC_RCC->RCC_CR.NMP         = nmp;     // [7:6]

LISOC_RCC->RCC_CR.NVC2_S2     = nvc2_s2; // [5:1]

LISOC_RCC->RCC_CR.FRAC_EN     = frac_en; // [0]

// 2. Set LISOC_RCC_DATPLLCFGR with 0x00000830  ([31:12]: reserved)

LISOC_RCC->RCC_DATPLLCFGR.RG_NPOST    = datpll_npost; // [6]

LISOC_RCC->RCC_DATPLLCFGR.RG_NDIV     = datpll_ndiv;  // [5:0]

LISOC_RCC->RCC_DATPLLCFGR.DA_EN       = datpll_da_en; // [11]

LISOC_RCC->RCC_DATPLLCFGR.DA_OUT_EN   = datpll_da_out_en; // [10]

// 3. Set LISOC_RCC_SYSPLLCFGR with 0x0000081d  ([31:12]: reserved)

LISOC_RCC->RCC_SYSPLLCFGR.RG_NPOST    = syspll_npost; // [6]

LISOC_RCC->RCC_SYSPLLCFGR.RG_NDIV     = syspll_ndiv;  // [5:0]

LISOC_RCC->RCC_SYSPLLCFGR.DA_EN       = syspll_da_en; // [11]

LISOC_RCC->RCC_SYSPLLCFGR.DA_OUT_EN   = syspll_da_out_en; // [10]

// 4. Set LISOC_RCC_RTC32KCTRL with 0x000011f7 ([31:13]: reserved)

LISOC_RCC->RCC_RTC32KCTRL.CNT_SEL     = cnt_sel;     // [12]

LISOC_RCC->RCC_RTC32KCTRL.EXT_CODE    = ext_code;    // [11:5]

LISOC_RCC->RCC_RTC32KCTRL.EXT_CODE_EN = ext_code_en; // [4]

LISOC_RCC->RCC_RTC32KCTRL.CAL_DIV     = cal_div;     // [3:2]

LISOC_RCC->RCC_RTC32KCTRL.CAL_ALLOW   = cal_allow;   // [1]

LISOC_RCC->RCC_RTC32KCTRL.CAL_EN      = cal_en;      // [0]

//LISOC_RCC->RCC_DATPLLCTRL.DPLL_LOCK_DLYTUNE = 1;

//LISOC_RCC->RCC_SYSPLLCTRL.SPLL_LOCK_DLYTUNE = 1;

//LISOC_RCC->RCC_DCXO_CTRL0.DCXO_MODE = 1;

// 5. Wait for pll lock

delay_us(100);

}

void rcc_switch(void)

{

// 6. Set LISOC_RCC_SYSCFGR with 0x3F to switch clock

LISOC_RCC->RCC_SYSCFGR.SYSSW = 1; // [0]

LISOC_RCC->RCC_SYSCFGR.AXISW = 1; // [1]

LISOC_RCC->RCC_SYSCFGR.BYTSW = 1; // [2]

LISOC_RCC->RCC_SYSCFGR.PIXSW = 1; // [3]

LISOC_RCC->RCC_SYSCFGR.VI1SW = 1; // [4]

LISOC_RCC->RCC_SYSCFGR.VI2SW = 1; // [5]

}

Chapter 3 LISoC_AHB_SYSCFG

1  Function list

- GPIO pin function multiplexing, tri-state and pull up control

- BOOT mode and XIP control

- Flash Cache control, cache bridge control

- SPI protocol mode control

- MIPI-CSI control

- Debug function control

2  Register list

Base address: 0x400F 1000

| Offset | Name | Access | Reset | Width | Description |
| --- | --- | --- | --- | --- | --- |
| 0x00 | GPIOA_OER | R/W | 0x00 | 32 | GPIO A output enable |
| 0x04 | GPIOA_PUPDR | R/W | 0xFFFF | 32 | GPIO A pull up/pull down |
| 0x08 | GPIOA_AFR | R/W | 0x00 | 32 | GPIO A alternative function control |
| 0x0c | GPIOB_OER | R/W | 0x00 | 32 | GPIO B output enable |
| 0x10 | GPIOB_PUPDR | R/W | 0xFFFF | 32 | GPIO B pull up/pull down |
| 0x14 | GPIOB_AFR | R/W | 0x00 | 32 | GPIO B alternative function control |
| 0x18 | BOOT_STATR | RO | - | 32 | Boot Pin Register Status |
| 0x1c | SOC_CCON0 | R/W | 0x06 | 32 | SOC chip control #0 |
| 0x20 | SOC_CCON1 | R/W | 0x00 | 32 | SOC chip control #1 |
| 0x24 | MRX_DBG0 | RO | 0x00 | 32 | MIPI-RX debug #0 |
| 0x28 | MRX_DBG1 | RO | 0x00 | 32 | MIPI-RX debug #1 |
| 0x2c | MRX_DBG2 | RO | 0x00 | 32 | MIPI-RX debug #2 |
| 0x30 | MRX_DBG3 | RO | 0x00 | 32 | MIPI-RX debug #3 |
| 0x34 | MTX_DBG0 | RO | 0x00 | 32 | MIPI-TX debug #0 |
| 0x38 | MTX_DBG1 | R/W | 0x00 | 32 | MIPI-TX debug #1 |
| 0x3c | MTX_DBG2 | R/W | 0x00 | 32 | MIPI-TX debug #2 |
| 0x40 | SOC_DBG0 | R/W | 0x00 | 32 | SOC debug control #0 |
| 0x44 | SOC_DBG1 | R/W | 0x00 | 32 | SOC debug control #1 |
| 0x48 | SOC_DBG2 | R/W | 0x00 | 32 | SOC debug control #2 |
| 0x4c | AF_CCON0 | R/W | 0x00 | 32 | AF control #0 |
| 0x50 | AF_CCON1 | R/W | 0x00 | 32 | AF control #1 |
| 0x54 | AF_COUNT | R/W | 0x00 | 32 | AF counter |

3  Register detailed definition

GPIOA_OER, GPIOB_OER

- Name: GPIO A/ GPIO B output enable register

- Size：32bits

- Addr Offset：0x00, 0x0c

- Read/Write access：read/write

- Reset Value: 0x0

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [31:16] | Reserved | RO | 0x00 | Reserved |
| [15:0] | GAOEN/GBOEN | R/W | 0x00 | When GPIO is set to normal mode GPIO A / GPIO B output is enabled 1: GPIO output function enabled 0: GPIO output is not enabled |

GPIOA_PUPDR, GPIOB_PUPDR

- Name: GPIO A/ GPIO B pull up pull down control register

- Size：32bits

- Addr Offset：0x04, 0x10

- Read/Write access：read/write

- Reset Value: 0xFFFF

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [31:16] | Reserved | RO | 0x00 | Reserved |
| [15:0] | GAPUEN /GBPUEN | R/W | 0xFFFF | GPIO A / GPIO B pull-up resistors are enabled ( the pull-up resistors work regardless of whether the GPIO pin is configured in normal GPIO mode ). The pull-up resistance value is approximately [TBD] Ω 1: Pull-up resistor enabled 0: Pull-up resistor is disabled |

GPIOA_AFR , GPIOB_AFR

- Name: GPIO A / GPIO B alternative function configure register

- Size：32bits

- Addr Offset：0x08, 0x14

- Read/Write access：read/write

- Reset Value: 0x0

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [31:30] | PA15AF , PB15AF | R/W | 0x00 | GPIO PA15 /PB15 multiplexing function selection |
| [29:28] | PA14AF, PB14AF | R/W | 0x00 | GPIO PA14 /PB14 multiplexing function selection |
| … |  |  |  | … |
| [1:0] | PA0AF,PB0AF | R/W | 0x00 | GPIO PA 0/PB0 multiplexing function selection |

GPIO A/GPIOB Alternative function configure table:

| GPIO# | AF == 0 (default) | IO | AF==1 | IO | AF==2 | IO | AF==3 | IO |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| P A15 | gpioa15 | IO |  |  | mipi_rx_tmo_mod[0] | I | mipi_tx_tmo_mod[0] | I |
| PA14 | SPISLV_MISO | O | gpioa14 | IO | mipi_rx_tmo_mod[1] | I | mipi_tx_tmo_mod[1] | I |
| PA13 | SPISLV_MOSI | I | gpioa13 | IO | mipi_rx_tmo_mod[2] | I | mipi_tx_tmo_mod[2] | I |
| PA12 | SPISLV_SSN | I | gpioa12 | IO | mipi_rx_tmo_mod[3] | I | mipi_tx_tmo_mod[3] | I |
| PA11 | SPISLV_SCK | I | gpioa11 | IO | mipi_rx_tmo_mod[4] | I | mipi_tx_tmo_mod[4] | I |
| PA10 | I2C0_SDA | IO | gpioa10 | IO | mipi_rx_tmo_mod[5] | I | mipi_tx_tmo_mod[5] | I |
| PA9 | I2C0_SCK | O | gpioa9 | IO | mipi_rx_tmo_mod[6] | I | mipi_tx_tmo_mod[6] | I |
| PA8 | DLCK_MASK | I | gpioa8 | IO | mipi_rx_tmo_mod[7] | I | mipi_tx_tmo_mod[7] | I |
| PA7 | MCO | O | gpioa7 | IO | mipi_rx_test_rda_da | I | mipi_tx_test_rda_da | I |
| PA6 | UART_TX0 | O | gpioa6 | IO | mipi_rx_test_rda_cs | I | mipi_tx_test_rda_cs | I |
| PA5 | UART_RX0 | I | gpioa5 | IO | mipi_rx_test_rda_en | I | mipi_tx_test_rda_en | I |
| PA4 | JTAGTDO | O | gpioa4 | IO | mipi_rx_tmo_pclk | O |  |  |
| PA3 | JTAGTDI | I | gpioa3 | IO | mipi_rx_test_clk | I |  |  |
| PA2 | JTAGRST | I | gpioa2 | IO |  |  |  |  |
| PA1 | SWCLK/JTAG_TCK | I | gpioa1 | IO |  |  |  |  |
| PA0 | SWDIO/JTAG_TMS | IO | gpioa0 | IO |  |  |  |  |
| PB15 | gpiob15 | IO | SPIMST_MOSI | O | mipi_rx_tmo_pdata[0] | O |  |  |
| PB14 | gpiob14 | IO | SPIMST_ MISO | I | mipi_rx_tmo_pdata[1] | O |  |  |
| PB13 | gpiob13 | IO | SPIMST_SSN | O | mipi_rx_tmo_pdata[2] | O |  |  |
| PB12 | gpiob12 | IO | SPIMST_SCK | O | mipi_rx_tmo_pdata[3] | O |  |  |
| PB11 | gpiob11 | IO | I2C2_SDA | IO | mipi_rx_tmo_pdata[4] | O |  |  |
| PB10 | gpiob10 | IO | I2C2_SCK | O | mipi_rx_tmo_pdata[5] | O |  |  |
| PB9 | gpiob9 | IO | UART_TX1 | O | mipi_rx_tmo_pdata[6] | O |  |  |
| PB8 | gpiob8 | IO | UART_RX1 | I | mipi_rx_tmo_pdata[7] | O |  |  |
| PB7 | gpiob7 | IO | I2C1_SDA | IO | mipi_rx_tmo_dat[0] | O | mipi_tx_tmo_dat[0] | O |
| PB6 | gpiob6 | IO | I2C1_SCK | O | mipi_rx_tmo_dat[1] | O | mipi_tx_tmo_dat[1] | O |
| PB5 | QSPI_DAT3 | IO | gpiob5 | IO | mipi_rx_tmo_dat[2] | O | mipi_tx_tmo_dat[2] | O |
| PB4 | QSPI_DAT2 | IO | gpiob4 | IO | mipi_rx_tmo_dat[3] | O | mipi_tx_tmo_dat[3] | O |
| PB3 | QSPI_DAT1 | IO | gpiob3 | IO | mipi_rx_tmo_dat[4] | O | mipi_tx_tmo_dat[4] | O |
| PB2 | QSPI_DAT0 | IO | gpiob2 | IO | mipi_rx_tmo_dat[5] | O | mipi_tx_tmo_dat[5] | O |
| PB1 | QSPI_SSN | O | gpiob1 | IO | mipi_rx_tmo_dat[6] | O | mipi_tx_tmo_dat[6] | O |
| PB0 | QSPI_SCLK | O | gpiob0 | IO | mipi_rx_tmo_dat[7] | O | mipi_tx_tmo_dat[7] | O |

BOOT_STATR

- Name: Boot status register

- Size：32bits

- Addr Offset：0x18

- Read/Write access：read only

- Reset Value: reflects BOOT PIN status

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [31:2] | Reserved | RO | 0x00 | Reserved |
| [1:0] | BOOT_STATUS | RO | -- | Reflect BOOT pin status Bit 1 corresponds to BOOT1 pin Bit 0 corresponds to BOOT0 pin |

SOC_CCON0

- Name: SOC chip control #0 register

- Size：32bits

- Addr Offset：0x1c

- Read/Write access：read/write

- Reset Value: 0x6

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [31:4] | Reserved | RO | 0x00 | Reserved |
| [3] | Cache_bridge_en | R/W | 0 | 1: Enable cache-bridge. Note that when Cache-IP is enabled ( EN bit == 1 in flash cache controller CCR register), this bit must be enabled. 0: Disable cache-bridge. However, the XIP function can be enabled independently |
| [2] | SPIMST_SS_IN_N | R/W | 1 | Determines the polarity of the default level output by the SPI master SSN pin 0: When the SSN pin is not strobed, it outputs low level by default (when the SPI master's serial communication protocol uses SSP mode, you must ensure that this bit value is 0) 1: When the SSN pin is not strobed, it outputs high level by default (when the SPI master's serial communication protocol uses SPI or Microwire, you must ensure that this bit value is 1) |
| [1] | QSPI_SS_IN_N | R/W | 1 | Determines the polarity of the QSPI SSN pin output default level 0: When the SSN pin is not strobed, it outputs low level by default (when the QSPI serial communication protocol uses SSP mode, you must ensure that this bit value is 0) 1: When the SSN pin is not strobed, it outputs high level by default (when the QSPI serial communication protocol uses SPI or Microwire, you must ensure that this bit value is 1) |
| [0] | XIP_EN | R/W | 0 | Enable QSPI XIP function |

SOC_CCON1

- Name: SOC chip control #1 register

- Size：32bits

- Addr Offset：0x20

- Read/Write access：read/write

- Reset Value: 0x0

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [31:9] | Reserved | RO | 0x00 | Reserved |
| [11:8] | tpgen_sel | R/W | 0 | tpgen select |
| [7:5] | Reserved | RO | 0x00 | Reserved |
| [4] | mipi_rx_tx_thru | R/W | 0 | mipi through |
| [3:0] | Soc_ctrl | R/W | 0 | [TBD] |

MRX_DBG0

- Name: MIPI-RX debug #0 register

- Size：32bits

- Addr Offset：0x24

- Read/Write access：read only

- Reset Value: 0x0

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [31:24] | Reserved | RO | 0 | Reserved |
| [23:16] | Csi2_ecc | RO | 0 | Reflects the current MIPI Rx CSI2 ECC value |
| [ 15:8 ] | Csi2_data_id | RO | 0 | Reflects the current MIPI Rx CSI2 data id value |
| [ 7:3 ] | Reserved | RO | 0 | Reserved |
| [ 2:0 ] | Csi2_state | RO | 0 | Reflects the current MIPI Rx CSI2 state value |

MRX_DBG 1

- Name: MIPI-RX debug #1 register

- Size：32bits

- Addr Offset：0x28

- Read/Write access：read only

- Reset Value: 0x0

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [31:16] | Csi_line_length | RO | 0 | Reflects the current MIPI Rx CSI2 line length value |
| [15:0] | Csi2_wc | RO | 0 | Reflects the current MIPI Rx CSI2 WC value |

MRX_DBG 2

- Name: MIPI-RX debug # 2 register

- Size：32bits

- Addr Offset: 0x2c

- Read/Write access: read only

- Reset Value: 0x 0

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [31:16] | Csi2_err_ecc | RO | 0 | Reflects the current MIPI Rx CSI2 error ECC value |
| [15:0] | Csi2_pkt_count | RO | 0 | Reflects the current MIPI Rx CSI2 packet count value |

MRX_DBG 3

- Name: MIPI-RX debug # 3 register

- Size：32bits

- Addr Offset: 0x30

- Read/Write access: read only

- Reset Value: 0x 0

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [31:16] | Reserved | RO | 0 | Reserved |
| [15:0] | Csi2_err_cs | RO | 0 | Reflects the current MIPI Rx CSI2 error CS value |

MTX_DBG 0

- Name: MIPI-TX debug #0 register

- Size：32bits

- Addr Offset：0x34

- Read/Write access：read only

- Reset Value: 0x0

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [31:12] | Reserved | RO | 0 | Reserved |
| [11:8] | Csi2_tx_fsm_phyd | RO | 0 | Reflects the current MIPI Tx CSI2 FSM phyd value |
| [7:4] | csi2_tx_fsm_link | RO | 0 | Reflects the current MIPI Tx CSI2 FSM link value |
| [3:0] | Csi2_tx_fsm_app | RO | 0 | Reflects the current MIPI Tx CSI2 FSM app value |

MTX_DBG 1

- Name: MIPI-TX debug #1 register

- Size：32bits

- Addr Offset：0x38

- Read/Write access：read write

- Reset Value: 0x0

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [31:10] | Reserved | RO | 0 | Reserved |
| [9] | Dphy_tx_deskew_man | R/W | 0 | Set MIPI Tx CSI2 DPHY TX deskew manual adjustment |
| [8] | Csi2_tx_ck_man | R/W | 0 | Set MIPI Tx CSI2 DPHY TX ck manual adjustment |
| [7:2] | csi2_tx_data_id | R/W | 0 | Set MIPI Tx CSI2 data id value |
| [1:0] | Csi2_tx_ccp2_vci | R/W | 0 | Set MIPI Tx CSI2 CCP2 VCI value |

MTX_DBG 2

- Name: MIPI-TX debug #2 register

- Size：32bits

- Addr Offset：0x3c

- Read/Write access：read write

- Reset Value: 0x0

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [31:16] | csi2_tx_line_length | R/W | 0 | Set MIPI Tx CSI2 Tx line length value |
| [15 : 0] | Csi2_tx_wc | R/W | 0 | Set MIPI Tx CSI2 WC value |

SOC_DBG 0 , SOC_DBG1, SOC_DBG2

- Name: LISOC debug #0, #1, #2 register

- Size：32bits

- Addr Offset：0x40, 0x44, 0x48

- Read/Write access：read write

- Reset Value: 0x0

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [31:16] | Reserved | RO | 0 | Reserved |
| [15:0] | Soc_debug0, Soc_debug1, Soc_debug2 | R/W | 0 | Set LISOC debug #0 #1 #2 value |

AF_CCON0

- Name: AF control#0 register

- Size：32bits

- Addr Offset：0x4c

- Read/Write access：read write

- Reset Value: 0x1F03E8

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [31:22] | Reserved | RO | 0 | Reserved |
| [21] | EN | R/W | 0 | 1: Global Enable AF Drive Function 0 disable |
| [20] | mask4 | R/W | 1 | CW mask bit When mask4=1, then the CW (AF0) output is 0 When mask4=0, the CW (AF0) output is controlled by the bit in AF_CCON1 |
| [19] | mask3 | R/W | 1 | enable mask bit When mask3=1, then enable (AF1) output is 0 When mask3=0, the enable (AF1) output is controlled by the bit in AF_CCON1 |
| [18] | mask2 | R/W | 1 | mode0 mask bit When mask2=1, then the output of mode0 (AF2) is 0 When mask2=0, then the output of mode0 (AF2) is controlled by the bit in AF_CCON1 |
| [17] | mask1 | R/W | 1 | mode1 mask bit When mask1=1, then the output of mode1 (AF3) is 0 When mask1=0, then the output of mode1 (AF3) is controlled by the bits in AF_CCON1 |
| [16] | mask0 | R/W | 1 | mode2 mask bit When mask0=1, then the output of mode2 (AF4) is 0 When mask0=0, then the output of mode2 (AF4) is controlled by the bits in AF_CCON1 |
| [15:0] | pre_div | R/W | 0x3E8 | The clock output frequency division in the AF module. Divide SOC_CLK by frequency based on the value of pre-div. Pay attention to keeping pre-div [0] at 0 If pre-div [15:0]=16'h0004, it means the clock is divided by 4. Pre-div [15:0]=16'h0008 indicates that the clock is divided by 8. |

AF_CCON1

- Name: AF control#1 register

- Size：32bits

- Addr Offset：0x50

- Read/Write access：read write

- Reset Value: 0x0

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [31:5] | Reserved | RO | 0 | Reserved |
| [4] | CW | R/W | 0 |  |
| [3] | enable | R/W | 0 |  |
| [2] | mode0 | R/W | 0 |  |
| [1] | mode1 | R/W | 0 |  |
| [0] | mode2 | R/W | 0 | Interface for driving PWM IC |

AF_COUNT

- Name: AF counter register

- Size：32bits

- Addr Offset：0x54

- Read/Write access：read write

- Reset Value: 0x0

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [31:16] | Reserved | RO | 0 | Reserved |
| [15:0] | af_count | R/W | 0 | Count the rising edge pulses input from the external port |

4  Program Guide

If the user intends to disable the Flash cache while using the XIP function, it is recommended to set the registers in the following order:

- Prerequisite: The XIP_EN bit field of the SOC_CCON0 register is 0 (XIP is disabled)

- Set the EN bit field of the Flash cache controller CCR register to 0 to turn off the Flash cache

- Set the Cache_bridge_en bit field of the SOC_CCON0 register to 0 to turn off the cache bridge

- Set the XIP_EN bit field of the SOC_CCON0 register to 1 to enable XIP mode operation.

- . . .

- Set the XIP_EN bit field of the SOC_CCON0 register to 0 to disable XIP mode operation.

The sample code is as follows:

#define CCR				0x40090000

#define SOC_CCON0		0x400F101C

// Disable flash cache

(*(volatile uint32_t *) CCR) &= ~(uint32_t)0x01;

(*(volatile uint32_t *) SOC_CCON0) &= ~(uint32_t)0x08;

// Enable XIP

(*(volatile uint32_t *) SOC_CCON0) |= (uint32_t)0x01;

...

// Disable XIP

(*(volatile uint32_t *) SOC_CCON0) &= ~(uint32_t)0x01;

If the user plans to enable the Flash cache while using the XIP function, it is recommended to set the registers in the following order:

- Prerequisite: The XIP_EN bit field of the SOC_CCON0 register is 0 (XIP is disabled)

- Set the EN bit field of the Flash cache controller CCR register to 1 to enable the Flash cache

- Set the Cache_bridge_en bit field of the SOC_CCON0 register to 1 to enable cache bridge

- Set the XIP_EN bit field of the SOC_CCON0 register to 1 to enable XIP mode operation (the flash cache is activated at this time).

- . . .

- Set the XIP_EN bit field of the SOC_CCON0 register to 0 to disable XIP mode operation.

The sample code is as follows:

#define CCR				0x40090000

#define SOC_CCON0		0x400F101C

// Disable flash cache

(*(volatile uint32_t *) CCR) |= (uint32_t)0x01;

(*(volatile uint32_t *) SOC_CCON0) |= (uint32_t)0x08;

// Enable XIP

(*(volatile uint32_t *) SOC_CCON0) |= (uint32_t)0x01;

...

// Disable XIP

(*(volatile uint32_t *) SOC_CCON0) &= ~(uint32_t)0x01;

Chapter 4 LISoC_AHB_NVIC

1 Function list

- Interrupt sources and priority differentiation

The interrupt controller supports a total of 21 an interrupt source. By configuring the priority register of the interrupt controller, each interrupt source can be Assign one of 8 priorities (the highest priority is 7 , the lowest priority is 0 ). By default, each interrupt source is The precedence is all 0 . The interrupt controller uses the priority of the interrupt source to differentiate interrupt signals delivered to the processor.

LISOC interrupt vector table

| Region | IRQ No. | IRQ Name | Description |
| --- | --- | --- | --- |
| Cortex M3 | -14 | NonMaskableInt_IRQn | SV#2 Non maskable interrupt |
| Cortex M3 | -13 | HardFault_IRQn | SV #3 Hard fault interrupt |
| Cortex M3 | -12 | MemoryManagement_IRQn | SV #4 Memory management interrupt |
| Cortex M3 | -11 | BusFault_IRQn | SV #5 Bus fault interrupt |
| Cortex M3 | -10 | UsageFault_IRQn | SV #6 Usage fault interrupt |
| Cortex M3 | -5 | SVCall_IRQn | SV #11 Service call interrupt |
| Cortex M3 | -4 | DebugMonitor_IRQn | SV #12 Debug monitor interrupt |
| Cortex M3 | -3 | Reserved | - |
| Cortex M3 | -2 | PendSV_IRQn | SV #14 Pending IRQ interrupt |
| Cortex M3 | -1 | SysTick_IRQn | SV #15 System tick interrupt |
| Peripherals | 0 | Uart0_irq | UART 0 interrupt |
| Peripherals | 1 | Uart1_irq | UART 1 interrupt |
| Peripherals | 2 | Spimst_irq | SPI master interrupt |
| Peripherals | 3 | Spislv_irq | SPI slave interrupt |
| Peripherals | 4 | qspi_irq | QSPI master interrupt |
| Peripherals | 5 | rtc_irq | Real Time Clock interrupt |
| Peripherals | 6 | wdt_irq | Watch Dog Timer interrupt |
| Peripherals | 7 | gpioa_irq | GPIO A interrupt |
| Peripherals | 8 | gpiob_irq | GPIO B interrupt |
| Peripherals | 9 | Timer0_irq | Timer 0 interrupt |
| Peripherals | 10 | Timer1_irq | Timer 1 interrupt |
| Peripherals | 11 | Timer2_irq | Timer 2 interrupt |
| Peripherals | 12 | I2c0_irq | I2C 0 interrupt |
| Peripherals | 13 | I2c1_irq | I2C 1 interrupt |
| Peripherals | 14 | I2c2_irq | I2C 2 interrupt |
| Peripherals | 15 | Isp_irq | Image Signal Processor IP AE/AWB interrupt (Auto exposure / Auto white balance) |
| Peripherals | 16 | Cache_irq | Cache interrupt |
| Peripherals | 17 | Mipi_rx_err_ecc_irq | MIPI-Rx ECC error interrupt |
| Peripherals | 18 | Mipi_rx_err_cs_irq | MIPI-Rx CS error interrupt |
| Peripherals | 19 | Af_int_cmpl_irq | AF interrupt |
| Peripherals | 20 | Mipi_rx_int_herr_out_irq | MIPI Rx hsize error interrupt |
| Peripherals | 21 | Mipi_rx_int_verr_out_irq | MIPI Rx vsize error interrupt |
| Peripherals | 22 |  | Reserved |

2 Register List

Table 2-1 NVIC register list

Base address：0xE000 E000

| Offset | Name | Access | Reset | Width | Description |
| --- | --- | --- | --- | --- | --- |
| 0x010 | STCSR | R/W | 0x0 | 32 | SysTick Control and Status Register |
| 0x014 | STRVR | R/W | 0x0 | 32 | SysTick Reload Value Register |
| 0x018 | STCVR | R/W | 0x0 | 32 | SysTick Current Value Register |
| 0x100 | NVIC_ISER | R/W | 0x0 | 32 | Interrupt Set Enable Registers (1 bit each) |
| 0x180 | NVIC_ICER | R/W | 0x0 | 32 | Interrupt Clear Enable Registers (1 bit each) |
| 0x200 | NVIC_ISPR | R/W | 0x0 | 32 | Interrupt Set Pending Registers (1 bit each) |
| 0x280 | NVIC_ICPR | R/W | 0x0 | 32 | Interrupt Clear Pending Registers (1 bit each) |
| 0x300 | NVIC_IABR | R/W | 0x0 | 32 | Interrupt Active bit Registers (1 bit each) |
| 0x400 | NVIC_IP0 | R/W | 0x0 | 32 | Interrupt Priority Register 0~3 (8Bit each) |
| 0x404 | NVIC_IP4 | R/W | 0x0 | 32 | Interrupt Priority Register 4~7 (8Bit each) |
| 0x408 | NVIC_IP8 | R/W | 0x0 | 32 | Interrupt Priority Register 8~11 (8Bit each) |
| 0x40C | NVIC_IP12 | R/W | 0x0 | 32 | Interrupt Priority Register 12~15 (8Bit each) |
| 0x410 | NVIC_IP16 | R/W | 0x0 | 32 | Interrupt Priority Register 16~19 (8Bit each) |
| 0xD04 | SCB_ICSR | R/W | 0x0 | 32 | System Interrupt Control and State Register |
| 0xD08 | SCB_VTOR | R/W | 0x0 | 32 | Vector Table Offset Register |
| 0xD0C | SCB_AIRCR | R/W | 0xFA050000 | 32 | Application Interrupt and Reset Control Register |
| 0xD10 | SCB_SCR | R/W | 0x0 | 32 | System Control Register |
| 0xD14 | SCB_CCR | R/W | 0x200 | 32 | System Configuration Control Register |
| 0xD18 | SCB_SHP4 | R/W | 0x0 | 32 | System Handlers Priority Registers 4-7 |
| 0xD1C | SCB_SHP8 | R/W | 0x0 | 32 | System Handlers Priority Registers 8-11 |
| 0xD20 | SCB_SHP12 | R/W | 0x0 | 32 | System Handlers Priority Registers 12-15 |
| 0xF00 | NVIC_STIR | WO | 0x0 | 32 | Software Trigger Interrupt Register |

3 Detailed definition of registers

STCSR

Name: SysTick Control and Status Register

Address: 0xE000E010

Access: read / write

Data width: 32 bit

Reset value: 0x0

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [31:17] | Reserved | RO | 0x00 | - |
| [16] | COUNT_FLAG | RO | 0x00 | Indicates whether the counter has counted to 0 since the last read of this register: 0 = Timer has not counted to 0 1 = Timer has counted to 0. COUNTFLAG is set to 1 by a count transition from 1 to 0. COUNTFLAG is cleared to 0 by a read of this register, and by any write to the Current Value register. |
| [15:3] | Reserved | RO | 0x00 | - |
| [2] | CLKSOURCE | R/W | 0x00 | Indicates the SysTick clock source: 0 = SysTick uses the external reference clock. 1 = SysTick uses the processor clock. If no external clock is provided, this bit reads as 1 and ignores writes. |
| [1] | TICKINT | R/W | 0x00 | Indicates whether counting to 0 causes the status of the SysTick exception to change to pending: 0 = Count to 0 does not affect the SysTick exception status. 1 = Count to 0 changes the SysTick exception status to pending. Changing the value of the counter to 0 by writing zero to the SysTick Current Value register to 0 never changes the status of the SysTick exception. |
| [0] | ENABLE | R/W | 0x00 | Indicates the enabled status of the SysTick counter: 0 = Counter is disabled 1 = Counter is operating. |

STRVR

Name: SysTick Reload Value Register

Address: 0xE000E014

Access: read / write

Data width: 32 bit

Reset value: 0x0

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [31:24] | Reserved | RO | 0x00 | Reserved. Read as zero. Writing is ignored. |
| [23:0] | RELOAD | R/W | 0x00 | The value to load into the SYST_CVR when the counter reaches 0. |

STCVR

Name: SysTick Current Value Register

Address: 0xE000E018

Access: read / write

Data width: 32 bit

Reset value: 0x0

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [31:24] | Reserved | RO | 0x00 | Reserved. Read as zero. Writing is ignored. |
| [23:0] | CURRENT | R/W | 0x00 | Current counter value. This is the value of the counter at the time it is sampled. • Any write to the register clears the register to zero. • The counter does not provide read-modify-write protection. • Unsupported bits are read as zero, see SysTick Reload Value Register. |

NVIC_ISER

Name: Interrupt Set Enable Registers (1 bit each)

Address: 0xE000E100

Access: read / write

Data width: 32 bit

Reset value: 0x0

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [31:20] | Reserved | RO | 0x00 | Reserved. Read as zero. Writing is ignored. |
| [19:0] | SETENA | R/W | 0x00 | Enables or shows the current enabled state of interrupt #n (n = 0~19): On reads 0 = Interrupt disabled. 1 = Interrupt enabled. On writes 0 = No effect. 1 = Enable interrupt. Software can enable multiple interrupts in a single write to NVIC_ISER. |

NVIC_ICER

Name: Interrupt Clear-Enable Registers (1 bit each)

Address: 0xE000E180

Access: read / write

Data width: 32 bit

Reset value: 0x0

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [31:20] | Reserved | RO | 0x00 | Reserved. Read as zero. Writing is ignored. |
| [19:0] | CLRENA | R/W | 0x00 | Disables or shows the current enabled state of interrupt #n (n = 0~19): On reads 0 = Interrupt disabled. 1 = Interrupt enabled. On writes 0 = No effect. 1 = Disable interrupt. Software can disable multiple interrupts in a single write to NVIC_ICER. |

NVIC_ISPR

Name: Interrupt Set Pending Registers (1 bit each)

Address: 0xE000E200

Access: read / write

Data width: 32 bit

Reset value: 0x0

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [31:20] | Reserved | RO | 0x00 | Reserved. Read as zero. Writing is ignored. |
| [19:0] | SETPEND | R/W | 0x00 | Changes the state of interrupt #n (n = 0~19) to pending, or shows whether the state of the interrupt is pending: On reads 0 = Interrupt is not pending. 1 = Interrupt is pending. On writes 0 = No effect. 1 = Changes state of interrupt to pending. Software can set multiple interrupts to pending state in a single write to NVIC_ISPR. |

NVIC_ICPR

Name: Interrupt Clear Pending Registers (1 bit each)

Address: 0xE000E280

Access: read / write

Data width: 32 bit

Reset value: 0x0

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [31:20] | Reserved | RO | 0x00 | Reserved. Read as zero. Writing is ignored. |
| [19:0] | CLRPEND | R/W | 0x00 | Clears the pending state of interrupt #n (n = 0~19), or shows whether the state of the interrupt is pending: On reads 0 = Interrupt is not pending. 1 = Interrupt is pending. On writes 0 = No effect. 1 = Clears the pending state of the interrupt. Software can clear the pending state of multiple interrupts in a single write to NVIC_ICPR. |

NVIC_IABR

Name: Interrupt Active bit Registers (1 bit each)

Address: 0xE000E300

Access: read / write

Data width: 32 bit

Reset value: 0x0

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [31:20] | Reserved | RO | 0x00 | Reserved. Read as zero. Writing is ignored. |
| [19:0] | ACTIVE | R/W | 0x00 | Shows whether interrupt #n (n = 0~19) is active: 0 = interrupt is not active 1 = interrupt is active |

NVIC_IP0 / 4 / 8 / 12 / 16

Name: Interrupt Priority Register 0~3 / 4~7 / 8~11 / 12~15 / 16~19

Address: 0xE000E400, 0xE000E404, 0xE000E408, 0xE000E40C, 0xE000E410

Access: read / write

Data width: 32 bit

Reset value: 0x0

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [31:24] | PRI_N3 | R/W | 0x00 | Priority of interrupt number n+3 (n=0,4,8,12,16) e.g. for NVIC_IP0, these bits represents priority of IRQ#3; for NVIC_IP4, these bits represents priority of IRQ#7 |
| [23:16] | PRI_N2 | R/W | 0x00 | Priority of interrupt number n+2 (n=0,4,8,12,16) e.g. for NVIC_IP0, these bits represents priority of IRQ#2; for NVIC_IP4, these bits represents priority of IRQ#6 |
| [15:8] | PRI_N1 | R/W | 0x00 | Priority of interrupt number n+1 (n=0,4,8,12,16) e.g. for NVIC_IP0, these bits represents priority of IRQ#1; for NVIC_IP4, these bits represents priority of IRQ#5 |
| [7:0] | PRI_N0 | R/W | 0x00 | Priority of interrupt number n+0 (n=0,4,8,12,16) e.g. for NVIC_IP0, these bits represents priority of IRQ#0; for NVIC_IP4, these bits represents priority of IRQ#4 |

SCB_ICSR

Name: System Interrupt Control and State Register

Purpose: Provides software control of the NMI, PendSV, and SysTick exceptions, and provides interrupt status information.

Address: 0xE000ED04

Access: read / write

Data width: 32 bit

Reset value: 0x0

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [31] | NMIPENDSET | R/W | 0x00 | On writes, makes the NMI exception active. On reads, indicates the state of the exception: 0 = On writes, has no effect. On reads, NMI is inactive. 1 = On writes, make the NMI exception active. On reads, NMI is active. Because NMI is higher priority than other exceptions, if the processor is not already executing the NMI handler, it enters the NMI exception handler as soon as it recognizes the write to this bit. |
| [30:29] | Reserved | RO | 0x00 | - |
| [28] | PENDSVSET | R/W | 0x00 | On writes, sets the PendSV exception as pending. On reads, indicates the current state of the exception: 0 = On writes, has no effect. On reads, PendSV is not pending. 1 = On writes, make PendSV exception pending. On reads, PendSV is pending. Normally, software writes 1 to this bit to request a context switch. |
| [27] | PENDSVCLR | WO | 0x00 | Removes the pending status of the PendSV exception: 0 = No effect. 1 = Remove pending status. |
| [26] | PENDSTSET | R/W | 0x00 | On writes, sets the SysTick exception as pending. On reads, indicates the current state of the exception: 0 = On writes, has no effect. On reads, SysTick is not pending. 1 = On writes, make SysTick exception pending. On reads, SysTick is pending. |
| [25] | PENDSTCLR | WO | 0x00 | Removes the pending status of the SysTick exception: 0 = No effect. 1 = Remove pending status. |
| [24] | Reserved | RO | 0x00 | - |
| [23] | ISRPREEMPT | RO | 0x00 | Indicates whether a pending exception will be serviced on exit from debug halt state: 0 = Will not service. 1 = Will service a pending exception. |
| [22] | ISRPENDING | RO | 0x00 | Indicates whether an external interrupt, generated by the NVIC, is pending: 0 = No external interrupt pending. 1 = External interrupt pending. |
| [21] | Reserved | RO | 0x00 | - |
| [20:12] | VECTPENDING | RO | 0x00 | The exception number of the highest priority pending exception. A value of 0 indicates that there is no pending exception. |
| [11] | RETTOBASE | RO | 0x00 | In Handler mode, indicates whether there is an active exception other than the exception indicated by the current value of the IPSR: 0 = There is an active exception other than the exception shown by IPSR. 1 = There is no active exception other than any exception shown by IPSR. In Thread mode the value of this bit is UNKNOWN. |
| [10:9] | Reserved | RO | 0x00 | - |
| [8:0] | VECTACTIVE | RO | 0x00 | The exception number of the current executing exception. A value of 0 indicates that the processor is in Thread mode. |

The effect is unpredictable if a write to the ICSR:

• sets both PENDSVSET and PENDSVCLR to 1

• sets both PENDSTSET and PENDSTCLR to 1.

The value of the VECTACTIVE field is the same as the IPSR[8:0] which is part of PSR register in ARM cortext M3 core.

SCB_VTOR

Name: Vector Table Offset Register

Purpose: Holds the vector table address.

Address: 0xE000ED08

Access: read / write

Data width: 32 bit

Reset value: 0x0

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [31:7] | NMIPENDSET | R/W | 0x00 | Bits [31:7] of the vector table address. |
| [6:0] | Reserved | RO | 0x00 | - |

SCB_AIRCR

Name: Application Interrupt and Reset Control Register

Purpose: Sets or returns interrupt control data.

Address: 0xE000ED0C

Access: read / write

Data width: 32 bit

Reset value: 0xFA050000

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [31:16] | VECTKEY VECTKEYSTAT | R/W | 0xFA05 | Vector Key. Register writes must write 0x05FA to this field, otherwise the write is ignored. On reads, returns 0xFA05. |
| [15] | ENDIANNESS | RO | 0x00 | Indicates the memory system endianness: 0 = Little endian 1 = Big endian. Always read as zero. |
| [14:11] | Reserved | RO | 0x00 | - |
| [10:8] | PRIGROUP | R/W | 0x00 | Priority grouping, indicates the binary point position. See ARMv7 architecture manual for more info. |
| [7:3] | Reserved | RO | 0x00 | - |
| [2] | SYSRESETREQ | R/W | 0x00 | System Reset Request: 0 = Do not request a reset. 1 = Request reset. Writing 1 to this bit asserts a signal to the external system to request a Local reset. A Local or Power-on reset clears this bit to 0. |
| [1] | VECTCLRACTIVE | WO | 0x00 | Writing 1 to this bit clears all active state information for fixed and configurable exceptions. This includes clearing the IPSR to zero. The effect of writing a 1 to this bit if the processor is not halted in Debug state is UNPREDICTABLE. |
| [0] | VECTRESET | RO | 0x00 | Reserved. Writing 1 to this bit is isgnored. Reading always returns zero. |

SCB_SCR

Name: System Control Register

Purpose: Sets or returns system control data.

Address: 0xE000ED10

Access: read / write

Data width: 32 bit

Reset value: 0x0

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [31:5] | Reserved | RO | 0x00 | - |
| [4] | SEVONPEND | R/W | 0x00 | Determines whether an interrupt transition from inactive state to pending state is a wakeup event: 0 = Transitions from inactive to pending are not wakeup events. 1 = Transitions from inactive to pending are wakeup events. |
| [3] | Reserved | RO | 0x00 | - |
| [2] | SLEEPDEEP | RO | 0x00 | Provides a qualifying hint indicating that waking from sleep might take longer. 0 = Selected sleep state is not deep sleep. 1 = Selected sleep state is deep sleep. This bit is not implemented because LISOC does not support deep sleep. This bit always read as zero and writing to it is ignored. |
| [1] | SLEEPONEXIT | R/W | 0x00 | Determines whether, on an exit from an ISR that returns to the base level of execution priority, the processor enters a sleep state: 0 = Do not enter sleep state. 1 = Enter sleep state. |
| [0] | Reserved | RO | 0x00 | - |

SCB_CCR

Name: System Configuration Control Register

Purpose: Sets or returns configuration and control data.

Address: 0xE000ED14

Access: read / write

Data width: 32 bit

Reset value: 0x200

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [31:10] | Reserved | RO | 0x00 | - |
| [9] | STKALIGN | RO | 0x1 | Determines whether the exception entry sequence guarantees 8-byte stack frame alignment, adjusting the SP if necessary before saving state: 0 = Guaranteed SP alignment is 4-byte, no SP adjustment is performed. 1 = 8-byte alignment guaranteed, SP adjusted if necessary This bit is read-only. It always read as 1. |
| [8] | BFHFNMIGN | R/W | 0x00 | Determines the effect of precise data access faults on handlers running at priority -1 or priority -2: 0 = Precise data access fault causes a lockup. 1 = Handler ignores the fault. |
| [7:5] | Reserved | RO | 0x00 | - |
| [4] | DIV_0_TRP | R/W | 0x00 | Controls the trap on divide by 0: 0 = Trapping disabled. 1 = Trapping enabled. |
| [3] | UNALIGN_TRP | R/W | 0x00 | Controls the trapping of unaligned word or halfword accesses: 0 = Trapping disabled. 1 = Trapping enabled. Unaligned load-store multiples and word or halfword exclusive accesses always fault. |
| [2] | Reserved | RO | 0x00 | - |
| [1] | USERSETMPEND | RO | 0x00 | Controls whether unprivileged software can access the STIR: 0 = Unprivileged software cannot access the STIR. 1 = Unprivileged software can access the STIR. |
| [0] | NONBASETHRDENA | R/W | 0x00 | Controls whether the processor can enter Thread mode at an execution priority level other than base level: 0 = Any attempt to enter Thread mode at an execution priority level of other than base level faults. 1 = The processor can enter Thread mode at any execution priority level because of a controlled return value. |

SCB_SHP4

Name: System Handler Priority Registers #4-#7

Purpose: Sets or returns priority for system handlers 4-7.

Address: 0xE000ED18

Access: read / write

Data width: 32 bit

Reset value: 0x0

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [31:24] | PRI_7 | R/W | 0x0 | Reserved for priority of system handler 7 |
| [23:16] | PRI_6 | R/W | 0x0 | Priority of system handler 6, Usage Fault |
| [15:8] | PRI_5 | R/W | 0x0 | Priority of system handler 5, BusFault |
| [7:0] | PRI_4 | R/W | 0x0 | Priority of system handler 4, MemManage |

SCB_SHP8

Name: System Handler Priority Registers #8-#11

Purpose: Sets or returns priority for system handlers 8-11.

Address: 0xE000ED1C

Access: read / write

Data width: 32 bit

Reset value: 0x0

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [31:24] | PRI_11 | R/W | 0x0 | Priority of system handler 11, SVCall |
| [23:16] | PRI_10 | R/W | 0x0 | Reserved for priority of system handler 10 |
| [15:8] | PRI_9 | R/W | 0x0 | Reserved for priority of system handler 9 |
| [7:0] | PRI_8 | R/W | 0x0 | Reserved for priority of system handler 8 |

SCB_SHP12

Name: System Handler Priority Registers #12-#15

Purpose: Sets or returns priority for system handlers 12-15.

Address: 0xE000ED20

Access: read / write

Data width: 32 bit

Reset value: 0x0

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [31:24] | PRI_15 | R/W | 0x0 | Priority of system handler 15, Systick |
| [23:16] | PRI_14 | R/W | 0x0 | Priority of system handler 14, PendSV |
| [15:8] | PRI_13 | R/W | 0x0 | Reserved for priority of system handler 13 |
| [7:0] | PRI_12 | R/W | 0x0 | Priority of system handler 12, DebugMonitor |

NVIC_STIR

Name: Software Triggered Interrupt Register

Purpose: Provides a mechanism for software to generate an interrupt..

Address: 0xE000EF00

Access: read / write

Data width: 32 bit

Reset value: 0x0

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [31:9] | Reserved. | RO | 0x0 | - |
| [8:0] | INTID | R/W | 0x0 | Indicates the interrupt to be triggered.The value written is (ExceptionNumber - 16), i.e. the “irq no” column in previous interrupt vector table. Writing to this register has the same effect as setting the NVIC ISPR bit corresponding to the interrupt to 1, see Interrupt Set-Pending Registers, NVIC_ISPR . |

4 Program Guide

After a system reset, all interrupts are set to an inactive state. In order to correctly handle interrupt requests, the system's interrupt configuration

The setup process can be divided into the following 3 steps:

Step 1 : Configure the interrupt controller. By default, the interrupt sources connected to the interrupt controller are set to invalid and

And the priority is 0 . Each interrupt source can be configured as one of 8 priority levels.

Step 2 : Configure the interrupt controller and turn on the exception and interrupt enable bits that need to be taken over.

Step 3: Configure the peripheral interface module. Peripheral interface modules are generally enabled through their internal interrupt enable and mask registers

and turn off interrupts. Therefore, the internal interrupt register of the peripheral interface module needs to be enabled so that the interrupt source can normally generate an interrupt.

Chapter 5 LISoC_AHB_QSPI

1  Function list

AHB_QSPI is a programmable Synchronous Serial Interface (SSI) peripheral. This is an AMBA 2.0-compliant AHB (Advanced High-performance Bus) component.The host processor accesses data, control, and status information on the AHB_QSPI through the AHB interface.

AHB_QSPI has the following features:

■ AHB data width fixed to 32 bits.

■ Serial communication fixed at master mode.

■ Programmable serial interface operation – user can choose from:

❑ Motorola Serial Peripheral Interface (SPI)

❑ Texas instruments Synchronous Serial Protocol (SSP)

❑ National Semiconductors Microwire

■ Programmable delay on the sample time of received serial data bit (rxd), when configured in Master Mode; enables programmable control of routing delays resulting in higher serial data-bit rates.

■ Utilization of the negative edge of the clock - Using advance RX

■ Programmable features:

❑Serial Interface operation – Choice of Motorola SPI, Texas Instruments Synchronous Serial Protocol or National Semiconductors Microwire.

Clock bit rate – Dynamic control of the serial bit rate of the data transfer under the control of the programmer.

❑ Clock stretching support in enhanced SPI transfers.

❑ Data item size (4 to 32 bits) – Item size of each data transfer under control of the programmer.

■ Configurable features:

❑ Receive and transmit FIFO buffers depth is 32. The FIFO width is fixed at 32 bits.

❑ Number of slave select outputs – When operating as a serial master, Only 1 serial slave-select output signals can be generated.

❑ Hardware/software slave-select – Dedicated hardware slave-select lines can be used or software control can be used to target the serial-slave device.

❑ Combined interrupt line from the AHB_QSPI to the interrupt controller.

❑ Interrupt polarity –The active level of the output interrupt lines is active high.

❑ Serial clock polarity – This configuration option selects the serial-clock polarity of the SPI format directly after reset.

❑ Serial clock phase – This configuration option selects the serial-clock phase of the SPI format directly after reset.

❑ Data Pre-fetch - This configuration option pre-fetches the data for the successive burst duringthe current XIP transaction.

■ Enhanced protocol features in SPI mode of operation:

❑ Dual, Quad and Octal SPI support in master and slave modes (with only data phase) of operation.

❑ Programmable Instruction, Address length, wait cycles and data frame size

❑ Programmable option to skip Address and Instruction phase in enhanced SPI modes.

■ Execute in place mode support

■ Execute in Place (XIP) mode support features:

❑ Programmable Instruction and Address length in XIP mode.

❑ Data frame size mapping directly from AHB transfers.

❑ Fixed data frame size transfer support.

❑ Continuous transfer mode support.

2  Register list

Base address: 0x1f00 0000

| Offset | Register name | reset value | mode | Bit width (bits) | describe |
| --- | --- | --- | --- | --- | --- |
| 0x00 | CTRLR0 | 0x00004007 | R/W | 32 | Control Register 0 : This register controls serial data transfer. This register cannot be written after AHB_QSPI is enabled. AHB_QSPI is enabled by writing to the SSIENR register. |
| 0x04 | CTRLR1 | 0x0 | R/W | 32 | Control Register 1 : Control Register 1 is used to control the end of serial transmission in receive-only mode. This register cannot be written after AHB_QSPI is enabled. AHB_QSPI is enabled by writing to the SSIENR register. |
| 0x08 | SSIENR | 0x0 | R/W | 32 | SSI Enable Register |
| 0x0c | MWCR | 0x0 | R/W | 32 | Microwire Control Register : This register controls the direction of the data word for the half-duplex Microwire serial protocol. This register is not writable after AHB_QSPI is enabled, and AHB_QSPI is enabled by writing to the SSIENR register. |
| 0x10 | SER | 0x1 | R/W | 32 | Slave Enable Register : The register enables the individual slave select output lines from the AHB_QSPI master. O nly one slave-select output signals are available on the AHB_QSPI master. You cannot write to this register when AHB_QSPI is busy. |
| 0x14 | BAUDR | 0x4 | R/W | 32 | Baud Rate Select : The register derives the frequency of the serial clock that regulates the data transfer. The 16-bit field in this register defines the ssi_clk divider value. This register is not writable after AHB_QSPI is enabled, which is enabled by writing to the SSIENR register. |
| 0x18 | TXFTLR | 0x0 | R/W | 32 | Transmit FIFO Threshold Level : This register controls the threshold value for the transmit FIFO memory. This register is not writable after AHB_QSPI is enabled, and AHB_QSPI is enabled by writing to the SSIENR register. |
| 0x1c | RXFTLR | 0x0 | R/W | 32 | Receive FIFO Threshold Level : This register controls the threshold value for the receive FIFO memory. This register cannot be written after AHB_QSPI is enabled, and AHB_QSPI is enabled by writing the SSIENR register. |
| 0x20 | TXFLR | 0x0 | R | 32 | Transmit FIFO Level Register |
| 0x24 | RXFLR | 0x0 | R | 32 | Receive FIFO Level Register |
| 0x28 | SR | 0x6 | R | 32 | Status Register : This is a read-only register used to indicate the current transfer status, FIFO status, and any transmission/reception errors that may have occurred. The status register may be read at any time. None of the bits in this register request an interrupt. |
| 0x2c | IMR | 0x3f | R/W | 32 | Interrupt Mask Register |
| 0x30 | ISR | 0x0 | R | 32 | Interrupt Register Status |
| 0x34 | RISR | 0x0 | R | 32 | Raw Interrupt Status Register |
| 0x38 | TXOICR | 0x0 | R | 32 | Transmit FIFO Overflow Interrupt Clear Register |
| 0x3c | RXOICR | 0x0 | R | 32 | Receive FIFO Overflow Interrupt Clear Register |
| 0x40 | RXUICR | 0x0 | R | 32 | Receive FIFO Underflow Interrupt Clear Register |
| 0x44 | MSTICR | 0x0 | R | 32 | Multi-Master Interrupt Clear Register |
| 0x48 | ICR | 0x0 | R | 32 | Interrupt Clear Register |
| 0x58 | IDR | 0xFFFFFFFF | R | 32 | Identification Register |
| 0x5c | SSI_VERSION_ID | 0x3130322a | R/W | 32 | coreKit Version ID Register |
| 0x60+i*0x04 | DRx (for i = 0; i <= 31) | 0x0 | R/W | 32 | Data Register 0~31: The APB_SSI data register is a 16-bit read/write buffer for the transmit/receive FIFOs. When the register is read, data in the receive FIFO buffer is accessed. When it is written to, data are moved into the transmit FIFO buffer; a write can occur only when SSI_EN = 1. FIFOs are reset when SSI_EN = 0. |
| 0xf0 | RX_SAMPLE_DELAY | 0x0 | R/W | 32 | RX Sample Delay Register |
| 0xf4 | SPI_CTRLR0 | 0x08100200 | R/W | 32 | SPI Control Register |
| 0xfc | XIP_MODE_BITS | 0x0 | R/W | 32 | Mode bits |
| 0x100 | XIP_INCR_INST | 0x0 | R/W | 32 | XIP INCR transfer opcode |
| 0x104 | XIP_WRAP_INST | 0x0 | R/W | 32 | XIP WRAP transfer opcode |

- Register detailed definition

- CTRLR0

■Name: Control Register 0

■Description: This register controls the serial data transfer. It is impossible to write to this register when the AHB_QSPI is enabled.

■Size: 32 bits

■Offset: 0x0

■Reset Value: 0x0000 4007

| 31 | 30:25 | 24 | 23:22 | 21:20 | 19:16 | 15 | 14 | 13 | 12 | 11:10 | 9 | 8 | 7:6 | 5 | 4:0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SSI_IS_MST | RSVD_CTRLR0_25_31 | SPI_HYPERBUS_EN | SPI_FRF | RSVD_CTRLR0_20_21 | CFS | RSVD_CTRLR0_15 | SSTE | SRL | SLV_OE | TMOD | SCPOL | SCPH | FRF | RSVD_CTRLR0_5 | DFS |

Table 3-1 Fields for Register: CTRLR0

| Bits | Name | MemoryAccess | Description |
| --- | --- | --- | --- |
| 31 | SSI_IS_MST | R | This field selects if AHB_QSPI is working in Master or Slave mode Values: ■0x1 (MASTER): AHB_QSPI is Master ■0x0 (SLAVE): AHB_QSPI is Slave Value After Reset: 0x1 |
| 30:25 | RSVD_CTRLR0_25_31 | R | Reserved bits - Read Only Value After Reset: 0x0 |
| 24 | SPI_HYPERBUS_EN | R | SPI Hyperbus Frame format enable. Selects if data frame format for Transmitting/Receiving the data is in Hyperbus mode. This field is effective only when CTRLRO.FRF is set to SPI frame format. Values: ■0x0 (DISABLE): Disable Hyperbus Format ■0x1 (ENABLE): Enable Hyperbus Format Value After Reset: 0x0 |
| 23:22 | SPI_FRF | R/W | SPI Frame Format Selects data frame format for Transmitting/Receiving the data. Bits only valid when SSIC_SPI_MODE is either set to "Dual" or "Quad" or "Octal" mode. Values: ■0x0 (SPI_STANDARD): Standard SPI Format ■0x1 (SPI_DUAL): Dual SPI Format ■0x2 (SPI_QUAD): Quad SPI Format ■0x3 (SPI_OCTAL): Octal SPI Format Value After Reset: 0x0 |
| 21:20 | RSVD_CTRLR0_20_21 | R | Reserved bits - Read Only Value After Reset: 0x0 Exists: Always |
| 19:16 | CFS | R/W | Control Frame Size. Selects the length of the control word for the Microwire frame format. Values: ■0x0 (SIZE_01_BIT): 01-bit Control Word ■0x1 (SIZE_02_BIT): 02-bit Control Word ■0x2 (SIZE_03_BIT): 03-bit Control Word ■0x3 (SIZE_04_BIT): 04-bit Control Word ■0x4 (SIZE_05_BIT): 05-bit Control Word ■0x5 (SIZE_06_BIT): 06-bit Control Word ■0x6 (SIZE_07_BIT): 07-bit Control Word ■0x7 (SIZE_08_BIT): 08-bit Control Word ■0x8 (SIZE_09_BIT): 09-bit Control Word ■0x9 (SIZE_10_BIT): 10-bit Control Word ■0xa (SIZE_11_BIT): 11-bit Control Word ■0xb (SIZE_12_BIT): 12-bit Control Word ■0xc (SIZE_13_BIT): 13-bit Control Word ■0xd (SIZE_14_BIT): 14-bit Control Word ■0xe (SIZE_15_BIT): 15-bit Control Word ■0xf (SIZE_16_BIT): 16-bit Control Word Value After Reset: 0x0 |
| 15 | RSVD_CTRLR0_15 | R | Reserved bits - Read Only Value After Reset: 0x0 |
| 14 | SSTE | R/W | Slave Select Toggle Enable. While operating in SPI mode with clock phase (SCPH) set to 0, this register controls the behavior of the slave select line (ss_*_n) between data frames. Values: ■0x1 (TOGGLE_EN): ss_*_n line will toggle between consecutive data frames, with the serial clock (sclk) being held to its default value while ss_*_n is high ■0x0 (TOGGLE_DISABLE): ss_*_n will stay low and sclk will run continuously for the duration of the transfer Value After Reset: 0x1 |
| 13 | SRL | R/W | Shift Register Loop. Used for testing purposes only. When internally active, connects the transmit shift register output to the receive shift register input. Can be used in both serial-slave and serial-master modes. When the AHB_QSPI is configured as a slave in loopback mode, the ss_in_n and ssi_clk signals must be provided by an external source. In this mode, the slave cannot generate these signals because there is nothing to which to loop back. Values: ■0x1 (TESTING_MODE): Test Mode Operation ■0x0 (NORMAL_MODE): Normal mode operation Value After Reset: 0x0 |
| 12 | SLV_OE | R/W | Slave Output Enable. Relevant only when the AHB_QSPI is configured as a serial-slave device. When configured as a serial master, this bit field has no functionality. This bit enables or disables the setting of the ssi_oe_n output from the AHB_QSPI serial slave. When SLV_OE = 1, the ssi_oe_n output can never be active. When the ssi_oe_n output controls the tri-state buffer on the txd output from the slave, a high impedance state is always present on the slave txd output when SLV_OE = 1. This is useful when the master transmits in broadcast mode (master transmits data to all slave devices). Only one slave may respond with data on the master rxd line. This bit is enabled after reset and must be disabled by software (when broadcast mode is used), if you do not want this device to respond with data. Values: ■0x1 (DISABLED): Slave Output is disabled ■0x0 (ENABLED): Slave Output is enabled Value After Reset: 0x0 |
| 11:10 | TMOD | R/W | Transfer Mode. Selects the mode of transfer for serial communication. This field does not affect the transfer duplicity. Only indicates whether the receive or transmit data are valid. In transmit-only mode, data received from the external device is not valid and is not stored in the receive FIFO memory; it is overwritten on the next transfer. In receive-only mode, transmitted data are not valid. After the first write to the transmit FIFO, the same word is retransmitted for the duration of the transfer. In transmit-and-receive mode, both transmit and receive data are valid. The transfer continues until the transmit FIFO is empty. Data received from the external device are stored into the receive FIFO memory, where it can be accessed by the host processor. Values: ■0x0 (TX_AND_RX): Transmit & Receive; Not Applicable in enhanced SPI operating mode or when SSIC_HAS_TX_RX_EN is set to 0 ■0x1 (TX_ONLY): Transmit only mode; Or Write in enhanced SPI operating mode ■0x2 (RX_ONLY): Receive only mode; Or Read in enhanced SPI operating mode ■0x3 (EEPROM_READ): EEPROM Read mode; Not Applicable in enhanced SPI operating mode Value After Reset: 0x0 |
| 9 | SCPOL | R/W | Serial Clock Polarity. Valid when the frame format (FRF) is set to Motorola SPI. Used to select the polarity of the inactive serial clock, which is held inactive when the AHB_QSPI master is not actively transferring data on the serial bus. Values: ■0x0 (INACTIVE_HIGH): Inactive state of serial clock is low ■0x1 (INACTIVE_LOW): Inactive state of serial clock is high Value After Reset:0x0 |
| 8 | SCPH | R/W | Serial Clock Phase. Valid when the frame format (FRF) is set to Motorola SPI. The serial clock phase selects the relationship of the serial clock with the slave select signal. When SCPH = 0, data are captured on the first edge of the serial clock. When SCPH = 1, the serial clock starts toggling one cycle after the slave select line is activated, and data are captured on the second edge of the serial clock. Values: ■0x1 (START_BIT): Serial clock toggles at start of first bit ■0x0 (MIDDLE_BIT): Serial clock toggles in middle of first bit Value After Reset: 0x0 |
| 7:6 | FRF | R/W | Frame Format. Selects which serial protocol transfers the data. ■0x0 (SPI): Motorola SPI Frame Format ■0x1 (SSP): Texas Instruments SSP Frame Format ■0x2 (MICROWIRE): National Semiconductors Microwire Frame Format ■0x3 (RESERVED): Reserved Value After Reset: 0x0 |
| 5 | RSVD_CTRLR0_5 | R | Reserved bits - Read Only Value After Reset: 0x0 |
| 4:0 | DFS | R/W | Data Frame Size. Selects the data frame length. When the data frame size is programmed to be less than 32 bits, the receive data is automatically right-justified by the receive logic, with the upper bits of the receive FIFO zero-padded. You must right-justify transmit data before writing into the transmit FIFO. The transmit logic ignores the upper unused bits when transmitting the data. Note: When SSIC_SPI_MODE is set to "Dual", "Quad" or "Octal" mode and SPI_FRF is not set to 2'b00: ■DFS value must be a multiple of 2 if SPI_FRF = 01 ■DFS value must be multiple of 4 if SPI_FRF = 10 ■DFS value must be multiple of 8 if SPI_FRF = 11 Values: ■0x0 (DFS_01_BIT): Reserved ■0x1 (DFS_02_BIT): Reserved ■0x2 (DFS_03_BIT): Reserved ■0x3 (DFS_04_BIT): 04-bit serial data transfer ■0x4 (DFS_05_BIT): 05-bit serial data transfer ■0x5 (DFS_06_BIT): 06-bit serial data transfer ■0x6 (DFS_07_BIT): 07-bit serial data transfer ■0x7 (DFS_08_BIT): 08-bit serial data transfer ■0x8 (DFS_09_BIT): 09-bit serial data transfer ■0x9 (DFS_10_BIT): 10-bit serial data transfer ■0xa (DFS_11_BIT): 11-bit serial data transfer ■0xb (DFS_12_BIT): 12-bit serial data transfer ■0xc (DFS_13_BIT): 13-bit serial data transfer ■0xd (DFS_14_BIT): 14-bit serial data transfer ■0xe (DFS_15_BIT): 15-bit serial data transfer ■0xf (DFS_16_BIT): 16-bit serial data transfer ■0x10 (DFS_17_BIT): 17-bit serial data transfer ■0x11 (DFS_18_BIT): 18-bit serial data transfer ■0x12 (DFS_19_BIT): 19-bit serial data transfer ■0x13 (DFS_20_BIT): 20-bit serial data transfer ■0x14 (DFS_21_BIT): 21-bit serial data transfer ■0x15 (DFS_22_BIT): 22-bit serial data transfer ■0x16 (DFS_23_BIT): 23-bit serial data transfer ■0x17 (DFS_24_BIT): 24-bit serial data transfer ■0x18 (DFS_25_BIT): 25-bit serial data transfer ■0x19 (DFS_26_BIT): 26-bit serial data transfer ■0x1a (DFS_27_BIT): 27-bit serial data transfer ■0x1b (DFS_28_BIT): 28-bit serial data transfer ■0x1c (DFS_29_BIT): 29-bit serial data transfer ■0x1d (DFS_30_BIT): 30-bit serial data transfer ■0x1e (DFS_31_BIT): 31-bit serial data transfer ■0x1f (DFS_32_BIT): 32-bit serial data transfer Value After Reset: 0x7 |

- CTRLR1

■Name: Control Register 1

■Description: When the AHB_QSPI is configured as a serial slave, writing to this location has no effect; reading from this location returns 0. Control register 1 controls the end of serial transfers when in receive-only mode. It is impossible to write to this register when the AHB_QSPI is enabled.

■Size: 32 bits

■Offset: 0x4

■Reset Value: 0x0

| 31:16 | 15:0 |
| --- | --- |
| RSVD_CTRLR1 | NDF |

Table 3-2 Fields for Register: CTRLR1

| Bits | Name | Memory Access | Description |
| --- | --- | --- | --- |
| 31:16 | RSVD_CTRLR1 | R | Reserved bits - Read Only Value After Reset: 0x0 |
| 15:0 | NDF | R/W | Number of Data Frames. When TMOD = 10 or TMOD = 11, this register field sets the number of data frames to be continuously received by the AHB_QSPI. The AHB_QSPI continues to receive serial data until the number of data frames received is equal to this register value plus 1, which enables you to receive up to 64 KB of data in a continuous transfer. When the AHB_QSPI is configured as a serial slave, the transfer continues for as long as the slave is selected. Therefore, this register serves no purpose and is not present when the AHB_QSPI is configured as a serial slave. Value After Reset: 0x0 |

- SSIENR

■Name: SSI Enable Register

■Description: This register enables and disables the AHB_QSPI.

■Size: 32 bits

■Offset: 0x8

■Reset Value: 0x0

| 31:1 | 0 |
| --- | --- |
| RSVD_SSIENR | SSIC_EN |

Table 5-8Fields for Register: SSIENR

| Bits | Name | Memory Access | Description |
| --- | --- | --- | --- |
| 31:1 | RSVD_SSIENR | R | Reserved bits - Read Only Value After Reset: 0x0 |
| 0 | SSIC_EN | R/W | SSI Enable. Enables and disables all AHB_QSPI operations. When disabled, all serial transfers are halted immediately. Transmit and receive FIFO buffers are cleared when the device is disabled. It is impossible to program some of the AHB_QSPI control registers when enabled. When disabled, the ssi sleep output is set (after delay) to inform the system that it is safe to remove the ssi_clk, thus saving power consumption in the system. Values: ■0x1 (ENABLED): Enables AHB_QSPI ■0x0 (DISABLE): Disables AHB_QSPI Value After Reset: 0x0 |

- MWCR

■Name: Microwire Control Register

■Description: This register controls the direction of the data word for the half-duplex Microwire serial protocol. It is impossible to write to this register when the AHB_QSPI is enabled.

■Size: 32 bits

■Offset: 0xc

■Reset Value: 0x0

| 31:3 | 2 | 1 | 0 |
| --- | --- | --- | --- |
| RSVD_MWCR | MHS | MDD | MWMOD |

Table 3-4Fields for Register: MWCR

| Bits | Name | Memory Access | Description |
| --- | --- | --- | --- |
| 31:3 | RSVD_MWCR | R | Reserved bits - Read Only Value After Reset: 0x0 |
| 2 | MHS | R/W | Microwire Handshaking. Relevant only when the AHB_QSPI is configured as a serial-master device. When configured as a serial slave, this bit field has no functionality. Used to enable and disable the busy/ready handshaking interface for the Microwire protocol. When enabled, the AHB_QSPI checks for a ready status from the target slave, after the transfer of the last data/control bit, before clearing the BUSY status in the SR register. Values: ■0x1 (ENABLED): handshaking interface is enabled ■0x0 (DISABLE): handshaking interface is disabled Value After Reset: 0x0 |
| 1 | MDD | R/W | Microwire Control. Defines the direction of the data word when the Microwire serial protocol is used. When this bit is set to 0, the data word is received by the AHB_QSPI MacroCell from the external serial device. When this bit is set to 1, the data word is transmitted from the AHB_QSPI MacroCell to the external serial device. Values: ■0x1 (TRANSMIT): SSI transmits data ■0x0 (RECEIVE): SSI receives data Value After Reset: 0x0 |
| 0 | MWMOD | R/W | Microwire Transfer Mode. Defines whether the Microwire transfer is sequential or non-sequential. When sequential mode is used, only one control word is needed to transmit or receive a block of data words. When non-sequential mode is used, there must be a control word for each data word that is transmitted or received. Values: ■0x1 (SEQUENTIAL): Sequential Transfer ■0x0 (NON_SEQUENTIAL): Non-Sequential Transfer Value After Reset:0x0 |

- SER

■Name: Slave Enable Register

■Description: This register is valid only when the AHB_QSPI is configured as a master device. When the AHB_QSPI is configured as a serial slave, writing to this location has no effect; reading from this location returns 0. The register enables the individual slave select output lines from the AHB_QSPI master. Only 1 slave-select output pins are available on the AHB_QSPI master. You cannot write to this register when AHB_QSPI is busy and when SSIC_EN = 1.

■Size: 32 bits

■Offset: 0x10

■Reset Value: 0x1

| 31:1 | 0 |
| --- | --- |
| RSVD_SER | SER |

Table 3-5 Fields for Register: SER

| Bits | Name | Memory Access | Description |
| --- | --- | --- | --- |
| 31：1 | RSVD_SER | R | Reserved bits - Read Only Value After Reset: 0x0 |
| 0 | SER | R/W | Slave Select Enable Flag. Each bit in this register corresponds to a slave select line (ss_x_n) from the AHB_QSPI master. When a bit in this register is set (1), the corresponding slave select line from the master is activated when a serial transfer begins. It should be noted that setting or clearing bits in this register have no effect on the corresponding slave select outputs until a transfer is started. Before beginning a transfer, you should enable the bit in this register that corresponds to the slave device with which the master wants to communicate. When not operating in broadcast mode, only one bit in this field should be set. ■1: Selected ■0: Not Selected Value After Reset: 0x1 |

- BAUDR

■Name: Baud Rate Select

■Description: This register is valid only when the AHB_QSPI is configured as a master device. When the AHB_QSPI is configured as a serial slave, writing to this location has no effect; reading from this location returns 0. The register derives the frequency of the serial clock that regulates the data transfer. The 16-bit field in this register defines the ssi_clk divider value. It is impossible to write to this register when the AHB_QSPI is enabled.

■Size: 32 bits

■Offset: 0x14

■Reset Value: 0x04

| 31:16 | 15:0 |
| --- | --- |
| RSVD_BAUDR_16_31 | SCKDV |

Table 3-6 Fields for Register: BAUDR

| Bits | Name | Memory Access | Description |
| --- | --- | --- | --- |
| 31:16 | RSVD_BAUDR_16_31 | R | Reserved bits - Read Only Value After Reset: 0x0 Exists: Always |
| 15:0 | SCKDV | R/W | SSI Clock Divider. The LSB for this field is always set to 0 and is unaffected by a write operation, which ensures an even value is held in this register. If the value is 0, the serial output clock (sclk_out) is disabled. The frequency of the sclk_out is derived from the following equation: Fsclk_out = Fssi_clk/SCKDV where SCKDV is any even value between 2 and 65534. For example: for Fssi_clk = 3.6864MHz and SCKDV =2 Fsclk_out = 3.6864/2 = 1.8432MHz Value After Reset: 4 |

- TXFTLR

■Name: Transmit FIFO Threshold Level

■Description: This register controls the threshold value for the transmit FIFO memory. The AHB_QSPI is enabled and disabled by writing to the SSIENR register.

Reset Value: 0x0

■Size: 32 bits

■Offset: 0x18

■Reset Value: 0x0

| 31:21 | 20:16 | 15:5 | 4:0 |
| --- | --- | --- | --- |
| RSVD_TXFTLR | TXFTHR | RSVD_TXFTLR | TFT |

Table 3-7 Fields for Register: TXFTLR

| Bits | Name | Memory Access | Description |
| --- | --- | --- | --- |
| 31:21 | RSVD_TXFTHR | R | Reserved bits - Read Only Value After Reset: 0x0 |
| 20:16 | TXFTHR | R/W | Transfer start FIFO level. Used to control the level of entries in transmit FIFO above which transfer will start on serial line. This register can be used to ensure that sufficient data is present in transmit FIFO before starting a write operation on serial line. These field is valid only for Master mode of operation. Value After Reset: 0x0 |
| 15:5 | RSVD_TXFTLR | R | Reserved bits - Read Only Value After Reset: 0x0 |
| 4:0 | TFT | R/W | Transmit FIFO Threshold. Controls the level of entries (or below) at which the transmit FIFO controller triggers an interrupt. The FIFO depth is 32; this register is sized to the number of address bits needed to access the FIFO. If you attempt to set this value greater than or equal to the depth of the FIFO, this field is not written and retains its current value. When the number of transmit FIFO entries is less than or equal to this value, the transmit FIFO empty interrupt is triggered. Value After Reset: 0x0 |

- RXFTLR

■Name: Receive FIFO Threshold Level

■Description: This register controls the threshold value for the receive FIFO memory..

■Size: 32 bits

■Offset: 0x1c

■Reset Value: 0x0

| 31:16 | 15:0 |
| --- | --- |
| RSVD_RXFTLR | RFT |

Table 3-8 Fields for Register: RXFTLR

| Bits | Name | Memory Access | Description |
| --- | --- | --- | --- |
| 31:5 | RSVD_RXFTLR | R | Reserved bits - Read Only Value After Reset: 0x0 |
| 4:0 | RFT | R/W | Receive FIFO Threshold. Controls the level of entries (or above) at which the receive FIFO controller triggers an interrupt. The FIFO depth is configurable in the range 8-256. This register is sized to the number of address bits needed to access the FIFO. If you attempt to set this value greater than the depth of the FIFO, this field is not written and retains its current value. When the number of receive FIFO entries is greater than or equal to this value + 1, the receive FIFO full interrupt is triggered. Value After Reset: 0x0 |

- TXFLR

■Name: Transmit FIFO Level Register

■Description: This register contains the number of valid data entries in the transmit FIFO memory.

■Size: 32 bits

■Offset: 0x20

■Reset Value: 0x0

| 31:16 | 15:0 |
| --- | --- |
| RSVD_TXFLR | TXTFL |

Table 3-9 Fields for Register: TXFLR

| Bits | Name | Memory Access | Description |
| --- | --- | --- | --- |
| 31:6 | RSVD_TXFLR | R | Reserved bits - Read Only Value After Reset: 0x0 |
| 5:0 | TXTFL | R | Transmit FIFO Level. Contains the number of valid data entries in the transmit FIFO. Value After Reset: 0x0 |

- RXFLR

■Name: Receive FIFO Level Register

■Description: This register contains the number of valid data entries in the receive FIFO memory. This register can be read at any time.

■Size: 32 bits

■Offset: 0x24

■Reset Value: 0x0

| 31:16 | 15:0 |
| --- | --- |
| RSVD_RXFLR | RXTFL |

Table 3-10 Fields for Register: RXFLR

| Bits | Name | Memory Access | Description |
| --- | --- | --- | --- |
| 31:6 | RSVD_RXFLR | R | Reserved bits - Read Only Value After Reset: 0x0 |
| 5:0 | RXTFL | R | Receive FIFO Level. Contains the number of valid data entries in the receive FIFO. Value After Reset: 0x0 |

- SR

■Name: Status Register

■Description: This is a read-only register used to indicate the current transfer status, FIFO status, and any transmission/reception errors that may have occurred. The status register may be read at any time. None of the bits in this register request an interrupt.

■Size: 32 bits

■Offset: 0x28

■Reset Value: 0x06

| 31:7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| RSVD_SR | DCOL | TXE | RFF | RFNE | TFE | TFNF | BUSY |

Table 3-11 Fields for Register: SR

| Bits | Name | Memory Access | Description |
| --- | --- | --- | --- |
| 31:7 | RSVD_SR | R | Reserved bits - Read Only Value After Reset: 0x0 |
| 6 | DCOL | R | Data Collision Error. Relevant only when the AHB_QSPI is configured as a master device. This bit will be set if ss_in_n input is asserted by other master, when the AHB_QSPI master is in the middle of the transfer. This informs the processor that the last data transfer was halted before completion. This bit is cleared when read. Values: ■0x1 (TX_COLLISION_ERROR): Transmit Data Collision Error ■0x0 (NO_ERROR_CONDITION): No Error Value After Reset: 0x0 |
| 5 | TXE | R | Transmission Error. Set if the transmit FIFO is empty when a transfer is started. This bit can be set only when the AHB_QSPI is configured as a slave device. Data from the previous transmission is resent on the txd line. This bit is cleared when read. Values: ■0x1 (TX_ERROR): Transmission Error ■0x0 (NO_ERROR): No Error Value After Reset: 0x0 |
| 4 | RFF | R | Receive FIFO Full. When the receive FIFO is completely full, this bit is set. When the receive FIFO contains one or more empty location, this bit is cleared. Values: ■0x1 (FULL): Receive FIFO is full ■0x0 (NOT_FULL): Receive FIFO is not full Value After Reset: 0x0 |
| 3 | RFNE | R | Receive FIFO Not Empty. Set when the receive FIFO contains one or more entries and is cleared when the receive FIFO is empty. This bit can be polled by software to completely empty the receive FIFO. Values: ■0x1 (NOT_EMPTY): Receive FIFO is not empty ■0x0 (EMPTY): Receive FIFO is empty Value After Reset: 0x0 |
| 2 | TFE | R | Transmit FIFO Empty. When the transmit FIFO is completely empty, this bit is set. When the transmit FIFO contains one or more valid entries, this bit is cleared. This bit field does not request an interrupt. Values: ■0x1 (EMPTY): Transmit FIFO is empty ■0x0 (NOT_EMPTY): Transmit FIFO is not empty Value After Reset: 0x1 |
| 1 | TFNF | R | Transmit FIFO Not Full. Set when the transmit FIFO contains one or more empty locations, and is cleared when the FIFO is full. Values: ■0x1 (NOT_FULL): Tx FIFO is not Full ■0x0 (FULL): Tx FIFO is full Value After Reset: 0x1 |
| 0 | BUSY | R | SSI Busy Flag. When set, indicates that a serial transfer is in progress; when cleared indicates that the AHB_QSPI is idle or disabled. Values: ■0x1 (ACTIVE): AHB_QSPI is actively transferring data ■0x0 (INACTIVE): AHB_QSPI is idle or disabled Value After Reset: 0x0 |

- IMR

■Name: Interrupt Mask Register

■Description: This read/write register masks or enables all interrupts generated by the AHB_QSPI. When the AHB_QSPI is configured as a slave device, the MSTIM bit field is not present. This changes the reset value from 0x3F for serial-master configurations to 0x1F for serial-slave configurations.

■Size: 32 bits

■Offset: 0x2c

■Reset Value: 0x3F

| 31:7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| RSVD_IMR | XRXOIM | MSTIM | RXFIM | RXOIM | RXUIM | TXOIM | TXE |

Table 3-12 Fields for Register: IMR

| Bits | Name | Memory Access | Description |
| --- | --- | --- | --- |
| 31:7 | RSVD_IMR | R | Reserved bits - Read Only Value After Reset: 0x0 |
| 6 | XRXOIM | R | XIP Receive FIFO Overflow Interrupt Mask Values: ■0x1 (UNMASKED): ssi_xrxo_intr interrupt is not masked ■0x0 (MASKED): ssi_xrxo_intr interrupt is masked Value After Reset: 0 |
| 5 | MSTIM | R/W | Multi-Master Contention Interrupt Mask. This bit field is not present if the AHB_QSPI is configured as a serial-master device. Values: ■0x1 (UNMASKED): ssi_mst_intr interrupt is not masked ■0x0 (MASKED): ssi_mst_intr interrupt is masked Value After Reset:0x1 |
| 4 | RXFIM | R/W | Receive FIFO Full Interrupt Mask 0 - ssi_rxf_intr interrupt is masked 1 - ssi_rxf_intr interrupt is not masked Values: ■0x1 (UNMASKED): ssi_rxf_intr interrupt is not masked ■0x0 (MASKED): ssi_rxf_intr interrupt is masked Value After Reset: 0x1 |
| 3 | RXOIM | R/W | Receive FIFO Overflow Interrupt Mask Values: ■0x1 (UNMASKED): ssi_rxo_intr interrupt is not masked ■0x0 (MASKED): ssi_rxo_intr interrupt is masked Value After Reset: 0x1 |
| 2 | RXUIM | R/W | Receive FIFO Underflow Interrupt Mask Values: ■0x1 (UNMASKED): ssi_rxu_intr interrupt is not masked ■0x0 (MASKED): ssi_rxu_intr interrupt is masked Value After Reset: 0x1 |
| 1 | TXOIM | R/W | Transmit FIFO Overflow Interrupt Mask Values: ■0x1 (UNMASKED): ssi_txo_intr interrupt is not masked ■0x0 (MASKED): ssi_txo_intr interrupt is masked Value After Reset: 0x1 |
| 0 | TXEIM | R/W | Transmit FIFO Empty Interrupt Mask Values: ■0x1 (UNMASKED): ssi_txe_intr interrupt is not masked ■0x0 (MASKED): ssi_txe_intr interrupt is masked Value After Reset: SSIC_IS_DFLT_TXEIM |

- ISR

■Name: Interrupt Status Register

■Description: This register reports the status of the AHB_QSPI interrupts after they have been masked.

■Size: 32 bits

■Offset: 0x30

■Reset Value: 0x0

| 31:7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| RSVD_ISR | XRXOIS | MSTIS | RXFIS | RXOIS | RXUIS | TXOIS | TXEIS |

Table 3-13 Fields for Register: ISR

| Bits | Name | Memory Access | Description |
| --- | --- | --- | --- |
| 31:7 | RSVD_ISR | R | Reserved bits - Read Only Value After Reset: 0x0 |
| 6 | XRXOIS | R | XIP Receive FIFO Overflow Interrupt Status Values: ■0x1 (ACTIVE): ssi_xrxo_intr interrupt is active after masking ■0x0 (INACTIVE): ssi_xrxo_intr interrupt is not active after masking Value After Reset: 0x0 |
| 5 | MSTIS | R | Multi-Master Contention Interrupt Status. This bit field is not present if the AHB_QSPI is configured as a serial-slave device. Values: ■0x1 (ACTIVE): ssi_mst_intr interrupt is active after masking ■0x0 (INACTIVE): ssi_mst_intr interrupt is not active after masking Value After Reset: 0x0 |
| 4 | RXFIS | R | Receive FIFO Full Interrupt Status Values: ■0x1 (ACTIVE): ssi_rxf_intr interrupt is active after masking ■0x0 (INACTIVE): ssi_rxf_intr interrupt is not active after masking Value After Reset: 0x0 |
| 3 | RXOIS | R | Receive FIFO Overflow Interrupt Status Values: ■0x1 (ACTIVE): ssi_rxo_intr interrupt is active after masking ■0x0 (INACTIVE): ssi_rxo_intr interrupt is not active after masking Value After Reset: 0x0 |
| 2 | RXUIS | R | Receive FIFO Underflow Interrupt Status Values: ■0x1 (ACTIVE): ssi_rxu_intr interrupt is active after masking ■0x0 (INACTIVE): ssi_rxu_intr interrupt is not active after masking Value After Reset: 0x0 |
| 1 | TXOIS | R | Transmit FIFO Overflow Interrupt Status Values: ■0x1 (ACTIVE): ssi_txo_intr interrupt is active after masking ■0x0 (INACTIVE): ssi_txo_intr interrupt is not active after masking Value After Reset: 0x0 |
| 0 | TXEIS | R | Transmit FIFO Empty Interrupt Status Values: ■0x1 (ACTIVE): ssi_txe_intr interrupt is active after masking ■0x0 (INACTIVE): ssi_txe_intr interrupt is not active after masking Value After Reset: 0x0 |

- RISR

■Name: Raw Interrupt Status Register

■Description: Raw Interrupt Status Register

■Size: 32 bits

■Offset: 0x34

■Reset Value: 0x0

| 31:7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| RSVD_RISR | XRXOIR | MSTIR | RXFIR | RXOIR | RXUIR | TXOIR | TXEIR |

Table 3-14 Fields for Register: RISR

| Bits | Name | Memory Access | Description |
| --- | --- | --- | --- |
| 31:7 | RSVD_RISR | R | Reserved bits - Read Only Value After Reset: 0x0 |
| 6 | XRXOIR | R | XIP Receive FIFO Overflow Raw Interrupt Status Values: ■0x1 (ACTIVE): ssi_xrxo_intr interrupt is active prior to masking ■0x0 (INACTIVE): ssi_xrxo_intr interrupt is not active prior masking Value After Reset: 0x0 |
| 5 | MSTIR | R | Multi-Master Contention Raw Interrupt Status. This bit field is not present if the AHB_QSPI is configured as a serial-slave device. Values: ■0x1 (ACTIVE): ssi_mst_intr interrupt is active prior to masking ■0x0 (INACTIVE): ssi_mst_intr interrupt is not active prior masking Value After Reset: 0x0 |
| 4 | RXFIR | R | Receive FIFO Full Raw Interrupt Status Values: ■0x1 (ACTIVE): ssi_rxf_intr interrupt is active prior to masking ■0x0 (INACTIVE): ssi_rxf_intr interrupt is not active prior masking Value After Reset: 0x0 |
| 3 | RXOIR | R | Receive FIFO Overflow Raw Interrupt Status Values: ■0x1 (ACTIVE): ssi_rxo_intr interrupt is active prior to masking ■0x0 (INACTIVE): ssi_rxo_intr interrupt is not active prior masking Value After Reset: 0x0 |
| 2 | RXUIR | R | Receive FIFO Underflow Raw Interrupt Status Values: ■0x1 (ACTIVE): ssi_rxu_intr interrupt is active prior to masking ■0x0 (INACTIVE): ssi_rxu_intr interrupt is not active prior masking Value After Reset: 0x0 |
| 1 | TXOIR | R | Transmit FIFO Overflow Raw Interrupt Status Values: ■0x1 (ACTIVE): ssi_txo_intr interrupt is active prior to masking ■0x0 (INACTIVE): ssi_txo_intr interrupt is not active prior masking Value After Reset: 0x0 |
| 0 | TXEIR | R | Transmit FIFO Empty Raw Interrupt Status Values: ■0x1 (ACTIVE): ssi_txe_intr interrupt is active prior to masking ■0x0 (INACTIVE): ssi_txe_intr interrupt is not active prior masking Value After Reset: 0x0 |

- RXOICR

■Name: Receive FIFO Overflow Interrupt Clear Register

■Description: Receive FIFO Overflow Interrupt Clear Register

■Size: 32 bits

■Offset: 0x3c

■Reset Value: 0x0

| 31:1 | 0 |
| --- | --- |
| RSVD_RXOICR | RXOICR |

Table 3-15 Fields for Register: RXOICR

| Bits | Name | Memory Access | Description |
| --- | --- | --- | --- |
| 31:1 | RSVD_RXOICR | R | Reserved bits - Read Only Value After Reset: 0x0 |
| 0 | RXOICR | R | Clear Receive FIFO Overflow Interrupt. This register reflects the status of the interrupt. A read from this register clears the ssi_rxo_intr interrupt; writing has no effect. Value After Reset: 0x0 |

- RXUICR

■Name: Receive FIFO Underflow Interrupt Clear Register

■Description: Receive FIFO Underflow Interrupt Clear Register

■Size: 32 bits

■Offset: 0x40

■Reset Value: 0x0

| 31:1 | 0 |
| --- | --- |
| RSVD_RXUICR | RXUICR |

Table 3-16 Fields for Register: RXUICR

| Bits | Name | Memory Access | Description |
| --- | --- | --- | --- |
| 31:1 | RSVD_RXUICR | R | Reserved bits - Read Only Value After Reset: 0x0 |
| 0 | RXUICR | R | Clear Receive FIFO Underflow Interrupt. This register reflects the status of the interrupt. A read from this register clears the ssi_rxu_intr interrupt; writing has no effect. Value After Reset: 0x0 |

- MSTICR

■Name: Multi-Master Interrupt Clear Register

■Description: Multi-Master Interrupt Clear Register

■Size: 32 bits

■Offset: 0x44

■Reset Value: 0x0

| 31:1 | 0 |
| --- | --- |
| RSVD_MSTICR | MSTICR |

Table 3-17 Fields for Register: MSTICR

| Bits | Name | Memory Access | Description |
| --- | --- | --- | --- |
| 31:1 | RSVD_MSTICR | R | Reserved bits - Read Only Value After Reset: 0x0 |
| 0 | MSTICR | R | Clear Multi-Master Contention Interrupt. This register reflects the status of the interrupt. A read from this register clears the ssi_mst_intr interrupt; writing has no effect. Value After Reset: 0x0 |

- ICR

■Name: Interrupt Clear Register

■Description: Interrupt Clear Register

■Size: 32 bits

■Offset: 0x48

■Reset Value: 0x0

| 31:1 | 0 |
| --- | --- |
| RSVD_ICR | ICR |

Table 5-18 Fields for Register: ICR

| Bits | Name | Memory Access | Description |
| --- | --- | --- | --- |
| 31:1 | RSVD_ICR | R | Reserved bits - Read Only Value After Reset: 0x0 |
| 0 | ICR | R | Clear Interrupts. This register is set if any of the interrupts below are active. A read clears the ssi_txo_intr, ssi_rxu_intr, ssi_rxo_intr, and the ssi_mst_intr interrupts. Writing to this register has no effect. Value After Reset: 0x0 |

- IDR

■Name: Identification Register

■Description: This register contains the peripherals identification code, which is written into the register at configuration time using coreConsultant.

■Size: 32 bits

■Offset: 0x58

■Reset Value: 0xFFFFFFFF

31:0

IDCODE

Table 3-19 Fields for Register: IDR

| Bits | Name | Memory Access | Description |
| --- | --- | --- | --- |
| 31:0 | IDCODE | R | Identification code. The register contains the peripheral's identification code, which is written into the register at configuration time using CoreConsultant. Value After Reset: 0xFFFFFFFF |

- SSIC_VERSION_ID

■Description: This read-only register stores the specific AHB_QSPI component version.

■Size: 32 bits

■Offset: 0x5c

■Reset Value: 0x3130322A

31:0

SSIC_COMP_VERSION

Table 3-20 Fields for Register: SSIC_VERSION_ID

| Bits | Name | Memory Access | Description |
| --- | --- | --- | --- |
| 31:0 | SSIC_COMP_VERSION | R | Contains the hex representation of the Synopsys component version. Consists of ASCII value for each number in the version, followed by *. For example 32_30_31_2A represents the version 2.01*. Value After Reset: 0x3130322A |

- DRx(for i=0; i <=31)

■Name: AHB_QSPI Data Register

■Description: The AHB_QSPI data register is a 32-bit read/write buffer for the transmit/receive FIFOs. When the register is read, data in the receive FIFO buffer is accessed. When it is written to, data are moved into the transmit FIFO buffer; a write can occur only when SSIC_EN = 1. FIFOs are reset when SSIC_EN = 0.

Note The DR register in the AHB_QSPI occupies thirty-six 32-bit address locations of the memory map to facilitate AHB burst transfers. Writing to any of these address locations has the same effect as pushing the data from the pwdata bus into the transmit FIFO. Reading from any of these locations has the same effect as popping data from the receive FIFO onto the hrdata bus. The FIFO buffers on the AHB_QSPI are not addressable.

■Size: 32 bits

■Offset: 0x60 +i*0x4

■Reset Value: 0x0

31:0

DR

Table 3-21 Fields for Register: DRx (for i = 0; i <= 31)

| Bits | Name | Memory Access | Description |
| --- | --- | --- | --- |
| 31:0 | DR | R/W | Data Register. When writing to this register, you must right-justify the data. Read data are automatically right-justified. Read = Receive FIFO buffer Write = Transmit FIFO buffer. Value After Reset: 0x0 |

- RX_SAMPLE_DELAY

■Name: RX Sample Delay Register

■Description:This register control the number of ssi_clk cycles that are delayed (from the default sample time) before the actual sample of the rxd input occurs. It is impossible to write to this register when the AHB_QSPI is enabled.

■Size: 32 bits

■Offset: 0xf0

■Reset Value: 0x0

| 31:17 | 16 | 15:8 | 7:0 |
| --- | --- | --- | --- |
| RSVD1_RX_SAMPLE_DLY | SE | RSVD0_RX_SAMPLE_DLY | RSD |

Table 3-22 Fields for Register: RX_SAMPLE_DELAY

| Bits | Name | Memory Access | Description |
| --- | --- | --- | --- |
| 31:17 | RSVD1_RX_SAMPLE_DLY | R | Reserved bits - Read Only Value After Reset: 0x0 |
| 16 | SE | R | Receive Data (rxd) Sampling Edge. This register is used to decide the sampling edge for RXD signal with ssi_clk. Then this bit is set to 1 then negative edge of ssi_clk will be used to sample the incoming data, otherwise positive edge will be used for sampling. Value After Reset: 0x0 |
| 15:8 | RSVD0_RX_SAMPLE_DLY | R | Reserved bits - Read Only Value After Reset: 0x0 |
| 7:0 | RSD | R/W | Receive Data (rxd) Sample Delay. This register is used to delay the sample of the rxd input port. Each value represents a single ssi_clk delay on the sample of rxd. Note; If this register is programmed with a value that exceeds the depth of the internal shift registers (4) zero delay will be applied to the rxd sample. Value After Reset: 0x0 |

- SPI_CTRLR0

■Name: SPI_CTRLR0 - SPI Control Register

■Description: This register is used to control the serial data transfer in enhanced SPI mode of operation. The register is relevant only when SPI_FRF (in CTRLR0) is set to either 01 or 10 or 11. It is not possible to write to this register when the AHB_QSPI is enabled (SSIC_EN=1).

■Size: 32 bits

■Offset: 0xf4

■Reset Value: 0x0810 0200

| 31 | 30 | 29 | 28 | 27:26 | 25 | 24 | 23:22 | 21 | 20 | 19 | 18 | 17 | 16 | 15:11 | 10 | 9:8 | 7 | 6 | 5:2 | 1:0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| RSVD_SPI_CTRLR0 | CLK_STRETCH_EN | XIP_PREFETCH_EN | RSVD_SPI_CTRLR0_28 | XIP_MBL | SPI_RXDS_SIG_EN | SPI_DM_EN | RSVD_SPI_CTRLR0_22_23 | SSIC_XIP_CONT_XFER_EN | XIP_INST_EN | XIP_DFS_HC | SPI_RXDS_EN | INST_DDR_EN | SPI_DDR_EN | WAIT_CYCLES | RSVD_SPI_CTRLR0_10 | INST_L | XIP_MD_BIT_EN | RSVD_SPI_CTRLR0_6 | ADDR_L | TRANS_TYPE |

Table 3-23 Fields for Register: SPI_CTRLR0

| Bits | Name | Memory Access | Description |
| --- | --- | --- | --- |
| 31 | RSVD_SPI_CTRLR0 | R | Reserved bits - Read Only Value After Reset: 0x0 |
| 30 | CLK_STRETCH_EN | R/W | Enables clock stretching capability in SPI transfers. In case of write, if the FIFO becomes empty AHB_QSPI will stretch the clock until FIFO has enough data to continue the transfer. In case of read, if the receive FIFO becomes full AHB_QSPI will stop the clock until data has been read from the FIFO. Value After Reset: 0x0 |
| 29 | XIP_PREFETCH_EN | R/W | Enables XIP pre-fetch functionality in AHB_QSPI. Once enabled AHB_QSPI will pre-fetch data frames from next contigous location, to reduce the latency for the upcoming contiguous transfer. If the next XIP request is not contigous then pre-fetched bits will be discarded. Value After Reset: 0x0 |
| 28 | RSVD_SPI_CTRLR0_28 | R | Reserved bits - Read Only Value After Reset: 0x0 Exists: Always |
| 27:26 | XIP_MBL | R/W | XIP Mode bits length. Sets the length of mode bits in XIP mode of operation. These bits are valid only when SPI_CTRLR0.XIP_MD_BIT_EN is set to 1. Values: ■0x0 (MBL_2): Mode bits length equal to 2 ■0x1 (MBL_4): Mode bits length equal to 4 ■0x2 (MBL_8): Mode bits length equal to 8 ■0x3 (MBL_16): Mode bits length equal to 16 Value After Reset:0x2 |
| 25 | SPI_RXDS_SIG_EN | R | Enable rxds signaling during address and command phase of Hypebus transfer. This bit enables rxds signaling by Hyperbus slave devices during Command-Address (CA) phase. If the rxds signal is set to 1 during the CA phase of transfer, AHB_QSPI transmits (2*SPI_CTRLR0.WAIT_CYCLES-1) wait cycles after the address phase is complete. Value After Reset: 0x0 |
| 24 | SPI_DM_EN | R | SPI data mask enable bit. When this bit is enabled, the txd_dm signal is used to mask the data on the txd data line. Value After Reset: 0x0 |
| 23:22 | RSVD_SPI_CTRLR0_22_23 | R | Reserved bits - Read Only Value After Reset: 0x0 Exists: Always |
| 21 | SSIC_XIP_CONT_XFER_EN | R/W | Enable continuous transfer in XIP mode. If this bit is set to 1 then continuous transfer mode in XIP will be enabled, in this mode AHB_QSPI will keep slave selected until a non-XIP transfer is detected on the AHB interface. Value After Reset: 0x0 |
| 20 | XIP_INST_EN | R/W | XIP instruction enable bit. If this bit is set to 1 then XIP transfers will also have instruction phase. The instruction op-codes will be chosen from XIP_INCR_INST or XIP_WRAP_INST registers bases on AHB transfer type. Value After Reset:0x1 |
| 19 | XIP_DFS_HC | R/W | Fix DFS for XIP transfers. If this bit is set to 1 then data frame size for XIP transfers will be fixed to the programmed value in CTRLR0.DFS. The number of data frames to fetch will be determined by HSIZE and HBURST signals. If this bit is set to 0 then data frame size and number of data frames to fetch will be determined by HSIZE and HBURST signals Value After Reset: 0x0 |
| 18 | SPI_RXDS_EN | R | Read data strobe enable bit. Once this bit is set to 1 AHB_QSPI will use Read data strobe (rxds) to capture read data in DDR mode. Value After Reset: 0x0 |
| 17 | INST_DDR_EN | R | Instruction DDR Enable bit. This will enable Dual-data rate transfer for Instruction phase. Value After Reset:0x0 |
| 16 | SPI_DDR_EN | R | SPI DDR Enable bit. This will enable Dual-data rate transfers in Dual/Quad/Octal frame formats of SPI. Value After Reset: 0x0 |
| 15:11 | WAIT_CYCLES | R/W | Wait cycles in Dual/Quad/Octal mode between control frames transmit and data reception. Specified as number of SPI clock cycles. Value After Reset: 0x0 |
| 10 | RSVD_SPI_CTRLR0_10 | R | Reserved bits - Read Only Value After Reset: 0x0 |
| 9:8 | INST_L | R/W | Dual/Quad/Octal mode instruction length in bits. Values: ■0x0 (INST_L0): No Instruction ■0x1 (INST_L4): 4 bit Instruction length ■0x2 (INST_L8): 8 bit Instruction length ■0x3 (INST_L16): 16 bit Instruction length Value After Reset: 0x2 |
| 7 | XIP_MD_BIT_EN | R/W | Mode bits enable in XIP mode. If this bit is set to 1, then in XIP mode of operation AHB_QSPI will insert mode bits after the address phase. These bits are set in register XIP_MODE_BITS register. The length of mode bits is always set to 8 bits. Value After Reset: 0x0 |
| 6 | RSVD_SPI_CTRLR0_6 | R | Reserved bits - Read Only Value After Reset: 0x0 Exists: Always |
| 5:2 | ADDR_L | R/W | This bit defines Length of Address to be transmitted. Only after this much bits are programmed in to the FIFO the transfer can begin. Values: ■0x0 (ADDR_L0): No Address ■0x1 (ADDR_L4): 4 bit Address length ■0x2 (ADDR_L8): 8 bit Address length ■0x3 (ADDR_L12): 12 bit Address length ■0x4 (ADDR_L16): 16 bit Address length ■0x5 (ADDR_L20): 20 bit Address length ■0x6 (ADDR_L24): 24 bit Address length ■0x7 (ADDR_L28): 28 bit Address length ■0x8 (ADDR_L32): 32 bit Address length ■0x9 (ADDR_L36): 36 bit Address length ■0xa (ADDR_L40): 40 bit Address length ■0xb (ADDR_L44): 44 bit Address length ■0xc (ADDR_L48): 48 bit Address length ■0xd (ADDR_L52): 52 bit Address length ■0xe (ADDR_L56): 56 bit Address length ■0xf (ADDR_L60): 60 bit Address length Value After Reset: SSIC_DFLT_ADDR_L |
| 1:0 | TRANS_TYPE | R/W | Address and instruction transfer format. Selects whether AHB_QSPI will transmit instruction/address either in Standard SPI mode or the SPI mode selected in CTRLR0.SPI_FRF field. Values: ■0x0 (TT0): Instruction and Address will be sent in Standard SPI Mode. ■0x1 (TT1): Instruction will be sent in Standard SPI Mode and Address will be sent in the mode specified by CTRLR0.SPI_FRF. ■0x2 (TT2): Both Instruction and Address will be sent in the mode specified by SPI_FRF. ■0x3 (TT3): Reserved. Value After Reset: 0x0 |

- XIP_MODE_BITS

■Name: eXecute in Place - Mode bits

■Description: This register carries the mode bits which are sent in the XIP mode of operation after address phase. This is a 8 bit register and can only be written when SSIENR register is set to 0.

■Size: 32 bits

■Offset: 0xfc

■Reset Value: 0x0

| 31:16 | 15:0 |
| --- | --- |
| RSVD_XIP_MD_BITS | XIP_MD_BITS |

Table 3-24 Fields for Register: XIP_MODE_BITS

| Bits | Name | Memory Access | Description |
| --- | --- | --- | --- |
| 31:16 | RSVD_XIP_MD_BITS | R | Reserved bits Value After Reset: 0x0 |
| 15:0 | XIP_MD_BITS | R/W | XIP mode bits to be sent after address phase of XIP transfer. Value After Reset: 0x0 |

- XIP_INCR_INST

■Name: XIP_INCR_INST - XIP INCR transfer opcode

■Description: This register is used to store the instruction op-code to be used in INCR transactions when the same is requested on AHB interface. It is not possible to write to this register when the AHB_QSPI is enabled (SSIC_EN=1).

■Size: 32 bits

■Offset: 0x100

■Reset Value: 0x0

| 31:16 | 15:0 |
| --- | --- |
| RSVD_INCR_INST | INCR_INST |

Table 3-25 Fields for Register: XIP_INCR_INST

| Bits | Name | Memory Access | Description |
| --- | --- | --- | --- |
| 31:16 | RSVD_INCR_INST | R | Reserved bits - Read Only Value After Reset: 0x0 |
| 15:0 | INCR_INST | R/W | XIP INCR transfer opcode. When SPI_CTRLR0.XIP_INST_EN bit is set to 1, AHB_QSPI sends instruction for all XIP transfers, this register field stores the instruction op-code to be sent when an INCR type transfer is requested on AHB bus. The number of bits to be send in instruction phase is determined by SPI_CTRL0.INST_L field. Value After Reset: 0X0 |

- XIP_WRAP_INST

■Name: XIP_WRAP_INST - XIP WRAP transfer opcode

■Description: This Register is valid only when SSIC_XIP_INST_EN is equal to 1. This register is used to store the instruction op-code to be used in WRAP transactions when the same is requested on AHB interface. It is not possible to write to this register when the AHB_QSPI is enabled (SSIC_EN=1).

■Size: 32 bits

■Offset: 0x104

■Reset Value: 0x0

| 31:16 | 15:0 |
| --- | --- |
| RSVD_WRAP_INST | WRAP_INST |

Table 3-26 Fields for Register: XIP_WRAP_INST

| Bits | Name | Memory Access | Description |
| --- | --- | --- | --- |
| 31:16 | RSVD_WRAP_INST | R | Reserved bits - Read Only Value After Reset: 0x0 Exists: Always |
| 15:0 | WRAP_INST | R/W | XIP WRAP transfer opcode. When SPI_CTRLR0.XIP_INST_EN bit is set to 1, AHB_QSPI sends instruction for all XIP transfers, this register field stores the instruction op-code to be sent when an WRAP type transfer is requested on AHB bus. The number of bits to be send in instruction phase is determined by SPI_CTRL0.INST_L field. Value After Reset: 0X0 |

- Program Guide

- Programming Considerations

The following functions should be programmed during configuration

■AHB data bus width

■ Depth of receive and transmit FIFO buffers

■ Peripheral ID code

■ Whether the interrupt level is active high or active low

■ Whether interrupts are individual or combined

■ Whether to generate a clock enable input for the ssi_clk

■ Whether or not pclk and ssi_clk are synchronous

■ Whether to hardcode the frame format, and what type of frame format:

❑ Motorola SPI – Requires setting the serial clock polarity and phase

❑ Texas Instruments Synchronous Serial Protocol

❑ National Semiconductor Microwire

■ Whether to use Enhanced SPI mode of operation and advanced features

- Host SPI and SSP serial transfer

The typical software flow to complete SPI or SSP serial transfer from the AHB_QSPI serial host is summarized as follows:

1. If the AHB_QSPI is enabled, disable it by writing 0 to the SSI Enable register (SSIENR).

2. Set up the AHB_QSPI control registers for the transfer; these registers can be set in any order.

❑Write Control Register 0 (CTRLR0). For SPI transfers, the serial clock polarity and serial clock phase parameters must be set identical to target slave device.

❑ If the transfer mode is receive only, write CTRLR1 (Control Register 1) with the number of frames in the transfer minus 1; for example, if you want to receive four data frames, write this register with 3.

❑ Write the Baud Rate Select Register (BAUDR) to set the baud rate for the transfer.

❑ Write the Transmit and Receive FIFO Threshold Level registers (TXFTLR and RXFTLR, respectively) to set FIFO threshold levels.

❑ Write the TXFTLR.TXFTHR (TX FIFO Start Transfer Threshold) accordingly.

❑ Write the IMR register to set up interrupt masks.

❑ The Slave Enable Register (SER) register can be written here to enable the target slave for selection. If a slave is enabled here, the transfer begins as soon as one valid data entry is present in the transmit FIFO. If no slaves are enabled prior to writing to the Data Register (DR), the transfer does not begin until a slave is enabled.

3. Enable the AHB_QSPI by writing 1 to the SSIENR register.

4. Write data for transmission to the target slave into the transmit FIFO (write DR). If no slaves were enabled in the SER register at this point, enable it now to begin the transfer.

5. Poll the BUSY status to wait for completion of the transfer. The BUSY status cannot be polled immediately. If a transmit FIFO empty interrupt request is made, write the transmit FIFO (write DR). If a receive FIFO full interrupt request is made, read the receive FIFO (read DR).

6. The transfer is stopped by the shift control logic when the transmit FIFO is empty. If the transfer mode is receive only (TMOD = 2'b10), the transfer is stopped by the shift control logic when the specified number of frames have been received. When the transfer is done, the BUSY status is reset to 0.

7. If the transfer mode is not transmit only (TMOD != 01), read the receive FIFO until it is empty.

8. Disable the AHB_QSPI by writing 0 to SSIENR.

Note EEPROM read mode is not supported when the AHB_QSPI is configured to be in the SSP mode

Figure 4-1 shows a typical software flow for starting a AHB_QSPI master SPI/SSP serial transfer. The diagram also shows the hardware flow inside the serial-master component.

Figure 4-1 AHB_QSPI Master SPI/SSP Transfer Flow

- Host Microwire serial transmission

The typical software process for completing Microwire serial transmission from the AHB_QSPI serial host is summarized as follows:

- If the AHB_QSPI is enabled, disable it by writing 0 to SSIENR.

- Set up the AHB_QSPI control registers for the transfer. These registers can be set in any order. Write CTRLR0 to set transfer parameters.

❑ If the transfer is sequential and the AHB_QSPI master receives data, write CTRLR1 with the number of frames in the transfer minus 1; for instance, if you want to receive four data frames, write this register with 3.

❑ Write BAUDR to set the baud rate for the transfer.

❑ Write TXFTLR and RXFTLR to set FIFO threshold levels.

❑ Write the IMR register to set up interrupt masks. You can write the SER register to enable the target slave for selection. If a slave is enabled here, the transfer begins as soon as one valid data entry is present in the transmit FIFO. If a slave is enabled here, the transfer begins as soon as one valid data entry is present in the transmit FIFO. no slaves are enabled prior to writing to the DR register, the transfer does not begin until a slave is enabled.

3. Enable the AHB_QSPI by writing 1 to the SSIENR register.

4. If the AHB_QSPI master transmits data, write the control and data words into the transmit FIFO (write DR). If the AHB_QSPI master receives data, write the control word(s) into the transmit FIFO. If no slaves were enabled in the SER register at this point, enable now to begin the transfer.

5. Poll the BUSY status to wait for completion of the transfer. The BUSY status cannot be polled immediately. If a transmit FIFO empty interrupt request is made, write the transmit FIFO (write DR). If a receive FIFO full interrupt request is made , read the receive FIFO (read DR).

6. The transfer is stopped by the shift control logic when the transmit FIFO is empty. If the transfer mode is sequential and the AHB_QSPI master receives data, the transfer is stopped by the shift control logic when the specified number of data frames is received. When the transfer is done, the BUSY status is reset to 0.

7. If the AHB_QSPI master receives data, read the receive FIFO until it is empty.

8. Disable the AHB_QSPI by writing 0 to SSIENR.

Figure 4-2 shows a typical software flow for starting a AHB_QSPI master Microwire serial transfer. The diagram also shows the hardware flow inside the serial-master component.

Figure 4-2 AHB_QSPI Master Microwire Transfer Flow

- XIP transport

The following figure illustrates the transmission process of XIP transmission:

Figure 4-3 eXecute In Place (XIP) Transfer Flow

Chapter 6 LISoC_AHB_FlashCache

1  Function list

The LISoC AHB_FlashCache is an instruction cache designed to reside between the bus interconnect and the QSPI-flash controller. The LISoC AHB_FlashCache has the following characteristics:

• 8KB cache size , 2-way set associative cache ( 4K bytes/way) , Four words per cacheline .

• 256KB address bus size (within the 1MB QSPI flash address space, only the first 256KB area is cacheable) .

• Supports automatic and manual SRAM power up and power down (with simple handshaking).

If valid data is in the powered-down cache because the cache is in a low-power state, the cache contents will not be invalidated on wake up. The software can therefore save energy by avoiding invaliding the cache RAMs on wake up.

• Supports automatic or manual cache invalidate in the enabling sequence.

• Interrupt request generated on SRAM power or manual invalidation errors.

• Optional run-time support for prefetch to improve performance when executing a sequence of code that has not been read before.

• Performance counters that measure cache hits and misses.

2  Register list

Base address：0x4009 0000

| Offset | Name | Access | Reset | Width | Description |
| --- | --- | --- | --- | --- | --- |
| 0x00 | CCR | R/W | 0x40 | 32 | Configuration and Control Register |
| 0x04 | SR | RO | 0x00 | 32 | Status Register |
| 0x08 | IRQMASK | R/W | 0x00 | 32 | Interrupt Request Mask register |
| 0x0c | IRQSTAT | R/W | 0x00 | 32 | Interrupt Request Status register |
| 0x14 | CSHR | R/W | 0x00 | 32 | Cache Statistic Hit Register |
| 0x18 | CSMR | R/W | 0x00 | 32 | Cache Statistic Miss Register |

3  Register detailed definition

CCR

- Name: Configuration and control register

- Size：32bits

- Addr Offset：0x00

- Read/Write access：write/read

- Reset Value: 0x40

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [31:7] | Reserved | RO | 0 | reserved |
| [6] | STATISTIC_EN | R/W | 1 | Enable statistics logic: 0: Disabled. Counters are stalled. 1: Enable. Counters are running. |
| [5] | SET_PREFETCH | R/W | 0 | Cache Prefetch Setting: 0: Disable prefetch. 1: Enable prefetch. |
| [4] | SET_MAN_INV | R/W | 0 | Cache Invalidate Setting: 0: Automatic cache invalidate when cache enabled. 1: Manual cache invalidate mode. |
| [3] | SET_MAN_POW | R/W | 0 | Power Control Setting: 0: Automatic. 1: Manual. |
| [2] | POW_REQ | R/W | 0 | Manual SRAM power request. |
| [1] | INV_REQ | R/W | 0 | Manual invalidate request. Functional only when SET_MAN_INV is set. Automatically cleared when invalidation is finished or power or invalidation error occurs. Cannot be cleared manually. Manual invalidation request must be set only when the cache is disabled, otherwise it causes an invalidation error interrupt. |
| [0] | EN | R/W | 0 | Cache Enable: 0: Disable cache. 1: Enable cache. |

SR

- Name: Status register.

- Size：32bits

- Addr Offset：0x04

- Read/Write access：read only

- Reset Value: 0x0

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [31:5] | Reserved | RO | 0 | reserved |
| [4] | POW_STAT | RO | 0 | SRAM power acknowledges. Real-time registered value of RAMPWRUPACK port. |
| [3] | Reserved | RO | 0 | Reserved |
| [2] | INV_STAT | RO | 0 | Invalidating Status. Indicates if invalidation process is ongoing. |
| [1:0] | CS | RO | 0 | Cache status: 0: Cache disabled. 1: Cache enabling. 2: Cache enabled. 3: Cache disabling. |

IRQMASK

- Name: Interrupt request mask register

- Size：32bits

- Addr Offset：0x08

- Read/Write access：write/read

- Reset Value: 0x0

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [31:2] | Reserved | RO | 0 | reserved |
| [1] | MAN_INV_ERR | RW | 0 | Mask interrupt request on manual invalidation error indication (IRQSTAT.MAN_INV_ERR is set). |
| [0] | POW_ERR | RW | 0 | Mask interrupt request on Power Error indication (IRQSTAT.POW_ERR is set). |

IRQSTAT

- Name: Interrupt Request Status Register

- Size：32bits

- Addr Offset：0x0c

- Read/Write access：write/read

- Reset Value: 0x0

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [31:2] | Reserved | RO | 0 | reserved |
| [1] | MAN_INV_ERR | RW | 0 | Manual invalidation error status. Set when manual invalidation is requested meanwhile the cache is not disabled. Write 1 to clear. |
| [0] | POW_ERR | RW | 0 | SRAM power error. Write 1 to clear. Power acknowledge de-asserted during operation. Manual power request de-asserted while cache is enabled and operating in manual power mode. |

CSHR

- Name: Cache Statistic Hit Register

- Size：32bits

- Addr Offset：0x14

- Read/Write access：write/read

- Reset Value: 0x0

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [31:0] | CSHR | RW | 0 | Counts the number of cache hits during cache look up. Only cacheable read transactions are looked up by the Flach Cache Controller. Writing to the register clears the contents. |

The count value in the registers saturate at 0xFFFF_FFFF. Perform a write access to the register to clear the counter.

CSMR

- Name: Cache Statistic Miss Register

- Size：32bits

- Addr Offset：0x18

- Read/Write access：write/read

- Reset Value: 0x0

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [31:0] | CSMR | RW | 0 | Counts the number of cache misses during cache look up. Only cacheable read transactions are looked up by the Flach Cache Controller. Writing to the register clears the contents. |

The count value in the registers saturate at 0xFFFF_FFFF. Perform a write access to the register to clear the counter.

4  Program Guide

When QSPI-flash XIP is executed, it is recommended to keep the flash cache controller in the enabled and automatically invalidated and automatic power management working states. At this time, you only need to write 1 to bit 0 of the CCR register of the Flash cache controller and keep the other bits of the CCR register at their default values.

When QSPI-flash stores a large amount of user data and is used for data reading and writing, it is recommended that the flash cache controller be in the disable state. At this time, you only need to write 0 to bit 0 of the CCR register.

Flash Cache controller software operation example:

// Enable flash cache

#define CCR		0x40090000

(*(volatile uint32_t *) CCR) |= (uint32_t)0x01;

// Disable flash cache

#define CCR		0x40090000

(*(volatile uint32_t *) CCR) &= ~(uint32_t)0x01;

Chapter 7 LISoC_AHB_BOOTROM

1 Function list

- Supports 8-bit, 16-bit and 32-bit ROM instructions or data read operations

- Supports single-cycle reading and writing of BOOTROM

- Capacity 32KB

2 Register list

none

3 Detailed definition of registers

none

4 Program Guide

The address space mapping of BOOTROM is as follows:

| name | size | start addr | end addr |
| --- | --- | --- | --- |
| BOOT ROM | 64K | 0x0000 0000 | 0x0000 FFFF |

Chapter 8 LISoC_AHB_SramC

1 Function list

- Supports 8-bit, 16-bit and 32-bit SRAM data read and write operations

- Supports single-cycle read and write of SRAM

- Support low-power operation (8-bit/16-bit operation): In a memory composed of multiple SRAMs, the system selects one or more SRAMs according to different addresses, and the unselected SRAM slice is in low-power standby mode

- Support DFT/BIST function

2 Register list

none

3 Detailed definition of registers

none

4 Program Guide

The three SRAMs in this chip each have 128KB of storage space. Each block of SRAM can be accessed from two different memory map intervals. It is convenient for software programmers to use one area as program image storage space and another area as operation data storage space. The address space mapping of each SRAM is as follows:

| name | size | start addr | end addr |
| --- | --- | --- | --- |
| SRAM0 | 128K | 0x1FF0 0000 | 0x1FF1 FFFF |
| SRAM1 | 128K | 0x1FF2 0000 | 0x1FF3 FFFF |
| SRAM2 | 128K | 0x1FF4 0000 | 0x1FF5 FFFF |
| SRAM0 (mirror) | 128K | 0x2000 0000 | 0x2001 FFFF |
| SRAM1 (mirror) | 128K | 0x2002 0000 | 0x2003 FFFF |
| SRAM2 (mirror) | 128K | 0x2004 0000 | 0x2005 FFFF |

Chapter 9 LISoC_APB_Timers

1 Function list

This module provides 3 timers (timers/counters) for the chip to count or generate timed interrupts.

● Three programmable same timers , Timer 0 to Timer2

● Timers counter width: 32 bits

● Support for two operation modes: free-running and user-defined count

● Support Interrupt Mask and Clear

● Support for independent cloc king of timers

● Interrupt output is active -high and 1 bit

2 Register list

Timer0 Base address：0x40000800

Timer1 Base address：0x40000C00

Timer2 Base address：0x40001000

| Offset | Register | R/W | Size | Description |
| --- | --- | --- | --- | --- |
| 0x0 | TimerNLoadCount(for N = 0; N < 3) | R/W | 32 bits | Timer1CountLoadValue Range: 0 to 232-1 Reset value:32’b0 |
| 0x4 | TimerNCurrentValue(for N = 0; N < 3) | R | 32 bits | Timer1CurrentCountValueRegister Range: 0 to 232-1 Reset value: 32'b0 |
| 0x8 | TimerNControlReg(for N = 0; N < 3) | R/W | 32 bits | Timer1Control Register Reset value: 3'b0 |
| 0xc | TimerNEOI(for N = 0; N < 3) | R | 32bit | Timer1ClearsInterruptReagister Reset value: 1'b0 |
| 0x10 | TimersNIntStatus(for N = 0; N < 3) | R | 32 bit | Timer1Interrupt StatusRegister Reset value: 1'b0 |
| 0xa8 | TimerNRawIntStatus(for N = 0; N < 3) | R | 32 bits | TimersRawInterruptStatusRegister Reset value: 1'b0 |
| 0xac | TIMERS_N_COMP_VERSION(for N = 0; N < 3) | R | 32bits | TimersCompentVersionRegister Reset value: 32'b3231322a |

3 Register detailed definition

Note: In the following description, N is 0, 1 or 2, TIMER_WIDTH =32, TIMER_NUM= 1

- Timer N LoadCount (for N = 0; N < 3)

● Name: Timer N Load Count Register

● Size: 32bits

● Address Offset: 0x00

● Read/write access: read/write

● Reset Value: 0x0

| Bits | Name | R/W | Description |
| --- | --- | --- | --- |
| 31:0 | TimerN Load Count Register | R/W | Value to be loaded into TimerN. This is the value from which counting commences. Any value written to this register is loaded into the associated timer. Value After Reset: 0x0 |

- TimerNCurrentValue(for N = 0; N < 3)

● Name: TimerN Current Value Register

● Size: 32bits

● Address Offset:0x04

● Read/write access: read

● Reset Value: 0x0

| Bits | Name | R/W | Description |
| --- | --- | --- | --- |
| 31:0 | Timer N Current Value | R | Current Value of TimerN. This register is supported only when timer_N_clk is synchronous to pclk. Reading this register when using independent clocks results in an undefined value. Value After Reset: 0x0 |

- TimerNControlReg(for N = 0; N < 3)

● Name: TimerN Control Register

● Size: 3 bits

● Address Offset: 0x08

● Read/write access: read/write

● Reset Value: 0x0

| Bits | Name | R/W | Description |
| --- | --- | --- | --- |
| 31:3 | Reserved, read as zero |  |  |
| 2 | Timer Interrupt Mask | R/W | Timer interrupt mask for TimerN. 0: not masked 1: masked Value After Reset: 0x0 |
| 1 | Timer Mode | R/W | Timer mode for TimerN. 0: free-running mode 1: user-defined count mode NOTE: You must set the TimerNLoadCount register to all 1s before enabling the timer in free-running mode Value After Reset: 0x0 |
| 0 | Timer Enable | R/W | Timer enable bit for TimerN. 0: disable 1: enable Value After Reset: 0x0 |

Note:

a) User-defined count mode

Timer loads the current value of the TimerNLoadCount register. Use this mode if you want a fixed, timed interrupt. Designate this mode by writing a “1” to bit 1 of TimerNControlReg.

b) Free-running mode:

Timer loads the maximum value, which is dependent on the timer width; that is, the TimerNLoadCount register is comprised of 2TIMER_WIRDTH_N-1,all of which are loaded with 1s. The timer counter wrapping to its maximum value allows time to reprogram or disable the timer before another interrupt occurs. Use this mode if you want a single timed interrupt. Designate this mode by writing a “0” to bit 1 of TimerNControlReg.

- TimerNEOI(for N = 0; N < 3)

● Name: TimerN End-of-Interrupt Register

● Size: 32 bit

● Address Offset: 0x0C

● Read/write access: read

● Reset Value: 0x0

| Bits | Name | R/W | Description |
| --- | --- | --- | --- |
| 31:1 | Reserved, read as zero |  |  |
| 0 | TimerN End of- Interrupt Register | R | Reading from this register returns all zeroes (0) and clears the interrupt from TimerN. Value After Reset: 0x0 |

- TimerNIntStatus(for N = 0; N < 3)

● Name: TimerN Interrupt Status Register

● Size: 32 bit

● Address Offset: 0x10

● Read/write access: read

● Reset Value: 0x0

| Bits | Name | R/W | Description |
| --- | --- | --- | --- |
| 31:1 | Reserved, read as zero |  |  |
| 0 | TimerN Interrupt Status Register | R | Contains the interrupt status for TimerN Value After Reset: 0x0 |

- TimeNRawIntStatus(for N = 0; N < 3)

● Name: Timers Raw Interrupt Status Register

● Size: 32 bits

● Address Offset: 0xa8

● Read/write access: read

● Reset Value: 0x0

| Bits | Name | R/W | Description |
| --- | --- | --- | --- |
| 0 | Timers Raw Interrupt Status Register | R | The register contains the unmasked interrupt status of all timers in the component. 0 = timer_intr is not active prior to masking 1 = timer_intr is active prior to masking Value After Reset: 0x0 |

- TIMERS_N_COMP_VERSION(for N = 0; N < 3)

● Name: Timers Component Version

● Size: 32 bits

● Address Offset: 0xac

● Read/write access: read

● Reset Value: 0x3231 322a

| Bits | Name | R/W | Description |
| --- | --- | --- | --- |
| 31:0 | Timers Component Version | R | Current revision number of the DW_apb_timers component Reset Value: 0x3231_322a |

4 Program Guide

1. You can use timers according to the following steps, divided into three steps:

(1)Initialize the Timer N ControlReg control register

a) Turn off the timer: write 0 in bit0

b) Select the counting mode: write 0 or 1 in bit1, indicating free counting and 				user-defined counting modes respectively.

c) Select interrupt mask enable: write 0 or 1 in bit2, which corresponds to interrupt 	non-mask or interrupt mask mode respectively.

(2) Write the count value (32bits count width) into the Timer N LoadReg register

(3) Turn on the timer: write 1 to bit0

In short, before enabling the timer, you should first ensure that the timer is Disable and 		load the initial count value.

(Note: After the system is reset, the default value of the control register is 0x0, that is, 			the timer is off)

2. Interrupt query and clearing:

(1) Query the status of the interrupt by reading the register Timer N IntStatus or Timer N RawIntStatus

(2) If an interrupt is found, it can be cleared by reading the register Timer N EOI.

3. Note:

(1) After querying the interrupt source, if it is not reconfigured, the counter will continue to count according to the original configuration until the count is completed and an interrupt is issued again. If reconfiguration is required, the timer should be turned off (disable) after clearing the interrupt, then configured, and finally turned on (enabled).

(2) Before closing (diable) the timer, if there are still interrupts, it is best to clear them first and then close the timer.

(3) The counter does not have a pause counting function, only Enable and Diable. After closing, the current count value is 0x0.

(4) The maximum counting bit width of Timer0/ Timer1/Timer2 is 32/32bits respectively . The counting time also depends on the frequency of timer_N_clk . Timing interrupt time = count value */timer_N_clk. For example, at 20MHZ, counting for 10ms, the count value can be calculated as: 0x30d40.

(5) In the free counting mode, every time after the counting channel is enabled, the counter first loads the count value (TimerNLoadCount), and then starts counting down, and an interrupt is generated after counting to 0; it starts from this moment Load the maximum value 0xffff_ffff (32 bits) to start the next counting cycle. Afterwards, the maximum value is continuously loaded and counted until the counting channel is closed (Disable).

That is to say, when TimerNLoadCount is not written ( default is 0x0 after reset), an interrupt will be generated after being enabled for the first time.

Therefore, in free counting mode, it is necessary to write TimerNLoadCount=0xffff_ffff.

(6) The Timers interrupt signal is timer_intr (independent interrupt, Active-High), with a 			width of 1 bits.

Chapter 10 LISoC_APB_WDT

1Function list

This module is used to provide a system reset signal to the system to prevent system deadlock.

● Counter counts down from a preset value to 0 to indicate the occurrence of a timeout.

● If a timeout occurs the WDT can perform one of the following operations:

❍ Generate a system reset

❍ First generate an interrupt and if this is not cleared by the service routine by the time a second timeout occurs then generate a system reset

● Support p rogrammable timeout range (period)

● Support the restart counter(feeddog)

● Once enable wdt, software couldn ' t disable it

● Support for Pause mode with the use of external pause enable signal.

2 Register list

Base address：0x4000 3000

| Offset | Name | R/W | Width | Description |
| --- | --- | --- | --- | --- |
| 0x0 | WDT_CR | R/W | 32bits | Control register Reset Value:0x13 |
| 0x4 | WDT_TORR | R/W | 32bits | Timeout range register Reset Value: 0x0 |
| 0x8 | WDT_CCVR | R | 32bits | Current counter value register Reset Value: 0xffff |
| 0xc | WDT_CRR | W | 32bits | Counter restart register Reset Value: 0x0 |
| 0x10 | WDT_STAT | R | 32bits | Interrupt status register Reset Value: 0x0 |
| 0x14 | WDT_EOI | R | 32bits | Interrupt clear register Reset Value: 0x0 |
| 0xf8 | WDT_COMP_VERSION | R | 32bits | Component Version register Reset Value:0x3131_312a |

3 Register detailed definition

Note: In the description below, WDTT_CNT_WIDTH is 32

- WDT_CR

● Name: Control Register

● Size: 32 bits

● Address Offset: 0x00

● Read/write access: R/W

● Reset Value: 0x13

| Bits | Name | R/W | Description |
| --- | --- | --- | --- |
| 31:6 | Reserved and read as zero (0). |  |  |
| 5 | <no name> | R/W | Redundant R/W bit. Included for ping test purposes, as it is the only R/W register bit that is in every configuration of the DW_apb_wdt |
| 4:2 | RPL | R | Reset pulse length. This is used to select the number of pclk cycles for which the system reset stays asserted. The range of values available is 2 to 256 pclk cycles. Note: The total reset pulse length also includes the reset synchronization delay and the time taken for pclk to be made available. Values: ■0x0 (PCLK_CYCLES_2): 2 pclk cycles ■0x1 (PCLK_CYCLES_4): 4 pclk cycles ■0x2 (PCLK_CYCLES_8): 8 pclk cycles ■0x3 (PCLK_CYCLES_16): 16 pclk cycles ■0x4 (PCLK_CYCLES_32): 32 pclk cycles ■0x5 (PCLK_CYCLES_64): 64 pclk cycles ■0x6 (PCLK_CYCLES_128): 128 pclk cycles ■0x7 (PCLK_CYCLES_256): 256 pclk cycles Value After Reset: 0x4 |
| 1 | RMOD | R/W | Response mode. Selects the output response generated to a timeout. Values: ■0x0 (RESET): Generate a system reset ■0x1 (INTERRUPT): First generate an interrupt and even if it is cleared by the time a second timeout occurs then generate a system reset Value After Reset: 0x1 |
| 0 | WDT_EN | R | WDT enable. This bit is used to enable and disable the apb_wdt. When disabled, the counter does not decrement. Thus, no interrupts or system resets are generated. The apb_wdt is used to prevent system lock-up. To prevent a software bug from disabling the apb_wdt, once this bit has been enabled, it can be cleared only by a system reset. Values: ■0x0 (DISABLED): Watchdog timer disabled ■0x1 (ENABLED): Watchdog timer enabled Value After Reset: 0x1 |

- WDT_TORR

● Name: Timeout RangeRegister

● Size: 32 bits

● Address Offset: 0x04

● Read/write access: read

● Reset Value: 0x00

| Bits | Name | R/W | Description |
| --- | --- | --- | --- |
| 31:8 | Reserved and read as zero (0) |  |  |
| 7:4 | TOP_INIT | R | Timeout period for initialization. Used to select the timeout period that the watchdog counter restarts from for the first counter restart (kick). This register should be written after reset and before the WDT is enabled. A change of the TOP_INIT is seen only once the WDT has been enabled, and any change after the first kick is not seen as subsequent kicks use the period SPECIFIED by the TOP bits. Reset Value: 0x0 |
| 3:0 | TOP | R/W | Timeout period. This field is used to select the timeout period from which the watchdog counter restarts. A change of the timeout period takes effect only after the next counter restart (kick). Reset Value: 0x 0 |

Note: WDT_TORR, only bit[3:0] exists (bit[7:4] does not exist).

The count values corresponding to TOP[3:0] are as described in the following table:

| WDT_TORR | 对应的计数值 |
| --- | --- |
| TOP[3:0] | Main Timeout |
| 0x0 | 0xffff |
| 0x1 | 0x1_ffff |
| 0x2 | 0x3_ffff |
| 0x3 | 0x7_ffff |
| 0x4 | 0xf_ffff |
| 0x5 | 0x1f_ffff |
| 0x6 | 0x3f_ffff |
| 0x7 | 0x7f_ffff |
| 0x8 | 0xff_ffff |
| 0x9 | 0x1ff_ffff |
| 0xa | 0x3ff_ffff |
| 0xb | 0x7ff_ffff |
| 0xc | 0xfff_ffff |
| 0xd | 0x1fff_ffff |
| 0xe | 0x3fff_ffff |
| 0xf | 0x7fff_ffff |

- WDT_CCVR

● Name: Current Counter Value Register

● Size: 32 bits

● Address Offset: 0x08

● Read/write access: read

● Reset Value: 0xffff

| Bits | Name | R/W | Description |
| --- | --- | --- | --- |
| 31:0 | Current Counter Value Register | R | This register, when read, is the current value of the internal counter Reset Value: 0xffff |

- WDT_CRR

● Name: Counter Restart Register

● Size: 32 bits

● Address Offset: 0x0c

● Read/write access: write

● Reset Value: 0x00

| Bits | Name | R/W | Description |
| --- | --- | --- | --- |
| 31:8 | Reserved and read as zero |  |  |
| 7:0 | Counter Restart Register | W | This register is used to restart the WDT counter. As a safety feature to prevent accidental restarts, the value 0x76 must be written. A restart also clears the WDT interrupt. Reading this register returns zero Reset Value: 0x0 |

- WDT_STAT

● Name: Interrupt Status Register

● Size: 32 bits

● Address Offset: 0x10

● Read/write access: read

● Reset Value: 0x0

| Bits | Name | R/W | Description |
| --- | --- | --- | --- |
| 31:1 | Reserved and read as zero |  |  |
| 0 | Interrupt Status Register | R | This register shows the interrupt status of the WDT. 1 = Interrupt is active regardless of polarity. 0 = Interrupt is inactive. Reset Value: 0x0 |

- WDT_EOI

● Name: Interrupt Clear Register

● Size: 32 bit

● Address Offset: 0x14

● Read/write access: read

● Reset Value: 0x0

| Bits | Name | R/W | Description |
| --- | --- | --- | --- |
| 31:1 | Reserved and read as zero |  |  |
| 0 | Interrupt Clear Register | R | Clears the watchdog interrupt. This can be used to clear the interrupt without restarting the watchdog counter Reset Value: 0x0 |

- WDT_COMP_VERSION

● Name: Component Version Register

● Size: 32 bits

● Address Offset: 0xf8

● Read/write access: read

● Reset Value: 0x3131 312a

| Bits | Name | R/W | Description |
| --- | --- | --- | --- |
| 31:0 | WDT Component Version | R | V ersion N umber Reset Value: 0x3131_312a |

4 Program Guide

1 WatchDog Timer can be used as shown in Figure 3 .

Note:

1. Selected required timeout period (WDT_TORR)

2. Set reset pulse length, response mode, and enable WDT (WDT_CR)

3. Write 0x76 to WDT_CRR

4. Started back to selected timeout period

5. Can clear by reading WDT_EOI or restarting(kicking) the counter by write 0x76 to WDT_CRR

Figure 3 WDT o peration f low

2 The specific steps are as follows:

- Write counter load value

Write register WDT_TORR (0x0~0xf, corresponding to 15 count values)

- Load the count value

Write the register WDT_CRR=0x76 (that is, restart the WDT counter (or feed the dog) and load the value 	in (1) into the counter)

(3)Configuration control register

Bit [1] of WDT_CR to set the output response mode (system reset only - 0 or interrupt + system reset - 1),

For example, if the count value 0xffff is selected, only system reset is selected, and the system reset 	pulse length is 32 pclk, the driver is as follows:

WDT_TORR = 0x0;

WDT_CRR = 0x76;

WDT_CR = 0x11 ;

3Related instructions (must read)

(1) After writing 1 to bit[0] to enable, it cannot be disabled by writing 0.

(2) Each time the value is written to WDT_TORR, it will be loaded into the counter immediately for use 	only after Restarting the counter (i.e. WDT_CRR=0x76) (regardless of whether Enbale WDT is enabled or 	not, the new count value can take effect as long as the Counter is restarted)

(3) After the WDT itself is reset, the count value is 0xffff; the output mode is interrupt + reset; the reset 	pulse width length is 32pclk.

(4) If the output mode is interrupt + system reset. The system reset signal can be generated even if the 	interrupt signal has not been cleared.

(5) After Enbale WDT, writing the WDT_CR (control register) RPL bit can still take effect. That is, 	WDT_CR is not affected by EnableWDT.

(6) The interrupt signal wdt_intr is Active-high; the system reset signal wdt_sys_rst_n is Active-low.

(7) WDT counting will be paused when the WDT_pause pin is high .

Chapter 11 LISoC_APB_SSI Master

1 Function list

The APB_SSI master is a full-duplex master serial interface. The host processor accesses data, control, and status information on the APB_SSI master through the APB interface. The APB_SSI master can connect to any serial-slave peripheral device using one of the following interfaces:

- Motorola Serial Peripheral Interface (SPI)

- TexasInstruments Serial Protocol (SSP)

- National Semiconductor Microwire

The feature list is as follows:

- Enable serial communication with serial-slave peripheral devices. Each is to be selected using hardware.

- Support independent masking of interrupts . Polarity for each individual interrupt is active-high.

- Receive and transmit FIFO buffers d epth is 8.

- Programmable features:

  - Clock bit-rate – Dynamic control of the serial bit rate of the data transfer .

  - Provide three kinds of transfer mode : Transmit and Receive , Transmit Only and Receive Only .

  - Serial interface operation – Choice of Motorola SPI, Texas Instruments Synchronous Serial Protocol or National Semiconductor Microwire.

  - Data Item size (4 to 16 bits) – Item size of each data transfer under the control of the programmer.

- Support AMBA 2.0 .

- Supports 32 bits APB data bus widths.

2Register List

Base address: 0x 4000 2400

| Offset | Register name | reset value | mode | Bit width (bits) | describe |
| --- | --- | --- | --- | --- | --- |
| 0x00 | CTRLR0 | 0x1000007 | R/W | 32 | Control Register 0 : This register controls serial data transfer. This register cannot be written after APB_SSI is enabled. APB_SSI is enabled by writing to the SSIENR register. |
| 0x04 | CTRLR1 | 0x0 | R/W | 32 | Control Register 1 : Control Register 1 is used to control the end of serial transmission in receive-only mode. This register cannot be written after APB_SSI is enabled. APB_SSI is enabled by writing to the SSIENR register. |
| 0x08 | SSIENR | 0x0 | R/W | 32 | SSI Enable Register |
| 0x0c | MWCR | 0x0 | R/W | 32 | Microwire Control Register : This register controls the direction of the data word for the half-duplex Microwire serial protocol. This register is not writable after APB_SSI is enabled, and APB_SSI is enabled by writing to the SSIENR register. |
| 0x10 | SER | 0x0 | R/W | 32 | Slave Enable Register : The register enables the individual slave select output lines from the APB_SSI master. 2 slave-select output signals are available on the APB_SSI master. You cannot write to this register when APB_SSI is busy. |
| 0x14 | BAUDR | 0x0 | R/W | 32 | Baud Rate Select : The register derives the frequency of the serial clock that regulates the data transfer. The 16-bit field in this register defines the ssi_clk divider value. This register is not writable after APB_SSI is enabled, and APB_SSI is enabled by writing to the SSIENR register. |
| 0x18 | TXFTLR | 0x0 | R/W | 32 | Transmit FIFO Threshold Level : This register controls the threshold value for the transmit FIFO memory. This register is not writable after APB_SSI is enabled, and APB_SSI is enabled by writing to the SSIENR register. |
| 0x1c | RXFTLR | 0x0 | R/W | 32 | Receive FIFO Threshold Level : This register controls the threshold value for the receive FIFO memory. This register is not writable after APB_SSI is enabled, and APB_SSI is enabled by writing the SSIENR register. |
| 0x20 | TXFLR | 0x0 | R | 32 | Transmit FIFO Level Register |
| 0x24 | RXFLR | 0x0 | R | 32 | Receive FIFO Level Register |
| 0x28 | SR | 0x6 | R | 32 | Status Register : This is a read-only register used to indicate the current transfer status, FIFO status, and any transmission/reception errors that may have occurred. The status register may be read at any time. None of the bits in this register request an interrupt. |
| 0x2c | IMR | 0x3f | R/W | 32 | Interrupt Mask Register |
| 0x30 | ISR | 0x0 | R | 32 | Interrupt Register Status |
| 0x34 | RISR | 0x0 | R | 32 | Raw Interrupt Status Register |
| 0x38 | TXOICR | 0x0 | R | 32 | Transmit FIFO Overflow Interrupt Clear Register |
| 0x3c | RXOICR | 0x0 | R | 32 | Receive FIFO Overflow Interrupt Clear Register |
| 0x40 | RXUICR | 0x0 | R | 32 | Receive FIFO Underflow Interrupt Clear Register |
| 0x44 | MSTICR | 0x0 | R | 32 | Multi-Master Interrupt Clear Register |
| 0x48 | ICR | 0x0 | R | 32 | Interrupt Clear Register |
| 0x58 | IDR | 0xffffffff | R | 32 | Identification Register |
| 0x5c | SSI_VERSION_ID | 0x3430322a | R/W | 32 | coreKit Version ID Register |
| 0x60 ~ 0x7c | DR0 ~ DR7 | 0x0 | R/W | 32 | Data Register 0~ 7 : The APB_SSI data register is a 16-bit read/write buffer for the transmit/receive FIFOs. When the register is read, data in the receive FIFO buffer is accessed. When it is written to, data are moved into the transmit FIFO buffer; a write can occur only when SSI_EN = 1. FIFOs are reset when SSI_EN = 0. |

3 Detailed definition of registers

- CTRLR0

● Name: Control Register 0

● Description: This register controls the serial data transfer. It is impossible to write to this register when the APB_SSI is enabled. The APB_SSI is enabled and disabled by writing to the SSIENR register.

● Size: 32 bits

● Address Offset: 0x0

● Read/write access: read/write

● Reset Value: 0x0100 0007

| 31:25 | twenty four | 23:16 | 15:12 | 11 | 10 | 9:8 | 7 | 6 | 5:4 | 3:0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| RSVD_CTRLR0 | SSTE | RSVD_CTRLR0_16_23 | CFS | SRL | RSVD_CTRLR0_10 | TMOD | SCPOL | SCPH | FRF | DFS |

Table 3-1Fields for Register: CTRLR0

| Bits | Name | Reset Value | Memory Access | Description |
| --- | --- | --- | --- | --- |
| 24 | SSTE | 0x1 | R/W | Slave Select Toggle Enable. When operating in SPI mode with clock phase (SCPH) set to 0, this register controls the behavior of the slave select line (ss_*_n) between data frames. If this register field is set to 1 the ss_*_n line will toggle between consecutive data frames, with the serial clock (sclk) being held to its default value while ss_*_n is high; if this register field is set to 0 the ss_*_n will stay low and sclk will run continuously for the duration of the transfer . |
| 15:12 | CFS | 0x0 | R/W | Control Frame Size . Selects the length of the control word for the Microwire frame format. Values: ■0x0 (SIZE_01_BIT): 1-bit Control Word ■0x1 (SIZE_02_BIT): 2-bit Control Word ■0x2 (SIZE_03_BIT): 3-bit Control Word ■0x3 (SIZE_04_BIT): 4-bit Control Word ■0x4 (SIZE_05_BIT): 5-bit Control Word ■0x5 (SIZE_06_BIT): 6-bit Control Word ■0x6 (SIZE_07_BIT): 7-bit Control Word ■0x7 (SIZE_08_BIT): 8-bit Control Word ■0x8 (SIZE_09_BIT): 9-bit Control Word ■0x9 (SIZE_10_BIT): 10-bit Control Word ■0xa (SIZE_11_BIT): 11-bit Control Word ■0xb (SIZE_12_BIT): 12-bit Control Word ■0xc (SIZE_13_BIT): 13-bit Control Word ■0xd (SIZE_14_BIT): 14-bit Control Word ■0xe (SIZE_15_BIT): 15-bit Control Word ■0xf (SIZE_16_BIT): 16-bit Control Word |
| 11 | SRL | 0x0 | R/W | Shift Register Loop. Used for testing purposes only. When internally active, connects the transmit shift register output to the receive shift register input. Can be used in both serial-slave and serial-master modes. Values: ■ 0 – Normal Mode Operation ■ 1 – Test Mode Operation |
| 9:8 | TMOD | 0x0 | R/W | Transfer Mode . Selects the mode of transfer for serial communication. This field does not affect the transfer duplicity. Only indicates whether the receive or transmit data are valid. In transmit-only mode , data received from the external device is not valid and is not stored in the receive FIFO memory; it is overwritten on the next transfer. In receive-only mode , transmitted data are not valid. After the first write to the transmit FIFO, the same word is retransmitted for the duration of the transfer. In transmit-and-receive mode , both transmit and receive data are valid. The transfer continues until the transmit FIFO is empty. Data received from the external device are stored into the receive FIFO memory, where it can be accessed by the host processor. Values: ■ 00 –- Transmit & Receive ■ 01 –- Transmit Only ■ 10 – Receive Only ■ 11 –- Reserved |
| 7 | SCPOL | 0x0 | R/W | Serial Clock Polarity . Valid when the frame format (FRF) is set to Motorola SPI. Used to select the polarity of the inactive serial clock, which is held inactive when the APB_SSI master is not actively transferring data on the serial bus. Values: ■ 0 – Inactive state of serial clock is low ■ 1 – Inactive state of serial clock is high |
| 6 | SCPH | 0x0 | R/W | Serial Clock Phase . Valid when the frame format (FRF) is set to Motorola SPI. The serial clock phase selects the relationship of the serial clock with the slave select signal. When SCPH = 0, data are captured on the first edge of the serial clock. When SCPH = 1, the serial clock starts toggling one cycle after the slave select line is activated, and data are captured on the second edge of the serial clock. Values: ■ 0: Serial clock toggle in middle of first data bit ■ 1: Serial clock toggle at start of first data bit |
| 5:4 | FRF | 0x00 | R/W | Frame Format . Selects which serial protocol transfers the data. Values: ■ 00 – Motorola SPI ■ 01 –- Texas Instruments SSP ■ 10 –- National Semiconductors Microwire ■ 11 –- Reserved |
| 3:0 | DFS | 0x7 | R/W | Data Frame Size . Selects the data frame length. When the data frame size is programmed to be less than 16 bits, the receive data are automatically right-justified by the receive logic, with the upper bits of the receive FIFO zero-padded. You must right-justify transmit data before writing into the transmit FIFO. The transmit logic ignores the upper unused bits when transmitting the data. Values: ■0x3 (FRAME_04BITS): 4-bit serial data transfer ■0x4 (FRAME_05BITS): 5-bit serial data transfer ■0x5 (FRAME_06BITS): 6-bit serial data transfer ■0x6 (FRAME_07BITS): 7-bit serial data transfer ■0x7 (FRAME_08BITS): 8-bit serial data transfer ■0x8 (FRAME_09BITS): 9-bit serial data transfer ■0x9 (FRAME_10BITS): 10-bit serial data transfer ■0xa (FRAME_11BITS): 11-bit serial data transfer ■0xb (FRAME_12BITS): 12-bit serial data transfer ■0xc (FRAME_13BITS): 13-bit serial data transfer ■0xd (FRAME_14BITS): 14-bit serial data transfer ■0xe (FRAME_15BITS): 15-bit serial data transfer ■0xf (FRAME_16BITS): 16-bit serial data transfer |

- CTRLR1

● Name: Control Register 1

● Description: Control register 1 controls the end of serial transfers when in receive-only mode. It is impossible to write to this register when the APB_SSI is enabled. The APB_SSI is enabled and disabled by writing to the SSIENR register..

● Size e: 32 b its

● Address Offset: 0x04

● Read/write access: read/write

● Reset Value: 0x0

| 31:16 | 15:0 |
| --- | --- |
| Reserved | NDF |

Table 3-2Fields for Register: CTRLR1

| Bits | Name | Reset Value | Memory Access | Description |
| --- | --- | --- | --- | --- |
| 15:0 | NDF | 0x0 | R/W | Number of Data Frames . When TMOD = 10 , this register field sets the number of data frames to be continuously received by the APB_SSI. The APB_SSI continues to receive serial data until the number of data frames received is equal to this register value plus 1, which enables you to receive up to 64 KB of data in a continuous transfer. |

- SSIENR

● Name: SSI Enable Register

● Description: This register enables and disables the APB_SSI.

● Size: 32 bits

● Address Offset: 0x08

● Read/write access: read/write

● Reset Value: 0x0

| 31:1 | 0 |
| --- | --- |
| Reserved | SSI_EN |

Table 3-3Fields for Register: SSIENR

| Bits | Name | Reset Value | Memory Access | Description |
| --- | --- | --- | --- | --- |
| 0 | SSI_EN | 0x0 | R/W | SSI Enable . Enables and disables all APB_SSI operations. When disabled, all serial transfers are halted immediately. Transmit and receive FIFO buffers are cleared when the device is disabled. It is impossible to program some of the APB_SSI control registers when enabled. When disabled, the ssi_sleep output is set (after delay) to inform the system that it is safe to remove the ssi_clk, thus saving power consumption in the system. Values: ■0x0 (DISABLE): Disables Serial Transfer ■0x1 (ENABLED): Enables Serial Transfer |

- MWCR

● Name: Microwire Control Register

● Description: This register controls the direction of the data word for the half-duplex Microwire serial protocol. It is impossible to write to this register when the APB_SSI is enabled. The APB_SSI is enabled and disabled by writing to the SSIENR register.

● Size : 32 bits

● Address Offset: 0x0C

● Read/write access: read/write

● Reset Value: 0x0

| 31:3 | 2 | 1 | 0 |
| --- | --- | --- | --- |
| Reserved | MHS | MDD | MWMOD |

Table 3-4Fields for Register: MWCR

| Bits | Name | Reset Value | Memory Access | Description |
| --- | --- | --- | --- | --- |
| 2 | MHS | 0x0 | R/W | Microwire Handshaking . Used to enable and disable the “busy/ready” handshaking interface for the Microwire protocol. When enabled, the APB_SSI checks for a ready status from the target slave, after the transfer of the last data/control bit, before clearing the BUSY status in the SR register. Values: ■ 0: handshaking interface is disabled ■ 1: handshaking interface is enabled |
| 1 | MDD | 0x0 | R/W | Microwire Control . Defines the direction of the data word when the Microwire serial protocol is used. When this bit is set to 0, the data word is received by the APB_SSI MacroCell from the external serial device. When this bit is set to 1, the data word is transmitted from the APB_SSI MacroCell to the external serial device. Values: ■0x0 (RECEIVE): SSI receives data ■0x1 (TRANSMIT): SSI transmits data |
| 0 | MWMOD | 0x0 | R/W | Microwire Transfer Mode . Defines whether the Microwire transfer is sequential or non-sequential. When sequential mode is used, only one control word is needed to transmit or receive a block of data words. When non-sequential mode is used, there must be a control word for each data word that is transmitted or received. Values: ■ 0 – non-sequential transfer ■ 1 – sequential transfer |

- SER

● Name: Slave Enable Register

● Description: The register enables the individual slave select output lines from the APB_SSI master. Only one slave-select output pins are available on the APB_SSI master. Register bits can be set or cleared when SSI_EN=0.

If SSI_EN=1, then register bits can be set (to delay the slave select assertion while TX FIFO is getting filled) but cannot be cleared.

● Size: 32bits

● Address Offset: 0x10

● Read/write access: read/write

● Reset Value: 0x0

| 31:1 | 0 |
| --- | --- |
| Reserved | SER |

Table 3-5Fields for Register: SER

| Bits | Name | Reset Value | Memory Access | Description |
| --- | --- | --- | --- | --- |
| 0 | SER | 0x0 | R/W | Slave Select Enable Flag . Each bit in this register corresponds to a slave select line (ss_ xn) from the APB_SSI master. When a bit in this register is set (1), the corresponding slave select line from the master is activated when a serial transfer begins. It should be noted that setting or clearing bits in this register have no effect on the corresponding slave select outputs until a transfer is started. Before beginning a transfer, you should enable the bit in this register that corresponds to the slave device with which the master wants to communicate. When not operating in broadcast mode, only one bit in this field should be set. Values: ■ 1: Selected ■ 0: Not Selected |

- BAUDR

● Name: Baud Rate Select

● Description: The register derives the frequency of the serial clock that regulates the data transfer. The 16-bit field in this register defines the ssi_clk divider value. It is impossible to write to this register when the APB_SSI is enabled. The APB_SSI is enabled and disabled by writing to the SSIENR register.

● Size: 32 b its

● Address Offset: 0x14

● Read/write access: read/write

● Reset Value: 0x0

| 31:16 | 15:0 |
| --- | --- |
| Reserved | SKDV |

Table 3-6Fields for Register: BAUDR

| Bits | Name | Reset Value | Memory Access | Description |
| --- | --- | --- | --- | --- |
| 15:0 | SKDV | 0x0 | R/W | SSI Clock Divider . The LSB for this field is always set to 0 and is unaffected by a write operation, which ensures an even value is held in this register. If the value is 0, the serial output clock (sclk_out) is disabled. The frequency of the sclk_out is derived from the following equation: Fsclk_out = Fssi_clk / SCKDV where SCKDV is any even value between 2 and 65534. |

- TXFTLR

● Name: Transmit FIFO Threshold Level

● Description: This register controls the threshold value for the transmit FIFO memory. The APB_SSI is enabled and disabled by writing to the SSIENR register..

● Size : 32bits

● Address Offset: 0x18

● Read/write access: read/write

● Reset Value: 0x0

| 31:3 | 2:0 |
| --- | --- |
| Reserved | TFT |

Table 3-7Fields for Register: TXFTLR

| Bits | Name | Reset Value | Memory Access | Description |
| --- | --- | --- | --- | --- |
| 2:0 | TFT | 0x0 | R/W | Transmit FIFO Threshold . Controls the level of entries (or below) at which the transmit FIFO controller triggers an interrupt. The FIFO depth is 16 ; If you attempt to set this value greater than or equal to the depth of the FIFO, this field is not written and retains its current value. When the number of transmit FIFO entries is less than or equal to this value, the transmit FIFO empty interrupt is triggered. |

- RXFTLR

● Name: Receive FIFO Threshold Level

● Description: This register controls the threshold value for the receive FIFO memory. The APB_SSI is enabled and disabled by writing to the SSIENR register.

● Size: 32 bits

● Address Offset: 0x1C

● Read/write access: read/write

● Reset Value: 0x0

| 31: 3 | 2:0 |
| --- | --- |
| Reserved | RFT |

Table 3-8Fields for Register: RXFTLR

| Bits | Name | Reset Value | Memory Access | Description |
| --- | --- | --- | --- | --- |
| 2 : 0 | RFT | 0x0 | R/W | Receive FIFO Threshold . Controls the level of entries (or above) at which the receive FIFO controller triggers an interrupt. The FIFO depth is 16. If you attempt to set this value greater than the depth of the FIFO, this field is not written and retains its current value. When the number of receive FIFO entries is greater than or equal to this value + 1, the receive FIFO full interrupt is triggered. |

- TxFLR

● Name: Transmit FIFO Level Register

● Description: This register contains the number of valid data entries in the transmit FIFO memory.

● Size: 32 bits

● Address Offset: 0x20

● Read/write access: read-only

● Reset Value: 0x0

| 31: 4 | 3 :0 |
| --- | --- |
| Reserved | TXTFL |

Table 3-9Fields for Register: TXFLR

| Bits | Name | Reset Value | Memory Access | Description |
| --- | --- | --- | --- | --- |
| 3 :0 | TXTFL | 0x0 | R | Transmit FIFO Level . Contains the number of valid data entries in the transmit FIFO. |

- RXFLR

● Name: Receive FIFO Level Register

● Description: This register contains the number of valid data entries in the receive FIFO memory. This register can be read at any time.

● Size: 32bits

● Address Offset: 0x24

● Read/write access: read-only

● Reset Value: 0x0

| 31: 4 | 3 :0 |
| --- | --- |
| Reserved | RXTFL |

Table 3-10Fields for Register: RXFLR

| Bits | Name | Reset Value | Memory Access | Description |
| --- | --- | --- | --- | --- |
| 4:0 | RXTFL | 0x0 | R | Receive FIFO Level . Contains the number of valid data entries in the receive FIFO. |

- SR

● Name: Register Status

● Description: This is a read-only register used to indicate the current transfer status, FIFO status, and any transmission/reception errors that may have occurred. The status register may be read at any time. None of the bits in this register request an interrupt.

● Size: 32 bits

● Address Offset: 0x28

● Read/write access: read-only

● Reset Value: 0x06

| 31:7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Reserved | DCOL | Reserved | RFE | RFNE | TFE | TNF | BUSY |

Table 3-11Fields for Register: SR

| Bits | Name | Reset Value | Memory Access | Description |
| --- | --- | --- | --- | --- |
| 6 | DCOL | 0x0 | R | Data Collision Error . This bit is set if the APB_SSI master is actively transmitting when another master selects this device as a slave. This informs the processor that the last data transfer was halted before completion. This bit is cleared when read. Values: ■ 0 – No error ■ 1 – Transmit data collision error |
| 4 | RFF | 0x0 | R | Receive FIFO Full . When the receive FIFO is completely full, this bit is set. When the receive FIFO contains one or more empty location, this bit is cleared. Values: ■ 0 – Receive FIFO is not full ■ 1 – Receive FIFO is full |
| 3 | RFNE | 0x0 | R | Receive FIFO Not Empty . Set when the receive FIFO contains one or more entries and is cleared when the receive FIFO is empty. This bit can be polled by software to completely empty the receive FIFO. Values: ■ 0 – Receive FIFO is empty ■ 1 – Receive FIFO is not empty |
| 2 | TFE | 0x1 | R | Transmit FIFO Empty . When the transmit FIFO is completely empty, this bit is set. When the transmit FIFO contains one or more valid entries, this bit is cleared. This bit field does not request an interrupt. Values: ■ 0 – Transmit FIFO is not empty ■ 1 – Transmit FIFO is empty |
| 1 | TNF | 0x1 | R | Transmit FIFO Not Full . Set when the transmit FIFO contains one or more empty locations, and is cleared when the FIFO is full. Values: ■ 0 – Transmit FIFO is full ■ 1 – Transmit FIFO is not full |
| 0 | BUSY | 0x0 | R | SSI Busy Flag . When set, indicates that a serial transfer is in progress; when cleared indicates that the APB_SSI is idle or disabled. Values: ■ 0 – APB_SSI is idle or disabled ■ 1 – APB_SSI is actively transferring data |

- IMR

● Name: Interrupt Mask Register

● Description: This read/write reigster masks or enables all interrupts generated by the APB_SSI.

● Size: 32 bits

● Address Offset: 0x2C

● Read/write access: read/write

● Reset Value: 0x3F

| 31:6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- |
| Reserved | MSTIM | RXFIM | RXOIM | RXUIM | TXOIM | TXEIM |

Table 3-12Fields for Register:IMR

| Bits | Name | Reset Value | Memory Access | Description |
| --- | --- | --- | --- | --- |
| 5 | MSTIM | 0x1 | R/W | Multi-Master Contention Interrupt Mask. Values: ■ 0 – ssi_mst_intr interrupt is masked ■ 1 – ssi_mst_intr interrupt is not masked |
| 4 | RXFIM | 0x1 | R/W | Receive FIFO Full Interrupt Mask Values: ■ 0 – ssi_rxf_intr interrupt is masked ■ 1 – ssi_rxf_intr interrupt is not masked |
| 3 | RXOIM | 0x1 | R/W | Receive FIFO Overflow Interrupt Mask Values: ■ 0 – ssi_rxo_intr interrupt is masked ■ 1 – ssi_rxo_intr interrupt is not masked |
| 2 | RXUIM | 0x1 | R/W | Receive FIFO Underflow Interrupt Mask Values: ■ 0 – ssi_rxu_intr interrupt is masked ■ 1 – ssi_rxu_intr interrupt is not masked |
| 1 | TXOIM | 0x1 | R/W | Transmit FIFO Overflow Interrupt Mask Values: ■ 0 – ssi_txo_intr interrupt is masked ■ 1 – ssi_txo_intr interrupt is not masked |
| 0 | TXEIM | 0x1 | R/W | Transmit FIFO Empty Interrupt Mask Values: ■ 0 – ssi_txe_intr interrupt is masked ■ 1 – ssi_txe_intr interrupt is not masked |

- ISR

● Name: Interrupt Status Register

● Description: This register reports the status of the APB_SSI interrupts after they have been masked.

● Size : 32bits

● Address Offset: 0x30

● Read/write access: read-only

● Reset Value: 0x0

| 31:6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- |
| Reserved | MSTIS | RXFIS | RXOIS | RXUIS | TxOIS | TxEIS |

Table 3-13Fields for Register:ISR

| Bits | Name | Reset Value | Memory Access | Description |
| --- | --- | --- | --- | --- |
| 5 | MSTIS | 0x0 | R | Multi-Master Contention Interrupt Status . Values: ■ 0 = ssi_mst_intr interrupt not active after masking ■ 1 = ssi_mst_intr interrupt is active after masking |
| 4 | RXFIS | 0x0 | R | Receive FIFO Full Interrupt Status Values: ■ 0 = ssi_rxf_intr interrupt is not active after masking ■ 1 = ssi_rxf_intr interrupt is full after masking |
| 3 | RXOIS | 0x0 | R | Receive FIFO Overflow Interrupt Status Values: ■ 0 = ssi_rxo_intr interrupt is not active after masking ■ 1 = ssi_rxo_intr interrupt is active after masking |
| 2 | RXUIS | 0x0 | R | Receive FIFO Underflow Interrupt Status Values: ■ 0 = ssi_rxu_intr interrupt is not active after masking ■ 1 = ssi_rxu_intr interrupt is active after masking |
| 1 | TxOIS | 0x0 | R | Transmit FIFO Overflow Interrupt Status Values: ■ 0 = ssi_txo_intr interrupt is not active after masking ■ 1 = ssi_txo_intr interrupt is active after masking |
| 0 | TxEIS | 0x0 | R | Transmit FIFO Empty Interrupt Status Values: ■ 0 = ssi_txe_intr interrupt is not active after masking ■ 1 = ssi_txe_intr interrupt is active after masking |

- RISR

● Name: Raw Interrupt Status Register

● Description: This read-only register reports the status of the APB_SSI interrupts prior to masking.

● Size: 32bits

● Address Offset: 0x34

● Read/write access: read-only

● Reset Value: 0x0

| 31:6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- |
| Reserved | MSTIR | RXFIR | RXOIR | RXUIR | TxOIR | TXEIR |

Table 3 -14Fields for Register: RISR

| Bits | Name | Reset Value | Memory Access | Description |
| --- | --- | --- | --- | --- |
| 5 | MSTIR | 0x0 | R | Multi-Master Contention Raw Interrupt Status. Values: ■ 0 = ssi_mst_intr interrupt is not active prior to masking ■ 1 = ssi_mst_intr interrupt is active prior masking |
| 4 | RXFIR | 0x0 | R | Receive FIFO Full Raw Interrupt Status Values: ■ 0 = ssi_rxf_intr interrupt is not active prior to masking Values: ■ 1 = ssi_rxf_intr interrupt is active prior to masking |
| 3 | RXOIR | 0x0 | R | Receive FIFO Overflow Raw Interrupt Status Values: ■ 0 = ssi_rxo_intr interrupt is not active prior to masking ■ 1 = ssi_rxo_intr interrupt is active prior masking |
| 2 | RXUIR | 0x0 | R | Receive FIFO Underflow Raw Interrupt Status Values: ■ 0 = ssi_rxu_intr interrupt is not active prior to masking ■ 1 = ssi_rxu_intr interrupt is active prior to masking |
| 1 | TxOIR | 0x0 | R | Transmit FIFO Overflow Raw Interrupt Status Values: ■ 0 = ssi_txo_intr interrupt is not active prior to masking ■ 1 = ssi_txo_intr interrupt is active prior masking |
| 0 | TXEIR | 0x0 | R | Transmit FIFO Empty Raw Interrupt Status Values: ■ 0 = ssi_txe_intr interrupt is not active prior to masking ■ 1 = ssi_txe_intr interrupt is active prior masking |

- TxOICR

● Name: Transmit FIFO Overflow Interrupt Clear Register

● Description: Transmit FIFO Overflow Interrupt Clear Register.

● Size: 32 bits

● Address Offset: 0x38

● Read/write access: read-only

● Reset Value: 0x0

| 31:1 | 0 |
| --- | --- |
| Reserved | TxOICR |

Table 3-15Fields for Register: TXOICR

| Bits | Name | Reset Value | Memory Access | Description |
| --- | --- | --- | --- | --- |
| 0 | TxOICR | 0x0 | R | Clear Transmit FIFO Overflow Interrupt . This register reflects the status of the interrupt. A read from this register clears the ssi_txo_intr interrupt; writing has no effect. |

- RXOICR

● Name: Receive FIFO Overflow Interrupt Clear Register

● Description: Receive FIFO Overflow Interrupt Clear Register.

● Size: 32 bits

● Address Offset: 0x3C

● Read/write access: read-only

● Reset Value: 0x0

| 31:1 | 0 |
| --- | --- |
| Reserved | RXOICR |

Table 3-16 Fields for Register: RXOICR

| Bits | Name | Reset Value | Memory Access | Description |
| --- | --- | --- | --- | --- |
| 0 | RXOICR | 0x0 | R | Clear Receive FIFO Overflow Interrupt . This register reflects the status of the interrupt. A read from this register clears the ssi_rxo_intr interrupt; writing has no effect. |

- RXUICR

● Name: Receive FIFO Underflow Interrupt Clear Register

● Description: Receive FIFO Underflow Interrupt Clear Register.

● Size: 32 bits

● Address Offset: 0x40

● Read/write access: read-only

● Reset Value: 0x0

| 31:1 | 0 |
| --- | --- |
| Reserved | RXUICR |

Table 3-17 Fields for Register: RXUICR

| Bits | Name | Reset Value | Memory Access | Description |
| --- | --- | --- | --- | --- |
| 0 | RXUICR | 0x0 | R | Clear Receive FIFO Underflow Interrupt . This register reflects the status of the interrupt. A read from this register clears the ssi_rxu_intr interrupt; writing has no effect. |

- MSTICR

● Name: Multi-Master Interrupt Clear Register

● Description:

● Size: 32 bits

● Address Offset: 0x44

● Read/write access: read-only

● Reset Value: 0x0

| 31:1 | 0 |
| --- | --- |
| Reserved | MSTICR |

Table 3-18 Fields for Register: MSTICR

| Bits | Name | Reset Value | Memory Access | Description |
| --- | --- | --- | --- | --- |
| 0 | MSTICR | 0x0 | R | Clear Multi-Master Contention Interrupt . This register reflects the status of the interrupt. A read from this register clears the ssi_mst_intr interrupt; writing has no effect. |

- ICR

● Name: Interrupt Clear Register

● Description: Multi-Master Interrupt Clear Register.

● Size: 32 bits

● Address Offset: 0x48

● Read/write access: read-only

● Reset Value: 0x0

| 31:1 | 0 |
| --- | --- |
| Reserved | ICR |

Table 3-19 Fields for Register: ICR

| Bits | Name | Reset Value | Memory Access | Description |
| --- | --- | --- | --- | --- |
| 0 | ICR | 0x0 | R | Clear Interrupts . This register is set if any of the interrupts below are active. A read clears the ssi_txo_intr, ssi_rxu_intr, ssi_rxo_intr, and the ssi_mst_intr interrupts. Writing to this register has no effect. |

- IDR

● Name: Identification Register

● Description: This register contains the peripherals identification code, which is written into the register at configuration time using coreConsultant..

● Size: 32 bits

● Address Offset: 0x58

● Read/write access: read-only

● Reset Value: 0xffffffff

31:0

IDCODE

Table 3-19 Fields for Register: IDR

| Bits | Name | Reset Value | Memory Access | Description |
| --- | --- | --- | --- | --- |
| 31: 0 | IDCODE | 0xffffffff | R | The register contains the peripheral's identification code, which is written into the register at configuration time using CoreConsultant |

- SSI_VERSION_ID

● Name: coreKit version ID Registe

● Description: This read-only register stores the specific APB_SSI component version.

● Size: 32 bits

● Address Offset: 0x5c

● Read/write access: read-only

● Reset Value: 0x3430 322a

31:0

SSI_COMP_VERSION

Table 3-19 Fields for Register: IDR

| Bits | Name | Reset Value | Memory Access | Description |
| --- | --- | --- | --- | --- |
| 31: 0 | SSI_COMP_VERSI ON | 0x 3430322a | R | The register contains the peripheral's identification code, which is written into the register at configuration time using CoreConsultant |

- DR x(for x=0; x< 8)

● Name: Data Register 0~15

● Description: The APB_SSI data register is a 16-bit read/write buffer for the transmit/receive FIFOs. When the register is read, data in the receive FIFO buffer is accessed. When it is written to, data are moved into the transmit FIFO buffer; a write can occur only when SSI_EN = 1. FIFOs are reset when SSI_EN = 0.

● Size: 32 bits

● Address Offset: 0x60 ~0x7C

● Read/write access: read/write

● Reset Value: 0x0

| 31:16 | 15:0 |
| --- | --- |
| Reserved | DR |

Table 3-20 Fields for Register: DRx

| Bits | Name | Reset Value | Memory Access | Description |
| --- | --- | --- | --- | --- |
| 15:0 | DR | 0x0 | R/W | Data Register . When writing to this register, you must right-justify the data. Read data are automatically right-justified. Read = Receive FIFO buffer Write = Transmit FIFO buffer |

4 Program Guide

4.1 Detailed description of some functions of SSI host

- Configurable clock frequency

The frequency of the APB_SSI serial input clock (ssi_clk) must be less than or equal to the frequency of pclk, which ensures that the control signal of the ssi_clk clock domain is synchronized with the pclk clock domain.

The maximum frequency of the APB_SSI master bit rate clock (sclk_out) is half the ssi_clk frequency. This allows the transfer control logic to capture data on one clock edge of sclk_out and send data on the opposite clock edge, as shown in Figure 3. sclk_out is only maintained while a valid transfer is in progress. At any other time, it remains inactive.

Figure 3: Maximum sclk_out/ssi_clk Ratio

The frequency of sclk_out can be obtained by the following equation:

Fsclk_out = Fssi_clk / SCKDV

SCKDV is a bit in the programmable register BAUDR, which can be any even number between 0 and 65534. If SCKDV is 0, sclk_out is disabled.

- Provides three transmission modes: send and receive, send only, receive only

TMOD bit of configuration control register 0 (CTRLR0) .

  - Send and receive

TMOD= 2’b00, both sending and receiving logic are valid. The data transfer occurs as normal according to the selected frame format (serial protocol).Transmit data are popped from the transmit FIFO and sent through the txd line to the target device, which replies with data on the rxd line. The receive data from the target device is moved from the receive shift register into the receive FIFO at the end of each data frame.

  - send only

TMOD = 2'b01, the received data is invalid and data cannot be stored in the receiving FIFO. The data transfer occurs as normal, according to the selected frame format (serial protocol). Transmit data are popped from the transmit FIFO and sent through the txd line to the target device, which replies with data on the rxd line. At the end of the data frame, the receive shift register does not load its newly received data into the receive FIFO. The data in the receive shift register is overwritten by the next transfer. You should mask interrupts originating from the receive logic when this mode is entered.

  - only receive

TMOD = 2'b10 , sending data is invalid . Data from a previous transfer is retransmitted from the shift register . The receive data from the target device is moved from the receive shift register into the receive FIFO at the end of each data frame. You should mask interrupts originating from the transmit logic when this mode is entered.

    - Note: When APB_SSI is disabled (SSI_EN = 0) or reset (psesetn = 0), the transmit and receive FIFOs are cleared. When the SSI Master is in receive-only mode, a dummy word needs to be written to the transmit FIFO (data register) to initiate serial transmission. The data is sent repeatedly so that the Master can receive the data until the number of received data reaches the required value (CTRLR1+1).

- Supports three transmission interface protocols (SPI, SSP, Mwire)

- SPI protocol

For the SPI protocol , the configurable parameter clock polarity (SCPOL) determines whether the serial clock inactive level is high or low. In order to achieve data transfer, both SPI peripherals must have consistent clock phase (SCPH) and clock polarity values (SCPOL). The data frame length can be 4-16 bits.

When SCPH=0, data transfer begins on the falling edge of the slave select signal, and the first data bit is captured by the master and slave on the first edge of the serial clock. Therefore valid data must appear on the txd and rxd lines before the first serial clock edge. The figure below shows the timing of a single SPI data transmission when SCPH=0, SCPOL=0 or =1.

Figure 4 : SPI Serial Format (SCPH = 0)

When SCPH=0, since data transmission starts on the falling edge of the slave select signal, continuous data transmission requires the slave select signal to flip before starting the transmission of the next data frame, as shown in the following figure:

Figure5: SPI Serial Format Continuous Transfers (SCPH = 0)

When SCPH=1, both the master and slave peripherals start transmitting data on the first serial clock edge after the slave select signal is valid. The first data bit is captured on the second clock edge of the serial clock. Data is given by the master and slave peripherals on the first edge of the serial clock. During the transmission of consecutive data frames, the slave select signal remains active low until the last bit of the last data frame is captured. The following figure shows the timing of SPI mode when SCPH=1:

Figure6: SPI Serial Format (SCPH = 1)

Continuous data frames and single data frames are transmitted in the same way. The MSB of the next data follows directly after the LSB of the current data frame . The slave select signal remains active during the transfer. The figure below shows continuous SPI transmission when SPCH=1.

Figure 7 : SPI Serial Format Continuous Transfer (SCPH = 1)

- SSP protocol

For the SSP protocol, data transmission begins by asserting the data frame detection line ( ss_0_n/ss_in_n ) for one serial clock cycle. The data to be transmitted is driven to the transmit data line (txd) after one serial clock cycle. Similarly, the data from the slave is driven to the receive data line (rxd). Data is generated on the rising edge of the serial clock ( sclk_out/sclk_in ) and sampled on the falling edge. The length of the data frame can be 4-16 bits. The figure below shows the transmission timing diagram of a single data frame of the SSP protocol.

Figure 8 : SSP Serial Format

Continuous data transfers occur in the same manner as single data transfers. Data frame detection declares a cycle when the LSB of the current transmission is the same clock cycle, indicating that another data frame follows. The figure below is the timing diagram of continuous data frame transmission in the SSP protocol.

Figure 9 : SSP Serial Format Continuous Transfer

- Mwire protocol

For the Mwire protocol, data transmission starts with the falling edge of the slave select signal (ss_0_n). Half a clock cycle (sclk_out) later, the first bit of the control word is sent to the txd line. The length of the control word can be from 1 to 16 bits, set by the CFS[15:12 ] bits of CTRLR0. The remaining bits of the control word are sent through the APB_SSI master (sent on the falling edge of sclk_out). During this period, no data appears on the host's rxd line (it is in a high-impedance state).

The direction of the data is controlled by MDD (1 bit of MWCR). When MDD=0, it means the host receives data from the external serial slave. After the LSB of the control word is transmitted one clock late, the slave device feeds back 0 bits of dummy data, and then transmits the data frame, which can be 4 to 16 bits long. Data is sent on the falling edge of the serial clock and captured on the rising edge.

The slave select line remains active low during the transmission and is pulled high half a cycle after the data transmission ends. The following figure shows the operation timing diagram of the APB_SSI host reading a single data from the peripheral slave:

Figure 10 : Single APB_SSI Master Microwire Serial Transfer (MDD=0)

Continuous Mwire protocol transmission can be continuous transmission or discontinuous transmission, which is controlled by bit 0 MMWMOD of MWCR. The figure below shows discontinuous transmission. The control word of the next transmission follows the LSB of the currently transmitted data.

Figure 11 : Continuous Nonsequential Microwire Transfer

(receiving data frame)

To perform a continuous nonsequential transfer , the only modification required is to write more control words to the transmit FIFO.

In a continuous transfer, only one control word is sent from the host, and the transfer proceeds in the same manner as a non-continuous read operation. But the clock for reading data continues to execute. The slave device automatically increments the address pointer value, pointing to the next address space, and consecutive data is given from that address space. Any number of address spaces can be read in this manner. The APB_SSI host stops transmitting when the number of received data is equal to the value of CTRLR1 + 1. The following timing diagram and example is a read transfer that reads two data consecutively from the peripheral:

Figure 12 : Continuous Sequential Microwire Transfer

(receiving data frame)

When MDD=1, it indicates that APB_SSI sends data to the peripheral slave. After the LSB of the control word is sent, APB_SSI immediately starts sending data frames to the peripheral. The timing diagram below shows the operation of APB_SSI writing a single data to the peripheral.

Figure 13 : Single Microwire Transfer (transmitting data frame)

    - Note: APB_SSI does not support continuous write operations in Mwire mode, that is, MDD=1, MMWOD=1.

The figure below shows continuous data transmission. The control word of the next transmission follows the LSB of the currently transmitted data. The only modification required to perform a continuous transmission is to write more control words and data to the transmit FIFO. The picture below shows writing two words to the peripheral.

Figure 14 : Continuous Microwire Transfer (transmitting data frame)

For write operations from the APB_SSI master to the peripheral slave, the Mwire handshake interface can be enabled. The handshake interface can be enabled by writing 1 to bit 2 MHS of MWCR. When MHS is set to 1, the APB_SSI master checks the ready status of the slave device before completing the transmission. Or send the next control word of the continuous transmission. The following figure shows the continuous Mwire transmission after enabling the handshake interface:

Figure 1 5: Continuous Microwire Transfer with Handshaking

(transmitting data frame)

After the first data sent to the slave device is sent, the APB_SSI host pulls the rxd input high and waits for the ready status of the slave device. As soon as the ready signal is received, the host starts transmitting the next control word. After the transmission of the last data frame is completed, APB_SSI sends a start bit to clear the ready state of the slave before completing the transmission.

- A combined interrupt is the logical OR of individual interrupts

The APB_SSI interrupt list is as follows:

| Interrupt name | Interrupt abbreviation | Interrupt generation conditions | Interrupt clear | Remark |
| --- | --- | --- | --- | --- |
| Send FIFO empty interrupt | ssi_txe_intr | Set when the transmit FIFO is equal to or less than its limit value , and service is requested to prevent underflow. The limit value is set by the software programmable register TXFTLR to determine the level at which the transmit FIFO entry generates an interrupt. | This interrupt is cleared by hardware when data is written to send FIFO caches values greater than the threshold level . | / |
| Send FIFO overflow interrupt | ssi_txo_intr | when APB attempts to write data to an already filled transmit FIFO . | This interrupt is valid until the transmit FIFO overflow interrupt clear register (TXOICR) is read . | When valid, data written by APB is discarded. |
| Receive FIFO full interrupt | ssi_rxf_intr | Set when the receive FIFO is equal to or greater than its limit value plus one, and service is requested to prevent overflow . The limit value is set by the software programmable register RXFTLR to determine the level at which the receive FIFO entry generates an interrupt. | This interrupt is cleared by hardware when data is received from Read from the FIFO so that its value is less than the limit value ( threshold level ). | / |
| Receive FIFO overflow interrupt | ssi_rxo_intr | Set when the receive logic attempts to fill data into an already filled receive FIFO . | This interrupt is active until the receive FIFO overflow interrupt clear register (RXOICR) is read . | When valid, newly received data is discarded. |
| Receive FIFO underflow interrupt | ssi_rxu_intr | Set when an APB access attempts to read data from an empty receive FIFO . | This interrupt is valid until the receive FIFO underflow interrupt clear register is read. (RXUICR) . | When valid , 0 is read from the receive FIFO . |
| Multi-host contention interrupt | ssi_mst_intr | When another serial master on the serial bus selects the APB_SSI master as a serial slave device and it is transmitting data. This notifies the processor that there may be contention on the serial bus. | This interrupt remains active until the Multi-Master Interrupt Clear Register ( MSTIC R) is read . |  |
| Combined interrupt request | ssi_intr | Logical OR of all requested interrupts masked. | / | In order to mask this interrupt signal, all other APB_SSI interrupt requests must be masked |

4.2 Host SPI and SSP serial transmission

- The typical software flow of the APB_SSI host performing SPI and SSP serial transmission is as follows:

- If the APB_SSI is enabled, disable it by writing 0 to the SSI Enable register (SSIENR).

- Set up the APB_SSI control registers for the transfer; these registers can be set in any order.

  - Write Control Register 0 (CTRLR0). For SPI transfers, the serial clock polarity and serial clock phase parameters must be set identical to target slave device.

  - If the transfer mode is receive only , write CTRLR1 (Control Register 1) with the number of frames in the transfer minus 1; for example, if you want to receive four data frames, write this register with 3.

  - Write the Baud Rate Select Register (BAUDR) to set the baud rate for the transfer.

  - Write the Transmit and Receive FIFO Threshold Level registers (TXFTLR and RXFTLR, respectively) to set FIFO threshold levels.

  - Write the IMR register to set up interrupt masks.

  - The Slave Enable Register (SER) register can be written here to enable the target slave for selection. If a slave is enabled here, the transfer begins as soon as one valid data entry is present in the transmit FIFO. If no slaves are enabled prior to writing to the Data Register (DR), the transfer does not begin until a slave is enabled.

- Enable the APB_SSI by writing 1 to the SSIENR register.

- Write data for transmission to the target slave into the transmit FIFO (write DR). If no slaves were enabled in the SER register at this point, enable it now to begin the transfer.

- Poll the BUSY status to wait for completion of the transfer. The BUSY status cannot be polled immediately; If a transmit FIFO empty interrupt request is made, write the transmit FIFO (write DR). If a receive FIFO full interrupt request is made, read the receive FIFO (read DR).

- The transfer is stopped by the shift control logic when the transmit FIFO is empty. If the transfer mode is receive only (TMOD = 2'b10), the transfer is stopped by the shift control logic when the specified number of frames have been received. When the transfer is done, the BUSY status is reset to 0.

- If the transfer mode is not transmit only (TMOD != 01), read the receive FIFO until it is empty.

- Disable the APB_SSI by writing 0 to SSIENR.

Figure 1 2 shows a typical software flow for starting a APB_SSI master SPI/SSP serial transfer. The diagram also shows the hardware flow inside the serial-master component.

Figure 16: APB_SSI Master SPI/SSP Transfer Flow

- Note: When choosing to use the SSP protocol for data transmission, you need to first configure an SPI_PROTOCAL_SEL register of the sys_ctrl module (SPI_PROTOCAL_SEL=0x0). By default, the register value is 0x1, which is used for data transmission using the SPI protocol and Microwire protocol. (See LISoCSYS_CTRL datasheet for details)

The following is an example of a program that configures the host SPI to only send query mode:

void test_spi_mt()

{

unsigned int trans_data[10] ;

unsigned int i;

for (i=0;i<10;i++)

{

trans_data[i] = i;

}

SSI0_SSIENR = 0; 		// Disable SSI module

// 	Configure related control registers

SSI0_CTRLR0 = 0x010f; 	// The size of the data frame is 16 bits, SPI protocol transmission, send only mode, //The polarity and phase are both 0

SSI0_BAUDR = 0x2; // Configure the bit rate divided by 2

SSI0_IMR = 0x0; 		// Work in query mode and shield all interrupts

for (i=0; i<10; i++) 	// Fill data into Master's sending data FIFO

{

SSI0_DR0 = trans_data[i];

}

if(!(SSI0_SR & 0x1)) 	// Check whether the SSI module is in busy state,

//Enable transmission if it is not busy

{

SSI0_SER = 0x01; 		// Select slave0

SSI0_SSIENR = 0x1; // Enable SSI module

}

while(SSI0_SR & 0x1);

SSI0_SSIENR = 0x0; // Detect whether the transmission is completed and disable the SSI module after completion

}

4.3 Host Microwire serial transmission

- The typical software process for the APB_SSI host to perform Microwire serial transmission is as follows:

- If the APB_SSI is enabled, disable it by writing 0 to SSIENR.

- Set up the APB_SSI control registers for the transfer. These registers can be set in any order. Write CTRLR0 to set transfer parameters.

  - If the transfer is sequential and the APB_SSI master receives data, write CTRLR1 with the number of frames in the transfer minus 1 .

  - Write BAUDR to set the baud rate for the transfer.

  - Write TXFTLR and RXFTLR to set FIFO threshold levels.

  - Write the IMR register to set up interrupt masks.

You can write the SER register to enable the target slave for selection. If a slave is enabled here, the transfer begins as soon as one valid data entry is present in the transmit FIFO. If no slaves are enabled prior to writing to the DR register, the transfer does not begin until a slave is enabled.

- Enable the APB_SSI by writing 1 to the SSIENR register.

- If the APB_SSI master transmits data, write the control and data words into the transmit FIFO (write DR). If the APB_SSI master receives data, write the control word(s) into the transmit FIFO.If no slaves were enabled in the SER register at this point, enable now to begin the transfer.

- Poll the BUSY status to wait for completion of the transfer. The BUSY status cannot be polled immediately . If a transmit FIFO empty interrupt request is made, write the transmit FIFO (write DR). If a receive FIFO full interrupt request is made, read the receive FIFO (read DR).

- The transfer is stopped by the shift control logic when the transmit FIFO is empty. If the transfer mode is sequential and the APB_SSI master receives data, the transfer is stopped by the shift control logic when the specified number of data frames is received. When the transfer is done, the BUSY status is reset to 0.

- If the APB_SSI master receives data, read the receive FIFO until it is empty.

- Disable the APB_SSI by writing 0 to SSIENR.

Figure 13 shows a typical software flow for starting a APB_SSI master Microwire serial transfer. The diagram also shows the hardware flow inside the serial-master component.

Figure 17 : APB_SSI Master Microwire Transfer Flow

- The following example is pseudo code that illustrates how to use software to select the target slave.

Chapter 12 LISoC_APB_SSI Slave

1 Function list

The APB_SSI slave is a full-duplex slave-synchronous serial interface. The host processor accesses data, control, and status information on the APB_SSI slave through the APB interface. The APB_SSI slave can connect to any serial-master peripheral device using one of the following interfaces:

- Motorola Serial Peripheral Interface (SPI)

- TexasInstruments Serial Protocol (SSP)

- National Semiconductor Microwire

The feature list is as follows:

  - Support independent masking of interrupts . Polarity for each individual interrupt is active-high.

  - Receive and transmit FIFO buffers d epth is 8.

  - Programmable features:

    - Serial interface operation – Choice of Motorola SPI, Texas Instruments Synchronous Serial Protocol or National Semiconductor Microwire.

    - Data Item size (4 to 16 bits) – Item size of each data transfer under the control of the programmer.

    - Provide three kinds of transfer mode : Transmit and Receive , Transmit Only , Receive Only .

- Support AMBA 2.0 .

- Supports 32 bits APB data bus widths.

2 Register list

Base address: 0x4000 2000

| Offset | Register name | reset value | Mode | Bit width (bits) | describe |
| --- | --- | --- | --- | --- | --- |
| 0x00 | CTRLR0 | 0x10007 | R/W | 32 | Control Register 0 : This register controls serial data transfer. This register cannot be written after APB_SSI is enabled. APB_SSI is enabled by writing to the SSIENR register. |
| 0x08 | SSIENR | 0x0 | R/W | 32 | SSI Enable Register |
| 0x0c | MWCR | 0x0 | R/W | 32 | Microwire Control Register : This register controls the direction of the data word for the half-duplex Microwire serial protocol. This register is not writable after APB_SSI is enabled, and APB_SSI is enabled by writing to the SSIENR register. |
| 0x18 | TXFTLR | 0x0 | R/W | 32 | Transmit FIFO Threshold Level : This register controls the threshold value for the transmit FIFO memory. This register is not writable after APB_SSI is enabled, and APB_SSI is enabled by writing to the SSIENR register. |
| 0x1c | RXFTLR | 0x0 | R/W | 32 | Receive FIFO Threshold Level : This register controls the threshold value for the receive FIFO memory. This register is not writable after APB_SSI is enabled, and APB_SSI is enabled by writing the SSIENR register. |
| 0x20 | TxFLR | 0x0 | R | 32 | Transmit FIFO Level Register |
| 0x24 | RXFLR | 0x0 | R | 32 | Receive FIFO Level Register |
| 0x28 | SR | 0x6 | R | 32 | Status Register : This is a read-only register used to indicate the current transfer status, FIFO status, and any transmission/reception errors that may have occurred. The status register may be read at any time. None of the bits in this register request an interrupt. |
| 0x2c | IMR | 0x1f | R/W | 32 | Interrupt Mask Register |
| 0x30 | ISR | 0x0 | R | 32 | Interrupt Register Status |
| 0x34 | RISR | 0x0 | R | 32 | Raw Interrupt Status Register |
| 0x38 | TxOICR | 0x0 | R | 32 | Transmit FIFO Overflow Interrupt Clear register |
| 0x3c | RXOICR | 0x0 | R | 32 | Receive FIFO Overflow Interrupt Clear Register |
| 0x40 | RXUICR | 0x0 | R | 32 | Receive FIFO Underflow Interrupt Clear Register |
| 0x44 | MSTICR | 0x0 | R | 32 | Multi-Master Interrupt Clear Register |
| 0x48 | ICR | 0x0 | R | 32 | Interrupt Clear Register |
| 0x58 | IDR | 0xffffffff | R | 32 | Identification Register |
| 0x5c | SSI_VERSION_ID | 0x3430322a | R/W | 32 | coreKit Version ID Register |
| 0x60 ~ 0x7c | DR0 ~ DR 7 | 0x0 | R/W | 32 | Data Register 0~ 7 : The APB_SSI data register is a 16-bit read/write buffer for the transmit/receive FIFOs. When the register is read, data in the receive FIFO buffer is accessed. When it is written to, data are moved into the transmit FIFO buffer; a write can occur only when SSI_EN = 1. FIFOs are reset when SSI_EN = 0. |

- Register detailed definition

- CTRLR0

● Name: Control Register 0

● Description: This register controls the serial data transfer. It is impossible to write to this register when the APB_SSI is enabled. The APB_SSI is enabled and disabled by writing to the SSIENR register.

● Size: 32 bits

● Address Offset: 0x0

● Read/write access: read/write

● Reset Value: 0x0100 0007

| 31:25 | twenty four | 23:16 | 15:12 | 11 | 10 | 9:8 | 7 | 6 | 5:4 | 3:0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| RSVD_CTRLR0 | SSTE | RSVD_CTRLR0_16_23 | CFS | SRL | SLV_OE | TMOD | SCPOL | SCPH | FRF | DFS |

Table 3-1Fields for Register: CTRLR0

| Bits | Name | Reset Value | Memory Access | Description |
| --- | --- | --- | --- | --- |
| twenty four | SSTE | 0x1 | R/W | Slave Select Toggle Enable. When operating in SPI mode with clock phase (SCPH) set to 0, this register controls the behavior of the slave select line (ss_*_n) between data frames. If this register field is set to 1 the ss_*_n line will toggle between consecutive data frames, with the serial clock (sclk) being held to its default value while ss_*_n is high; if this register field is set to 0 the ss_*_n will stay low and sclk will run continuously for the duration of the transfer . |
| 15:12 | CFS | 0x0 | R/W | Control Frame Size . Selects the length of the control word for the Microwire frame format. Values: ■0x0 (SIZE_01_BIT): 1-bit Control Word ■0x1 (SIZE_02_BIT): 2-bit Control Word ■ 0x2 (SIZE_03_BIT): 3-bit Control Word ■0x3 (SIZE_04_BIT): 4-bit Control Word ■0x4 (SIZE_05_BIT): 5-bit Control Word ■0x5 (SIZE_06_BIT): 6-bit Control Word ■0x6 (SIZE_07_BIT): 7-bit Control Word ■0x7 (SIZE_08_BIT): 8-bit Control Word ■0x8 (SIZE_09_BIT): 9-bit Control Word ■0x9 (SIZE_10_BIT): 10-bit Control Word ■0xa (SIZE_11_BIT): 11-bit Control Word ■0xb (SIZE_12_BIT): 12-bit Control Word ■0xc (SIZE_13_BIT): 13-bit Control Word ■0xd (SIZE_14_BIT): 14-bit Control Word ■0xe (SIZE_15_BIT): 15-bit Control Word ■0xf (SIZE_16_BIT): 16-bit Control Word |
| 11 | SRL | 0x0 | R/W | Shift Register Loop. Used for testing purposes only. When internally active, connects the transmit shift register output to the receive shift register input. Can be used in both serial-slave and serial-master modes. Values: ■ 0 – Normal Mode Operation ■ 1 – Test Mode Operation |
| 10 | SLV_OE | 0x0 | R/W | Slave Output Enable. This bit enables or disables the setting of the ssi_oe_n output from the APB_SSI serial slave. When SLV_OE = 1, the ssi_oe_n output can never be active. When the ssi_oe_n output controls the tri-state buffer on the txd output from the slave, a high impedance state is always present on the slave txd output when SLV_OE = 1. This is useful when the master transmits in broadcast mode (master transmits data to all slave devices). Only one slave may respond with data on the master rxd line. This bit is enabled after reset and must be disabled by software (when broadcast mode is used), if you do not want this device to respond with data. Values: ■0x1 (DISABLED): Slave Output is disabled ■0x0 (ENABLED): Slave Output is enabled |
| 9:8 | TMOD | 0x0 | R/W | Transfer Mode . Selects the mode of transfer for serial communication. This field does not affect the transfer duplicity. Only indicates whether the receive or transmit data are valid. In transmit-only mode , data received from the external device is not valid and is not stored in the receive FIFO memory; it is overwritten on the next transfer. In receive-only mode , transmitted data are not valid. After the first write to the transmit FIFO, the same word is retransmitted for the duration of the transfer. In transmit-and-receive mode , both transmit and receive data are valid. The transfer continues until the transmit FIFO is empty. Data received from the external device are stored into the receive FIFO memory, where it can be accessed by the host processor. Values: ■ 00 –- Transmit & Receive ■ 01 –- Transmit Only ■ 10 – Receive Only ■ 11 –- Reserved |
| 7 | SCPOL | 0x0 | R | Serial Clock Polarity . Valid when the frame format (FRF) is set to Motorola SPI. Used to select the polarity of the inactive serial clock, which is held inactive when the APB_SSI master is not actively transferring data on the serial bus. Values: ■ 0 – Inactive state of serial clock is low ■ 1 – Inactive state of serial clock is high |
| 6 | SCPH | 0x0 | R | Serial Clock Phase . Valid when the frame format (FRF) is set to Motorola SPI. The serial clock phase selects the relationship of the serial clock with the slave select signal. When SCPH = 0, data are captured on the first edge of the serial clock. When SCPH = 1, the serial clock starts toggling one cycle after the slave select line is activated, and data are captured on the second edge of the serial clock. Values: ■ 0: Serial clock toggle in middle of first data bit ■ 1: Serial clock toggle at start of first data bit |
| 5:4 | FRF | 0x00 | R/W | Frame Format . Selects which serial protocol transfers the data. Values: ■ 00 – Motorola SPI ■ 01 –- Texas Instruments SSP ■ 10 –- National Semiconductors Microwire ■ 11 –- Reserved |
| 3 :0 | DFS | 0x7 | R/W | Data Frame Size . Selects the data frame length. When the data frame size is programmed to be less than 16 bits, the receive data are automatically right-justified by the receive logic, with the upper bits of the receive FIFO zero-padded. You must right-justify transmit data before writing into the transmit FIFO. The transmit logic ignores the upper unused bits when transmitting the data. Values: ■0x3 (FRAME_04BITS): 4-bit serial data transfer ■0x4 (FRAME_05BITS): 5-bit serial data transfer ■0x5 (FRAME_06BITS): 6-bit serial data transfer ■0x6 (FRAME_07BITS): 7-bit serial data transfer ■0x7 (FRAME_08BITS): 8-bit serial data transfer ■0x8 (FRAME_09BITS): 9-bit serial data transfer ■0x9 (FRAME_10BITS): 10-bit serial data transfer ■0xa (FRAME_11BITS): 11-bit serial data transfer ■0xb (FRAME_12BITS): 12-bit serial data transfer ■0xc (FRAME_13BITS): 13-bit serial data transfer ■0xd (FRAME_14BITS): 14-bit serial data transfer ■0xe (FRAME_15BITS): 15-bit serial data transfer ■0xf (FRAME_16BITS): 16-bit serial data transfer |

- SSIENR

● Name: SSI Enable Register

● Description: This register enables and disables the APB_SSI.

● Size: 32 bits

● Address Offset: 0x08

● Read/write access: read/write

● Reset Value: 0x0

| 31:1 | 0 |
| --- | --- |
| Reserved | SSI_EN |

Table 3-2Fields for Register: SSIENR

| Bits | Name | Reset Value | Memory Access | Description |
| --- | --- | --- | --- | --- |
| 0 | SSI_EN | 0x0 | R/W | SSI Enable . Enables and disables all APB_SSI operations. When disabled, all serial transfers are halted immediately. Transmit and receive FIFO buffers are cleared when the device is disabled. It is impossible to program some of the APB_SSI control registers when enabled. When disabled, the ssi_sleep output is set (after delay) to inform the system that it is safe to remove the ssi_clk, thus saving power consumption in the system. Values: ■0x0 (DISABLE): Disables Serial Transfer ■0x1 (ENABLED): Enables Serial Transfer |

- MWCR

● Name: Microwire Control Register

● Description: This register controls the direction of the data word for the half-duplex Microwire serial protocol. It is impossible to write to this register when the APB_SSI is enabled. The APB_SSI is enabled and disabled by writing to the SSIENR register.

● Size : 32 bits

● Address Offset: 0x0C

● Read/write access: read/write

● Reset Value: 0x0

| 31:2 | 1 | 0 |
| --- | --- | --- |
| Reserved | MDD | MWMOD |

Table 3-3Fields for Register: MWCR

| Bits | Name | Reset Value | Memory Access | Description |
| --- | --- | --- | --- | --- |
| 1 | MDD | 0x0 | R/W | Microwire Control . Defines the direction of the data word when the Microwire serial protocol is used. When this bit is set to 0, the data word is received by the APB_SSI MacroCell from the external serial device. When this bit is set to 1, the data word is transmitted from the APB_SSI MacroCell to the external serial device. Values: ■0x0 (RECEIVE): SSI receives data ■0x1 (TRANSMIT): SSI transmits data |
| 0 | MWMOD | 0x0 | R/W | Microwire Transfer Mode . Defines whether the Microwire transfer is sequential or non-sequential. When sequential mode is used, only one control word is needed to transmit or receive a block of data words. When non-sequential mode is used, there must be a control word for each data word that is transmitted or received. Values: ■ 0 – non-sequential transfer ■ 1 – sequential transfer |

- TXFTLR

● Name: Transmit FIFO Threshold Level

● Description: This register controls the threshold value for the transmit FIFO memory. The APB_SSI is enabled and disabled by writing to the SSIENR register..

● Size : 32bits

● Address Offset: 0x18

● Read/write access: read/write

● Reset Value: 0x0

| 31:3 | 2:0 |
| --- | --- |
| Reserved | TFT |

Table 3-4Fields for Register: TXFTLR

| Bits | Name | Reset Value | Memory Access | Description |
| --- | --- | --- | --- | --- |
| 2:0 | TFT | 0x0 | R/W | Transmit FIFO Threshold . Controls the level of entries (or below) at which the transmit FIFO controller triggers an interrupt. The FIFO depth is 16 ; If you attempt to set this value greater than or equal to the depth of the FIFO, this field is not written and retains its current value. When the number of transmit FIFO entries is less than or equal to this value, the transmit FIFO empty interrupt is triggered. |

- RXFTLR

● Name: Receive FIFO Threshold Level

● Description: This register controls the threshold value for the receive FIFO memory. The APB_SSI is enabled and disabled by writing to the SSIENR register.

● Size: 32 bits

● Address Offset: 0x1C

● Read/write access: read/write

● Reset Value: 0x0

| 31: 3 | 2:0 |
| --- | --- |
| Reserved | RFT |

Table 3-5Fields for Register: RXFTLR

| Bits | Name | Reset Value | Memory Access | Description |
| --- | --- | --- | --- | --- |
| 2 : 0 | RFT | 0x0 | R/W | Receive FIFO Threshold . Controls the level of entries (or above) at which the receive FIFO controller triggers an interrupt. The FIFO depth is 16. If you attempt to set this value greater than the depth of the FIFO, this field is not written and retains its current value. When the number of receive FIFO entries is greater than or equal to this value + 1, the receive FIFO full interrupt is triggered. |

- TxFLR

● Name: Transmit FIFO Level Register

● Description: This register contains the number of valid data entries in the transmit FIFO memory.

● Size: 32 bits

● Address Offset: 0x20

● Read/write access: read-only

● Reset Value: 0x0

| 31: 4 | 3 :0 |
| --- | --- |
| Reserved | TXTFL |

Table 3-6Fields for Register: TXFLR

| Bits | Name | Reset Value | Memory Access | Description |
| --- | --- | --- | --- | --- |
| 3 :0 | TXTFL | 0x0 | R | Transmit FIFO Level . Contains the number of valid data entries in the transmit FIFO. |

- RXFLR

● Name: Receive FIFO Level Register

● Description: This register contains the number of valid data entries in the receive FIFO memory. This register can be read at any time.

● Size: 32bits

● Address Offset: 0x24

● Read/write access: read-only

● Reset Value: 0x0

| 31: 4 | 3 :0 |
| --- | --- |
| Reserved | RXTFL |

Table 3-7Fields for Register: RXFLR

| Bits | Name | Reset Value | Memory Access | Description |
| --- | --- | --- | --- | --- |
| 4:0 | RXTFL | 0x0 | R | Receive FIFO Level . Contains the number of valid data entries in the receive FIFO. |

- SR

● Name: Register Status

● Description: This is a read-only register used to indicate the current transfer status, FIFO status, and any transmission/reception errors that may have occurred. The status register may be read at any time. None of the bits in this register request an interrupt.

● Size: 32 bits

● Address Offset: 0x28

● Read/write access: read-only

● Reset Value: 0x06

| 31:6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- |
| Reserved | TXE | RFE | RFNE | TFE | TNF | BUSY |

Table 3-8Fields for Register: SR

| Bits | Name | Reset Value | Memory Access | Description |
| --- | --- | --- | --- | --- |
| 5 | TXE | 0x0 | R | Transmission Error. Set if the transmit FIFO is empty when a transfer is started. This bit can be set only when the APB_SSI is configured as a slave device. Data from the previous transmission is resent on the txd line. This bit is cleared when read. Values: ■0x0 (NO_ERROR): No Error ■0x1 (TX_ERROR): Transmission Error |
| 4 | RFF | 0x0 | R | Receive FIFO Full . When the receive FIFO is completely full, this bit is set. When the receive FIFO contains one or more empty location, this bit is cleared. Values: ■ 0 – Receive FIFO is not full ■ 1 – Receive FIFO is full |
| 3 | RFNE | 0x0 | R | Receive FIFO Not Empty . Set when the receive FIFO contains one or more entries and is cleared when the receive FIFO is empty. This bit can be polled by software to completely empty the receive FIFO. Values: ■ 0 – Receive FIFO is empty ■ 1 – Receive FIFO is not empty |
| 2 | TFE | 0x1 | R | Transmit FIFO Empty . When the transmit FIFO is completely empty, this bit is set. When the transmit FIFO contains one or more valid entries, this bit is cleared. This bit field does not request an interrupt. Values: ■ 0 – Transmit FIFO is not empty ■ 1 – Transmit FIFO is empty |
| 1 | TNF | 0x1 | R | Transmit FIFO Not Full . Set when the transmit FIFO contains one or more empty locations, and is cleared when the FIFO is full. Values: ■ 0 – Transmit FIFO is full ■ 1 – Transmit FIFO is not full |
| 0 | BUSY | 0x0 | R | SSI Busy Flag . When set, indicates that a serial transfer is in progress; when cleared indicates that the APB_SSI is idle or disabled. Values: ■ 0 – APB_SSI is idle or disabled ■ 1 – APB_SSI is actively transferring data |

- IMR

● Name: Interrupt Mask Register

● Description: This read/write reigster masks or enables all interrupts generated by the APB_SSI.

● Size: 32 bits

● Address Offset: 0x2C

● Read/write access: read/write

● Reset Value: 0x1F

| 31:5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- |
| Reserved | RXFIM | RXOIM | RXUIM | TXOIM | TXEIM |

Table 3-9Fields for Register:IMR

| Bits | Name | Reset Value | Memory Access | Description |
| --- | --- | --- | --- | --- |
| 4 | RXFIM | 0x1 | R/W | Receive FIFO Full Interrupt Mask Values: ■ 0 – ssi_rxf_intr interrupt is masked ■ 1 – ssi_rxf_intr interrupt is not masked |
| 3 | RXOIM | 0x1 | R/W | Receive FIFO Overflow Interrupt Mask Values: ■ 0 – ssi_rxo_intr interrupt is masked ■ 1 – ssi_rxo_intr interrupt is not masked |
| 2 | RXUIM | 0x1 | R/W | Receive FIFO Underflow Interrupt Mask Values: ■ 0 – ssi_rxu_intr interrupt is masked ■ 1 – ssi_rxu_intr interrupt is not masked |
| 1 | TXOIM | 0x1 | R/W | Transmit FIFO Overflow Interrupt Mask Values: ■ 0 – ssi_txo_intr interrupt is masked ■ 1 – ssi_txo_intr interrupt is not masked |
| 0 | TXEIM | 0x1 | R/W | Transmit FIFO Empty Interrupt Mask Values: ■ 0 – ssi_txe_intr interrupt is masked ■ 1 – ssi_txe_intr interrupt is not masked |

- ISR

● Name: Interrupt Status Register

● Description: This register reports the status of the APB_SSI interrupts after they have been masked.

● Size : 32bits

● Address Offset: 0x30

● Read/write access: read-only

● Reset Value: 0x0

| 31:5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- |
| Reserved | RXFIS | RXOIS | RXUIS | TxOIS | TxEIS |

Table 3-10Fields for Register:ISR

| Bits | Name | Reset Value | Memory Access | Description |
| --- | --- | --- | --- | --- |
| 4 | RXFIS | 0x0 | R | Receive FIFO Full Interrupt Status Values: ■ 0 = ssi_rxf_intr interrupt is not active after masking ■ 1 = ssi_rxf_intr interrupt is full after masking |
| 3 | RXOIS | 0x0 | R | Receive FIFO Overflow Interrupt Status Values: ■ 0 = ssi_rxo_intr interrupt is not active after masking ■ 1 = ssi_rxo_intr interrupt is active after masking |
| 2 | RXUIS | 0x0 | R | Receive FIFO Underflow Interrupt Status Values: ■ 0 = ssi_rxu_intr interrupt is not active after masking ■ 1 = ssi_rxu_intr interrupt is active after masking |
| 1 | TxOIS | 0x0 | R | Transmit FIFO Overflow Interrupt Status Values: ■ 0 = ssi_txo_intr interrupt is not active after masking ■ 1 = ssi_txo_intr interrupt is active after masking |
| 0 | TxEIS | 0x0 | R | Transmit FIFO Empty Interrupt Status Values: ■ 0 = ssi_txe_intr interrupt is not active after masking ■ 1 = ssi_txe_intr interrupt is active after masking |

- RISR

● Name: Raw Interrupt Status Register

● Description: This read-only register reports the status of the APB_SSI interrupts prior to masking.

● Size: 32bits

● Address Offset: 0x34

● Read/write access: read-only

● Reset Value: 0x0

| 31:5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- |
| Reserved | RXFIR | RXOIR | RXUIR | TxOIR | TXEIR |

Table 3-11Fields for Register: RISR

| Bits | Name | Reset Value | Memory Access | Description |
| --- | --- | --- | --- | --- |
| 4 | RXFIR | 0x0 | R | Receive FIFO Full Raw Interrupt Status Values: ■ 0 = ssi_rxf_intr interrupt is not active prior to masking Values: ■ 1 = ssi_rxf_intr interrupt is active prior to masking |
| 3 | RXOIR | 0x0 | R | Receive FIFO Overflow Raw Interrupt Status Values: ■ 0 = ssi_rxo_intr interrupt is not active prior to masking ■ 1 = ssi_rxo_intr interrupt is active prior masking |
| 2 | RXUIR | 0x0 | R | Receive FIFO Underflow Raw Interrupt Status Values: ■ 0 = ssi_rxu_intr interrupt is not active prior to masking ■ 1 = ssi_rxu_intr interrupt is active prior to masking |
| 1 | TxOIR | 0x0 | R | Transmit FIFO Overflow Raw Interrupt Status Values: ■ 0 = ssi_txo_intr interrupt is not active prior to masking ■ 1 = ssi_txo_intr interrupt is active prior masking |
| 0 | TXEIR | 0x0 | R | Transmit FIFO Empty Raw Interrupt Status Values: ■ 0 = ssi_txe_intr interrupt is not active prior to masking ■ 1 = ssi_txe_intr interrupt is active prior masking |

- TxOICR

● Name: Transmit FIFO Overflow Interrupt Clear Register

● Description: Transmit FIFO Overflow Interrupt Clear Register.

● Size: 32 bits

● Address Offset: 0x38

● Read/write access: read-only

● Reset Value: 0x0

| 31:1 | 0 |
| --- | --- |
| Reserved | TxOICR |

Table 3-15Fields for Register: TXOICR

| Bits | Name | Reset Value | Memory Access | Description |
| --- | --- | --- | --- | --- |
| 0 | TxOICR | 0x0 | R | Clear Transmit FIFO Overflow Interrupt . This register reflects the status of the interrupt. A read from this register clears the ssi_txo_intr interrupt; writing has no effect. |

- RXOICR

● Name: Receive FIFO Overflow Interrupt Clear Register

● Description: Receive FIFO Overflow Interrupt Clear Register.

● Size: 32 bits

● Address Offset: 0x3C

● Read/write access: read-only

● Reset Value: 0x0

| 31:1 | 0 |
| --- | --- |
| Reserved | RXOICR |

Table 3-13 Fields for Register: RXOICR

| Bits | Name | Reset Value | Memory Access | Description |
| --- | --- | --- | --- | --- |
| 0 | RXOICR | 0x0 | R | Clear Receive FIFO Overflow Interrupt . This register reflects the status of the interrupt. A read from this register clears the ssi_rxo_intr interrupt; writing has no effect. |

- RXUICR

● Name: Receive FIFO Underflow Interrupt Clear Register

● Description: Receive FIFO Underflow Interrupt Clear Register.

● Size: 32 bits

● Address Offset: 0x40

● Read/write access: read-only

● Reset Value: 0x0

| 31:1 | 0 |
| --- | --- |
| Reserved | RXUICR |

Table 3-14 Fields for Register: RXUICR

| Bits | Name | Reset Value | Memory Access | Description |
| --- | --- | --- | --- | --- |
| 0 | RXUICR | 0x0 | R | Clear Receive FIFO Underflow Interrupt . This register reflects the status of the interrupt. A read from this register clears the ssi_rxu_intr interrupt; writing has no effect. |

- MSTICR

● Name: Multi-Master Interrupt Clear Register

● Description:

● Size: 32 bits

● Address Offset: 0x44

● Read/write access: read-only

● Reset Value: 0x0

| 31:1 | 0 |
| --- | --- |
| Reserved | MSTICR |

Table 3-1 5 Fields for Register: MSTICR

| Bits | Name | Reset Value | Memory Access | Description |
| --- | --- | --- | --- | --- |
| 0 | MSTICR | 0x0 | R | Clear Multi-Master Contention Interrupt . This register reflects the status of the interrupt. A read from this register clears the ssi_mst_intr interrupt; writing has no effect. |

- ICR

● Name: Interrupt Clear Register

● Description: Multi-Master Interrupt Clear Register.

● Size: 32 bits

● Address Offset: 0x48

● Read/write access: read-only

● Reset Value: 0x0

| 31:1 | 0 |
| --- | --- |
| Reserved | ICR |

Table 3-16 Fields for Register: ICR

| Bits | Name | Reset Value | Memory Access | Description |
| --- | --- | --- | --- | --- |
| 0 | ICR | 0x0 | R | Clear Interrupts . This register is set if any of the interrupts below are active. A read clears the ssi_txo_intr, ssi_rxu_intr, ssi_rxo_intr, and the ssi_mst_intr interrupts. Writing to this register has no effect. |

- IDR

● Name: Identification Register

● Description: This register contains the peripherals identification code, which is written into the register at configuration time using coreConsultant..

● Size: 32 bits

● Address Offset: 0x58

● Read/write access: read-only

● Reset Value: 0xffffffff

31:0

IDCODE

Table 3-19 Fields for Register: IDR

| Bits | Name | Reset Value | Memory Access | Description |
| --- | --- | --- | --- | --- |
| 31: 0 | IDCO DE | 0xffffffff | R | The register contains the peripheral's identification code, which is written into the register at configuration time using CoreConsultant |

- SSI_VERSION_ID

● Name: coreKit version ID Registe

● Description: This read-only register stores the specific APB_SSI component version.

● Size: 32 bits

● Address Offset: 0x5c

● Read/write access: read-only

● Reset Value: 0x3430 322a

31:0

SSI_COMP_VERSION

Table 3-19 Fields for Register: IDR

| Bits | Name | Reset Value | Memory Access | Description |
| --- | --- | --- | --- | --- |
| 31: 0 | SSI_COMP_VERSION | 0x3430322a | R | The register contains the peripheral's identification code, which is written into the register at configuration time using CoreConsultant |

- DRx(for x=0; x< 8)

● Name: Data Register 0~ 7

● Description: The APB_SSI data register is a 16-bit read/write buffer for the transmit/receive FIFOs. When the register is read, data in the receive FIFO buffer is accessed. When it is written to, data are moved into the transmit FIFO buffer; a write can occur only when SSI_EN = 1. FIFOs are reset when SSI_EN = 0.

● Size: 32 bits

● Address Offset: 0x60 ~0x7C

● Read/write access: read/write

● Reset Value: 0x0

| 31:16 | 15:0 |
| --- | --- |
| Reserved | DR |

Table 3-17 Fields for Register: DRx

| Bits | Name | Reset Value | Memory Access | Description |
| --- | --- | --- | --- | --- |
| 15:0 | DR | 0x0 | R/W | Data Register . When writing to this register, you must right-justify the data. Read data are automatically right-justified. Read = Receive FIFO buffer Write = Transmit FIFO buffer |

4 Program Guide

4.1 Detailed description of some functions of SSI slave

- Clock frequency settings

The clock frequency of APB_SSI Slave is determined by the host, and the minimum value of ssi_clk frequency of APB_SSI slave depends on the operation of the slave peripherals. If the slave device is receive-only, the minimum frequency of ssi_clk is 8 times the maximum frequency of the expected host input bit rate clock (sclk_in); if the slave device is receive and transmit, the minimum frequency of ssi_clk is the maximum frequency of the expected host input bit rate clock (sclk_in). sclk_in) 1 2 times the maximum frequency ; to ensure that the data on the host rxd line remains stable before the host transfer logic captures the data.

The summary is as follows:

Slave ( receive only ): Fssi_clk >= 8 × (maximum Fsclk_in)

Slave machine :      Fssi_clk >= 12 × (maximum Fsclk_in)

- Provides three transmission modes: send and receive, send only, receive only

TMOD bit of configuration control register 0 (CTRLR0) .

  - Send and receive

TMOD= 2’b00, both sending and receiving logic are valid. The data transfer occurs as normal according to the selected frame format (serial protocol).Transmit data are popped from the transmit FIFO and sent through the txd line to the target device, which replies with data on the rxd line. The receive data from the target device is moved from the receive shift register into the receive FIFO at the end of each data frame.

  - send only

TMOD = 2'b01, the received data is invalid and data cannot be stored in the receiving FIFO. The data transfer occurs as normal, according to the selected frame format (serial protocol). Transmit data are popped from the transmit FIFO and sent through the txd line to the target device, which replies with data on the rxd line. At the end of the data frame, the receive shift register does not load its newly received data into the receive FIFO. The data in the receive shift register is overwritten by the next transfer. You should mask interrupts originating from the receive logic when this mode is entered.

  - only receive

TMOD = 2'b10 , sending data is invalid . When configured as a slave, the transmit FIFO is never popped in Receive Only mode. Data from a previous transfer is retransmitted from the shift register . The receive data from the target device is moved from the receive shift register into the receive FIFO at the end of each data frame. You should mask interrupts originating from the transmit logic when this mode is entered.

    - Note: When APB_SSI is disabled (SSI_EN = 0) or reset (psesetn = 0), the transmit and receive FIFOs are cleared. When the SSI Slave is in receive-only mode, there is no need to fill in the send FIFO with data. When the SSI Master is in receive and transmit or transmit-only mode, it must be ensured that there is data in the Slave's transmit FIFO before serial transmission is initiated.

- Supports three transmission interface protocols (SPI, SSP, Mwire)

For details of the agreement, please see the Datasheet of SSI Master

    - Note: When selecting the SPI protocol, the polarity and phase settings of the APB_SSI Slave serial clock must be consistent with the host, and the length setting of the transmission data frame must also be consistent with the host. for correct data transfer.

- A combined interrupt is the logical OR of individual interrupts

The APB_SSI interrupt list is as follows:

| Interrupt name | Interrupt abbreviation | Interrupt generation conditions | Interrupt clear | Remark |
| --- | --- | --- | --- | --- |
| Send FIFO empty interrupt | ssi_txe_intr | Set when the transmit FIFO is equal to or less than its limit value , and service is requested to prevent underflow. The limit value is set by the software programmable register TXFTLR to determine the level at which the transmit FIFO entry generates an interrupt. | This interrupt is cleared by hardware when data is written to send FIFO caches values greater than the threshold level . | / |
| Send FIFO overflow interrupt | ssi_txo_intr | when APB attempts to write data to an already filled transmit FIFO . | This interrupt is valid until the transmit FIFO overflow interrupt clear register (TXOICR) is read . | When valid, data written by APB is discarded. |
| Receive FIFO full interrupt | ssi_rxf_intr | Set when the receive FIFO is equal to or greater than its limit value plus one, and service is requested to prevent overflow . The limit value is set by the software programmable register RXFTLR to determine the level at which the receive FIFO entry generates an interrupt. | This interrupt is cleared by hardware when data is received from Read from the FIFO so that its value is less than the limit value ( threshold level ). | / |
| Receive FIFO overflow interrupt | ssi_rxo_intr | Set when the receive logic attempts to fill data into an already filled receive FIFO . | This interrupt is active until the receive FIFO overflow interrupt clear register (RXOICR) is read . | When valid, newly received data is discarded. |
| Receive FIFO underflow interrupt | ssi_rxu_intr | Set when an APB access attempts to read data from an empty receive FIFO . | This interrupt is valid until the receive FIFO underflow interrupt clear register (RXUICR) is read . | When valid, 0 is read from the receive FIFO . |
| Combined interrupt request | ssi_intr | Logical OR of all requested interrupts masked. | / | In order to mask this interrupt signal, all other APB_SSI interrupt requests must be masked |

4.2 Slave SPI and SSP serial transmission

A typical software flow for completing a continuous serial transfer from a serial master to the APB_SSI slave is described as follows:

1 ) . If the APB_SSI is enabled, disable it by writing 0 to SSIENR.

2 ) . Set up the APB_SSI control registers for the transfer. These registers can be set in any order.

a. Write CTRLR0 (for SPI transfers SCPH and SCPOL must be set identical to the master device).

b. Write TXFTLR and RXFTLR to set FIFO threshold levels.

c. Write the IMR register to set up interrupt masks.

3 ) . Enable the APB_SSI by writing 1 to the SSIENR register.

4 ) . If the transfer mode is transmit and receive (TMOD=2'b00) or transmit only (TMOD=2'b01), write data for transmission to the master into the transmit FIFO (Write DR).

If the transfer mode is receive only (TMOD=2'b10), there is no need to write data into the transmit FIFO; the current value in the transmit shift register is retransmitted.

5 ) . The APB_SSI slave is now ready for the serial transfer. The transfer begins when the APB_SSI slave is selected by a serial-master device.

6 ) . When the transfer is underway, the BUSY status can be polled to return the transfer status. If a transmit FIFO empty interrupt request is made, write the transmit FIFO (write DR). If a receive FIFO full interrupt request is made, read the receive FIFO (read DR).

7 ) . The transfer ends when the serial master removes the select input to the APB_SSI slave. When the transfer is completed, the BUSY status is reset to 0.

8 ) . If the transfer mode is not transmit only (TMOD != 01), read the receive FIFO until empty.

9 ) . Disable the APB_SSI by writing 0 to SSIENR.

Figure 14 shows a typical software flow for a APB_SSI slave SPI or SSP serial transfer. The diagram also shows the hardware flow inside the serial-slave component.

Figure 3: APB_SSI Slave SPI/SSP Transfer Flow

4.3 Slave Microwire Serial Transfers

When the APB_SSI is configured as a slave device, the Microwire protocol operates in much the same way as the SPI protocol. See SSI Master datasheet for details .

- The following example is pseudo code that illustrates how to use software to select the target slave.

Chapter 13 LISoC_ APB_UART

1 function list

UART is the simplest and most direct debugging tool mainly used for chips. By configuring the corresponding registers according to the transmission protocol, the relevant information can be printed out on the interface.

- AMBA 2.0 APB interface supports the implementation of AMBA SOC integration ;

- Two UART IP instances (UART0 and UART1) are integrated into the chip

- Configurable number of transmission data bits (5~8 bits);

- Configurable baud rate: baud rate = ( serial clock frequency )/(16* divisor );

- Configurable parity bits: odd, even and no parity;

- Configurable stop bits: 1, 1.5 and 2 bits;

- All registers can be configured uniformly;

- Closed-loop self-transmitting and receiving can be realized.

2 Register list

UART0 B ase address: 0 x 4 0 00 0000 UART1 B ase address: 0x4000 0400

| offset address | Register name |  | Mode | width | Detailed description |
| --- | --- | --- | --- | --- | --- |
| 0x00 | RBR_THR_DLL | RBR | R | 32 bits | Receive Buffer Register Reset Value: 0x0 Dependencies: LCR[7] bit = 0 |
|  |  | THR | W | 32 bits | Transmit Holding Register Reset Value: 0x0 Dependencies: LCR[7] bit = 0 |
|  |  | DLL | R/W | 32 bits | Divisor Latch (Low) Reset Value: 0x0 Dependencies: LCR[7] bit = 1 |
| 0x04 | DLH |  | R/W | 32 bits | Divisor Latch (High) Reset Value: 0x0 Dependencies: LCR[7] bit = 1 |
| 0x04 | IER |  | R/W | 32 bits | Interrupt Enable Register Reset Value: 0x0 Dependencies: LCR[7] bit = 0 |
| 0x08 | IIR |  | R | 32 bits | Interrupt Identification Register Reset Value: 0x01 |
|  | FCR |  | W | 32 bits | FIFO Control Register Reset Value: 0x0 |
| 0x0C | LCR |  | R/W | 32 bits | Line Control Register Reset Value: 0x0 |
| 0x10 | MCR |  | R/W | 32 bits | Modem Control Register Reset Value: 0x0 |
| 0x14 | LSR |  | R | 32 bits | Line Status Register Reset Value: 0x60 |
| 0x1C | SCR |  | R/W | 32 bits | Scratchpad Register Reset Value: 0x0 |
| 0x7C | USR |  | R | 32 bits | UART Register Status Reset Value: 0x0 |
| 0xC0 | DLF |  | R/W | 32 bits | Divisor Latch (Fraction) Reset Value: 0x0 |

3 Register detailed definition

RBR_THR_DLL (RBR)

● Name: Receive Buffer Register

● Size: 32 bits

● Address Offset: 0x00

● Read/write access: read-only

● Reset Value: 0x0

| Bits | Name | R/W | Description |
| --- | --- | --- | --- |
| 31:8 | Reserved and read as zero (Bit 8 is reserved and may not be zero) |  |  |
| 7:0 | Receive Buffer Register | R | Data byte received on the serial input port (sin) in UART mode. The data in this register is valid only if the Data Ready (DR) bit in the Line Status Register (L S R) is set. If FIFOs are disabled (FCR[0] set to 0), the data in the RBR must be read before the next data arrives, otherwise it will be overwritten, resulting in an over-run error. If FIFOs are enabled (FCR[0] set to 1), this register accesses the head of the receive FIFO. If the receive FIFO is full and this register is not read before the next data character arrives, then the data already in the FIFO will be preserved but any incoming data will be lost and an over-run error occurs. Data is only valid when LSR[0]=1 (data ready) Reset Value: 0x0 |

RBR_THR_DLL ( THR )

● Name: Transmit Holding Register

● Size: 32 bits

● Address Offset: 0x00

● Read/write access: write-only

● Reset Value: 0x0

| Bits | Name | R/W | Description |
| --- | --- | --- | --- |
| 31:8 | Reserved and read as zero |  |  |
| 7:0 | Transmit Holding Register | W | Data to be transmitted on the serial output port (sout) in UART mode. Data should only be written to the THR when the THR Empty (THRE) bit (LSR[5]) is set , the output port sout(TX), the status register LSR[5] controls the transmission , and LSR[6] transmitter status confirmation whether the transmission is completed If FIFO's are disabled (FCR[0] set to zero) and THRE is set, writing a single character to the THR clears the THRE. Any additional writes to the THR before the THRE is set again causes the THR data to be overwritten. If FIFO's are enabled (FCR[0] set to one) and THRE is set, 16 number of characters of data may be written to the THR before the FIFO is full. Any attempt to write data when the FIFO is full results in the write data being lost. Reset Value: 0x0 |

RBR_THR_ DLL (DLL)

● Name: Divisor Latch Low

● Size: 32 bits

● Address Offset: 0x00

● Read/write access: read/write

● Reset Value: 0x0

| Bits | Name | R/W | Description |
| --- | --- | --- | --- |
| 31:8 | Reserved and read as zero |  |  |
| 7:0 | Divisor Latch (Low) | R/W | Lower 8 bits of a 16-bit, read/write, Divisor Latch register that contains the baud rate divisor for the UART. This register may only be accessed when the DLAB bit (LCR[7]) is set and the UART is not busy (USR[0] is zero). The output baud rate is equal to the serial clock (sclk) frequency divided by sixteen times the value of the baud rate divisor, as follows: baud rate = (serial clock freq)/(16 * d ivisor). Note that with the Divisor Latch Registers (DLL and DLH) set to zero, the baud clock is disabled and no serial communications occur. Also, once the DLL is set, at least 8 clock cycles of the slowest LISoCapb_uart clock should be allowed to pass before transmitting or receiving data. Reset Value: 0x0 |

DLH

● Name: Divisor Latch High

● Size: 32 bits

● Address Offset: 0x04

● Read/write access: read/write

● Reset Value: 0x0

| Bits | Name | R/W | Description |
| --- | --- | --- | --- |
| 31:8 | Reserved and read as zero |  |  |
| 7:0 | Divisor Latch (High) | R/W | Upper 8-bits of a 16-bit, read/write, Divisor Latch register that contains the baud rate divisor for the UART. This register may only be accessed when the DLAB bit (LCR[7]) is set and the UART is not busy (USR[0] is zero). The output baud rate is equal to the serial clock (sclk) frequency divided by sixteen times the value of the baud rate divisor, as follows: baud rate = (serial clock freq) /(16 * divisor). Note that with the Divisor Latch Registers (DLL and DLH) set to zero, the baud clock is disabled and no serial communications occur. Also, once the DLH is set, at least 8 clock cycles of the slowest LISoC_apb_uart clock should be allowed to pass before transmitting or receiving data. Reset Value: 0x0 |

IER

● Name: Interrupt Enable Register

● Size: 32 bits

● Address Offset: 0x04

● Read/write access: read/write

● Reset Value: 0x0

| Bits | Name | R/W | Description |
| --- | --- | --- | --- |
| 31: 8 | Reserved and read as zero |  |  |
| 7 | PTIME | R/W | Programmable THRE Interrupt Mode Enable. This is used to enable/disable the generation of THRE Interrupt. Values: ■0x0 (DISABLED): Disable Programmable THRE Interrupt Mode ■0x1 (ENABLED): Enable Programmable THRE Interrupt Mode Reset Value: 0x0 |
| 6 : 4 | Reserved and read as zero |  |  |
| 3 | EDSSI | R/W | Enable Modem Status Interrupt. This is used to enable/disable the generation of Modem Status Interrupt. This is the fourth highest priority interrupt. Values: ■0x0 (DISABLED): Disable Modem Status Interrupt ■0x1 (ENABLED): Enable Modem Status Interrupt Reset Value: 0x0 |
| 2 | ELSI | R/W | Enable Receiver Line Status Interrupt. This is used to enable/disable the generation of Receiver Line Status Interrupt. This is the highest priority interrupt. Values: ■0x0 (DISABLED): Disable Receiver Line Status Interrupt ■0x1 (ENABLED): Enable Receiver Line Status Interrupt Reset Value: 0x0 |
| 1 | ETBEI | R/W | Enable Transmit Holding Register Empty Interrupt. This is used to enable/disable the generation of Transmitter Holding Register Empty Interrupt. This is the third highest priority interrupt. Values: ■ 0x0 (DISABLED): Disable Transmit empty interrupt ■0x1 (ENABLED): Enable Transmit empty interrupt Reset Value: 0x0 |
| 0 | ERBFI | R/W | Enable Received Data Available Interrupt. This is used to enable/disable the generation of Received Data Available Interrupt and the Character Timeout Interrupt (if FIFO's enabled). These are the second highest priority interrupts. Values: ■0x0 (DISABLED): Disable Receive data Interrupt ■0x1 (ENABLED): Enable Receive data Interrupt Reset Value: 0x0 |

IIR

● Name: Interrupt Identification Register

● Size: 32 bits

● Address Offset: 0x08

● Read/write access: read only

● Reset Value: 0x1

| Bits | Name | R/W | Description |
| --- | --- | --- | --- |
| 31: 4 | Reserved and read as zero (bit 7:6 for test purpose and may not be equal to 0) |  |  |
| 3 :0 | ID | R | Interrupt ID. It indicates the highest priority pending interrupt which can be one of the following types specified in Values. Bit 3 indicates an interrupt can only occur when the FIFOs are enabled and used to distinguish a Character Timeout condition interrupt. Values: ■ 0x0 (MODEM_STATUS): modem status ■ 0x1 (NO_INTERRUPT_PENDING): no interrupt pending ■ 0x2 (THR_EMPTY): THR empty ■ 0x4 (RECEIVED_DATA_AVAILABLE): received data available ■ 0x6 (RECEIVER_LINE_STATUS): receiver line status ■ 0x7 (BUSY_DETECT): busy detect ■ 0xc (CHARACTER_TIMEOUT): character timeout Reset Value: 0x1 |

FCR

● Name: FIFO Control Register

● Size: 32 bits

● Address Offset: 0x08

● Read/write access: write only

● Reset Value: 0x0

| Bits | Name | R/W | Description |
| --- | --- | --- | --- |
| 31: 8 | Reserved and read as zero |  |  |
| 7:6 | RT | W | RCVR Trigger (or RT). Select the trigger level in the receiver FIFO at which the Received Data Available Interrupt will be generated. Values: ■0x0 (FIFO_CHAR_1): 1 character in FIFO ■0x1 (FIFO_QUARTER_FULL): FIFO 1/4 full ■0x2 (FIFO_HALF_FULL): FIFO 1/2 full ■0x3 (FIFO_FULL_2): FIFO 2 less than full Reset Value: 0x0 |
| 5:4 | TET | W | TX Empty Trigger (or TET). This is used to select the empty threshold level at which the THRE Interrupts will be generated when the mode is active. Values: ■0x0 (FIFO_EMPTY): FIFO Empty ■0x1 (FIFO_CHAR_2): 2 characters in FIFO ■ 0x2 (FIFO_QUARTER_FULL): FIFO 1/4 full ■0x3 (FIFO_HALF_FULL): FIFO 1/2 full Reset Value: 0x0 |
| 3 | DMAM | W | DMA Mode (or DMAM). This bit is reserved. Always write 0 to this bit. Reset Value: 0x0 |
| 2 | XFIFOR | W | XMIT FIFO Reset (or XFIFOR). This resets the control portion of the transmit FIFO and treats the FIFO as empty. Note that this bit is 'self-clearing' and it is not necessary to clear this bit. Values: ■0x1 (RESET): Transmit FIFO reset Reset Value: 0x0 |
| 1 | RFIFOR | W | RCVR FIFO Reset (or RFIFOR). This resets the control portion of the receive FIFO and treats the FIFO as empty. Note that this bit is 'self-clearing' and it is not necessary to clear this bit. Values: ■0x1 (RESET): Receive FIFO reset Reset Value: 0x0 |
| 0 | FIFOE | W | FIFO Enable (or FIFOE). This enables/disables the transmit (XMIT) and receive (RCVR) FIFOs. Whenever the value of this bit is changed both the XMIT and RCVR controller portion of FIFOs is reset. Values: ■0x0 (DISABLED): FIFO disabled ■0x1 (ENABLED): FIFO enabled Reset Value: 0x0 |

LCR

● Name: Line Control Register

● Size: 32 bits

● Address Offset: 0x0C

● Read/write access: read/write

● Reset Value: 0x0

| Bits | Name | R/W | Description |
| --- | --- | --- | --- |
| 31:8 | Reserved and read as zero |  |  |
| 7 | DLAB | R/W | Divisor Latch Access Bit. Writeable only when UART is not busy (USR[0] is zero). This bit is used to enable reading and writing of the Divisor Latch register (DLL and DLH) to set the baud rate of the UART. This bit must be cleared after initial baud rate setup in order to access other registers . Reset Value: 0x0 |
| 6 | Break (or BC) | R/W | Break Control Bit. This is used to cause a break condition to be transmitted to the receiving device. If set to one the serial output is forced to the spacing (logic 0) state. When not in Loopback Mode, as determined by MCR[4], the sout line is forced low until the Break bit is cleared. Values: ■0x0 (DISABLED): Serial output is released for data transmission ■0x1 (ENABLED): Serial output is forced to spacing state Reset Value: 0x0 |
| 5 | Stick Parity | Reserved and read as zero |  |
| 4 | EPS | R/W | Even Parity Select. Writeable only when UART is not busy (USR[0] is zero). This is used to select between even and odd parity, when parity is enabled (PEN set to one). If set to one, an even number of logic 1s is transmitted or checked. If set to zero, an odd number of logic 1s is transmitted or checked. Reset Value: 0x0 |
| 3 | PEN | R/W | Parity Enable. Writeable only when UART is not busy (USR[0] is zero). This bit is used to enable and disable parity generation and detection in transmitted and received serial character respectively. 0 = parity disabled 1 = parity enabled Reset Value: 0x0 |
| 2 | STOP | R/W | Number of stop bits. Writeable only when UART is not busy (USR[0] is zero). This is used to select the number of stop bits per character that the peripheral transmits and receives. If set to zero, one stop bit is transmitted in the serial data. If set to one and the data bits are set to 5 (LCR[1:0] set to zero) one and a half stop bits is transmitted. Otherwise, two stop bits are transmitted. Note that regardless of the number of stop bits selected, the receiver checks only the first stop bit. 0 = 1 stop bit 1 = 1.5 stop bits when DLS (LCR[1:0]) is zero, else 2 stop bits Reset Value: 0x0 |
| 1:0 | DLS (or CLS, as sed in legacy) | R/W | Data Length Select. Writeable only when UART is not busy (USR[0] is zero). This is used to select the number of data bits per character that the peripheral transmits and receives. The number of bit that may be selected are as follows: 00 = 5 bits 01 = 6 bits 10 = 7 bits 11 = 8 bits Reset Value: 0x0 |

MCR

● Name: Modem Control Register

● Size: 32 bits

● Address Offset: 0x10

● Read/write access: read/write

● Reset Value: 0x0

| Bits | Name | R/W | Description |
| --- | --- | --- | --- |
| 31: 5 | Reserved and read as zero |  |  |
| 4 | Loopback | R/W | LoopBack Bit This is used to put the UART into a diagnostic mode for test purposes. If operating, data on the sout line is held high, while serial data output is looped back to the sin line, internally. In this mode all the interrupts are fully functional . Values: ■0x0 (DISABLED): Loopback mode disabled ■0x1 (ENABLED): Loopback mode enabled Reset Value: 0x0 |
| 3: 0 | Reserved and read as zero |  |  |

LSR

● Name: Line Status Register

● Size: 32 bits

● Address Offset: 0x14

● Read/write access: read-only

● Reset Value: 0x60

| Bits | Name | R/W | Description |
| --- | --- | --- | --- |
| 31:8 | Reserved and read as zero |  |  |
| 7 | RFE | R | Receiver FIFO Error bit. This bit is only relevant when FIFO's are enabled (FCR[0] set to one). This is used to indicate if there is at least one parity error, framing error, or break indication in the FIFO. This bit is cleared when the LSR is read and the character with the error is at the top of the receiver FIFO and there are no subsequent errors in the FIFO. Values: ■0x0 (NO_RX_FIFO_ERROR): No error in RX FIFO ■0x1 (RX_FIFO_ERROR): Error in RX FIFO Reset Value: 0x0 |
| 6 | TEMT | R | Transmitter Empty bit. If FIFO's are enabled (FCR[0] set to one), this bit is set whenever the Transmitter Shift Register and the FIFO are both empty. If FIFO's are disabled, this bit is set whenever the Transmitter Holding Register and the Transmitter Shift Register are both empty. Values: ■0x0 (DISABLED): Transmitter not empty ■0x1 (ENABLED): Transmitter empty Reset Value: 0x1 |
| 5 | THRE | R | Transmit Holding Register Empty bit. If THRE_MODE_USER == Disabled or THRE mode is disabled (IER[7] set to zero) and regardless of FIFO's being implemented/enabled or not, this bit indicates that the THR or TX FIFO is empty. This bit is set whenever data is transferred from the THR or TX FIFO to the transmitter shift register and no new data has been written to the THR or TX FIFO. This also causes a THRE Interrupt to occur, if the THRE Interrupt is enabled. Transmit Holding Register Empty bit. If THRE mode is disabled (IER[7] set to zero) and regardless of FIFO's being implemented/enabled or not, this bit indicates that the THR or TX FIFO is empty. This bit is set whenever data is transferred from the THR or TX FIFO to the transmitter shift register and no new data has been written to the THR or TX FIFO. This also causes a THRE Interrupt to occur, if the THRE Interrupt is enabled. If both modes are active (IER[7] set to one and FCR[0] set to one respectively), the functionality is switched to indicate the transmitter FIFO is full, and no longer controls THRE interrupts, which are then controlled by the FCR[ 5:4] threshold setting. Values: ■0x0 (DISABLED): THRE interrupt control is disabled ■0x1 (ENABLED): THRE interrupt control is enabled Reset Value: 0x1 |
| 4 | BI | R | Break Interrupt bit. This is used to indicate the detection of a break sequence on the serial input data. It is set whenever the serial input, sin, is held in a logic '0' state for longer than the sum of start time + data bits + parity + stop bits. A break condition on serial input causes one and only one character, consisting of all zeros, to be received by the UART. In the FIFO mode, the character associated with the break condition is carried through the FIFO and is revealed when the character is at the top of the FIFO. Reading the LSR or RBR clears the BI bit. In the non-FIFO mode, the BI indication occurs immediately and persists until the LSR is read. Note: If a FIFO is full when a break condition is received, a FIFO overrun occurs. The break condition and all the information associated with it-parity and framing errors-is discarded; any information that a break character was received is lost. Values: ■0x0 (NO_BREAK): No break sequence detected ■0x1 (BREAK): Break sequence detected . Reset Value: 0x0 |
| 3 | FE | R | Framing Error bit. This is used to indicate the occurrence of a framing error in the receiver. A framing error occurs when the receiver does not detect a valid STOP bit in the received data. In the FIFO mode, since the framing error is associated with a character received, it is revealed when the character with the framing error is at the top of the FIFO. When a framing error occurs the UART will try resynchronize. It does this by assuming that the error was due to the start bit of the next character and then continues receiving the other bit ie data, and/or parity and stop. It should be noted that the Framing Error (FE) bit (LSR[3]) will be set if a break interrupt has occurred, as indicated by Break Interrupt (BI) bit (LSR[4]). This happens because the break character implicitly generates a framing error by holding the sin input to logic 0 for longer than the duration of a character. Reading the LSR or RBR clears the FE bit. Values: ■0x0 (NO_FRAMING_ERROR): no framing error ■0x1 (FRAMING_ERROR): framing error Reset Value: 0x0 |
| 2 | PE | R | Parity Error bit. This is used to indicate the occurrence of a parity error in the receiver if the Parity Enable (PEN) bit (LCR[3]) is set. In the FIFO mode, since the parity error is associated with a character received, it is revealed when the character with the parity error arrives at the top of the FIFO. It should be noted that the Parity Error (PE) bit (LSR[2]) will be set if a break interrupt has occurred, as indicated by Break Interrupt (BI) bit (LSR[4]). In this situation, the Parity Error bit is set if parity generation and detection is enabled (LCR[3]=1) and the parity is set to odd (LCR[4]=0). Reading the LSR or RBR clears the PE bit. Values: ■0x0 (NO_PARITY_ERROR): no parity error ■0x1 (PARITY_ERROR): parity error Reset Value: 0x0 |
| 1 | OE | R | Overrun error bit. This is used to indicate the occurrence of an overrun error. This occurs if a new data character was received before the previous data was read. In the non-FIFO mode, the OE bit is set when a new character arrives in the receiver before the previous character was read from the RBR. When this happens, the data in the RBR is overwritten. In the FIFO mode, an overrun error occurs when the FIFO is full and a new character arrives at the receiver. The data in the FIFO is retained and the data in the receive shift register is lost. Reading the LSR or RBR clears the OE bit. Values: ■0x0 (NO_OVER_RUN_ERROR): no overrun error ■0x1 (OVER_RUN_ERROR): overrun error Reset Value: 0x0 |
| 0 | DR | R | This is used to indicate that the receiver contains at least one character in the RBR or the receiver FIFO. This bit is cleared when the RBR is read in the non-FIFO mode, or when the receiver FIFO is empty, in the FIFO mode. 0 = no data ready 1 = data ready Reset Value: 0x0 |

SCR

● Name: Scratchpad Register

● Size: 32 bits

● Address Offset: 0x1C

● Read/write access: read/write

● Reset Value: 0x0

| Bits | Name | R/W | Description |
| --- | --- | --- | --- |
| 31: 8 | Reserved and read as zero |  |  |
| 7:0 | SCR | R/W | This register is for programmers to use as a temporary storage space. It has no defined purpose in the LISoC_apb_uart. Reset Value: 0x0 |

USR

● Name: UART Status Register

● Size: 32 bits

● Address Offset: 0x7C

● Read/write access: read-only

● Reset Value: 0x0

| Bits | Name | R/W | Description |
| --- | --- | --- | --- |
| 31:5 | Reserved and read as zero |  |  |
| 4 | RFF | R | Receive FIFO Full. (Not implemented. Always read as 0) Reset Value: 0x0 |
| 3 | RFNE | R | Receive FIFO Not Empty. (Not implemented. Always read as 0) Reset Value: 0x0 |
| 2 | TFE | R | Transmit FIFO Empty. (Not implemented. Always read as 0) Reset Value: 0x0 |
| 1 | TNF | R | Transmit FIFO Not Full. (Not implemented. Always read as 0) Reset Value: 0x0 |
| 0 | BUSY | R | UART Busy. This bit indicates that a serial transfer is in progress, when cleared, indicates that the LISoC_apb_uart is idle or inactive. This bit will be set to 1 (busy) under any of the following conditions: - Transmission in progress on serial interface - Transmit data present in THR, when the baud divisor is non-zero ({DLH,DLL} does not equal 0) when the divisor latch access bit is 0 (LCR.DLAB = 0) - Reception in progress on the interface -Receive data present in RBR Note: It is possible for the UART Busy bit to be cleared even though a new character may have been sent from another device. That is, if the LISoC_apb_uart has no data in the THR and RBR and there is no transmission in progress and a start bit of a new character has just reached the LISoC_apb_uart. This is due to the fact that a valid start is not seen until the middle of the bit period and this duration is dependent on the baud divisor that has been programmed. The assertion of this bit may also be delayed by several cycles of the slower clock. Values: ■0x0 (IDLE): LISoC_apb_uart is idle or inactive ■0x1 (BUSY): LISoC_apb_uart is busy (actively transferring data) Reset Value: 0x0 |

DLF

● Name: Divisor Latch (Fraction) Register

● Size: 32 bits

● Address Offset: 0xC0

● Read/write access: read/write

● Reset Value: 0x0

| Bits | Name | R/W | Description |
| --- | --- | --- | --- |
| 31: 4 | Reserved and read as zero |  |  |
| 3:0 | DLF | R/W | Fractional part of divisor. The fractional value is added to integer value set by DLH, DLL. Fractional value is determined by (Divisor Fraction value)/16. Reset Value: 0x0 |

4 Program Guide

Program is mainly divided into two parts: sending and receiving data:

UART data transfer protocol:

S serial protocol

- Send data:

1. Set baudrate . According to the required baud rate, configure the divisor on the registers RBR_THR_ DLL and DLH to get the required baud rate;

The divisor is obtained according to the formula baudrate = ( serial clock frequency )/(16* divisor ) ;

For example: known serial clock frequency : 20MHz, baudrate: 9600

divisor = 20M/(9600*16) ≈ 130 = 0x82

So the lower eight bits of divisor DLL=0x82, the upper eight bits DLH=0x00

Specific setup process:

(1). Query the USR[0] bit of the uart register. If it is " 0”, confirm it is idle; (optional)

(2). Set the register LCR to zero;

(3). Calculate the divisor according to the above formula to set RBR_THR_ DLL and DLH respectively.

If you need to use a more precise baud rate, you can set DLF together with DLL and DLH .

(4).Register LCR is set to zero.

2. Set the data length datasize , configure the register LCR, the optional data length is 5~8 bits;

Specific setup process:

(1). Query the USR[0] bit of the uart register. If it is " 0”, confirm it is idle; (optional)

(2) .Clear the register LCR;

(3). Set LCR[1:0] and select the setting according to the actual required data length.

00 = 5 bits , 01 = 6 bits , 10 = 7 bits , 11 = 8 bits

3. Set the parity bit , configure the register LCR, and select odd, even and no parity bits;

Specific setup process:

(1). Query the USR[0] bit of the uart register. If it is " 0”, confirm it is idl; (optional)

(2).Clear the register LCR;

(3). Parity check enable setting, set LCR[3] (PEN: Parity Enable ),

means " 1”with parity check, otherwise, there is no parity check"

(4). Parity selection setting, set LCR[4] (EPS: Even Parity Select )

" 1”is even parity, " 0”is odd parity

4. Set the number of stop bits stopbit , for register LCR configuration, optional 1, 1.5 and 2;

Specific setup process:

(1). Query the USR[0] bit of the uart register. If it is " 0”, confirm it is idle; (optional)

(2).Set LCR[2]( Number of stop bits ) and select the setting according to the required requirements

" 0”is 1 stop bit

" 1”When the data bits are 5 bits, it means 1.5 bits of stop bits

When the data bit is 6, 7 or 8, it represents a 2-bit stop bit.

5.uart configuration config, overall configuration of the register LCR,

This configuration is mainly for the settings of parity\datazise\stopbit. Since the three are settings for different bits of LCR, they are configured together to prevent errors caused by multiple operations on the same register;

6.uart initializes init and configures baudrate\parity\datazise\stopbit, where baudrate is an independent configuration, and the other three can be called directly (5);

7. Send data , write data to the register RBR_THR_DLL , and determine whether the data has been sent by reading the status of the status register LSR;

Specific process:

(1). Query the status register bit LSR[5] (THRE: Transmit Holding Register Empty bit ), ” 1”means the register RBR_THR_DLL is empty and data can be written

(2). Write data to register RBR_THR_DLL

(3). Confirm that the data transmission is complete, query the register bit LSR[6] ( Transmitter Empty bit. ), "" 1”indicates that the transmitter has completed sending data

- Receive data:

- The configuration part is the same as sending data 1~4;

- Receive data , determine by reading the status of the register LSR, and read the data from the register RBR;

Specific process:

(1). Query status register bit LSR[0]( DR: Data Ready bit ), ” 1”indicates that the data is ready and the data can be read; otherwise, continue to query and wait;

obtain the data by reading the value of register RBR_THR_DLL

(3). Confirm that the reception is completed, query LSR[0], if it is ", 0”it means the data reception is completed.

- Precautions:

1. Baud rate setting, the current baud rate is tentatively set with the CPU frequency of 20Mhz.

If the frequency changes in the future, please reconfigure it according to the existing formula and calculation method;

2. There is 16 byte FIFO in LISoC_apb_UART; currently all transfers only support query mode

3. Since the current hardware environment does not support the transmission of character data, all transmissions are integer data, so it is more

High-level driver development and application require some changes to the sending and receiving functions.

- This version adds uart configuration config and uart initialization init, where config integrates the configurations 2, 3, and 4 into one to avoid multiple operations of the register, so the configurations 2, 3, and 4 are not needed, and config is used directly. Yes; in addition, initialization init is the call of baudrate and config.

The following figure shows an example of the configuration:

Picture attached uart configuration example

Chapter 14 LISoC_APB_GPIO

1Function list

The GPIO module mainly provides SOC chips with rich I/O pin resources for data communication with peripherals. It has two major functions: I/O and interrupt.

● Two ports :Port A to Port B. Each port owns i independently configurable 16 signals . And each signal owns separate data register and data direction register .

● Both ports support interrupt mode

● Support debounce logic for input interrupt s to remove the glitch

● Support synchronization of interrupt (only level-sensitive)signals

● Support four interrupt input types: high -level,low-level,rising-edge,falling-edge

● I nterrupt output(to I n terrupt Controller) is active-high and 1 bit

● Support interrupt mask and clear (edge-sensitive)

2 Register list

Port A Base address：0x4000 3400

Port B Base address：0x4000 3800

| Offset | Register | R/W | Width | Description |
| --- | --- | --- | --- | --- |
| 0x00 | GPIOx_SWPORT_DR(x=A,B) | R/W | 32bits | data register ResetValue: 0x0 |
| 0x04 | GPIOx_SWPORT_DDR(x=A,B) | R/W | 32bits | data direction register Reset Value: 0x0 |
| 0x30 | GPIOx_INTEN(x=A,B) | R/W | 32 bits | Interrupt enable register Reset Value: 0x0 |
| 0x34 | GPIOx_INTMASK(x=A,B) | R/W | 32 bits | Interrupt mask register Reset Value: 0x0 |
| 0x38 | GPIOx_INTTYPE_LEVEL(x=A,B) | R/W | 32bits | Interrupt level register Reset Value: 0x0 |
| 0x3c | GPIOx_INT_POLARITY(x=A,B) | R/W | 32 bits | Interrupt polarity register Reset Value: 0x0 |
| 0x40 | GPIOx_INTSTATUS(x=A,B) | R | 32bits | Interrupt status register Reset Value: 0x0 |
| 0x44 | GPIOx_RAW_INTSTATUS(x=A,B) | R | 32 bits | Raw interrupt status (premasking) Reset Value: 0x0 |
| 0x48 | GPIOx_DEBOUNCE(x=A,B) | R/W | 32 bits | Debounce enable register Reset Value: 0x0 |
| 0x04c | GPIOx_PORT_EOI(x=A,B) | W | 32 bits | clear interrupt register Reset Value: 0x0 |
| 0x60 | GPIOx_LS_SYNC(x=A,B) | R/W | 32 bit | Level-sensitive synchronization enable register Reset Value: 0x0 |
| 0x68 | GPIOx_INT_BOTHEDGE(x=A,B) | R/W | 32bit | Interrupt Both Edge type Reset Value: 0x0 |
| 0x6c | GPIOx_VER_ID_CODE(x=A,B) | R | 32 bits | Component Version register Reset Value: 0x3231_332a |

3 Detailed definition of registers

- GPIOx_SWPORT_D R(x=A,B)

● Name: Data Register

● Description: Data Register

● Size: 32 bits

● Address Offset: 0x00

● Read/write access: read/write

● Reset Value: 0x0

| 31:16 | 15:0 |
| --- | --- |
| RSVD_GPIO_SWPORT_DR | GPIO_SWPORT_DR |

Table 3-1 Fields for Register: GPIOx_SWPORTA_DR(x=A,B)

| Bits | Name | R/W | Description |
| --- | --- | --- | --- |
| 31:16 | RSVD_GPIO_SWPORT_ D R | R | Reserved bits Reset Value: 0x0 |
| 15 :0 | GPIO_SWPORT_DR | R/W | Values written to this register are output on the I/O signals for Port if the corresponding data direction bits for Port are set to Output mode . The value read back is equal to the last value written to this register Reset Value: 0x0 |

- GPIOx_SWPORT_DDR (x=A,B)

● Name: Data Direction Register

● Description: Data Direction Register

● Size: 32 bits

● Address Offset: 0x04

● Read/write access: read/write

● Reset Value: 0x0

| 31:16 | 15:0 |
| --- | --- |
| RSVD_GPIO_SWPORT_D D R | GPIO_SWPORT_D D R |

Table 3-2Fields for Register: GPIOx_SWPORTA_DDR(x=A,B)

| Bits | Name | R/W | Description |
| --- | --- | --- | --- |
| 31:16 | RSVD_GPIO_SWPORT_D D R | R | Reserved bits Reset Value: 0x0 |
| 15 :0 | GPIO_SWPORT_D D R | R/W | Values written to this register independently control the direction of the corresponding data bit in Port Values: ■ 0 – Input (default) ■ 1 – Output Reset Value: 0x0 |

- GPIOx_INTEN (x=A,B)

● Name: Interrupt enable register

● Description: Interrupt enable register

● Size: 32 bits

● Address Offset: 0x30

● Read/write access: read/write

● Reset Value: 0x0

| 31:16 | 15:0 |
| --- | --- |
| RSVD_GPIO_INTEN | GPIO_INTE |

Table 3-3Fields for Register: GPIOx_INTEN (x=A,B)

| Bits | Name | R/W | Description |
| --- | --- | --- | --- |
| 31:16 | RSVD_GPIO_INTEN | R | Reserved bits Reset Value: 0x0 |
| 15 :0 | GPIO_INTE | R/W | Allows each bit of Port to be configured for interrupts. By default the generation of interrupts is disabled. Whenever a 1 is written to a bit of this register, it configures the corresponding bit on Port to become an interrupt; Otherwise, Port operates as a normal GPIO signal. Values: ■ 0 – Configure Port bit as normal GPIO signal (default) ■ 1 – Configure Port bit as interrupt Reset Value: 0x0 |

- GPIOx_INTMASK (x=A,B)

● Name: Interrupt mask reigster

● Description: Interrupt mask reigster

● Size: 32 bits

● Address Offset: 0x34

● Read/write access: read/write

● Reset Value: 0x0

| 31:16 | 15:0 |
| --- | --- |
| RSVD_GPIO_INTMASK | GPIO_INTMASK |

Table 3-4Fields for Register: GPIOx_INTMASK (x=A,B)

| Bits | Name | R/W | Description |
| --- | --- | --- | --- |
| 31:16 | RSVD_GPIO_INTMASK | R | Reserved bits Reset Value: 0x0 |
| 15 :0 | GPIO_INTMASK | R/W | Controls whether an interrupt on Port can create an interrupt for the interrupt controller by not masking it. By default, all interrupts bits are unmasked. Whenever a 1 is written to a bit in this register, it masks the interrupt generation capability for this signal; otherwise interrupts are allowed through. The unmasked status can be read as well as the resultant status after masking Values: ■ 0 – Interrupt bits are unmasked (default) ■ 1 – Mask interrupt Reset Value: 0x0 |

- GPIOx_INTTYPE_LEVEL (x=A,B)

● Name: Interrupt level

● Description: Interrupt level

● Size: 32 bits

● Address Offset: 0x38

● Read/write access: read/write

● Reset Value: 0x0

| 31:16 | 15:0 |
| --- | --- |
| RSVD_GPIO_INTTYPE_LEVEL | GPIO_INTTYPE_LEVEL |

Table 3-5Fields for Register: GPIOx_INTTYPE_LEVEL (x=A,B)

| Bits | Name | R/W | Description |
| --- | --- | --- | --- |
| 31:16 | RSVD_GPIO_INTTYPE_LEVEL | R | Reserved bits Reset Value: 0x0 |
| 15 :0 | GPIO_INTTYPE_LEVEL | R/W | Controls the type of interrupt that can occur on Port. Whenever a 0 is written to a bit of this register, it configures the interrupt type to be level-sensitive; otherwise, it is edge-sensitive Values: ■ 0 – Level-sensitive (default) ■ 1 – Edge-sensitive Reset Value: 0x0 |

- GPIOx_INT_POLARITY (x=A,B)

● Name: Interrupt polarity

● Description: Interrupt polarity

● Size: 32bits

● Address Offset: 0x3c

● Read/write access: read/write

● Reset Value: 0x0

| 31:16 | 15:0 |
| --- | --- |
| RSVD_GPIO_INT_POLARITY | GPIO_INT_POLARITY |

Table 3-6Fields for Register: GPIOx_INT_POLARITY(x=A,B)

| Bits | Name | R/W | Description |
| --- | --- | --- | --- |
| 31:16 | RSVD_GPIO_INT_POLARITY | R | Reserved bits Reset Value: 0x0 |
| 15 :0 | GPIO_INT_POLARITY | R/W | Controls the polarity of edge or level sensitivity that can occur on input of Port. Whenever a 0 is written to a bit of this register, it configures the interrupt type to falling-edge or active-low sensitive; otherwise, it is rising-edge or active-high sensitive Values: ■ 0 – Active-low (default) ■ 1 – Active-high Reset Value: 0x0 |

- GPIOx_INTSTATUS (x=A,B)

● Name: Interrupt status

● Description: Interrupt status

● Size: 32 bits

● Address Offset: 0x40

● Read/write access: read

● Reset Value: 0x0

| 31:16 | 15:0 |
| --- | --- |
| RSVD_GPIO_INTSTATUS | GPIO_INTSTATUS |

Table 3-7Fields for Register: GPIOx_INTSTATUS (x=A,B)

| Bits | Name | R/W | Description |
| --- | --- | --- | --- |
| 31:16 | RSVD_GPIO_INTSTATUS | R | Reserved bits Reset Value: 0x0 |
| 15 :0 | GPIO_INTSTATUS | R | Interrupt status of Port Values: ■0x0 (INACTIVE): Inactive ■0x1 (ACTIVE): Active Reset Value: 0x0 |

- GPIOx_RAW_INTSTATUS (x=A,B)

● Name: Raw interrupt status

● Description: Raw interrupt status

● Size: 32bits

● Address Offset: 0x44

● Read/write access: read

● Reset Value: 0x0

| 31:16 | 15:0 |
| --- | --- |
| RSVD_GPIO_RAW_INTSTATUS | GPIO_RAW_INTSTATUS |

Table 3-8Fields for Register: GPIOx_RAW_INTSTATUS (x=A,B)

| Bits | Name | R/W | Description |
| --- | --- | --- | --- |
| 31:1 6 | RSVD_GPIO_RAW_INTSTATUS | R | Reserved bits Reset Value: 0x0 |
| 15 :0 | GPIO_RAW_INTSTATUS | R | Raw interrupt of status of Port (premasking bits) Values: ■0x0 (INACTIVE): Inactive ■0x1 (ACTIVE): Active Reset Value: 0x0 |

- GPIOx_DEBOUNCE (x=A,B)

● Name: Debounce enable

● Description: Debounce enable

● Size: 32 bits

● Address Offset: 0x48

● Read/write access: read/write

● Reset Value: 0x0

| 31:16 | 15:0 |
| --- | --- |
| RSVD_GPIO_DEBOUNCE | GPIO_DEBOUNCE |

Table 3-9Fields for Register: GPIOx_DEBOUNCE (x=A,B)

| Bits | Name | R/W | Description |
| --- | --- | --- | --- |
| 31:1 6 | RSVD_GPIO_DEBOUNCE | R | Reserved bits Reset Value: 0x0 |
| 15 :0 | GPIO_DEBOUNCE | R/W | Controls whether an external signal that is the source of an interrupt needs to be debounced to remove any spurious glitches. Writing a 1 to a bit in this register enables the debouncing circuitry. A signal must be valid for two periods of an external clock before it is internally processed Values: ■ 0 – No debounce (default) ■ 1 – Enable debounce Reset Value: 0x0 |

- GPIOx_PORTA_EOI (x=A,B)

● Name: Clear interrupt

● Description: Clear interrupt

● Size: 32 bits

● Address Offset: 0x4c

● Read/write access: write

● Reset Value: 0x0

| 31:16 | 16:0 |
| --- | --- |
| RSVD_GPIO_PORTA_EOI | GPIO_PORTA_EOI |

Table 3-10Fields for Register: GPIOx_PORTA_EOI(x=A,B)

| Bits | Name | R/W | Description |
| --- | --- | --- | --- |
| 31:1 6 | RSVD_GPIO_PORTA_EOI | R | Reserved bits Reset Value: 0x0 |
| 15 :0 | GPIO_PORTA_EOI | W | Controls the clearing of edge type interrupts from Port. When a 1 is written into a corresponding bit of this register, the interrupt is cleared. All interrupts are cleared when Port A is not configured for interrupts Values: ■ 0 – No interrupt clear (default) ■ 1 – Clear interrupt Reset Value: 0x0 |

- GPIOx_EXT_PORT (x=A,B)

● Name: External Port

● Description: External Port

● Size: 32 bits

● Address Offset: 0x50

● Read/write access: read

● Reset Value: 0x0

| 31:16 | 15:0 |
| --- | --- |
| RSVD_GPIO_EXT_PORT | GPIO_EXT_PORT |

Table 3-11Fields for Register: GPIOx_EXT_PORTA (x=A,B)

| Bits | Name | R/W | Description |
| --- | --- | --- | --- |
| 31:1 6 | RSVD_GPIO_EXT_PORT | R | Reserved bits Reset Value: 0x0 |
| 15 :0 | GPIO_EXT_PORT | R | When Port is configured as Input, then reading this location reads the values on the signal. When the data direction of Port is set as Output, reading this location reads the data register for Port Reset Value: 0x0 |

- GPIOx_LS_SYNC (x=A,B)

● Name: Synchronization level

● Description:

● Size: 1 bit

● Address Offset: 0x60

● Read/write access: read/write

● Reset Value: 0x0

| 31: 1 | 0 |
| --- | --- |
| RSVD_GPIO_LS_SYNC | GPIO_LS_SYNC |

Table 3-12Fields for Register: GPIOx_LS_SYNC (x=A,B)

| Bits | Name | R/W | Description |
| --- | --- | --- | --- |
| 31:1 | RSVD_GPIO_LS_SYNC | R | Reserved bits Reset Value: 0x0 |
| 0 | GPIO_LS_SYNC | R/W | Writing a 1 to this register results in all level-sensitive interrupts being synchronized to pclk_intr Values: ■ 0 – No synchronization to pclk_intr (default) ■ 1 – Synchronize to pclk_intr Reset Value: 0x0 |

- GPIOx_INT_BOTHEDGE(x=A,B)

● Name: Name: Interrupt Both Edge type

● Description: Name: Interrupt Both Edge type

● Size: 32 bits

● Address Offset: 0x68

● Read/write access: read/write

● Reset Value: 0x0

| 31:16 | 15:0 |
| --- | --- |
| RSVD_GPIO_INT_BOTHEDGE | GPIO_INT_BOTHEDGE |

Table 3-13 Fields for Register: GPIOx_INT_BOTHEDGE(x=A,B)

| Bits | Name | R/W | Description |
| --- | --- | --- | --- |
| 31:1 6 | RSVD_GPIO_INT_BOTHEDGE | R | Reserved bits Reset Value: 0x0 |
| 15:0 | GPIO_INT_BOTHEDGE | R/W | Controls the edge type of interrupt that can occur on Port. - Whenever a particular bit is programmed to 1, it enables the generation of interrupts on both the rising edge and the falling edge of an external input signal corresponding to that bit on port. - The values programmed in the registers gpio_intype_level and gpio_int_polarity for this particular bit are not considered when the corresponding bit of this register is set to 1. - Whenever a particular bit is programmed to 0, the interrupt type depends on the value of the corresponding bits in the gpio_inttype_level and gpio_int_polarity registers. Values: ■0x0 (DISABLED): single edge sensitive ■ 0x1 (ENABLED): both edge sensitive Value After Reset: 0x0 |

- GPIOx_VER_ID_CODE (x=A,B)

● Name: GPIO Component Version

● Description: GPIO Component Version

● Size: 32 bits

● Address Offset: 0x6c

● Read/write access: read

● Reset Value: 0x3231 332a

31:0

GPIO_VER_ID_CODE

Table 3-14Fields for Register: GPIOx_VER_ID_CODE (x=A,B)

| Bits | Name | R/W | Description |
| --- | --- | --- | --- |
| 31:0 | GPIO_VER_ID_CODE | R | V ersion N umber Reset Value : 0x3231_332a |

4 Program Guide

According to the usage requirements, you can use this module as follows:

4.1 I/O port (portx,x=a,b)

1. Select port x . Except for the different base addresses, the registers corresponding to portA and portB have the same offset position and function .

2. Configure I/O direction

Configure the corresponding bit of the data direction register gpio xswport_ddr as input/output (0 - input; 1 - output).

if bit1 of portb is selected as input, then B_swport_ddr &= (0<<1); when it is output, then B_swport_ddr |= (1<<1); The B_swport_ddr register address of portb is 0x40003804 .

3. Write output data

Write the data you want to output to the corresponding bit of the output data register B_swport_dr

4. Read external input data

Read the corresponding bit of the input data register B_ext_port and read the data in

4.2 Interrupts

1. Configuration interruption

a ) Configure the direction of the porta port

Configure the corresponding bit of the data direction register gpio aswport_ddr as input (0 - input).

b) Select interrupt type and polarity

Write the corresponding bit of register gpio ainttype_level to select the interrupt type

(0 — level-sensitive;1 — edge-sensitive);

the corresponding bit of register gpio aint_polarity to select the interrupt polarity

(0 — active-low;1 — active-high);

There are 4 interrupt types after combination:

Rising——Write the corresponding bit of gpio ainttype_level to 1; write the corresponding bit of gpio aint_polarity to 1;

Falling——Write the corresponding bit of gpio ainttype_level to 1; write the corresponding bit of gpio aint_polarity to 0;

High——Write the corresponding bit of gpio ainttype_level to 0; write the corresponding bit of gpio aint_polarity to 1;

Low——Write the corresponding bit of gpio ainttype_level to 0; write the corresponding bit of gpio aint_polarity to 1.

c)Select debounce

Write the corresponding bit of register gpio adebounce to decide whether to include the circuit

(0 — No debounce ;1 — Enable debounce );

d)Level type interrupt synchronization enable

Write register gpio als_sync to 0 or 1 to select whether to include level interrupt synchronization to pclk_intr circuit

e) Interrupt Mask

Write the corresponding bit of register gpio aint_mask to set (0——Unmask; 1——Mask)

f) Enable interrupts

Write the corresponding bit of the register gpio ainten to 1 and enable the interrupt function.

For example, select the bit0 bit of porta as the interrupt, input the falling edge, enable debounce, and not mask the interrupt. The configuration is as follows:

gpio aswport_ddr &= (0<<0);

gpio ainttype_level |= (1<<0);

gpio aint_polarity &= (0<<0);

gpio adebounce |= (1<<0);

gpio aint_mask &= (0<<0);

gpio ainten |= (1<<0);

2. Query interrupt

Read register gpio aintstatus or gpio araw_intstatus

3. Clear interrupts

a) If the type selected at b) in 1 is edge-sensitive (rising or falling)

Write the corresponding bit of register gpio aport_eoi to 1 to clear the current interrupt;

You can also write the corresponding bit of the register gpio ainten to 0 to turn off the interrupt.

Note: The interrupt output signal gpioa_intr_flag is active-high, 1 bit.

4.3 Description

1. During system integration, GPIO PortA[15:1] and PortB[15:0] have a reuse relationship.

The corresponding relationship of multiplexed pins is as follows in Table 4-3-1 :

Table 4-3-1

| port | AF0(default) | AF1 |
| --- | --- | --- |
| PA0 | SWDIO/JTAG_TMS | gpioa0 |
| PA1 | SWCLK/JTAG_TCK | gpioa1 |
| PA2 | JTAGRST | gpioa2 |
| PA3 | JTAGTDI | gpioa3 |
| PA4 | JTAGTDO | gpioa4 |
| PA5 | UART_RX0 | gpioa5 |
| PA6 | UART_TX0 | gpioa6 |
| PA7 | MCO | gpioa7 |
| PA8 | DLCK_MASK | gpioa8 |
| PA9 | I2C0_SCK | gpioa9 |
| PA10 | I2C0_SDA | gpioa10 |
| PA11 | SPISLV_SCK | gpioa11 |
| PA12 | SPISLV_SSN | gpioa12 |
| PA13 | SPISLV_RX | gpioa13 |
| PA14 | SPISLV_TX | gpioa14 |
| PA15 | gpioa15 |  |
| PB0 | QSPI_SCLK | B0 |
| PB1 | QSPI_SSN | B1 |
| PB2 | QSPI_DAT0 | B2 |
| PB3 | QSPI_DAT1 | B3 |
| PB4 | QSPI_DAT2 | B4 |
| PB5 | QSPI_DAT3 | B5 |
| PB6 | B6 | I2C1_SCK |
| PB7 | B7 | I2C1_SDA |
| PB8 | B8 | UART_RX1 |
| PB9 | B9 | UART_TX1 |
| PB10 | B10 | I2C2_SCK |
| PB11 | B11 | I2C2_SDA |
| PB12 | B12 | SPIMST_SCK |
| PB13 | B13 | SPIMST_SSN |
| PB14 | B14 | SPIMST_RX |
| PB15 | B15 | SPIMST_TX |

2. If you need to configure the pin as a GPIO, you must first configure the sys_ctrl corresponding register GPIOA_AFR / B_AFR;

3. If the pin GPIO is configured as an output, you also need to configure the sys_ctrl output enable register GPIOA_OER/B_OER;

4. The function of the Debounce circuit is to remove the glitch of the input data (only under the interrupt function): if the input data width is less than 2 dbclk rising edges, the data will be invalid.

Chapter 15 LISoC_APB_I2C

1 Function list

This module provides a standard interface for 2-wire serial half-duplex transmission for the chip, and can be connected to 2Cvarious peripherals with I interfaces off-chip, such as EEPROM, etc.

● Two-wire I 2Cserial interface – consists of a serial data line (SDA) and a serial clock (SCL)

● Three Speed Mode :

❍ Standard mode (100 Kb/s)

❍ Fast mode (≤ 400 Kb/s) or fast mode plus (≤ 1000 Kb/s) 1

❍  High-speed mode (≤ 3.4 Mb/s)

● Master OR Slave I 2Coperation in an I 2Csystem

● 7-bit or 10-bit addressing

● 7- or 10-bit combined format transfers

● Bulk transmit mode

● Transmit and receive buffers : 16 depth, 8bits width FIFO

● Interrupt or polled-mode operation

● Support p rogrammable SCL

2 Register list

I2C0 Base address：0x4000 1400

I2C1 Base address：0x4000 1800

I2C2 Base address：0x4000 1C00

| Name | Offset | Width | R/W | Description |
| --- | --- | --- | --- | --- |
| ICx_CON(x=0,1,2) | 0x00 | 32 bits | R/W | I2C Control Reset Value: 0x7F |
| ICx_TAR(x=0,1,2) | 0x04 | 32 bits | R/W | I2C Target Address Reset Value: 0x55 |
| ICx_SAR(x=0,1,2) | 0x08 | 32bits | R/W | I2C Slave Address Reset Value: 0x55 |
| ICx_HAS_MADDR(x=0,1,2) | 0x0c | 32bits | R/W | I2C High Speed Master Mode Code Address Register Reset Value: 0x1 |
| ICx_DATA_CMD(x=0,1,2) | 0x10 | 32bits | R/W | I2C Rx/Tx Data Buffer and Command Reset Value: 0x0 |
| ICx_SS_SCL_HCNT(x=0,1,2) | 0x14 | 32 bits | R/W | Standard speed I2C Clock SCL High Count Reset Value: 0x0190 |
| ICx_SS_SCL_LCNT(x=0,1,2) | 0x18 | 32 bits | R/W | Standard speed I2C Clock SCL Low Count Reset Value: 0x01d6 |
| ICx_FS_SCL_HCNT(x=0,1,2) | 0x1C | 32 bits | R/W | Standard speed I2C Clock SCL High Count Reset Value: 0x3C |
| ICx_FS_SCL_LCNT(x=0,1,2) | 0x20 | 32 bits | R/W | Standard speed I2C Clock SCL Low Count Reset Value: 0x82 |
| ICx_HS_SCL_HCNT(x=0,1,2) | 0x24 | 32 bits | R/W | Standard speed I2C Clock SCL High Count Reset Value: 0x6 |
| ICx_HS_SCL_LCNT(x=0,1,2) | 0x28 | 32 bits | R/W | Standard speed I2C Clock SCL Low Count Reset Value: 0x10 |
| ICx_INTR_STAT(x=0,1,2) | 0x2C | 32 bits | R | I2C Interrupt Status Reset Value: 0x0 |
| ICx_INTR_MASK(x=0,1,2) | 0x30 | 32 bits | R/W | I2C Interrupt Mask Reset Value: 0x8ff |
| ICx_RAW_INTR_STAT(x=0,1,2) | 0x34 | 32 bits | R | I2C Raw Interrupt Status Reset Value: 0x0 |
| ICx_RX_TL(x=0,1,2) | 0x38 | 32 bits | R/W | I2C Receive FIFO Threshold Reset Value: 0 |
| ICx_TX_TL(x=0,1,2) | 0x3C | 32 bits | R/W | I2C Transmit FIFO Threshold Reset Value: 0 |
| ICx_CLR_INTR(x=0,1,2) | 0x40 | 32 bit | R | Clear Combined and Individual Interrupts Reset Value: 0x0 |
| ICx_CLR_RX_UNDER(x=0,1,2) | 0x44 | 32 bit | R | Clear RX_UNDER Interrupt Reset Value: 0x0 |
| ICx_CLR_RX_OVER(x=0,1,2) | 0x48 | 32 bit | R | Clear RX_OVER Interrupt Reset Value: 0x0 |
| ICx_CLR_TX_OVER(x=0,1,2) | 0x4C | 32 bit | R | Clear TX_OVER Interrupt Reset Value: 0x0 |
| ICx_CLR_RD_REQ(x=0,1,2) | 0X50 | 32bits | R | Clear RD_REQ Interrupt Register Reset Value: 0x0 |
| ICx_CLR_TX_ABRT(x=0,1,2) | 0x54 | 32 bit | R | Clear TX_ABRT Interrupt Reset Value: 0x0 |
| ICx_CLR_RX_DONE(x=0,1,2) | 0x58 | 32 bit | R | Clear RX_DONE Interrupt Reset Value: 0x0 |
| ICx_CLR_ACTIVITY(x=0,1,2) | 0x5c | 32 bit | R | Clear ACTIVITY Interrupt Reset Value: 0x0 |
| ICx_CLR_STOP_DET(x=0,1,2) | 0x60 | 32 bit | R | Clear STOP_DET Interrupt Reset Value: 0x0 |
| ICx_CLR_START_DET(x=0,1,2) | 0x64 | 32 bit | R | Clear START_DET Interrupt Reset Value: 0x0 |
| ICx_CLR_GEN_CALL(x=0,1,2) | 0x68 | 32 bit | R | Clear GEN_CALL Interrupt Reset Value: 0x0 |
| ICx_ENABLE(x=0,1,2) | 0x6C | 32 bit | R/W | I2C Enable Reset Value: 0x0 |
| ICx_STATUS(x=0,1,2) | 0x70 | 32 bit | R | I2C Status register Reset Value: 0x6 |
| ICx_TXFLR(x=0,1,2) | 0x74 | 32 bits | R | Transmit FIFO Level Register Reset Value: 0x0 |
| ICx_RXFLR(x=0,1,2) | 0x78 | 32 bits | R | Receive FIFO Level Register Reset Value: 0x0 |
| ICx_SDA_HOLD(x=0,1,2) | 0x7C | 32bits | R/W | I2C SDA Hold Time Length Register Reset Value: 0x1 |
| ICx_TX_ABRT_SOURCE(x=0,1,2) | 0x80 | 32 bits | R | I2C Transmit Abort Status Register Reset Value: 0x0 |
| ICx_ENABLE_STATUS(x=0,1,2) | 0x9C | 32 bits | R | I2C Enable Status Register Reset Value:0x0 |
| ICx_COMP_VERSION(x=0,1,2) | 0xf8 | 32 bits | R | Component Version ID Reset Value: 0x3230_322a |

3 Detailed definition of registers

- ICx_CON (x=0,1,2)

● Name: I 2CControl Register

● Description: I2C Control Register. This register can be written only when the apb_i2c is disabled, which corresponds to the IC_ENABLE[0] register being set to 0. Writes at other times have no effect.

● Size: 32 bits

● Address Offset: 0x00

● Read/Write Access: Read/Write

● Reset Value: 0x7F

| 31:11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2:1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| RSVD_IC_CON | STOP_DET_IF_MASTER_ACTIVE | RX_FIFO_FULL_HLD_CTRL | TX_EMPTY_CTRL | STOP_DET_IFADDRESSED | IC_SLAVE_DISABLE | IC_RESTART_EN | IC_10BITADDR_MASTER | IC_10BITADDR_SLAVE | SPEED | MASTER_MODE |

Table 3-1Fields for Register: ICx_CON (x=0,1,2)

| Bits | Name | R/W | Description |
| --- | --- | --- | --- |
| 15:11 | RSVD_IC_CON | N/A | Reserved |
| 10 | STOP_DET_IF_MASTER_ACTIVE | R | In Master mode: ■1'b1: issues the STOP_DET interrupt only when master is active. ■1'b0: issues the STOP_DET irrespective of whether master is active or not. Values: ■ 0x1 (ENABLED): Master issues the STOP_DET interrupt only when master is active ■0x0 (DISABLED): Master issues the STOP_DET interrupt irrespective of whether master is active or not Value After Reset: 0x0 |
| 9 | RX_FIFO_FULL_HLD_CTRL | R/W | This bit controls whether apb_i2c should hold the bus when the Rx FIFO is physically full to its RX_BUFFER_DEPTH(16). Values: ■0x1 (ENABLED): Hold bus when RX_FIFO is full ■0x0 (DISABLED): Overflow when RX_FIFO is full Value After Reset: 0x0 |
| 8 | TX_EMPTY_CTRL | R/W | This bit controls the generation of the TX_EMPTY interrupt, as described in the IC_RAW_INTR_STAT register. Values: ■0x1 (ENABLED): Controlled generation of TX_EMPTY interrupt ■0x0 (DISABLED): Default behavior of TX_EMPTY interrupt Value After Reset: 0x0 |
| 7 | STOP_DET_IFADDRESSED | R/W | In slave mode: ■1'b1: issues the STOP_DET interrupt only when it is addressed. ■0'b0: issues the STOP_DET irrespective of whether it's addressed or not. NOTE: During a general call address, this slave does not issue the STOP_DET interrupt if STOP_DET_IF_ADDRESSED = 1'b1, even if the slave responds to the general call address by generating ACK. The STOP_DET interrupt is generated only when the transmitted address matches the slave address (SAR). Values: ■0x1 (ENABLED): slave issues STOP_DET intr only if addressed ■0x0 (DISABLED): slave issues STOP_DET intr always Value After Reset: 0x0 |
| 6 | IC_SLAVE_DISABLE | R/W | This bit controls whether I 2Chas its slave disabled NOTE: Software should ensure that if this bit is written with 0, then bit 0 should also be written with a 0. Values: ■ 0: slave is enabled ■ 1: slave is disabled Reset value: 1 |
| 5 | IC_RESTART_EN | R/W | Determines whether RESTART conditions may be sent when acting as a master Values: ■ 0: disable ■ 1: enable When RESTART is disabled, the master is prohibited from perform the following functions: ● Change direction within a transfer (split) ● Send a START BYTE ● High-speed mode operation ● Combined format transfers in 7-bit addressing modes ● Read operation with a 10-bit address ● Send multiple bytes per transfer By replacing RESTART condition followed by a STOP and a subsequent START condition, split operations are broken down into multiple apb_i 2ctransfers. If the above operations are performed, it will result in setting bit 6 ( TX_ABRT ) of the IC_RAW_INTR_STAT register Reset value: 1 |
| 4 | IC_10BITADDR_MASTER | R/W | Controls whether the apb_i2c starts its transfers in 7- or 10-bit addressing mode when acting as a master. ■0: 7-bit addressing ■1: 10-bit addressing Values: ■0x1 (ADDR_10BITS): Master 10Bit addressing mode ■0x0 (ADDR_7BITS): Master 7Bit addressing mode Value After Reset: 0x1 |
| 3 | IC_10BITADDR_SLAVE | R/W | When acting as a slave, this bit controls whether the apb_i2c responds to 7- or 10-bit addresses Values: ■ 0: 7-bit addressing. The apb_i 2cignores transactions that involve 10-bit addressing; for 7-bit addressing, only the lower 7 bits of the IC_SAR register are compared ■ 1: 10-bit addressing. The apb_i 2cresponds to only 10-bit addressing transfers that match the full 10 bits of the IC_SAR register Reset value: 1 |
| 2:1 | SPEED | R/W | These bits control at which speed the apb_i2c operates; its setting is relevant only if one is operating the apb_i2c in master mode. Hardware protects against illegal values being programmed by software. These bits must be programmed appropriately for slave mode also, as it is used to capture correct value of spike filter as per the speed mode. 1: standard mode (100 kbit/s) 2: fast mode (<=400 kbit/s) or fast mode plus (<=1000Kbit/s) 3: high speed mode (3.4 Mbit/s) Values: ■0x1 (STANDARD): Standard Speed mode of operation ■0x2 (FAST): Fast or Fast Plus mode of operation ■ 0x3 (HIGH): High Speed mode of operation Value After Reset: 0x3 |
| 0 | MASTER_MODE | R/W | This bit controls whether the apb_i2c master is enabled. NOTE: Software should ensure that if this bit is written with '1' then bit 6 should also be written with a '1'. Values: ■0x1 (ENABLED): Master mode is enabled ■0x0 (DISABLED): Master mode is disabled Value After Reset: 0x01 |

- ICx_TAR (x=0,1,2)

● Name: I2C Target Address Register

● Description: I2C Target Address Register. This register can be written to only when IC_ENABLE[0] is set to 0.

● Size: 32 bits

● Address Offset: 0x04

● Read/Write Access: Read/Write

● Reset Value: 0x55

| 31 :1 2 | 11 | 10 | 9:0 |
| --- | --- | --- | --- |
| RSVD_IC_TAR | SPECIAL | GC_OR_START | IC_TAR |

Table 3-2 Fields for Register: ICx_TAR(x=0,1,2)

| Bits | Name | R/W | Description |
| --- | --- | --- | --- |
| 31:12 | Reserved | N/A | Reserved |
| 11 | SPECIAL | R/W | This bit indicates whether software performs a General Call or START BYTE command. 0: ignore bit 10 GC_OR_START and use IC_TAR normally 1: perform special I 2Ccommand as specified in GC_OR_START bit Reset value: 0x0 |
| 10 | GC_OR_START | R/W | If bit 11 ( SPECIAL ) is set to 1, then this bit indicates whether a General Call or START byte command is to be performed by the apb_i 2c. 0: General Call Address – after issuing a General Call, only writes may be performed. Attempting to issue a read command results in setting bit 6 (TX_ABRT) of the IC_RAW_INTR_STAT re gister. The apb_i 2cremains in General Call mode until the SPECIAL bit value (bit 11) is cleared. 1: START BYTE Reset value: 0x0 |
| 9:0 | IC_TAR | R/W | This is the target address for any master transaction. When transmitting a General Call, these bits are ignored. To generate a START BYTE, the CPU needs to write only once into these bits Reset value: 0x55 |

- ICx_SAR (x=0,1,2)

● Name: I2C Slave Address Register

● Description: I2C Slave Address Register . This register can be written only when the I2C interface is disabled, which corresponds to the IC_ENABLE[0] register being set to 0.

● Size: 32 bits

● Address Offset: 0x08

● Read/Write Access: Read/Write

● Reset Value: 0x55

| 31 :1 0 | 9:0 |
| --- | --- |
| RSVD_IC_SAR | IC_SAR |

Table 3-2 Fields for Register: ICx_SAR(x=0,1,2)

| Bits | Name | R/W | Description |
| --- | --- | --- | --- |
| 31:10 | RSVD_IC_SAR | R | IC_SAR Reserved bits - Read Only Exists: Always |
| 9:0 | IC_SAR | R/W | The IC_SAR holds the slave address when the I2C is operating as a slave. For 7-bit addressing, only IC_SAR[6:0] is used. Note: The default values cannot be any of the reserved address locations: that is, 0x00 to 0x07, or 0x78 to 0x7f. The correct operation of the device is not guaranteed if you program the IC_SAR or IC_TAR to a reserved value. Refer to Table "I2C/SMBus Definition of Bits in First Byte" for a complete list of these reserved values Value After Reset: 0x055 Exists: Always |

- ICx_HAS_MADDR (x=0,1,2)

● Name: I2C High Speed Master Mode Code Address Register

● Description: I2C High Speed Master Mode Code Address Register . This register can be written only when the I2C interface is disabled, which corresponds to the IC_ENABLE[0] register being set to 0. Writes at other times have no effect.

● Size: 32 bits

● Address Offset: 0x0C

● Read/Write Access: Read/Write

● Reset Value: 0x01

| 31 :3 | 2:0 |
| --- | --- |
| RSVD_IC_HS_MAR | IC_HS_MAR |

Table 3-4 Fields for Register: ICx_HAS_MADDR(x=0,1,2)

| Bits | Name | Memory Access | Description |
| --- | --- | --- | --- |
| 31:3 | RSVD_IC_HS_MAR | R | IC_HS_MAR Reserved bits - Read Only Exists: Always |
| 2:0 | IC_HS_MAR | R/W | This bit field holds the value of the I2C HS mode master code. HS-mode master codes are reserved 8-bit codes (00001xxx) that are not used for slave addressing or other purposes. Each master has its unique master code; up to eight high-speed mode masters can be present on the same I2C bus system. Value After Reset: 0x1 |

- ICx_DATA_CMD (x=0,1,2)

● Name: I2C Rx/Tx Data Buffer and Command Register

● Description: I2C Rx/Tx Data Buffer and Command Register; this is the register the CPU writes to when filling the TX FIFO and the CPU reads from when retrieving bytes from RX FIFO.

● Size: 32 bits

● Address Offset: 0x10

● Read/Write Access: Read/Write

● Reset Value: 0x0

| 31:1 1 | 10 | 9 | 8 | 7:0 |
| --- | --- | --- | --- | --- |
| RSVD_IC_DATA_CMD | RESTART | STOP | CMD | DAT |

Table 3-5 Fields for Register: ICx_DATA_CMD(x=0,1,2)

| Bits | Name | R/W | Description |
| --- | --- | --- | --- |
| 31:11 | Reserved | N/A | Reserved |
| 10 | RESTART | W | This bit controls whether a RESTART is issued before the byte is sent or received. 1 - RESTART is issued before the data is sent/received (according to the value of CMD), regardless of whether or not the transfer direction is changing from the previous command; 0 - RESTART is issued only if the transfer direction is changing from the previous command; Values: ■0x1 (ENABLE): Issue RESTART before this command ■ 0x0 (DISABLE): Donot Issue RESTART before this command Value After Reset: 0x0 |
| 9 | STOP | W | This bit controls whether a STOP is issued after the byte is sent or received. ■1 - STOP is issued after this byte, regardless of whether or not the Tx FIFO is empty. If the Tx FIFO is not empty, the master immediately tries to start a new transfer by issuing a START and arbitrating for the bus. ■0 - STOP is not issued after this byte, regardless of whether or not the Tx FIFO is empty. If the Tx FIFO is not empty, the master continues the current transfer by sending/receiving data bytes according to the value of the CMD bit . If the Tx FIFO is empty , the master holds the SCL line low and stalls the bus until a new command is available in the Tx FIFO. Values: ■0x1 (ENABLE): Issue STOP after this command ■0x0 (DISABLE): Donot Issue STOP after this command Value After Reset: 0x0 |
| 8 | CMD | W | This bit controls whether a read or a write is performed.It controls only the direction when it acts as a master. 1 = Read 0 = Write When a command is entered in the TX FIFO, this bit distinguishes the write and read commands When programming this bit, you should remember the following: attempting to perform a read operation after a General Call command has been sent results in a TX_ABRT interrupt (bit 6 of the IC_RAW_INTR_STAT register), unless bit 11 ( SPECIAL ) in the IC_TAR register has been cleared If a “ 1”is written to this bit after receiving a RD_REQ interrupt, then a TX_ABRT interrupt occurs Reset value: 0x0 |
| 7:0 | DAT | R/W | This register contains the data to be transmitted or received on the I 2Cbus. If you are writing to this register and want to perform a read, bits 7:0 ( DAT ) are ignored by the apb_i 2c. However, when you read this register, these bits return the value of data received on the apb_i 2cinterface Reset value: 0x0 |

- ICx_SS_SCL_HCNT (x=0,1,2)

● Name: Standard Speed I 2CClock SCL High Count Register

● Description: Standard Speed I2C Clock SCL High Count Register

● Size: 32 bits

● Address Offset: 0x14

● Read/Write Access: Read/Write

● Reset Value: 0x0190

| 31:16 | 15:0 |
| --- | --- |
| RSVD_ SS_SCL_HCNT | IC_SS_SCL_HCNT |

Table 3-6 Fields for Register: ICx_SS_SCL_HCNT(x=0,1,2)

| Bits | Name | R/W | Description |
| --- | --- | --- | --- |
| 31:16 | Reserved | N/A | Reserved |
| 15:0 | IC_SS_SCL_HCNT | R/W | This register must be set before any I 2Cbus transaction can take place to ensure proper I/O timing. This register sets the SCL clock high-period count for standard speed. The minimum valid value is 6 NOTE: This register must not be programmed to a value higher than 65525 Reset value : 0x0190 |

- ICx_SS_SCL_LCNT (x=0,1,2)

● Name: Standard Speed I 2CClock SCL Low Count Register

● Description: Standard Speed I2C Clock SCL Low Count Register

● Size: 32 bits

● Address Offset: 0x18

● Read/Write Access: Read/Write

● Reset Value: 0x01d6

| 31:16 | 15:0 |
| --- | --- |
| RSVD_ SS_SCL_LCNT | IC_SS_SCL_LCNT |

Table 3-7 Fields for Register: ICx_SS_SCL_LCNT(x=0,1,2)

| Bits | Name | R/W | Description |
| --- | --- | --- | --- |
| 31:16 | Reserved | N/A | Reserved |
| 15:0 | IC_SS_SCL_LCNT | R/W | This register must be set before any I 2Cbus transaction can take place to ensure proper I/O timing. This register sets the SCL clock low period count for standard speed. The minimum valid value is 8 Reset value : 0x01d6 |

- ICx_FS_SCL_HCNT(x=0,1,2)

● Name: Fast Mode or Fast Mode Plus I2C Clock SCL High Count Register

● Description: Fast Mode or Fast Mode Plus I2C Clock SCL High Count Register

● Size: 32 bits

● Address Offset: 0x1C

● Read/Write Access: Read/Write

● Reset Value: 0x3c

| 31:16 | 15:0 |
| --- | --- |
| RSVD_FS_SCL_HCNT | IC_FS_SCL_HCNT |

Table 3-6 Fields for Register: ICx_SS_SCL_HCNT(x=0,1,2)

| Bits | Name | R/W | Description |
| --- | --- | --- | --- |
| 31:16 | Reserved | N/A | Reserved |
| 15:0 | IC_FS_SCL_HCNT | R/W | This register must be set before any I2C bus transaction can take place to ensure proper I/O timing. This register sets the SCL clock high-period count for fast mode or fast mode plus. It is used in high-speed mode to send the Master Code and START BYTE or General CALL. For more information, refer to "IC_CLK Frequency Configuration". This register can be written only when the I2C interface is disabled, which corresponds to the IC_ENABLE[0] register being set to 0. Writes at other times have no effect. The minimum valid value is 6; hardware prevents values less than this being written, and if attempted results in 6 being set. Value After Reset: 0x3C |

- ICx_FS_SCL_LCNT(x=0,1,2)

● Name: Fast Mode or Fast Mode Plus I2C Clock SCL Low Count Register

● Description: Fast Mode or Fast Mode Plus I2C Clock SCL Low Count Register

● Size: 32 bits

● Address Offset: 0x20

● Read/Write Access: Read/Write

● Reset Value: 0x82

| 31:16 | 15:0 |
| --- | --- |
| RSVD_FS_SCL_LCNT | IC_FS_SCL_LCNT |

Table 3-7 Fields for Register: ICx_SS_SCL_LCNT(x=0,1,2)

| Bits | Name | R/W | Description |
| --- | --- | --- | --- |
| 31:16 | Reserved | N/A | Reserved |
| 15:0 | IC_FS_SCL_LCNT | R/W | This register must be set before any I2C bus transaction can take place to ensure proper I/O timing. This register sets the SCL clock low period count for fast speed. It is used in high-speed mode to send the Master Code and START BYTE or General CALL. For more information, refer to "IC_CLK Frequency Configuration". This register can be written only when the I2C interface is disabled, which corresponds to the IC_ENABLE[0] register being set to 0. Writes at other times have no effect. The minimum valid value is 8; hardware prevents values less than this being written, and if attempted results in 8 being set. Value After Reset: 0x82 |

- ICx_HS_SCL_HCNT(x=0,1,2)

● Name: High Speed I2C Clock SCL High Count Register

● Description: High Speed I2C Clock SCL High Count Register

● Size: 32 bits

● Address Offset: 0x24

● Read/Write Access: Read/Write

● Reset Value: 0x06

| 31:16 | 15:0 |
| --- | --- |
| RSVD_HS_SCL_HCNT | IC_HS_SCL_HCNT |

Table 3-6 Fields for Register: ICx_SS_SCL_HCNT(x=0,1,2)

| Bits | Name | R/W | Description |
| --- | --- | --- | --- |
| 31:16 | Reserved | N/A | Reserved |
| 15:0 | IC_HS_SCL_HCNT | R/W | This register must be set before any I2C bus transaction can take place to ensure proper I/O timing. This register sets the SCL clock high period count for high speed.refer to "IC_CLK Frequency Configuration". The SCL High time depends on the loading of the bus. For 100pF loading, the SCL High time is 60ns; for 400pF loading, the SCL High time is 120ns. This register can be written only when the I2C interface is disabled, which corresponds to the IC_ENABLE[0] register being set to 0. Writes at other times have no effect. The minimum valid value is 6; hardware prevents values less than this being written, and if attempted results in 6 being set. Value After Reset: 0x06 |

- ICx_HS_SCL_LCNT(x=0,1,2)

● Name: High Speed I2C Clock SCL Low Count Register

● Description: High Speed I2C Clock SCL Low Count Register

● Size: 32 bits

● Address Offset: 0x28

● Read/Write Access: Read/Write

● Reset Value: 0x10

| 31:16 | 15:0 |
| --- | --- |
| RSVD_HS_SCL_LCNT | IC_HS_SCL_LCNT |

Table 3-7 Fields for Register: ICx_SS_SCL_LCNT(x=0,1,2)

| Bits | Name | R/W | Description |
| --- | --- | --- | --- |
| 31:16 | Reserved | N/A | Reserved |
| 15:0 | IC_HS_SCL_LCNT | R/W | This register must be set before any I2C bus transaction can take place to ensure proper I/O timing. This register sets the SCL clock low period count for high speed. For more information, refer to "IC_CLK Frequency Configuration". The SCL low time depends on the loading of the bus. For 100pF loading, the SCL low time is 160ns; for 400pF loading, the SCL low time is 320ns. This register can be written only when the I2C interface is disabled, which corresponds to the IC_ENABLE[0] register being set to 0. Writes at other times have no effect. The minimum valid value is 8; hardware prevents values less than this being written, and if attempted results in 8 being set. Value After Reset: 0x10 |

- ICx_INTR_STAT (x=0,1,2)

● Name: I 2CInterrupt Status Register

● Description: I 2CInterrupt Status Register

● Size: 32 bits

● Address Offset: 0x2C

● Read/Write Access: Read

● Reset Value: 0x0

| 31:14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| RSVD_IC_INTR_STAT | R_MASTER_ON_HOLD | R_RESTART_DET | R_GEN_CALL | R_START_DET | R_STOP_DET | R_ACTIVITY | R_RX_DONE | R_TX_ABRT | R_RD_REQ | R_TX_EMPTY | R_TX_OVER | R_RX_FULL | R_RX_OVER | R_RX_UNDER |

Table 3-8 Fields for Register: ICx_INTR_STAT(x=0,1,2)

| Bits | Name | R/W | Description |
| --- | --- | --- | --- |
| 31:14 | RSVD_IC_INTR_STAT | R | IC_INTR_STAT Reserved bits - Read Only |
| 13 | R_MASTER_ON_HOLD | R | See IC_RAW_INTR_STAT for a detailed description of R_MASTER_ON_HOLD bit. Values: ■ 0x1 (ACTIVE): R_MASTER_ON_HOLD interrupt is active ■ 0x0 (INACTIVE): R_MASTER_ON_HOLD interrupt is inactive Value After Reset: 0x0 |
| 12 | R_RESTART_DET | R | See IC_RAW_INTR_STAT for a detailed description of R_RESTART_DET bit. Values: ■ 0x1 (ACTIVE): R_RESTART_DET interrupt is active ■ 0x0 (INACTIVE): R_RESTART_DET interrupt is inactive Value After Reset: 0x0 |
| 11 | R_GEN_CALL | R | See IC_RAW_INTR_STAT for a detailed description of R_GEN_CALL bit. Values: ■ 0x1 (ACTIVE): R_GEN_CALL interrupt is active ■ 0x0 (INACTIVE): R_GEN_CALL interrupt is inactive Value After Reset: 0x0 |
| 10 | R_START_DET | R | See IC_RAW_INTR_STAT for a detailed description of R_START_DET bit. Values: ■ 0x1 (ACTIVE): R_START_DET interrupt is active ■ 0x0 (INACTIVE): R_START_DET interrupt is inactive Value After Reset: 0x0 |
| 9 | R_STOP_DET | R | See IC_RAW_INTR_STAT for a detailed description of R_STOP_DET bit. Values: ■ 0x1 (ACTIVE): R_STOP_DET interrupt is active ■ 0x0 (INACTIVE): R_STOP_DET interrupt is inactive Value After Reset: 0x0 |
| 8 | R_ACTIVITY | R | See IC_RAW_INTR_STAT for a detailed description of R_ACTIVITY bit. Values: ■ 0x1 (ACTIVE): R_ACTIVITY interrupt is active ■ 0x0 (INACTIVE): R_ACTIVITY interrupt is inactive Value After Reset: 0x0 |
| 7 | R_RX_DONE | R | See IC_RAW_INTR_STAT for a detailed description of R_RX_DONE bit. Values: ■ 0x1 (ACTIVE): R_RX_DONE interrupt is active ■ 0x0 (INACTIVE): R_RX_DONE interrupt is inactive Value After Reset: 0x0 |
| 6 | R_TX_ABRT | R | See IC_RAW_INTR_STAT for a detailed description of R_TX_ABRT bit. Values: ■ 0x1 (ACTIVE): R_TX_ABRT interrupt is active ■ 0x0 (INACTIVE): R_TX_ABRT interrupt is inactive Value After Reset: 0x0 |
| 5 | R_RD_REQ | R | See IC_RAW_INTR_STAT for a detailed description of R_RD_REQ bit. Values: ■ 0x1 (ACTIVE): R_RD_REQ interrupt is active ■ 0x0 (INACTIVE): R_RD_REQ interrupt is inactive Value After Reset: 0x0 |
| 4 | R_TX_EMPTY | R | See IC_RAW_INTR_STAT for a detailed description of R_TX_EMPTY bit. Values: ■ 0x1 (ACTIVE): R_TX_EMPTY interrupt is active ■ 0x0 (INACTIVE): R_TX_EMPTY interrupt is inactive Value After Reset: 0x0 |
| 3 | R_TX_OVER | R | See IC_RAW_INTR_STAT for a detailed description of R_TX_OVER bit. Values: ■ 0x1 (ACTIVE): R_TX_OVER interrupt is active ■ 0x0 (INACTIVE): R_TX_OVER interrupt is inactive Value After Reset: 0x0 |
| 2 | R_RX_FULL | R | See IC_RAW_INTR_STAT for a detailed description of R_RX_FULL bit. Values: ■ 0x1 (ACTIVE): R_RX_FULL interrupt is active ■ 0x0 (INACTIVE): R_RX_FULL interrupt is inactive Value After Reset: 0x0 |
| 1 | R_RX_OVER | R | See IC_RAW_INTR_STAT for a detailed description of R_RX_OVER bit. Values: ■ 0x1 (ACTIVE): R_RX_OVER interrupt is active ■ 0x0 (INACTIVE): R_RX_OVER interrupt is inactive Value After Reset: 0x0 |
| 0 | R_RX_UNDER | R | See IC_RAW_INTR_STAT for a detailed description of R_RX_UNDER bit. Values: ■ 0x1 (ACTIVE): RX_UNDER interrupt is active ■ 0x0 (INACTIVE): RX_UNDER interrupt is inactive Value After Reset: 0x0 |

- ICx_INTR_MASK (x=0,1,2)

● Name: I 2CInterrupt Mask Register

● Description: I 2CInterrupt Mask Registe

● Size: 32 bits

● Address Offset: 0x30

● Read/Write Access: Read/Write

● Reset Value: 0x0

| 31:1 4 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| RSVD_IC_INTR_STAT | R_MASTER_ON_HOLD | R_RESTART_DET | R_GEN_CALL | R_START_DET | R_STOP_DET | R_ACTIVITY | R_RX_DONE | R_TX_ABRT | R_RD_REQ | R_TX_EMPTY | R_TX_OVER | R_RX_FULL | R_RX_OVER | R_RX_UNDER |

Table 3-9 Fields for Register: ICx_INTR_MASK(x=0,1,2)

| Bits | Name | Memory Access | Description |
| --- | --- | --- | --- |
| 31:14 | RSVD_IC_INTR_STAT | R | IC_INTR_STAT Reserved bits - Read Only |
| 13 | R_MASTER_ON_HOLD | R | See IC_RAW_INTR_STAT for a detailed description of R_MASTER_ON_HOLD bit. Values: ■ 0x1 (ACTIVE): R_MASTER_ON_HOLD interrupt is active ■ 0x0 (INACTIVE): R_MASTER_ON_HOLD interrupt is inactive Value After Reset: 0x0 |
| 12 | R_RESTART_DET | R | See IC_RAW_INTR_STAT for a detailed description of R_RESTART_DET bit. Values: ■ 0x1 (ACTIVE): R_RESTART_DET interrupt is active ■ 0x0 (INACTIVE): R_RESTART_DET interrupt is inactive Value After Reset: 0x0 |
| 11 | R_GEN_CALL | R | See IC_RAW_INTR_STAT for a detailed description of R_GEN_CALL bit. Values: ■ 0x1 (ACTIVE): R_GEN_CALL interrupt is active ■ 0x0 (INACTIVE): R_GEN_CALL interrupt is inactive Value After Reset: 0x1 |
| 10 | R_START_DET | R | See IC_RAW_INTR_STAT for a detailed description of R_START_DET bit. Values: ■ 0x1 (ACTIVE): R_START_DET interrupt is active ■ 0x0 (INACTIVE): R_START_DET interrupt is inactive Value After Reset: 0x0 |
| 9 | R_STOP_DET | R | See IC_RAW_INTR_STAT for a detailed description of R_STOP_DET bit. Values: ■ 0x1 (ACTIVE): R_STOP_DET interrupt is active ■ 0x0 (INACTIVE): R_STOP_DET interrupt is inactive Value After Reset: 0x0 Exists: Always Volatile: true |
| 8 | R_ACTIVITY | R | See IC_RAW_INTR_STAT for a detailed description of R_ACTIVITY bit. Values: ■ 0x1 (ACTIVE): R_ACTIVITY interrupt is active ■ 0x0 (INACTIVE): R_ACTIVITY interrupt is inactive Value After Reset: 0x0 |
| 7 | R_RX_DONE | R | See IC_RAW_INTR_STAT for a detailed description of R_RX_DONE bit. Values: ■ 0x1 (ACTIVE): R_RX_DONE interrupt is active ■ 0x0 (INACTIVE): R_RX_DONE interrupt is inactive Value After Reset: 0x1 |
| 6 | R_TX_ABRT | R | See IC_RAW_INTR_STAT for a detailed description of R_TX_ABRT bit. Values: ■ 0x1 (ACTIVE): R_TX_ABRT interrupt is active ■ 0x0 (INACTIVE): R_TX_ABRT interrupt is inactive Value After Reset: 0x1 |
| 5 | R_RD_REQ | R | See IC_RAW_INTR_STAT for a detailed description of R_RD_REQ bit. Values: ■ 0x1 (ACTIVE): R_RD_REQ interrupt is active ■ 0x0 (INACTIVE): R_RD_REQ interrupt is inactive Value After Reset: 0x1 |
| 4 | R_TX_EMPTY | R | See IC_RAW_INTR_STAT for a detailed description of R_TX_EMPTY bit. Values: ■ 0x1 (ACTIVE): R_TX_EMPTY interrupt is active ■ 0x0 (INACTIVE): R_TX_EMPTY interrupt is inactive Value After Reset: 0x1 |
| 3 | R_TX_OVER | R | See IC_RAW_INTR_STAT for a detailed description of R_TX_OVER bit. Values: ■ 0x1 (ACTIVE): R_TX_OVER interrupt is active ■ 0x0 (INACTIVE): R_TX_OVER interrupt is inactive Value After Reset: 0x1 |
| 2 | R_RX_FULL | R | See IC_RAW_INTR_STAT for a detailed description of R_RX_FULL bit. Values: ■ 0x1 (ACTIVE): R_RX_FULL interrupt is active ■ 0x0 (INACTIVE): R_RX_FULL interrupt is inactive Value After Reset: 0x1 |
| 1 | R_RX_OVER | R | See IC_RAW_INTR_STAT for a detailed description of R_RX_OVER bit. Values: ■ 0x1 (ACTIVE): R_RX_OVER interrupt is active ■ 0x0 (INACTIVE): R_RX_OVER interrupt is inactive Value After Reset: 0x1 |
| 0 | R_RX_UNDER | R | See IC_RAW_INTR_STAT for a detailed description of R_RX_UNDER bit. Values: ■ 0x1 (ACTIVE): RX_UNDER interrupt is active ■ 0x0 (INACTIVE): RX_UNDER interrupt is inactive Value After Reset: 0x1 |

- ICx_RAW_INTR_STAT (x=0,1,2)

● Name: I 2CRaw Interrupt Status Register

● Description: I2C Raw Interrupt Status Register

Unlike the IC_INTR_STAT register, these bits are not masked so they always show the true status of the apb_i2c.

● Size: 32 bits

● Address Offset: 0x34

● Read/Write Access: Read

● Reset Value: 0x0

| 31:1 4 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| RSVD_IC_RAW_INTR_STAT | MASTER_ON_HOLD | RESTART_DET | GEN_CALL | START_DET | STOP_DET | ACTIVITY | RX_DONE | TX_ABRT | RD_REQ | TX_EMPTY | TX_OVER | RX_FULL | RX_OVER | RX_UNDER |

Table 3-10 Fields for Register: ICx_RAW_INTR_STAT(x=0,1,2)

| Bits | Name | R/W | Description |
| --- | --- | --- | --- |
| 31:14 | Reserved | N/A | Reserved. |
| 13 | MASTER_IN_HOLD | R | Indicates whether master is holding the bus and TX FIFO is empty. Enabled only when I2C_DYNAMIC_TAR_UPDATE=1 and IC_EMPTYFIFO_HOLD_MASTER_EN=1. Values: ■0x1 (ACTIVE): MASTER_ON_HOLD interrupt is active ■0x0 (INACTIVE): MASTER_ON_HOLD interrupt is inactive Reset value : 0x0 |
| 12 | RESTART_D ET | R | Indicates whether a RESTART condition has occurred on the I2C interface when apb_i2c is operating in Slave mode and the slave is being addressed. Note: However, in high-speed mode or during a START BYTE transfer, the RESTART comes before the address field as per the I2C protocol. In this case, the slave is not the addressed slave when the RESTART is issued, therefore apb_i2c does not generate the RESTART_DET interrupt. Values: ■0x1 (ACTIVE): RESTART_DET interrupt is active ■0x0 (INACTIVE): RESTART_DET interrupt is inactive Reset value : 0x0 |
| 11 | GEN_CALL | R | Set only when a General Call address is received and it is acknowledged. It stays set until it is cleared either by disabling apb_i 2cor when the CPU reads bit 0 of the IC_CLR_GEN_CALL register. apb_i 2cstores the received data in the Rx buffer Values: ■0x1 (ACTIVE): GEN_CALL interrupt is active ■0x0 (INACTIVE): GEN_CALL interrupt is inactive Reset value: 0x0 |
| 10 | START_DET | R | Indicates whether a START or RESTART condition has o ccurred on the I2C interface Values: ■0x1 (ACTIVE): START_DET interrupt is active ■0x0 (INACTIVE): START_DET interrupt is inactive Reset value: 0x0 |
| 9 | STOP_DET | R | Indicates whether a STOP condition has occurred on the I2C interface regardless of whether apb_i2c is operating in slave or master mode. In Slave Mode: ■If IC_CON[7]=1'b1 (STOP_DET_IFADDRESSED), the STOP_DET interrupt will be issued only if slave is addressed. Note: During a general call address, this slave does not issue a STOP_DET interrupt if STOP_DET_IF_ADDRESSED=1'b1, even if the slave responds to the general call address by generating ACK. The STOP_DET interrupt is generated only when the transmitted address matches the slave address (SAR). ■If IC_CON[7]=1'b0 (STOP_DET_IFADDRESSED), the STOP_DET interrupt is issued irrespective of whether it is being addressed. In Master Mode: ■If IC_CON[10]=1'b1 (STOP_DET_IF_MASTER_ACTIVE), the STOP_DET interrupt will be issued only if Master is active. ■If IC_CON[10]=1'b0 (STOP_DET_IFADDRESSED), the STOP_DET interrupt will be issued irrespective of whether master is active or not. Values: ■0x1 (ACTIVE): STOP_DET interrupt is active ■0x0 (INACTIVE): STOP_DET interrupt is inactive Value After Reset: 0x0 |
| 8 | ACTIVITY | R | This bit captures apb_i 2cactivity and stays set until it is cleared. There are four ways to clear it: ● Disabling the apb_i2c ● Reading the IC_CLR_ACTIVITY register ● Reading the IC_CLR_INTR register ● System reset Values: ■0x1 (ACTIVE): RAW_INTR_ACTIVITY interrupt is active ■0x0 (INACTIVE): RAW_INTR_ACTIVITY interrupt is inactive Value After Reset: 0x0 Reset value: 0x0 |
| 7 | RX_DONE | R | When the apb_i 2cis acting as a slave-transmitter, this bit is set to 1 if the master does not acknowledge a transmitted byte. This occurs on the last byte of the transmission, indicating that the transmission is done Values: ■0x1 (ACTIVE): RX_DONE interrupt is active ■0x0 (INACTIVE): RX_DONE interrupt is inactive Value After Reset: 0x0 Reset value: 0x0 |
| 6 | TX_ABRT | R | This bit indicates if apb_i 2c, as an I 2Ctransmitter, is unable to complete the intended actions on the contents of the transmit FIFO. This situation can occur both as an I 2Cmaster or an I 2Cslave, and is referred to as a “transmit abort". When this bit is set to 1, the IC_TX_ABRT_SOURCE register indicates the reason why the transmit abort takes places. NOTE: The apb_i 2cflushes/resets/empties the TX FIFO whenever this bit is set. The TX FIFO remains in this flushed state until the register IC_CLR_TX_ABRT is read. Once this read is performed, the TX FIFO is then ready to accept more data bytes from the APB interface Values: ■0x1 (ACTIVE): TX_ABRT interrupt is active ■0x0 (INACTIVE): TX_ABRT interrupt is inactive Reset value: 0x0 |
| 5 | RD_REQ | R | This bit is set to 1 when apb_i 2cis acting as a slave and another I2C master is attempting to read data from apb_i 2c. The apb_i 2cholds the I 2Cbus in a wait state (SCL=0) until this interrupt is serviced, which means that the slave has been addressed by a remote master that is asking for data to be transferred. The processor must respond to this interrupt and then write the requested dat a to the IC_DATA_CMD register. This bit is set to 0 just after the processor reads the IC_CLR_RD_REQ register Values: ■0x1 (ACTIVE): RD_REQ interrupt is active ■0x0 (INACTIVE): RD_REQ interrupt is inactive Reset value: 0x0 |
| 4 | TX_EMPTY | R | The behavior of the TX_EMPTY interrupt status differs based on the TX_EMPTY_CTRL selection in the IC_CON register. ■When TX_EMPTY_CTRL = 0: This bit is set to 1 when the transmit buffer is at or below the threshold value set in the IC_TX_TL register. ■When TX_EMPTY_CTRL = 1: This bit is set to 1 when the transmit buffer is at or below the threshold value set in the IC_TX_TL register and the transmission of the address/data from the internal shift register for the most recently popped command is completed. It is automatically cleared by hardware when the buffer level goes above the threshold. When IC_ENABLE[0] is set to 0, the TX FIFO is flushed and held in reset. There the TX FIFO looks like it has no data within it, so this bit is set to 1, provided there is activity in the master or slave state machines. When there is no longer any activity, then with ic_en=0, this bit is set to 0. Values: ■0x1 (ACTIVE): TX_EMPTY interrupt is active ■0x0 (INACTIVE): TX_EMPTY interrupt is inactive Value After Reset: 0x0 |
| 3 | TX_OVER | R | Set during transmit if the transmit buffer is filled to 16( TX_BUFFER_DEPTH )and the processor attempts to another issue I 2Ccommand by writing to the IC_DATA_CMD register Values: ■0x1 (ACTIVE): TX_OVER interrupt is active ■0x0 (INACTIVE): TX_OVER interrupt is inactive Reset value: 0x0 |
| 2 | RX_FULL | R | Set when the receive buffer reaches or goes above the RX_TL threshold in the IC_RX_TL register. It is aut omatically cleared by hardware when buffer level goes below the threshold. If the module is disabled (IC_ENABLE[0]=0), the RX FIFO is flushed and held in reset; therefore the RX FIFO is not full Values: ■0x1 (ACTIVE): RX_FULL interrupt is active ■0x0 (INACTIVE): RX_FULL interrupt is inactive Reset value: 0x0 |
| 1 | RX_OVER | R | Set if the receive buffer is completely filled to 16( RX_BUFFER_DEPTH ) and an additional byte is received from an external I 2Cdevice. The apb_i2c acknowledges this, but any data bytes received after the FIFO is full are lost. If the module is disabled (IC_ENABLE[0]=0), this bit keeps its level until the master or slave state machines go into idle, and when ic_en goes to 0, this interrupt is cleared Note: If the bit 9 of the IC_CON register (RX_FIFO_FULL_HLD_CTRL) is programmed to HIGH, then the RX_OVER interrupt never occurs, because the Rx FIFO never overflows. Values: ■0x1 (ACTIVE): RX_OVER interrupt is active ■0x0 (INACTIVE): RX_OVER interrupt is inactive Value After Reset: 0x0 Reset value: 0x0 |
| 0 | RX_UNDER | R | Set if the processor attempts to read the receive buffer when it is empty by reading from t he IC_DATA_CMD register r. If the module is disabled (IC_ENABLE[0]=0), this bit keeps its level until the master or slave state machines go into idle, and when ic_en goes to 0, this interrupt is cleared Values: ■ 0x1 (ACTIVE): RX_UNDER interrupt is active ■0x0 (INACTIVE): RX_UNDER interrupt is inactive Reset value: 0x0 |

- IC x_RX_TL (x=0,1,2)

● Name: I 2CReceive FIFO Threshold Register

● Description: I 2CReceive FIFO Threshold Register

● Size: 32bits

● Address Offset: 0x38

● Read/Write Access: Read/Write

● Reset Value: 0x0

| 31:8 | 7:0 |
| --- | --- |
| RSVD_IC_RX_TL | RX_TL |

Table 3-11 Fields for Register: ICx_RX_TL(x=0,1,2)

| Bits | Name | R/W | Description |
| --- | --- | --- | --- |
| 31: 8 | Reserved | N/A | Reserved |
| 7:0 | RX_TL | R/W | Receive FIFO Threshold Level. Controls the level of entries (or above) that triggers the RX_FULL interrupt (bit 2 in IC_RAW_INTR_STAT register). The valid range is 0-15. A value of 0 sets the threshold for 1 entry, and a value of 15 sets the threshold for 16 entries. Reset value: 0x0 |

- ICx_TX_TL (x=0,1,2)

● Name: I 2CTransmit FIFO Threshold Register

● Description: I 2CTransmit FIFO Threshold Register

● Size: 32 bits

● Address Offset: 0x3c

● Read/Write Access: Read/Write

● Reset Value: 0x0

| 31: 8 | 7:0 |
| --- | --- |
| RSVD_IC_TX_TL | TX_TL |

Table 3-12 Fields for Register: ICx_TX_TL(x=0,1,2)

| Bits | Name | R/W | Description |
| --- | --- | --- | --- |
| 31: 8 | Reserved | N/A | Reserved |
| 7:0 | TX_TL | R/W | Transmit FIFO Threshold Level. Controls the level of entries (or below) that trigger the TX_EMPTY interrupt (bit 4 in IC_RAW_INTR_STAT register). The valid range is 0-15. A value of 0 sets the threshold for 0 entries, and a value of 15 sets the threshold for 15 entries. Reset value : 0x0 |

- IC x_CLR_INTR (x=0,1,2)

● Name: Clear Combined and Individual Interrupt Register

● Description: Clear Combined and Individual Interrupt Register

● Size: 32 bits

● Address Offset: 0x40

● Read/Write Access: Read

● Reset Value: 0x0

| 31:1 | 0 |
| --- | --- |
| RSVD_IC_CLR_INTR | CLR_INTR |

Table 3-13 Fields for Register: ICx_CLR_INTR(x=0,1,2)

| Bits | Name | R/W | Description |
| --- | --- | --- | --- |
| 31:1 | Reserved | N/A | Reserved |
| 0 | CLR_INTR | R | Read this register to clear the combined interrupt, all individual interrupts, and the IC_TX_ABRT_SOURCE register. This bit does not clear hardware clearable interrupts but software clearable interrupts. Refer to Bit 9 of the IC_TX_ABRT_SOURCE register for an exception to clearing IC_TX_ABRT_SOURCE Reset value: 0x0 |

- ICx_CLR_RX_UNDER (x=0,1,2)

● Name: Clear RX_UNDER Interrupt Register

● Description: Clear RX_UNDER Interrupt Register

● Size: 32 bits

● Address Offset: 0x44

● Read/Write Access: Read

● Reset Value: 0x0

| 31:1 | 0 |
| --- | --- |
| RSVD_IC_CLR_RX_UNDER | CLR_RX_UNDER |

Table 3-14 Fields for Register: ICx_CLR_RX_UNDER(x=0,1,2)

| Bits | Name | R/W | Description |
| --- | --- | --- | --- |
| 31:1 | Reserved | N/A | Reserved |
| 0 | CLR_RX_UNDER | R | Read this register to clear the RX_UNDER interrupt (bit 0) of the IC_RAW_INTR_STAT register Reset value: 0x0 |

- ICx_CLR_RX_OVER (x=0,1,2)

● Name: Clear RX_OVER Interrupt Register

● Description: Clear RX_OVER Interrupt Register

● Size: 32 bits

● Address Offset: 0x48

● Read/Write Access: Read

● Reset Value: 0x0

| 31:1 | 0 |
| --- | --- |
| RSVD_IC_CLR_RX_OVER | CLR_RX_OVER |

Table 3-15 Fields for Register: ICx_CLR_RX_OVER(x=0,1,2)

| Bits | Name | R/W | Description |
| --- | --- | --- | --- |
| 31:1 | Reserved | N/A | Reserved |
| 0 | CLR_RX_OVER | R | Read this register to clear the RX_OVER interrupt (bit 1) of t he IC_RAW_INTR_STAT register Reset value: 0x0 |

- ICx_CLR_TX_OVER (x=0,1,2)

● Name: Clear TX_OVER Interrupt Register

● Description: Clear TX_OVER Interrupt Register

● Size: 32 bits

● Address Offset: 0x4c

● Read/Write Access: Read

● Reset Value: 0x0

| 31:1 | 0 |
| --- | --- |
| RSVD_IC_CLR_TX_OVER | CLR_TX_OVER |

Table 3-16 Fields for Register: ICx_CLR_TX_OVER(x=0,1,2)

| Bits | Name | R/W | Description |
| --- | --- | --- | --- |
| 31:1 | Reserved | N/A | Reserved |
| 0 | CLR_TX_OVER | R | Read this register to clear the TX_OVER interrupt (bit 3) of the IC_RAW_INTR_STAT register Reset value: 0x0 |

- ICx_CLR_RD_REQ (x=0,1,2)

● Name: Clear RD_REQ Interrupt Register

● Description: Clear RD_REQ Interrupt Register

● Size: 32 bits

● Address Offset: 0x50

● Read/Write Access: Read

● Reset Value: 0x0

| 31:1 | 0 |
| --- | --- |
| RSVD_IC_CLR_RD_REQ | CLR_RD_REQ |

Table 3-17 Fields for Register: ICx_CLR_RD_REQ(x=0,1,2)

| Bits | Name | R/W | Description |
| --- | --- | --- | --- |
| 31:1 | Reserved | N/A | Reserved |
| 0 | CLR_RD_REQ | R | Read this register to clear the RD_REQ interrupt (bit 5) of the IC_RAW_INTR_STAT register Reset value: 0x0 |

- ICx_CLR_TX_ABRT (x=0,1,2)

● Name: Clear TX_ABRT Interrupt Register

● Description: Clear TX_ABRT Interrupt Register

● Size: 32 bits

● Address Offset: 0x54

● Read/Write Access: Read

● Reset Value: 0x0

| 31:1 | 0 |
| --- | --- |
| RSVD_IC_CLR_TX_ABRT | CLR_TX_ABRT |

Table 3-18 Fields for Register: ICx_CLR_TX_ABRT(x=0,1,2)

| Bits | Name | R/W | Description |
| --- | --- | --- | --- |
| 31:1 | Reserved | N/A | Reserved |
| 0 | CLR_TX_ABRT | R | Read this register to clear the TX_ABRT interrupt (bit 6) of the IC_RAW_INTR_STAT register, and the IC_TX_ABRT_SOURCE register. This also releases the TX FIFO from the flushed/reset state, allowing more writes to the TX FIFO. Refer to Bit 9 of the IC_TX_ABRT_SOURCE register for an exception to clearing IC_TX_ABRT_SOURCE Reset value : 0x0 |

- ICx_CLR_RX_DONE (x=0,1,2)

● Name: Clear RX_DONE Interrupt Register

● Description: Clear RX_DONE Interrupt Register

● Size: 32 bits

● Address Offset: 0x58

● Read/Write Access: Read

● Reset Value: 0x0

| 31:1 | 0 |
| --- | --- |
| RSVD_IC_CLR_RX_DONE | CLR_RX_DONE |

Table 3-19 Fields for Register: ICx_CLR_RX_DONE(x=0,1,2)

| Bits | Name | R/W | Description |
| --- | --- | --- | --- |
| 31:1 | Reserved | N/A | Reserved |
| 0 | CLR_RX_DONE | R | Read this register to clear the RX_DONE interrupt (bit 7) of the IC_RAW_INTR_STAT register Reset value: 0x0 |

- ICx_CLR_ACTIVITY (x=0,1,2)

● Name: Clear ACTIVITY Interrupt Register

● Description: Clear ACTIVITY Interrupt Register

● Size: 32 bits

● Address Offset: 0x5c

● Read/Write Access: Read

● Reset Value: 0x0

| 31: 1 | 0 |
| --- | --- |
| RSVD_IC_CLR_ACTIVITY | CLR_ACTIVITY |

Table 3-20 Fields for Register: ICx_CLR_ACTIVITY(x=0,1,2)

| Bits | Name | R/W | Description |
| --- | --- | --- | --- |
| 31:1 | Reserved | N/A | Reserved |
| 0 | CLR_ACTIVITY | R | Reading this register clears the ACTIVITY interrupt if the I 2Cis not active anymore. If the I 2Cmodule is still active on the bus, the ACTIVITY interrupt bit continues to be set. It is automatically cleared by hardware if the module is disabled and if there is no further activity on the bus. The value read from his register to get status of the ACTIVITY interrupt (bit 8) of the IC_RAW_INTR_STAT register Reset value: 0x0 |

- ICx_CLR_STOP_DET (x=0,1,2)

● Name: Clear STOP_DET Interrupt Register

● Description: Clear STOP_DET Interrupt Register

● Size: 32 bits

● Address Offset: 0x60

● Read/Write Access: Read

● Reset Value: 0x0

| 31 :1 | 0 |
| --- | --- |
| RSVD_IC_CLR_STOP_DET | CLR_STOP_DET |

Table 3-21 Fields for Register: ICx_CLR_STOP_DET(x=0,1,2)

| Bits | Name | R/W | Description |
| --- | --- | --- | --- |
| 31:1 | Reserved | N/A | Reserved |
| 0 | CLR_STOP_DET | R | Read this register to clear the STOP_DET interrupt (bit 9) of the IC_RAW_INTR_STAT register Reset value: 0x0 |

- ICx_CLR_START_DET (x=0,1,2)

● Name: Clear START_DET Interrupt Register

● Description: Clear START_DET Interrupt Register

● Size: 32 bits

● Address Offset: 0x64

● Read/Write Access: Read

● Reset Value: 0x0

| 31:1 | 0 |
| --- | --- |
| RSVD_IC_CLR_START_DET | CLR_START_DET |

Table 3-22 Fields for Register: ICx_CLR_START_DET(x=0,1,2)

| Bits | Name | R/W | Description |
| --- | --- | --- | --- |
| 31:1 | Reserved | N/A | Reserved. |
| 0 | CLR_START_DET | R | Read this register to clear the START_DET interrupt (bit 10) of the IC_RAW_INTR_STAT register Reset value: 0x0 |

- ICx_CLR_GEN_CALL (x=0,1,2)

● Name: Clear GEN_CALL Interrupt Register

● Description: Clear GEN_CALL Interrupt Register

● Size: 32 bits

● Address Offset: 0x68

● Read/Write Access: Read

● Reset Value: 0x0

● Reset Value: 0x0

| 31:1 | 0 |
| --- | --- |
| RSVD_IC_CLR_GEN_CALL | CLR_GEN_CALL |

Table 3-23 Fields for Register: ICx_CLR_GEN_CALL(x=0,1,2)

| Bits | Name | R/W | Description |
| --- | --- | --- | --- |
| 31:1 | Reserved | N/A | Reserved |
| 0 | CLR_START_DET | R | Read this register to clear the START_DET interrupt (bit 10 ) of the IC_RAW_INTR_STAT register Reset value: 0x0 |

Operation of the Interrupt Registers

| Interrupt Bit Fields | Set by Hardware/ Cleared by Software | Set and Cleared by Hardware |
| --- | --- | --- |
| MST_ON_HOLD | × | √ |
| RESTART_DET | √ | × |
| GEN_CALL | √ | × |
| START_DET | √ | × |
| STOP_DET | √ | × |
| ACTIVITY | √ | × |
| RX_DONE | √ | × |
| TX_ABRT | √ | × |
| RD_REQ | √ | × |
| TX_EMPTY | × | √ |
| TX_OVER | √ | × |
| RX_FULL | × | √ |
| RX_OVER | √ | × |
| RX_UNDER | √ | × |

- ICx_ENABLE (x=0,1,2)

● Name: I 2CEnable Register

● Size: 32 bits

● Address Offset: 0x6c

● Read/Write Access: Read/Write

● Reset Value: 0x0

| 31: 3 | 2 | 1 | 0 |
| --- | --- | --- | --- |
| RSVD_IC_ENABLE | TX_CMD_BLOCK | ABORT | ENABLE |

Table 3-24 Fields for Register: ICx_ENABLE(x=0,1,2)

| Bits | Name | R/W | Description |
| --- | --- | --- | --- |
| 31:3 | Reserved | N/A | Reserved |
| 2 | TX_CMD_BLOCK | R/W | ■1'b0: The transmission of data starts on I2C bus automatically, as soon as the first data is available in the Tx FIFO. Note: To block the execution of Master commands, set the TX_CMD_BLOCK bit only when Tx FIFO is empty (IC_STATUS[2]==1) and Master is in Idle state (IC_STATUS[5] == 0). Any further commands put in the Tx FIFO are not executed until TX_CMD_BLOCK bit is unset. Values: ■0x1 (BLOCKED): Tx Command execution blocked ■0x0 (NOT_BLOCKED): Tx Command execution not blocked Value After Reset: 0x0 |
| 1 | ABORT | R/W | When set, the controller initiates the transfer abort. ■0: ABORT not initiated or ABORT done ■1: ABORT operation in progress The software can abort the I2C transfer in master mode by setting this bit. The software can set this bit only when ENABLE is already set; otherwise, the controller ignores any write to ABORT bit. The software cannot clear the ABORT bit once set. In response to an ABORT, the controller issues a STOP and flushes the Tx FIFO after completing the current transfer, then sets the TX_ABORT interrupt after the abort operation. The ABORT bit is cleared automatically after the abort operation. For a detailed description on how to abort I2C transfers, refer to "Aborting I2C Transfers". Values: ■0x1 (ENABLED): ABORT operation in progress ■0x0 (DISABLE): ABORT operation not in progress Value After Reset: 0x0 |
| 0 | ENABLE | R/W | Controls whether the apb_i 2cis enabled. 0: Disables apb_i 2c(TX and RX FIFOs are held in an erased state) 1: Enables apb_i2c Software can disable apb_i 2cwhile it is active. However, it is important that care be taken to ensure that apb_i 2cis disabled properly. When apb_i 2cis disabled, the following occurs: • The TX FIFO and RX FIFO get flushed. • Status bits in the IC_INTR_STAT register are still active until apb_i 2cgoes into IDLE state. If the module is transmitting, it stops as well as deletes the contents of the transmit buffer after the current transfer is complete. If the module is receiving, the apb_i 2cstops the current transfer at the end of the current byte and does not acknowledge the transfer In systems with asynchronous pclk and ic_clk , there is a two ic_clk delay when enabling or disabling the apb_i2c. For a detailed description on how to disable apb_i2c, refer to "Disabling apb_i2c" Reset value: 0x0 |

- ICx_STATUS (x=0,1,2)

● Name: I 2CStatus Register

● Size: 32 bits

● Address Offset: 0x70

● Read/Write Access: Read

● Reset Value: 0x06

This is a read-only register used to indicate the current transfer status and FIFO status

| 31: 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| RSVD_IC_STATUS | SLV_ACTIVITY | MST_ACTIVITY | RFF | RFNE | TFE | TNF | ACTIVITY |

Table 3-25 Fields for Register: ICx_STATUS(x=0,1,2)

| Bits | Name | R/W | Description |
| --- | --- | --- | --- |
| 31 : 7 | Reserved | N/A | Reserved |
| 6 | SLV_ACTIVITY | R | Slave FSM Activity Status. When the Slave Finite State Machine (FSM) is not in the IDLE state, this bit is set. 0: Slave FSM is in IDLE state so the Slave part of apb_i 2cis not Active 1: Slave FSM is not in IDLE state so the Slave part of apb_i 2cis Active Reset value: 0x0 |
| 5 | MST_ACTIVITY | R | Master FSM Activity Status. When the Master Finite State Machine (FSM) is not in the IDLE state, this bit is set. 0: Master FSM is in IDLE state so the Master part of apb_i 2cis not Active 1: Master FSM is not in IDLE state so the Master part of apb_i 2cis Active Note IC_STATUS[0]—that is, ACTIVITY bit—is the OR of SLV_ACTIVITY and MST_ACTIVITY bits Reset value: 0x0 |
| 4 | RFF | R | Receive FIFO Completely Full. When the receive FIFO is completely full, this bit is set. When the receive FIFO contains one or more empty location, this bit is cleared 0: Receive FIFO is not full 1:Receive FIFO is full Reset value: 0x0 |
| 3 | RFNE | R | Receive FIFO Not Empty. This bit is set when the receive FIFO contains one or more entries; it is cleared when the receive FIFO is empty 0: Receive FIFO is empty 1: Receive FIFO is not empty Reset value: 0x0 |
| 2 | TFE | R | Transmit FIFO Completely Empty. When the transmit FIFO is completely empty, this bit is set. When it contains one or more valid entries, this bit is cleared. This bit field does not request an interrupt 0: Transmit FIFO is not empty 1: Transmit FIFO is empty Reset value: 0x1 |
| 1 | TNF | R | Transmit FIFO Not Full. Set when the transmit FIFO contains one or more empty locations, and is cleared when the FIFO is full 0: Transmit FIFO is full 1: Transmit FIFO is not full Reset value: 0x1 |
| 0 | ACTIVITY | R | I 2CActivity Status Reset value: 0x0 |

- ICx_TXFLR (x=0,1,2)

● Name: I 2CTransmit FIFO Level Register

● Description: I 2C Transmit FIFO Level Register

This register contains the number of valid data entries in the transmit FIFO buffer. It is cleared whenever:

❑ The I2C is disabled

❑ There is a transmit abort - that is, TX_ABRT bit is set in the IC_RAW_INTR_STAT register

❑ The slave bulk transmit mode is aborted

The register increments whenever data is placed into the transmit FIFO and decrements when data is taken from the transmit FIFO.

● Size: 32 bits

● Address Offset: 0x74

● Read/Write Access: Read

● Reset Value: 0x0

| 31:5 | 4:0 |
| --- | --- |
| RSVD_TXFLR | TxFLR |

Table 3-26 Fields for Register: ICx_TXFLR(x=0,1,2)

| Bits | Name | R/W | Description |
| --- | --- | --- | --- |
| 31: 5 | Reserved | N/A | Reserved |
| 4 : 0 | TxFLR | R | Transmit FIFO Level . Contains the number of valid data entries in the transmit FIFO Reset value: 0x0 |

- ICx_RXFLR (x=0,1,2)

● Name: I 2CReceive FIFO Level Register

● Description: I 2CReceive FIFO Level Register

This register contains the number of valid data entries in the receive FIFO buffer. It is cleared whenever:

❑ The I2C is disabled

❑ Whenever there is a transmit abort caused by any of the events tracked in IC_TX_ABRT_SOURCE

The register increments whenever data is placed into the receive FIFO and decrements when data is taken from the receive FIFO.

● Size: 32 bits

● Address Offset: 0x78

● Read/Write Access: Read

● Reset Value: 0x0

| 31:5 | 4:0 |
| --- | --- |
| RSVD_RXFLR | RXFLR |

Table 3-27 Fields for Register: ICx_RXFLR(x=0,1,2)

| Bits | Name | R/W | Description |
| --- | --- | --- | --- |
| 31: 5 | Reserved | N/A | Reserved |
| 4 :0 | RXFLR | R | Receive FIFO Level . Contains the number of valid data entries in the receive FIFO Reset value: 0x0 |

- ICx_SDA_HOLD(x=0,1,2)

● Name: I2C SDA Hold Time Length Register

● Description: I2C SDA Hold Time Length Register

The bits [15:0] of this register are used to control the hold time of SDA during transmit in both slave and master mode (after SCL goes from HIGH to LOW).

The bits [23:16] of this register are used to extend the SDA transition (if any) whenever SCL is HIGH in the receiver in either master or slave mode.

Writes to this register succeed only when IC_ENABLE[0]=0.

The values in this register are in units of ic_clk period. The value programmed in IC_SDA_TX_HOLD must be greater than the minimum hold time in each mode one cycle in master mode, seven cycles in slave mode for the value to be implemented.

The programmed SDA hold time during transmit (IC_SDA_TX_HOLD) cannot exceed at any time the duration of the low part of scl. Therefore the programmed value cannot be larger than N_SCL_LOW-2, where N_SCL_LOW is the duration of the low part of the scl period measured in ic_clk cycles.

● Size: 32 bits

● Address Offset: 0x7C

● Read/Write Access: Read/Write

● Reset Value: 0x01

| 31:24 | 23:16 | 15:0 |
| --- | --- | --- |
| RSVD_IC_SDA_HOLD | IC_SDA_RX_HOLD | IC_SDA_TX_HOLD |

Table 3-28 Fields for Register: ICx_SDA_HOLD(x=0,1,2)

| Bits | Name | R/W | Description |
| --- | --- | --- | --- |
| 31:24 | RSVD_IC_SDA_HOLD | R | IC_SDA_HOLD Reserved bits - Read Only |
| 23:16 | IC_SDA_RX_HOLD | R/W | Sets the required SDA hold time in units of ic_clk period, when apb_i2c acts as a receiver. Value After Reset: 0x0 |
| 15:0 | IC_SDA_TX_HOLD | R/W | Sets the required SDA hold time in units of ic_clk period, when apb_i2c acts as a transmitter. Value After Reset: 0x1 |

- ICx_TX_ABRT_SOURCE (x=0,1,2)

● Name: I 2CTransmit Abort Source Register

● Description: I2C Transmit Abort Source Register

This register has 32 bits that indicate the source of the TX_ABRT bit. Except for Bit 9, this register is cleared whenever the IC_CLR_TX_ABRT register or the IC_CLR_INTR register is read. To clear Bit 9, the source of the ABRT_SBYTE_NORSTRT must be fixed first; RESTART must be enabled (IC_CON[5]=1), the SPECIAL bit must be cleared (IC_TAR[11]), or the GC_OR_START bit must be cleared (IC_TAR[10]).

Once the source of the ABRT_SBYTE_NORSTRT is fixed, then this bit can be cleared in the same manner as other bits in this register. If the source of the ABRT_SBYTE_NORSTRT is not fixed before attempting to clear this bit, Bit 9 clears for one cycle and is then re-asserted.

● Size: 32 bits

● Address Offset: 0x80

● Read/Write Access: Read

● Reset Value: 0x0

This register has 16 bits that indicate the source of the TX_ABRT bit.

| 31:23 | 22:17 | 16 | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TX_FLUSH_CNT | RSVD_IC_TX_ABRT_SOURCE | ABRT_USER_ABRT | ABRT_SLLVRD_INTX | ABRT_SLV_ARBLOST | ABRT_SLVFLUSH_TXFIFO | ARB_LOST | ABRT_MASTER_DIS | ABRT_10B_RD_NORSTRT | ABRT_SBYTE_NORSTRT | ABRT_HS_NORSTRT | ABRT_SBYTE_ACKDET | ABRT_HS_ACKDET | ABRT_GCALL_READ | ABRT_GCALL_NOACK | ABRT_TXDATA_NOACK | ABRT_10ADDR2_NOACK | ABRT_10ADDR1_NOACK | ABRT_7B_ADDR_NOACK |

Table 3-29 Fields for Register: ICx_TX_ABRT_SOURCE(x=0,1,2)

| Bits | Name | R/W | Description | Role of apb_i2c |
| --- | --- | --- | --- | --- |
| 31:23 | TX_FLUSH_CNT | R | This field indicates the number of Tx FIFO Data Commands which are flushed due to TX_ABRT interrupt. It is cleared whenever I2C is disabled. Value After Reset: 0x0 | Master-Transmitter or Slave-Transmitter |
| 22:17 | Reserved | N/A | Reserved |  |
| 16 | ABRT_USER_ABRT | R | This is a master-mode-only bit. Master has detected the transfer abort (IC_ENABLE[1]) Values: ■0x1 (ABRT_USER_ABRT_GENERATED): Transfer abort detected by master ■0x0 (ABRT_USER_ABRT_VOID): Transfer abort detected by master- scenario not present Value After Reset: 0x0 | Master-Transmitter |
| 15 | ABRT_SLLVRD_INTX | R | 1: When the processor side responds to a slave mode request for data to be transmitted to a remote master and user writes a1 in CMD (bit 8) of IC_DATA_CMD register. Reset value: 0x0 | Slave-Transmitter |
| 14 | ABRT_SLV_ARBLOST | R | 1: Slave lost the bus while transmitting data to a remote master. IC_TX_ABRT_SOURCE [12] is set at the same time. Note: Even though the slave never “owns” the bus, something could go wrong on the bus bus. This is a fail safe check. For instance, during a data transmission at the low-to-high transition of SCL, if what is on the data bus is not what is supposed to be transmitted, then apb_i 2cno longer own the bus. Reset value: 0x0 | Slave-Transmitter |
| 13 | ABRT_SLVFLUSH_TXFIFO | R | 1: Slave has received a read command and some data exists in the TX FIFO so the slave issues a TX_ABRT interrupt to flush old data in TX FIFO. Reset value: 0x0 | Slave-Transmitter |
| 12 | ARB_LOST | R | 1: Master has lost arbitration, or if IC_TX_ABRT_SOURCE [14] is also set, then the slave transmitter has lost arbitration. Note: I 2Ccan be both master and slave at the same time. Reset value: 0x0 | Master-Transmitter or Slave-Transmitter |
| 11 | ABRT_MASTER_DIS | R | 1: User tries to initiate a Master operation with the Master mode disabled. Reset value: 0x0 | Master-Transmitter or Master-Receiver |
| 10 | ABRT_10B_RD_NORSTRT | R | 1: The restart is disabled ( IC_RESTART_EN bit (IC_CON[5]) = 0) and the master sends a read command in 10-bit addressing mode. Reset value: 0x0 | Master-Receiver |
| 9 | ABRT_SBYTE_NORSTRT | R | To clear Bit 9, the source of the ABRT_SBYTE_NORSTRT must be fixed first; restart must be enabled (IC_CON[5]=1), the SPECIAL bit must be cleared (IC_TAR[11]), or the GC_OR_START bit must be cleared (IC_TAR[10]). Once the source of the ABRT_SBYTE_NORSTRT is fixed, then this bit can be cleared in the same manner as other bits in this register. If the source of the ABRT_SBYTE_NORSTRT is not fixed before attempting to clear this bit, bit 9 clears for one cycle and then gets re-asserted. 1: The restart is disabled ( IC_RESTART_EN bit (IC_CON[5]) = 0) and the user is trying to send a START Byte. Reset value: 0x0 | Master |
| 8 | ABRT_HS_NORSTRT | R | 1: The restart is disabled ( IC_RESTART_EN bit (IC_CON[5]) = 0) and the user is trying to use the master to transfer data in High Speed mode. Reset value: 0x0 | Master-Transmitter or Master-Receiver |
| 7 | ABRT_SBYTE_ACKDET | R | 1: Master has sent a START Byte and the START Byte was acknowledged (wrong behavior). Reset value: 0x0 | Master |
| 6 | ABRT_HS_ACKDET | R | 1: Master is in High Speed mode and the High Speed Master code was acknowledged (wrong behavior). Reset value: 0x0 | Master |
| 5 | ABRT_GCALL_READ | R | 1: apb_i 2cin master mode sent a General Call but the user programmed the byte following the General Call to be a read from the bus ( IC_DATA_CMD [9] is set to1). Reset value: 0x0 | Master-Transmitter |
| 4 | ABRT_GCALL_NOACK | R | 1: apb_i 2cin master mode sent a General Call and no slave on the bus acknowledged the General Call. Reset value: 0x0 | Master-Transmitter |
| 3 | ABRT_TXDATA_NOACK | R | 1: This is a master-mode only bit. Master has received an acknowledgment for the address, but when it sends data byte(s) following the address, it did not receive an acknowledge from the remote slave(s). Reset value: 0x0 | Master-Transmitter |
| 2 | ABRT_10ADDR2_NOACK | R | 1: Master is in 10-bit address mode and the second address byte of the 10-bit address was not acknowledged by any slave. Reset value: 0x0 | Master-Transmitter or Master-Receiver |
| 1 | ABRT_10ADDR1_NOACK | R | 1: Master is in 10-bit address mode and the first 10-bit address byte was not acknowledged by any slave Reset value: 0x0 | Master-Transmitter or Master-Receiver |
| 0 | ABRT_7B_ADDR_NOACK | R | 1: Master is in 7-bit addressing mode and the address sent was not acknowledged by any slave. Reset value: 0x0 | Master-Transmitter or Master-Receiver |

- ICx_ACK_GENERAL_CALL(x=0,1,2)

● Name: I2C ACK General Call Register

● Description: I2C ACK General Call Register

The register controls whether apb_i2c responds with a ACK or NACK when it receives an I2C General Call address.

This register is applicable only when the apb_i2c is in slave mode.

● Size: 32 bits

● Address Offset: 0x98

● Read/Write Access: Read/Write

● Reset Value: 0x01

| 31 :1 | 0 |
| --- | --- |
| RSVD_IC_ACK_GEN_1_31 | ACK_GEN_CALL |

Table 3-30 Fields for Register: ICx_ACK_GENERAL_CALL(x=0,1,2)

| Bits | Name | Memory Access | Description |
| --- | --- | --- | --- |
| 31:1 | RSVD_IC_ACK_GEN_1_31 | R | RSVD_IC_ACK_GEN_1_31 Reserved bits - Read Only |
| 0 | ACK_GEN_CALL | R/W | ACK General Call. When set to 1, apb_i2c responds with a ACK (by asserting ic_data_oe) when it receives a General Call. Otherwise, apb_i2c responds with a NACK (by negating ic_data_oe). Values: ■ 0x1 (ENABLED): Generate ACK for a General Call ■ 0x0 (DISABLED): Generate NACK for General Call Value After Reset: 0x1 |

- ICx_ENABLE_STATUS (x=0,1,2)

● Name: I 2CEnable Status Register

● Size: 32 bits

● Address Offset: 0x9C

● Read/Write Access: Read

● Reset Value: 0x0

The register is used to report the apb_i 2chardware status when t he IC_ENABLE register is set from 1 to 0; that is, when apb_i 2cis disabled.

If IC_ENABLE has been set to 1, bits 2:1 are forced to 0, and bit 0 is forced to 1.

If IC_ENABLE has been set to 0, bits 2:1 is only be valid as soon as bit 0 is read as ' 0’.

Note

When IC_ENABLE has been written with '0,' a delay occurs for bit 0 to be read as '0’ because disabling the apb_i 2cdepends on I 2Cbus activities.

| 31:3 | 2 | 1 | 0 |
| --- | --- | --- | --- |
| RSVD_IC_ENABLE_STATUS | SLV_RX_DATA_LOST | SLV_DISABLED_WHILE_BUSY | IC_EN |

Table 3-31 Fields for Register: ICx_ENABLE_STATUS(x=0,1,2)

| Bits | Name | R/W | Description |
| --- | --- | --- | --- |
| 31: 1 | Reserved | N/A | Reserved |
| 2 | SLV_RX_DATA_LOST | R | Slave Received Data Lost. This bit indicates if a Slave- Receiver operation has been aborted with at least one data byte received from an I 2Ctransfer due to the setting of IC_ENABLE from 1 to 0. When read as 1, apb_i 2cis deemed to have been actively engaged in an aborted I 2Ctransfer (with matching address) and the data phase of the I 2Ctransfer has been entered, even though a data byte has been responded with a NACK. NOTE : If the remote I 2Cmaster terminates the transfer with a STOP condition before the apb_i 2chas a chance to NACK a transfer, and IC_ENABLE has been set to 0, then this bit is also set to 1. When read as 0, apb_i 2cis deemed to have been disabled without being actively involved in the data phase of a Slave-Receiver transfer. NOTE: The CPU can safely read this bit when IC_EN (bit 0) is read as 0. Reset value: 0x0 |
| 1 | SLV_DISABLED_WHI LE_BUSY | R | Slave Disabled While Busy (Transmit, Receive) This bit indicates if a potential or active Slave operation has been aborted due to the setting of the IC_ENABLE register from 1 to 0. This bit is set when the CPU writes a 0 to the IC_ENABLE register while: (a) apb_i 2cis receiving the address byte of the Slave-Transmitter operation from a remote master; OR, (b) address and data bytes of the Slave-Receiver operation from a remote master. When read as 1, apb_i 2cis deemed to have forced a NACK during any part of an I 2Ctransfer, irrespective of whether the I 2Caddress matches the slave address set in apb_i 2c(IC_SAR register) OR if the transfer is completed before IC_ENABLE is set to 0 but has not taken effect. NOTE : If the remote I 2Cmaster terminates the transfer with a STOP condition before the apb_i 2chas a chance to NACK a transfer, and IC_ENABLE has been set to 0, then this bit will also be set to 1. When read as 0, apb_i 2cis deemed to have been disabled when there is master activity, or when the I 2Cbus is idle. NOTE: The CPU can safely read this bit when IC_EN (bit 0) is read as 0. Reset value: 0x0 |
| 0 | IC_EN | R | ic_en Status . This bit always reflects the value driven on the output port ic_en. When read as 1, apb_i 2cis deemed to be in an enabled state. When read as 0, apb_i 2cis deemed completely inactive. NOTE: The CPU can safely read this bit anytime. When this bit is read as 0, the CPU can safely read SLV_RX_DATA_LOST (bit 2) and SLV_DISABLED_WHILE_BUSY (bit 1) Reset value: 0x0 |

- ICx_FS_SPKLEN(x=0,1,2)

● Name: I2C SS, FS or FM+ spike suppression limit

● Description: I2C SS, FS or FM+ spike suppression limit

This register is used to store the duration, measured in ic_clk cycles, of the longest spike that is filtered out by the spike suppression logic w hen the component is operating in SS, FS or FM+ modes. The relevant I2C requirement is tSP (table 4 ) as detailed in the I2C Bus Specification. This register must be programmed with a minimum value of 1.

● Size: 32 bits

● Address Offset: 0xA0

● Read/Write Access: Read/Write

● Reset Value: 0x05

| 31:8 | 7:0 |
| --- | --- |
| RSVD_IC_FS_SPKLEN | IC_FS_SPKLEN |

Table 3-32 Fields for Register: ICx_FS_SPKLEN(x=0,1,2)

| Bits | Name | Memory Access | Description |
| --- | --- | --- | --- |
| 31:8 | RSVD_IC_FS_SPKLEN | R | IC_FS_SPKLEN Reserved bits - Read Only |
| 7:0 | IC_FS_SPKLEN | R/W | This register must be set before any I2C bus transaction can take place to ensure stable operation. This register sets the duration, measured in ic_clk cycles, of the longest spike in the SCL or SDA lines that will be filtered out by the spike suppression logic. This register can be written only when the I2C interface is disabled which corresponds to the IC_ENABLE[0] register being set to 0. Writes at other times have no effect. The minimum valid value is 1; hardware prevents values less than this being written, and if attempted results in 1 being set. Value After Reset: 0x5 |

- ICx_HS_SPKLEN(x=0,1,2)

● Name: I2C HS spike suppression limit register

● Description: I2C HS spike suppression limit register

This register is used to store the duration, measured in ic_clk cycles, of the longest spike that is filtered out by the spike suppression logic when the component is operating in HS modes. The relevant I2C requirement is tSP (table 6) as detailed in the I2C Bus Specification. This register must be programmed with a minimum value of 1;

● Size: 32 bits

● Address Offset: 0Xa4

● Read/Write Access: Read/Write

● Reset Value: 0x01

| 31:8 | 7:0 |
| --- | --- |
| RSVD_IC_HS_SPKLEN | IC_HS_SPKLEN |

Table 3-33 Fields for Register: ICx_HS_SPKLEN(x=0,1,2)

| Bits | Name | Memory Access | Description |
| --- | --- | --- | --- |
| 31:8 | RSVD_IC_HS_SPKLEN | R | IC_HS_SPKLEN |
| 7:0 | IC_HS_SPKLEN | R/W | This register must be set before any I2C bus transaction can take place to ensure stable operation. This register sets the duration, measured in ic_clk cycles, of the longest spike in the SCL or SDA lines that will be filtered out by the spike suppression logic; for more information, refer to "Spike Suppression" This register can be written only when the I2C interface is disabled which corresponds to the IC_ENABLE[0] register being set to 0. Writes at other times have no effect. The minimum valid value is 1; hardware prevents values less than this being written, and if attempted results in 1 being set. Value After Reset: 0x1 |

- ICx_COMP_VERSION (x=0,1,2)

● Name: I 2CComponent Version Register

● Size: 32 bits

● Address Offset: 0xf8

● Read/Write Access: Read

● Reset Value: 0x3230 322a

31:0

IC_COMP_VERSION

Table 3-34 Fields for Register: ICx_COMP_VERSION(x=0,1,2)

| Bits | Name | R/W | Description |
| --- | --- | --- | --- |
| 31:0 | IC_COMP_VERSION | R | I 2CVersion Number Reset value : 0x3230_322a |

4 Program Guide

- a few terms

Transmitter —— the device that sends data to the bus. A transmitter can either be a device that initiates the data transmission to the bus (a master-transmitter ) or responds to a request from the master to send data to the bus (a slave-transmitter )

Receiver —— the device that receives data from the bus. A receiver can either be a device that receives data on its own request (a master-receiver ) or in response to a request from the master (a slave-receiver )

Master —— the component that initializes a transfer (START command), generates the clock (SCL) signal and terminates the transfer (STOP command). A master can be either a transmitter or a receiver

Slave —— the device addressed by the master. A slave can be either receiver or transmitter

Figure 3 Master/Slave and Transmitter/Receiver Relationships

SDA —— data signal line (Serial Data )

SCL —— clock signal line (Serial C l ock)

- I2C/SMBus Definition of Bits in First Byte

| Slave Address | R/W | Bit Description |
| --- | --- | --- |
| 0000 000 0 | 0 | General Call Address. apb_i2c places the data in the receive buffer and issues a General Call interrupt. |
| 0000 000 1 | 1 | START byte. |
| 0000 001X | X | CBUS address. apb_i2c ignores these accesses |
| 0000 010X | X | Reserved. |
| 0000 011X | X | Reserved. |
| 0000 1XX | X | High-speed master code. |
| 1111 1XX | X | Reserved. |
| 1111 0XX | X | 10-bit slave addressing. |
| 0001 000X | X | SMbus Host |
| 0001 100X | X | SMBus Alert Response Address |
| 1100 001X | X | SMBus Device Default Address |

- operating mode

- Master operation

- initialization

(1)Disable DW_apb_i2c

The register IC_ENABLE_STATUS is added to allow software to unambiguously determine when the hardware has completely shutdown in response to bit 0 of the IC_ENABLE register being set from 1 to 0. Only one register is required to be monitored, as opposed to monitoring two registers (IC_STATUS and IC_RAW_INTR_STAT) which is a requirement for apb_i2c versions 1.05a or earlier.

The apb_i2c Master can be disabled only if the current command being processed—when the ic_enable de-assertion occurs—has the STOP bit set to 1.

When an attempt is made to disable the apb_i2c Master while processing a command without the STOP bit set, the apb_i2c Master continues to remain active, holding the SCL line low until a new command is received in the Tx FIFO.

When the apb_i2c Master is processing a command without the STOP bit set, you can issue the ABORT (IC_ENABLE[1]) to relinquish the I2C bus and then disable apb_i2c.

(2) Write control register IC_CON

a)Set it as Master. Set B it[0] and bit[6] to 1 at the same time;

b) Set the maximum speed mode supported by the Device (bit[2:1]);

c) Select whether to turn on the Restart transmission function, bit[5] of IC_CON;

d) Set the addressing mode, bit[4] of IC_CON (which is 7/10 bits, depending on the Device to be addressed).

(3) Configure the high and low level times of the SCL clock ( refer to the standard mode as follows )

a) Write registers IC_SS_SCL_HCNT——(SCL high level number) and IC_SS_SCL_LCNT——(SCL low level number).

corresponding high level time of SCL (I 2Cserial transmission clock) = (IC_SS_SCL_HCNT+8)* Period i 2c_clk ;

Low level time=(IC_SS_SCL_LCNT+1)*Period i 2c_clk .

Note: SCL serial clock high and low level requirements:

The transmission rate is 100kb/s (Standard), the minimum high-level time of SCL is 4000ns, and the minimum low-level time is 4700ns.

The specific calculation is as follows: Assume that the ic_clk period is 46.666ns,

IC_SS_SCL_LCNT: Then 4700/46.666=100.7, rounded to 101. Subtract 1 from this value to get the minimum write value of the register 101-1=100.

IC_SS_SCL_HCNT = 4000/46.666=85.7, 86 after rounding. Subtract 8 from this value to get the minimum write value of this register 86-8=78.

(4) Write register IC_TAR (can also 2Cbe set after Enable I)

a )bit[11:10], indicating 2Cwhether I completed NORMAL, General Call or START BYTE transmission

b )bit[9:0], the address of the Device to be addressed

(5) Set the threshold of FIFO (can also 2Cbe set after Enable I)

Write register: IC_TX_TL (send FIFO)

IC_RX_TL (receive FIFO)

(6) Set the interrupt mask bit ( can also be set after Enable I 2C)

Write register IC_INTR_MASK

(7)Enable DW_apb_i2c

Write register IC_ENABLE to 1

(8) Write data and transmission direction (can only 2Cbe set after Enable I)

Write register IC_DATA_CMD

- Master sends data (Master-Transmitter)

(1) Write bit[8] of register IC_DATA_CMD ——0 ( write )

(2) Query the START interrupt status - interrupt status register IC_INTR_STAT or unmasked interrupt status register IC_RAW_INTR_STAT

(3) After detecting the START interrupt, continue to query the Tx_EMPTY (transmit FIFO empty) interrupt status

(4) After detecting the Tx_EMPTY interrupt, write data to the Tx FIFO (write the register IC_DATA_CMD[7:0], bit[8] cannot write 1, and the data sent thereafter must keep bit[8] as 0.)

(5) After writing the data to be sent, detect the STOP interrupt status

(6) After checking the STOP status, clear the interrupt status

(7)Disable I2C

(8) Query bit[0] (IC_EN bit) of the register IC_ENABLE_STATUS .

If it is 1, it means that I 2Chas not been truly closed, this transmission has not been completed, and the next transmission cannot be continued.

If 1 is found for a long time within a certain period of time, the software can return an error to indicate that I 2Cis not properly Disabled.

If it is 0, it means that I 2Cis indeed closed and the next transmission can be carried out.

- Master receives data (Master-Receiver)

(1) Write transfer direction

Write bit[8] of register IC_DATA_CMD ——1 ( read )

(2) Query the START interrupt status - query the interrupt status register IC_INTR_STAT or the unmasked interrupt status register IC_RAW_INTR_STAT

(3) After detecting the START interrupt, check whether the register IC_RXFLR (the number of data in the receive FIFO) is 0

(4) If it is detected that it is not 0, read the register IC_DATA_CMD and read the data away

(5) Detect STOP interrupt status

(6) After checking the STOP status, clear the interrupt status

(7)Disable I2C

(8) Query bit[0] (IC_EN bit) of the register IC_ENABLE_STATUS .

If it is 1, it means that I 2Chas not been truly closed, this transmission has not been completed, and the next transmission cannot be continued.

If 1 is found for a long time within a certain period of time, the software can return an error to indicate that I 2Cis not properly Disabled.

If it is 0, it means that I 2Cis indeed closed and the next transmission can be carried out.

Note: Because only one data can be read back at a time, the register IC_RXFLR is queried in c), but the RX_FULL (receive FIFO full) interrupt is not queried. To read multiple data, multiple read operations (1)~(6) need to be initiated.

- I2C/SMBus Definition of Bits in First Byte

When the apb_i2c is configured as a Standard (SS), Fast (FS)/Fast-Mode Plus (FM+), or High Speed (HS) master, the *CNT registers must be set before any I2C bus transaction can take place in order to ensure proper I/O timing. The *CNT registers are:

■ IC_SS_SCL_HCNT

■ IC_SS_SCL_LCNT

■ IC_FS_SCL_HCNT

■ IC_FS_SCL_LCNT

■ IC_HS_SCL_HCNT

■ IC_HS_SCL_LCNT

- Minimum High and Low Counts in SS, FS, FM+ and high speed Modes

When the apb_i2c operates as an I2C master, in both transmit and receive transfers:

■ IC_SS_SCL_LCNT and IC_FS_SCL_LCNT register values must be larger than IC_FS_SPKLEN + 7.

■ IC_SS_SCL_HCNT and IC_FS_SCL_HCNT register values must be larger than IC_FS_SPKLEN + 5.

■ IC_HS_SCL_LCNT register value must be larger than IC_HS_SPKLEN + 7.

■ IC_HS_SCL_HCNT register value must be larger than IC_HS_SPKLEN + 5.

Details regarding the apb_i2c high and low counts are as follows:

■ The minimum value of IC_*_SPKLEN + 7 for the *_LCNT registers is due to the time required for the apb_i2c to drive SDA after a negative edge of SCL.

■ The minimum value of IC_*_SPKLEN + 5 for the *_HCNT registers is due to the time required for the apb_i2c to sample SDA during the high period of SCL.

■ The apb_i2c adds one cycle to the programmed *_LCNT value in order to generate the low period of the SCL clock; this is due to the counting logic for SCL low counting to (*_LCNT + 1).

■ The apb_i2c adds IC_*_SPKLEN + 7 cycles to the programmed *_HCNT value in order to generate the high period of the SCL clock; this is due to the following factors:

❑ The counting logic for SCL high counts to (*_HCNT+1).

❑ The digital filtering applied to the SCL line incurs a delay of SPKLEN + 2 ic_clk cycles, where SPKLEN is:

■ IC_FS_SPKLEN if the component is operating in SS or FS

■ IC_HS_SPKLEN if the component is operating in high speed.

This filtering includes metastability removal and the programmable spike suppression on SDA and SCL edges.

❑ Whenever SCL is driven 1 to 0 by the apb_i2c—that is, completing the SCL high time—an

internal logic latency of three ic_clk cycles is incurred. Consequently, the minimum SCL low time

of which the apb_i2c is capable is nine (9) ic_clk periods (7 + 1 + 1), while the minimum SCL

high time is thirteen (13) ic_clk periods (6 + 1 + 3 + 3).

Chapter 16 LISoC_APB_RTC

1 Function list

This module provides a Real Time Clock (RTC) peripheral to the chip .

● Incrementing counter and comparator for interrupt generation

● Free-running pclk

● Counter enable mode

● Counter wrap mode

● Some uses of the apb_rtc are:

■ Real-time clock – used with software for keeping track of time

■ Long-term, exact chronometer – When clocked with a 1 Hz clock, it can keep track of time from now up to 136 years in the future

■ Alarm function – generates an interrupt after a programmed number of cycles

■ Long-time, base counter – clocked with a very slow clock signa

2 Register list

Base address: 0x 40002C00

| Offset | Register | R/W | Size | Description |
| --- | --- | --- | --- | --- |
| 0x0 | RTC_CCVR | R | 32bits | Current Counter Value Register Range: 0 to 2 32 -1 Reset value: 32’b0 |
| 0x4 | RTC_CMR | R/W | 32bits | Counter Match Register Range: 0 to 2 32 -1 Reset value: 32 'b0 |
| 0x8 | RTC_CLR | R/W | 32bits | Counter Load Register Reset value: 3'b0 |
| 0xc | RTC_CCR | R/W | 32 bits | Counter Control Register Reset value: 1'b0 |
| 0x10 | RTC_STAT | R | 32 bits | Interrupt Register Status Reset value: 1'b0 |
| 0x14 | RTC_RSTAT | R | 32bits | Interrupt Raw Status Register Reset value: 1 'b0 |
| 0x18 | RTC_EOI | R | 32bits | End Of Interrupt Register Reset value: 1 'b0 |
| 0x1C | RTC_COMP_VERSION | R | 32bits | Component Version Register Reset value: 32 'b3230372a |
| 0x20 | RTC_CPST | R/W | 32bits | Counter PreScaler Register Reset value: 16 'b2 |
| 0x24 | RTC_CPCVR | R | 32bits | Current Prescaler Counter Value Register Reset value: 16 'b0 |

3 Detailed definition of registers

- RTC_CCVR

● Name: Current Counter Value Register

● Description : Current Counter Value Register

● Size: 32bits

● Address Offset: 0x00

● Read/write access: read

● Reset Value: 0x0

31:0

Current_Counter_Value

| Bits | Name | Memory Access | Description |
| --- | --- | --- | --- |
| 31:0 | Current_Counter_Value | R | When read, this register is the current value of the internal counter. This value is always read coherently. Value After Reset: 0x0 |

- RTC_CMR

● Name: Counter Match Register

● Description : Counter Match Register

● Size: 32bits

● Address Offset: 0x04

● Read/write access: read/write

● Reset Value: 0x0

31:0

Counter_Match

| Bits | Name | Memory Access | Description |
| --- | --- | --- | --- |
| 31:0 | Counter_Match | R/W | Interrupt Match Register. When the internal counter matches this register, an interrupt is generated, provided interrupt generation is enabled. When appropriate, this value is written coherently. Only when all the bytes are written is the register used by the interrupt detection logic. Value After Reset: 0x0 |

- RTC_CLR

● Name: Counter Load Register

● Description : Counter Load Register

● Size: 32bits

● Address Offset: 0x08

● Read/write access: read/write

● Reset Value: 0x0

31:0

Counter_Load

| Bits | Name | Memory Access | Description |
| --- | --- | --- | --- |
| 31:0 | Counter_Load | R/W | Loaded into the counter as the loaded value, which is written coherently. Value After Reset: 0x0 |

- RTC_CCR

● Name: Counter Control Register

● Description : Counter Control Register

Note: If the RTC_RSTAT register indicates that a pending interrupt exists because a masked interrupt has been generated (rtc_mask bit of RTC_CCR is set to 1), then caution should be taken when programming RTC_CCR. That is, if both the rtc_mask and rtc_en bits of the RTC_CCR register are set to 0, then the interrupt asserts for one clock period. To avoid this scenario, you can use the following two-step process:

1. Program the rtc_en bit of the RTC_CCR register to 0, which clears the interrupt.

2. Then program the RTC_CCR register again to set the rtc_mask bit to 0, which unmasks any future interrupts

● Size: 32bits

● Address Offset: 0x0C

● Read/write access: read/write

● Reset Value: 0x0

| 31:5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- |
| RSVD_CCR | rtc_psclr_en | rtc_wen | rtc_en | rtc_mask | rtc_ien |

| Bits | Name | Memory Access | Description |
| --- | --- | --- | --- |
| 31:5 | RSVD_CCR | R | RTC_CCR 31to8 Reserved and read as 0. Value After Reset: 0x0 |
| 4 | rtc_psclr_en | R/W | Optional. Allows user to control the usage of RTC Prescaler feature. Values: ■ 0x0 (DISABLED): Disables the Prescaler counter ■ 0x1 (ENABLED): Enables the Prescaler counter Value After Reset: 0x0 |
| 3 | rtc_wen | R/W | Optional . Allows the user to force the counter to wrap when a match occurs instead of waiting until the maximum count is reached. Values: ■ 0x0 (DISABLED): Disables the WRAP ■ 0x1 (ENABLED): Enables the WRAP Value After Reset: 0x0 |
| 2 | rtc_en | R/W | Optional. Allows the user to control counting in the counter. Values: ■ 0x0 (DISABLED): Disables the counter ■ 0x1 (ENABLED): Enables the counter Value After Reset: 0x0 |
| 1 | rtc_mask | R/W | Allows the user to mask interrupt generation. Values: ■ 0x0 (UNMASKED): Interrupt unmasked ■ 0x1 (MASKED): Interrupt masked Value After Reset: 0x0 |
| 0 | rtc_ien | R/W | Allows the user to disable interrupt generation. Values: ■ 0x0 (DISABLED): Disables the interrupt generation ■ 0x1 (ENABLED): Enables the interrupt generation Value After Reset: 0x0 |

- RTC_STAT

● Name: Interrupt Status Register

● Description : Interrupt Status Register

● Size: 32bits

● Address Offset: 0x10

● Read/write access: read

● Reset Value: 0x0

| 31: 1 | 0 |
| --- | --- |
| RSVD_RTC_STAT | rtc_stat |

| Bits | Name | Memory Access | Description |
| --- | --- | --- | --- |
| 3 1:1 | RSVD_RTC_STAT | R | Reserved bits. V alue After Reset: 0x0 |
| 0 | rtc_stat | R | This register is the masked raw status. Values: ■0x0 (INACTIVE): Interrupt is inactive ■0x1 (ACTIVE): Interrupt is active (regardless of polarity) Value After Reset: 0x0 |

- RTC_RSTAT

● Name: Interrupt Raw Status Register

● Description : Interrupt R aw Status Register

● Size: 32bits

● Address Offset: 0x14

● Read/write access: read

● Reset Value: 0x0

| 31: 1 | 0 |
| --- | --- |
| RSVD_RTC_RSTAT | rtc_rstat |

| Bits | Name | Memory Access | Description |
| --- | --- | --- | --- |
| 3 1:1 | RSVD_RTC_RSTAT | R | Reserved bits. V alue After Reset:0x0 |
| 0 | rtc_rstat | R | This register is the raw status. Values: ■0x0 (INACTIVE): Interrupt is inactive ■0x1 (ACTIVE): Interrupt is active (regardless of polarity) Value After Reset: 0x0 |

- RTC_EOI

● Name: End Of Interrupt Register

● Description : End Of Interrupt Register

● Size: 32bits

● Address Offset: 0x18

● Read/write access: read

● Reset Value: 0x0

| 31: 1 | 0 |
| --- | --- |
| RSVD_RTC_EOI | rtc_eoi |

| Bits | Name | Memory Access | Description |
| --- | --- | --- | --- |
| 3 1:1 | RSVD_RTC_EOI | R | Reserved bits. V alue After Reset: 0x0 |
| 0 | rtc_eoi | R | By reading this location, the match interrupt is cleared. Performing read-to-clear on interrupt, the interrupt is cleared at the end of the read. Value After Reset: 0x0 |

- RTC_COMP_VERSION

● Name: Component Version Register

● Description : Component Version Register

● Size: 32bits

● Address Offset: 0x1c

● Read/write access: read

● Reset Value: 0x3230 372a

31: 0

Rtc_comp_version

| Bits | Name | Memory Access | Description |
| --- | --- | --- | --- |
| 31:0 | rtc_eoi | R | ASCII value for each number in the version, followed by *. For example, 32_30_31_2A represents the version 2.01*. Value After Reset: 0x3230372a |

- RTC_CPSR

● Name: Counter PreScaler Register

● Description : Counter PreScaler Register

● Size: 32bits

● Address Offset: 0x20

● Read/write access: read

● Reset Value: 0x2

| 31: 1 | 0 |
| --- | --- |
| RSVD_RTC_CPSR | Counter_Prescaler_Value |

| Bits | Name | Memory Access | Description |
| --- | --- | --- | --- |
| 3 1:16 | RSVD_RTC_CPSR | R | Reserved bits. V alue After Reset: 0x0 |
| 15:0 | Counter_Prescaler_Value | R/W | Counter Prescaler Register. The RTC counter will be updating at the rate of rtc_clk, rtc_clk_en, or pclk based on the configuration. This register is used to prescale the rate at which the RTC counter updates. When appropriate, this register is written coherently. Only when all the bytes are written, the register used by the prescaler counter logic. Value After Reset: 0x2 |

- RTC_CPCVR

● Name: Current Prescaler Counter Value Register

● Description : Current Prescaler Counter Value Register

● Size: 32bits

● Address Offset: 0x24

● Read/write access: read

● Reset Value: 0x0

| 31: 1 | 0 |
| --- | --- |
| RSVD_RTC_CPCVR | Counter_Prescaler_Counter_Value |

| Bits | Name | Memory Access | Description |
| --- | --- | --- | --- |
| 3 1:16 | RSVD_RTC_CPCVR | R | Reserved bits. V alue After Reset: 0x0 |
| 15:0 | Counter_Prescaler_Counter_Value | R/W | When read, this register provides the current value of the internal prescaler counter. This value always is read coherently. Value After Reset: 0x0 |

4 Program Guide

- Set the initial value and comparison value of the counter, and enable the interrupt.

- Enable the counter and wait for the interrupt.

- Check the interrupt status and clear it.

- Set the prescale mode and enable it.

- Reset the comparison value and wait for the interrupt.

- Check the interrupt status and clear it.

Chapter 17 LISoC_APB_DDRC

1 Function list

- Support 4 x 512MB DDR4 RAM. Supported data width is 64 bits.

- Support DDR calibration

- 二つの32ビットのDDRコントローラーを実装し、1600Mbpsスピードモードでactionする.

- 総スループット (theoretical limit): 2*1600*32/8= 12.8GBps

2 Register list

2.1 DDR PHY register list

BASEADDR: 0x 4001 0000 (DDR0_Phy) 0x4002 0000 (DDR1_Phy)

Offset: 0x0 ~ 0x438

2.2 DDR Control register list

BASEADDR: 0x 4003 0000 (DDR0_Ctrl) 0x4004 0000 (DDR1_Ctrl)

Offset: 0x0 ~ 0x414

3 Detailed definition of registers

3.1 Detailed definition of DDR PHY register

BASEADDR：0x4001 0000 (DDR0_Phy)   0x4002 0000 (DDR1_Phy)

Offset: 0x0

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [11:8] | reg_channel_en | R/W | 0xf | The byte enable signal of the PHY. High is valid. The defautl mode will open the four bytes at the same time. [0]: The byte0 enable signal, corresponding to A_DQ0~A_DQ7. [1]: The byte1 enable signal, corresponding to A_DQ8~A_DQ15. [2]: The byte2 enable signal, corresponding to B_DQ0~B_DQ7. [3]: The byte3 enable signal, corresponding to B_DQ8~B_DQ15. If the user only wants to use two bytes, we suggest to choose the byte0 and byte1. |
| [7] | reg_burst_sel | R/W | 0x1 | According to the SDRAM type to choose the burst type that the PHY needs to support. 1: Support the BL8/BL16 0: Reserved. |
| [6:4] | mem_select_t | R/W | 0x0 | Choose the SDRAM that the PHY need to support. 3’h0:DDR2 PHY mode. 3’h1:LPDDR2 PHY mode. 3’h2:DDR3 PHY mode. 3’h3:LPDDR3 PHY mode. 3’h4:DDR4 PHY mode. 3’h5: LPDDR4 PHY mode. Others: Reserved. |
| [2] | soft_reset1 | R/W | 0x1 | The reset signal of digital core, Active low. |
| [1] | soft_reset0 | R/W | 0x1 | The reset signal of analog logic, Active low. |
| [0] | soft_reset | R/W | 0x1 | The reset signal of digital core and analog logic. Active low. |

Offset: 0x4

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [31:16] | reg_wl_loadmode | R/W | 0x0 | [7:0]Write leveling load mode [7:0]. Related to the register 10’h002 [2]. The PHY needs to configure the load mode to enable the write leveling function. In case of that write leveling operation changs the load mode value in the SDRAM which is written by the MC, so we need to use this register to keep the same setting with the MC for the mode register which is related to the write leveling. This is the low 8bits of the mode register which is related to the write leveling. For DDR3/DDR4, this register should keep the same value with the MR1[7:0]. [15:8]"Write leveling load mode [15:8]. Relate to the register 10’h002 [2]. The PHY needs to configure the load mode to enable the write leveling function. In case of that write leveling operation changs the load mode value in the SDRAM which is written by the MC, so we need to use this register to keep the same setting with the MC for the mode register which is related to the write leveling. This is the low 8bits of the mode register which is related to the write leveling. For DDR3/DDR4, this register [5:0] should keep the same value with the MR1[13:8] and this register[7:6] should set 2'b01. |
| [7:6] | reg_wlcs_sel | R/W | 0x2 | The rank select signal of the write-leveling fuction. Low is valid. 2'b00: The write-leveling result will auto switch between the RANK0 and RANK1 according to the DFI interface command after the write-elveling training. 2'b01: Choose the RANK1. The chip which connects to CS1 will be chooses to enable the write-leveling training. 2'b10: Choose the RANK0. The chip which connects to CS0 will be chooses to enable the write-leveling training. 2'b11: Reserved. Note: Can't set to 2'b00 when enable the write-levleing training funciton. If the PHY needs to support two ranks, this register should set to 2'b00 after the write-leveling training. |
| [5] | reg_wl_bypass | R/W | 0x0 | The tx perbit-skew bypass function enable signal. High is valid. 0: Use the write-leveling training result to control the tx perbit-skew delay. The A_DQ0~A_DQ7/A_DM0 will use the A_DQS0 training result. The A_DQ8~A_DQ15/A_DM1 will use the A_DQS1 training result. The B_DQ0~A_DQ7/A_DM0 will use the B_DQS0 training result. The B_DQ8~A_DQ15/A_DM0 will use the B_DQS1 training result. 1: Use the register to control the tx perbit-skew delay. Each data pad has an independet controller register. |
| [4] | reg_wl_enable | R/W | 0x0 | The write-leveling training enable signal. High is valid. 0: Keep current state or exit the write-leveling training state. 1: Eanble the write-leveling training function. |
| [3:2] | reg_calcs_sel | R/W | 0x2 | The rank select signal of the rx-dqs calibration fuction. Low is valid. 2'b00: The rx-dqs calibration result will auto switch between the RANK0 and RANK1 according to the DFI interface command after the rx-dqs calibration training. 2'b01: Choose the RANK1. The chip which connects to CS1 will be chooses to enable the auto rx-dqs calibration training. 2'b10: Choose the RANK0. The chip which connects to CS0 will be chooses to enable the auto rx-dqs calibration training. 2'b11: Reserved. Note: Can't set to 2'b00 when enable the rx-dqs calibration training funciton. If the PHY needs to support two ranks, this register should set to 2'b00 after the rx-dqs calibration training. |
| [1] | reg_calib_bypass | R/W | 0x0 | The rx-dqs calibration control bypass enable signal. High is valid. 0: Use the auto rx-dqs calibration training result to control the rx-dqs gating signal. 1: Use the bypass register to control the rx-dqs gating signal. Each DQS and Each rank has independent bypass register. |
| [0] | reg_start_calib | R/W | 0x0 | The auto rx-dqs calibration training enable signal. High is valid. 0: Keep current state or exit the auto rx-dqs calibration training. 1: Enable the auto rx-dqs calibration training function. |

Offset: 0x8

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [29:24] | AL_FRE_OP0 | R/W | 0x0 | Setting the AL vlaue of the PHY for frequency point 0. When enable the Fast Frequency Change Mode, the PHY will choose AL_FEE_OP0 as the AL setting of the PHY when the reg_freq_choose_t == 2'b00.. For LPDDRn, this register should keep default value. For DDRn, this register should keep the same AL setting with the SDRAM. |
| [21:16] | AL_FRE_OP1 | R/W | 0x0 |  |
| [13:8] | AL_FRE_OP2 | R/W | 0x0 |  |
| [5:0] | AL_FRE_OP3 | R/W | 0x0 |  |

Offset: 0xC

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [29:24] | CL_FRE_OP0 | R/W | 0x6 | Setting the CL/RL vlaue of the PHY for frequency point 0. When enable the Fast Frequency Change Mode, the PHY will choose CL_FEE_OP0 as the CL/RL setting of the PHY when the reg_freq_choose_t == 2'b00. For LPDDRn,this register should keep the same RL setting with the SDRAM. For DDRn, this register should keep the same CL setting with the SDRAM. |
| [21:16] | CL_FRE_OP1 | R/W | 0x6 |  |
| [13:8] | CL_FRE_OP2 | R/W | 0x6 |  |
| [5:0] | CL_FRE_OP3 | R/W | 0x6 |  |

Offset: 0x10

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [28:24] | CWL_FRE_OP0 | R/W | 0x0 | Setting the CWL/WL vlaue of the PHY for frequency point 0. When enable the Fast Frequency Change Mode, the PHY will choose CWL_FEE_OP0 as the CWL/WL setting of the PHY when the reg_freq_choose_t == 2'b00. For LPDDRn,this register should keep the same WL setting with the SDRAM. For DDRn, this register should keep the same CWL setting with the SDRAM. |
| [20:16] | CWL_FRE_OP1 | R/W | 0x0 |  |
| [12:8] | CWL_FRE_OP2 | R/W | 0x0 |  |
| [4:0] | CWL_FRE_OP3 | R/W | 0x0 |  |

Offset: 0x14

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [28:24] | reg_fb1xclk_invdelaysel_dqcmd | R/W | 0x0 | Used to adjust the hold timing of the digital to analog interface. When increase this register, it will increase the hold time of the digital to analog interface. |
| [20:16] | reg_h4xclk_invdelaysel_dqcmd | R/W | 0x0 | Reserved. |
| [12:8] | reg_h4xclkdqs_invdelaysel_dqcmd | R/W | 0x0 | Reserved. |
| [4:0] | reg_h1xclk_invdelaysel_dqcmd | R/W | 0x0 | Reserved. |

Offset: 0x18

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [31:0] | reg_cmd_ph90en_bp | R/W | 0xffffffff | The command 90 degree control signal. High means eanble the command 90 degree conrol and low means the disalbe the command 90 degree control. [0]:A0 [1]:A1 [2]:A2 [3]:A3 [4]:A4 [5]:A5 [6]:A6 [7]:A7 [8]:A8 [9]:A9 [10]:A10 [11]:A11 [12]:A12 [13]:A13 [14]:A14 [15]:A15 [16]:A16 [17]:A17 [18]:ACTN [19]:BA0 [20]:BA1 [21]:BG0 [22]:BG1 [23]:CK [24]:CKB [25]:CKE0 [26]:CSB0 [27]:CSB1 [28]:ODT0 [29]:ODT1 [30]:CKE1 [31]:RESETN |

Offset: 0x1C

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [25] | reg_oscen_t | R/W | 0x0 | Reserved. |
| [24] | reg_x1clk_div_sel | R/W | 0x0 | Reserved. |
| [23] | reg_vt_comp_bp | R/W | 0x1 | The rx vt compensation disable signal. Active High. 1: Disable the rx vt compensation function. 0: Enable the rx vt compensation funciton. |
| [22] | reg_cmdout_mux | R/W | 0x1 | The timing control of the command path between the digital part and analog part. 1: The command signal from the digital part to the analog part is align to the posedge of the dfi_clk1x. 0: The command signal from the ditial part to the analog part is align to the negedge of the dfi_clk1x. |
| [21] | ph90en_bp | R/W | 0x0 | The 90 degree control bypass enable signal of the command path. In the normal mode, the 90 degree control of the command path is setting automatically based on the sdram type. The user also can use this register to choose the bypass mode, then use the register 0x58~0x5b to control the 90 degree of each command pad. 1: Choose the bypass mode. 0: Choose the auto mode. |
| [20] | ph90en_bp_dq | R/W | 0x0 | The 90 degree control bypass enable signal of the data path. In the normal mode, the 90 degree control of the data path is setting automatically based on the sdram type. The user also can use this register to choose the bypass mode, then use the register 0xa8/0xb8/c8/d8 to control the 90 degree of each dq pad. 1: Choose the bypass mode. 0: Choose the auto mode. |
| [19] | reg_rden_bypass | R/W | 0x0 | Then enable signal of the rx fifo read control bypass mode. High is valid. 0: Disable the bypass mode. 1: Enable the bypass mode and use the reg_rden_delay to control the rx fifo read timing. |
| [18:16] | reg_rden_delay | R/W | 0x2 | This register is used to control read timing of the rx fifo when enable the rx fifo read control bypass mode. When the dfi_rddata_en change to high, the phy will read out the rx data from the rx fifo afther the following delay. 0: 9+max(reg_ar_cycsel,reg_al_cycsel,reg_br_cycsel,reg_bl_cycsel) 1: 10+max(reg_ar_cycsel,reg_al_cycsel,reg_br_cycsel,reg_bl_cycsel) 2: 11+max(reg_ar_cycsel,reg_al_cycsel,reg_br_cycsel,reg_bl_cycsel) 3: 12+max(reg_ar_cycsel,reg_al_cycsel,reg_br_cycsel,reg_bl_cycsel) 4: 13+max(reg_ar_cycsel,reg_al_cycsel,reg_br_cycsel,reg_bl_cycsel) 5: 14+max(reg_ar_cycsel,reg_al_cycsel,reg_br_cycsel,reg_bl_cycsel) 6: 15+max(reg_ar_cycsel,reg_al_cycsel,reg_br_cycsel,reg_bl_cycsel) 7: 16+max(reg_ar_cycsel,reg_al_cycsel,reg_br_cycsel,reg_bl_cycsel) For auto mode, the delay = 11+max(reg_ar_cycsel,reg_al_cycsel,reg_br_cycsel,reg_bl_cycsel). |
| [14] | reg_rdodt_bypass | R/W | 0x0 | The bypass enable signal of the rx odt bypass function. When enable this function, the user can use the register 0xa9~0xac and register 0xd9~0xdc to adjust the timing of the rx odt. |
| [13] | reg_rxodt_st_bypass | R/W | 0x0 | The enable signal of the rx odt start point bypass mode, using to control the start point of the rx odt. High is valid. |
| [11:8] | reg_rxodt_stdelay | R/W | 0x0 | Using to control the rx odt start point when enable the rx odt start point bypass mode. Decrease this value will left shift the rx odt window, increase this value will right shift the rx odt window. Unit:dfi_clk1x. For auto mode, the start_point = (RL-1)>>1. |
| [7:4] | reg_rxodt_length | R/W | 0x2 | Used to control the length of the RX ODT. The step is 1x clock cycle. When the value +1, the length of RX ODT will increase 1x clock cycle. Related register 0x00f. It combines with the register 0x00f to decide the range of the RX ODT. The default RX ODT length is 4*dfi_clk1x cycles. |
| [3:0] | reg_rxodt_start_point | R/W | 0x3 | Used to control the start point of the RX ODT. When the value +1, the start point of RX ODT will increase 1x clock cycle. If the user wants to keep the length of the RX ODT unchanged, the users need to increase 1x clock cycle of the length of the RX ODT using the register 0x0e[7:4]. The default RX ODT start point is RL-2 in regard to the read command. |

Offset: 0x20

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [15:14] | reg_cat_rank_num | R/W | 0x3 | Define the rank number of the command bus train should support. 2’b11: Two ranks. 2’b01: Only one rank. Others: Not support. |
| [13:12] | reg_cat_channel_num | R/W | 0x3 | The command channel enable signal of command bus train. High is valid. 2’b11: Enable Channel A and Channel B. 2’b01: Only enable Channel A. 2’b10: Only enable Channel B. |
| [11:8] | reg_clk_div_cnt | R/W | 0x4 | This register is used to control the divider of the dfi_clk1x which used for the initialization of the SDRAM and command bus training. The dfi_clk1x will be clock divided to the low speed based on this register. Tclkinit = Tdfi_clk1x/(2*reg_clk_div_cnt). |
| [7:6] | reg_cat_bp_rank_sel | R/W | 0x0 | Used to choose the rank when enable the bypass command bus train or only enable one rank of auto command bus train. 2’b10: Choose the Rank0 to do the command bus train. 2’b01: Choose the Rank1 to do the command bus train. |
| [5] | reg_cat_bp_cmd_send | R/W | 0x0 | Used to send the command bus train command when enable the command bus training bpass mode. Posedge is valid. When this signal change from low to high, it will send the command bus train command to the SDRAM. The command bus value depends on the reg_cat_bp_mode(reg0x10[3], reg_cat_cs_train_value(reg0x21[5:0]) and reg_ca_train_value(reg0x22[5:0]). |
| [4] | reg_cat_bp_mode | R/W | 0x0 | Choose the command bus train stage for the byass mode. 0: Current is CS bypass command bus train. 1: Current is CA bypass command bus train. |
| [3] | reg_cat_bp_en | R/W | 0x0 | The Bypass command bus train enable signal. High is valid. When this bit set to high, the command path will choose the command bus train module and the tx delay line of command will be controlled by the register directly through the logic in the command bus training logic. |
| [2] | reg_cat_bp_start | R/W | 0x0 | The Bypass command bus train start signal. High is valid. When this bit set to high, the command bus module will go into the bypass command bus train flow. |
| [1] | reg_cat_start | R/W | 0x0 | The Auto command bus train start signal. High is valid. When this signal enable, it will begin the command bus train. |
| [0] | reg_cat_enable | R/W | 0x0 | The Auto command bus train enable signal. High is valid. When this signal enable, the command path will switch to the command bus train module. |

Offset: 0x24

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [29:25] | reg_txcbt | R/W | 0x4 | Used to control the timing parameter tcs_VREF/tCKCKEH/tMRZ. The delay should >= MAX(1.5ns,2nCK). |
| [24:20] | reg_tadr | R/W | 0xf | Used to control the check time of the read back data from the DQ. The delay should >= 20ns. |
| [19:15] | reg_tckelck | R/W | 0x4 | Used to control the clock and command valid after CKE low(tCKELCK). The delay should >= max(7.5ns,3nCK) |
| [14:10] | reg_tdstrain | R/W | 0x1 | Data Setup/Hold for Vref(CA) Training Mode. |
| [9:5] | reg_tmrw | R/W | 0xf | Used to control the delay between the MRW command and valid clock/CS requirement after CKE input LOW after MRW command. The delay >= MAX( 14ns, 10nCK). |
| [4:0] | reg_tcacd | R/W | 0x15 | Used to control the CA Bus Training Command to CA Bus Training Command Delay( tCACD). The delay should >= RU(20ns/tCK). |

Offset: 0x28

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [23:16] | reg_tvrefca_long | R/W | 0x28 | Used to control the Vref(CA) step time. |
| [15:8] | reg_tcaent | R/W | 0xf | Used to control the first CA Bus Training Command following CKE Low(tCAENT). The delay should >= 250ns. |
| [7:0] | reg_tfc | R/W | 0x1f | Used to control the frequency set point switching time(tFC). The delay should >= 250ns. |

Offset: 0x2C

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [31:24] | reg_mr1 | R/W | 0x0 | Used to control the LPDDR4 load mode value of MR1 when command bus train. Used to control the DDR4 load mode value of MR3 when read training(MPR), when ddr4 read training, set this register as current SDRAM MR3(A10~A3) value. The read training operation will write new data to the MR3, so we should keep the other bits the same with the current MR3. |
| [23:16] | reg_mr2 | R/W | 0x0 | Reserved for LPDDR4 load mode value setting. |
| [15:8] | reg_mr3 | R/W | 0x0 | Reserved for LPDDR4 load mode value setting. |
| [7:0] | reg_mr11 | R/W | 0x0 | Reserved for LPDDR4 load mode value setting. |

Offset: 0x30

Reserved for LPDDR4 load mode value setting.

Offset: 0x34

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [31] | reg_ca_vref_update | R/W | 0x0 | Reserved. |
| [30] | reg_cat_vref_scan_disable | R/W | 0x0 | Disable the CA_VREF train when do the Auto command but train. High is valid . 1: Only scan the timing window of the CA and CS based on the current CA_VREF setting. 0: Will scan the CA_VREF to find the best point. |
| [29:24] | reg_cat_vref_scan_max | R/W | 0x3f | Set the max value that the Vref train. When the Vref scan arrive to the reg_cat_vref_scan_max, it will stop the Vref scan. |
| [21:16] | reg_cat_vref_scan_min | R/W | 0x0 | Set the min value that the perbit skew train. The tx delay line scan will start from this delay point. |
| [14:8] | reg_cha_cat_vref_bp_value | R/W | 0x1f | Reserved. |
| [6:0] | reg_chb_cat_vref_bp_value | R/W | 0x1f | Reserved. |

Offset: 0x38

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [31:28] | reg_cs_perbit_skew_offest_fsp0 | R/W | 0x0 | The FSP[0] compensation value of CSB tx delay after the command bus train. When the PHY do the command bus train to scan the CSB delay window, the CSB has 75%*2UI. So after the command bus train, we need to make the compensation for the 25%*2UI. The compensation value is the half of the lost pulse width = 1/4UI. |
| [27:24] | reg_cs_perbit_skew_offest_fsp1 | R/W | 0x0 | The FSP[1] compensation value of CSB tx delay after the command bus train. When the PHY do the command bus train to scan the CSB delay window, the CSB has 75%*2UI. So after the command bus train, we need to make the compensation for the 25%*2UI. The compensation value is the half of the lost pulse width = 1/4UI. |
| [23:20] | reg_cs_perbit_skew_offest_fsp2 | R/W | 0x0 | The FSP[2] compensation value of CSB tx delay after the command bus train. When the PHY do the command bus train to scan the CSB delay window, the CSB has 75%*2UI. So after the command bus train, we need to make the compensation for the 25%*2UI. The compensation value is the half of the lost pulse width = 1/4UI. |
| [19:16] | reg_cs_perbit_skew_offest_fsp3 | R/W | 0x0 | The FSP[3] compensation value of CSB tx delay after the command bus train. When the PHY do the command bus train to scan the CSB delay window, the CSB has 75%*2UI. So after the command bus train, we need to make the compensation for the 25%*2UI. The compensation value is the half of the lost pulse width = 1/4UI. |
| [13:12] | reg_lpddr4_ca_odt | R/W | 0x1 | When enable the register control mode of CA ODT, the PHY will use this register to control the CA_ODT. |
| [11] | reg_lpddr4_ca_odt_sel | R/W | 0x0 | Choose the CA ODT control mode in LPDDR4 mode. 1: Use the register reg_lpddr4_ca_odt[1:0] to control. 0: Use the dfi_odt to control. |
| [10:8] | reg_cat_vref_scan_steps | R/W | 0x1 | Used to control the CA_VREF scan steps. When the reg_cat_vref_scan_disable is set to 1'b0 to open the CA_VREF scan mode, the use can use this register to control the scan steps to decrease the scan time. |
| [7] | reg_cs_pwc_disable | R/W | 0x0 | The disable signal of pulse width control of CS. High is valid. For command bus train, when the PHY do the command bus train of CS, if enable the pulse width control function, the pulse width will decrease 25% for command bus train. Before enter command bus train and after exit command bus train, the pulse width of CS will keep the normal pulse width. |
| [6] | reg_cat_skip_fspy | R/W | 0x0 | For the command bus train, the user can choose to set the FSP[Y]( assume that the current is FSP[X]) at the low speed based on the setting in the register ( reg_mr1~regmr22). This register is used to control to enable this function or not. 1: Enable the function to set the FSP[Y] before the command bus train. 0: Disable the function to set the FSP[Y]] before the command bus train. When the user chooses this mode, the user need to set the FSP[Y] before to enable the command bus trian. Note: For the first time of high speed command bus training, the phy will complete the sdram initialization and skip this operation automatically. |
| [5:4] | reg_cat_fspy_rank | R/W | 0x0 | If the user chooses to set the FSP[Y] in the command bus train flow by set tingthe reg_cat_ske_fspy = 1'b0, the use can use this register to choose the RANK which need to be set. 2'b00: Choose RANK0 and RANK1. 2'b01: Choose Rank1. 2'b10: Choose Rank0. 2'b11: Disable the setting. |
| [3] | reg_cat_fc_pd_en | R/W | 0x1 | Reserved. |
| [2] | reg_cat_skip_cs_train | R/W | 0x0 | The disable signal to skip the cs train flow of the auto command bus train. 1: Skip the CS train and go to the ca training directly. 0: Begin the CS train firstly and go to the ca training. |
| [1] | reg_cmd_perbit_skew_bp | R/W | 0x0 | The tx delay line of the command pad bypass enable signal. 1: For LPDDR4 mode, it can use the register to control the tx delay line of the command pad. 0: For LPDDR4 mode, it will use the command bus training module to control the tx delay line of the command pad. Note: For other sdram types, the tx delay line is always control by the register directly. This register is unsed. |
| [0] | reg_ca_perbit_skew_update | R/W | 0x0 | Used to update the perbit skew of the command in the command bus train module through the registers. Posedge is valid. When this signal change from low to high, it will update the perbit skew value setting by the register to the command bus train module according to the current reg_freq_choose(reg0x0c[3:2]). |

Offset: 0x3C

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [25] | reg_cat_cke_mode | R/W | 0x0 | Reserved. |
| [24] | reg_cat_ck_cke_odt_fix_perbit_skew | R/W | 0x1 | Reserved. |
| [23:16] | reg_cat_ca_scan_max | R/W | 0x3f | Set the max value that the perbit skew train. When the perbit skew scan arrive to the reg_cat_ca_scan_max, it will stop the tx delay line scan of the command. |
| [13:8] | reg_cat_ca_train_value | R/W | 0x15 | The signal is used to control the CA command bus train pattern. The user can use this register to control the training patter for CA command bus train. |
| [5:0] | reg_cat_cs_train_value | R/W | 0x6 | The signal is used to control the CS command bus train pattern. The user can use this register to control the command bus train patter for cs train stage. |

Offset: 0x40

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [29:24] | reg_cha_cat_cs_check_value | R/W | 0x6 | The check pattern of cs train mode for channel A. |
| [21:16] | reg_chb_cat_cs_check_value | R/W | 0x6 | The check pattern of cs train mode for channel B. |
| [13:8] | reg_cha_cat_ca_check_value | R/W | 0x15 | The check pattern of ca train mode for channel A. |
| [5:0] | reg_chb_cat_ca_check_value | R/W | 0x15 | The check pattern of ca train mode for channel B. |

Offset: 0x44

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [31:30] | reg_freq_choose_t | R/W | 0x0 | Used for Fast Frequency Changing. Valid only when reg_freq_choose_bypass(reg0x6a[7]) is high. 00: Freq Point 0. 01: Freq Point 1. 10: Freq Point 2. 11: Freq Point 3. In the normal mode, the current frequency point is set by the dfi_frequency[4:0] at the initialization and frequency change. If the user want to change the current working frequency or update the registers which used to control other frequency points. The user can use this register to set the frequency point firstly. After choose the freq point using this register, the user can use the update register to switch the training result belong to this freq point fastly. |
| [29] | reg_lpddr4_rd_preamble | R/W | 0x0 | Choose the read preamble mode when PHY is in LPDDR4 mode. 0: Static. 1: Toggle. Note: This setting should keep the same setting with the MR#1[3] of the LPDDR4. |
| [28] | reg_wl_freq_update | R/W | 0x0 | The data tx delay line control value update signal. Active high. In the default mode, the tx delay line is controlled by the write-levevling/write training result according to the frequency point. If the user wants to change the value to adjust the delay line, the user can use the bypass registers that show in the Table 4 12 to set the value and set this signal “1”. Then the bypass register value will be updated to the control register of the tx perbit skew base on the current frequency point. Note: Please recover to “0” after the setting. |
| [27] | reg_calib_freq_update | R/W | 0x0 | The RX DQS Gating control value update signal. Active high. In the default mode, after the RX DQS Gating, the training value will control the RX DQS Gating delay. If the user wants to change the value to adjust the RX DQS Gating window, the user can use the bypass registers that show in the Table 4 24 to set the control value. If this bit set to “1”, then the setting value will be update to the control register of the RX DQS Gating base on the current frequency point. Note: Please recover to “0” after the setting. |
| [26] | reg_cmd_abutobsmodeen | R/W | 0x0 | Reserved. |
| [25] | reg_sdram_vref_update | R/W | 0x0 | Reserved. |
| [24] | reg_calib_mode_sel | R/W | 0x0 | The DQS-Gating model sel. 1: Use the Read Preamble Training mode (Only used for DDR4). 0: Use the Normal Read mode (Can used for DDR2/3/4 and LPDDR2/3). When the DQS Gating mode chooses the Normal Read mode for DDR4, the Read DQS will be pulling down to 1’b0. So the register reg_a_l_weakpub_reg/reg_a_h_weakpub_reg/reg_b_l_weakpub_reg/reg_b_h_weakpub_reg, should set to 2’b00 to pull down the Read DQS when it is in “high z” state. |
| [23] | reg_pwrok_pi | R/W | 0x0 | Reserved. |
| [22:20] | reg_lpddr4_cmd_gap_for_diff_rank | R/W | 0x3 | Reserved. |
| [19:16] | reg_b0_cmdobsmuxsel | R/W | 0x0 | Reserved. |
| [15] | reg_b0_obsdataen | R/W | 0x0 | Reserved. |
| [14] | bist_calibst | R/W | 0x0 | Reserved. |
| [13] | reg_fiford_delay | R/W | 0x0 | Reserved. |
| [12] | bist_ck_select | R/W | 0x0 | The register used to choose the clk which used to sample the bist command when enable the phy noraml bist. 1: Choose the feedback signal from the CK pad as the clock. 0: Choose the feedback signal from the CKB pad as the clock. |
| [11] | reg_mch_odt | R/W | 0x0 | Reserved. |
| [10] | mux_sync_sel | R/W | 0x0 | Reserved. |
| [9] | reg_sync_en | R/W | 0x0 | Reserved. |
| [8] | reg_rdptr_delay | R/W | 0x1 | Reserved. |
| [7] | reg_scr_rcvmodsel | R/W | 0x0 | Reserved. |
| [6] | reg_scr_sckdimm0dis | R/W | 0x0 | Reserved. |
| [5] | reg_scr_sdhsclkb_pos1neg0sel | R/W | 0x1 | Reserved. |
| [4] | reg_scr_sdhsclk_pos1neg0sel | R/W | 0x0 | Reserved. |
| [3] | reg_scr_odttrien | R/W | 0x0 | Reserved. |
| [2] | reg_scr_csbtrien | R/W | 0x0 | Reserved. |
| [1] | reg_scr_cmdtrien | R/W | 0x0 | Reserved. |
| [0] | reg_scr_cketrirnk0 | R/W | 0x0 | Reserved. |

Offset: 0x48

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [12] | reg_ram_vref1_pd | R/W | 0x0 | The power down signal of RAM VREF. 1: Power Down. 0: Open. |
| [11:8] | bypassen | R/W | 0x0 | Reserved. |
| [7:0] | reg_mpr_cnt | R/W | 0x3f | Used to control the timing of the MPR command for DDR4 rx dqs calibration. Timing interval between MPR = reg_mpr_cnt * dficlk1x. |

Offset: 0x4C

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [23:16] | reg_max_rdvalue | R/W | 0x40 | Use to set the max number of the read command when do the DQS Gating. When the read commands are more than the max number, it is the DQS Gating Error and this state can be read from the register 0x91[5]. |
| [15:0] | reg_calib_timeout | R/W | 0x1000 | The bit[7:0] of the timer used for DQS Gating. When the clock cycle exceed the bits[15:0] of the timer, it is the DQS Gating Error and this state can be read from the register 0x91[5]. The bit[15:8] of the timer used for DQS Gating. When the clock cycle exceed the bits[15:0] of the timer, it is the DQS Gating Error and this state can be read from the register 0x91[5]. |

Offset: 0x50

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [10] | reg_rrankdly_4x_dec | R/W | 0x1 | The delay control signal of the read rank switch. Unit: dfi_clk4x. 1: Decrease 1 dfi_clk4x delay for the read rank switch. 0: Keep current state. |
| [9:8] | reg_rdrank_4xdly | R/W | 0x0 | Reserved. |
| [7] | reg_rdrank_delay_bp | R/W | 0x0 | The bypass mode of read rank switch point at 4x clock cycle precision. The user can use the register 0xa6 and 0xd6 to adjust the read rank switch point. |
| [6:4] | reg_rdrank_1xdly | R/W | 0x1 | Reserved. |
| [2:0] | reg_wrrank_1xdly | R/W | 0x1 | The delay control signal of the write rank witch. Unit:dfi_clk1x. |

Offset: 0x54~0x78

These registers are used to configure the pin remap of DDR PHY – DDR control connections. Reserved for internal use.

Offset: 0x7C

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [24:16] | reg_pllfbdiv_dqcmd | R/W | 0x0 | The control signal of feedback div signal. |
| [8:4] | reg_pllprediv_dqcmd | R/W | 0x1 | Internal PLL charge bump work frequency adjusts, it can be used to adjust PLL band width, 5’h1 has maximum band width. |
| [3] | reg_pllpostdiven_dqcmd | R/W | 0x0 | The reigister used to control the post div enable for low speed( which will be used for CA Training and PHY initialization). For different freuency range, the state of post div enable state is different. The user needs to based on the frequency of the low speed to set this register. |
| [2:0] | reg_pllpostdiv_dqcmd | R/W | 0x0 | The reigister used to control the post div for low speed (which will be used for CA Training and PHY initializaiton). For different freuency range, the value of post div enable state is different. The user needs to based on the frequency of the low speed to set this register. |

Offset: 0x80

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [19:18] | reg_plltestsel_dqcmd | R/W | 0x0 | Reserved. |
| [17:16] | reg_pllgvco_bias_dqcmd | R/W | 0x1 | Reserved. |
| [14:12] | reg_pllcpi_bias_dqcmd | R/W | 0x3 | Reserved. |
| [10:8] | reg_pllcpp_bias_dqcmd | R/W | 0x2 | Reserved. |
| [7] | reg_plltestouten_dqcmd | R/W | 0x0 | Reserved. |
| [6] | reg_lockenb_dqcmd | R/W | 0x0 | The power down enable signal of the pll. Active low. 0: Enable the pll. 1: Disalbe the pll. |
| [5] | reg_pllrstbsel_dqcmd | R/W | 0x0 | Resrved. |
| [4] | reg_pllref_clk_byp_dqcmd | R/W | 0x0 | Reserved. |
| [3] | reg_ssc_rstn | R/W | 0x1 | Reserved. |
| [2] | reg_pllincz_dqcmd | R/W | 0x0 | Reserved. |
| [1] | reg_pllclkouten_dqcmd_t | R/W | 0x1 | The clock output enable signal of pll. Active high. |
| [0] | reg_pllpd_dqcmd_t | R/W | 0x1 | Internal PLL power down. Set to “0” to enable the PLL. |

Offset: 0x84

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [28] | reg_dfi_clk_gate_bp | R/W | 0x0 | dfi clk1x in gate , bypass |
| [27] | reg_hclk_train_sel | R/W | 0x0 | Used to choose the clock state of train logic. 1: Keep clock for train logic when it is in no-train state. 0: Gate clock for train logic when it is in no-train state. |
| [26] | reg_hclk_bist_sel | R/W | 0x0 | The clock gate control signal of the bist module. 1: Keep the clock of the bist module even the module is not in working state. 0: Auto disable the clock of the bist module when the module is not in working state. |
| [25] | reg_hclk_zqcalib_sel | R/W | 0x0 | The clock gate control signal of the zqcalib module. 1: Keep the clock of the zqcalib module even the module is not in the training mode. 0: Auto disable the clock of the zqcalib module when the module is not in the training model. |
| [24] | reg_hclk_byte0_sel | R/W | 0x0 | The enable signal of the clock gate for the digital part of byte0. 1: Keep the clock when the byte0 is in idle state. 0: Automatically gate the clock when the byte0 is in idle state. |
| [23] | reg_hclk_byte1_sel | R/W | 0x0 | The enable signal of the clock gate for the digital part of byte1. 1: Keep the clock when the byte1 is in idle state. 0: Automatically gate the clock when the byte1 is in idle state. |
| [22] | reg_hclk_byte2_sel | R/W | 0x0 | The enable signal of the clock gate for the ditial part of byte2. 1: Keep the clock when the byte2 is in idle state. 0: Automatically gate the clock when the byte2 is in idle state. |
| [21] | reg_hclk_byte3_sel | R/W | 0x0 | The enable signal of the clock gate for the digital part of byte3. 1: Keep the clock when the byte3 is in idle state. 0: Automatically gate the clock when the byte3 is in idle state. |
| [20] | reg_hclk_byte_sel | R/W | 0x0 | The clock gate control signal of the DQ path common logic. 1: Keep the clock of the common logic at idle state. 0: Auto disable the common logic at idle state. |
| [19] | reg_hclk_ca_sel | R/W | 0x0 | Used to choose the clock state of command logic. 1: Keep clock of command logic when it is in idle state. 0: Gate clock of command logic when it is in idle state. |
| [18] | reg_train_reg_update_en | R/W | 0x1 | Disable the clock gate of the train logic. Active high. 1: It will enable the clock of the train logic. 0: Automatically gate the clock when the train logic is in idle state. If the users wants to use the register to update the training result through the bypass mode. This register should set to 1’b1 to enable the clock of the training control logic, then the bypass value can be updated to the lock register. |
| [17] | reg_dqclken_t | R/W | 0x1 | The enable signal of clock for DQ path in the analog circuit. This signal will be used to disable the clk of the DQ module to save the power when the DDR2/3/4 SDRAM goes to power down mode. 1: Keep the clock of the DQ path in the analog circuit. 0: Gate the clock of the DQ path in the analog circuit. |
| [16] | reg_outclken | R/W | 0x1 | The gating control signal of 1x clock of PHY. Active Low. 0: It will gate the 1x clock of the PHY. 1: The 1x clock of the PHY will be gate automatically in dfi low power mode. |
| [15] | reg_lp_bypass | R/W | 0x0 | The dfi low power disable signal. When set to high, it will ignore the signals of dfi low power interface. |
| [14] | reg_deep_lp_en | R/W | 0x1 | The deep low power mode enable signal. 1: Enable the deep low power. 0: Disable the deep low power. |
| [13] | reg_lp_wakeup_sel | R/W | 0x0 | Reserved. |
| [12] | reg_lp_vref_ctrl_en | R/W | 0x1 | Disable the bypass mode of the VREF control. 1: Using auto mode the disable VREF at dfi low power mode. 0: Using the register 0x114[4]/0x124[4]/0x134[4]/0x144[4] to disable the VREF. |
| [11:8] | reg_lp_wakeup_threhold | R/W | 0x9 | Configure the low power mode threshold. When the dfi_lp_wakeup<= threshold, it will go into the normal low power mode; when dfi_lp_wakeup> threshold, it will go into the deep low power mode. |
| [7:6] | reg_lp_io_dis_ctrl | R/W | 0x1 | Adjust the timing from disable the IO to disable the clock when begin the low power flow. |
| [5] | reg_lp_dq_clk_ctrl_en | R/W | 0x1 | Choose the control mode of the DQ clock path when in low power mode. 1: Auto mode. 0: Bypass mode. Use the register 0x00[1] to control. |
| [4] | reg_lp_dig_rst_ctrl_en | R/W | 0x0 | Choose the control mode of the reset of the digital core. 1: Auto mode. 0: Not reset. |
| [3] | reg_lp_spll_clktree_ctrl_en | R/W | 0x1 | Choose the control mode of the clock tree of the pll. 1: Auto mode. 0: Not close. |
| [2] | reg_lp_dig_clk_ctrl_en | R/W | 0x0 | Choose the control mode of the clock of the digital core. 1: Auto mode. 0: Not close. |
| [1] | reg_lp_io_ctrl_en | R/W | 0x1 | Choose the control mode of the IO. 1: Auto mode. 0: Not close. |
| [0] | reg_lp_pllpd_ctrl_en | R/W | 0x1 | Choose the power down mode of the PLL. 1: Auto mode. 0: Not power down. |

Offset: 0x88

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [23:20] | reg_lp_stvalue | R/W | 0x1 | The response timing when high the dfi_lp_ack. Unit: dfi_clk1x. |
| [19:16] | reg_lp_ackvalue | R/W | 0x6 | The response timing when detect the dfi_lp_req is high. Unit: dfi_clk1x. |
| [15:0] | reg_wait_cnt | R/W | 0x1388 | The bit[7:0] of pll lock wait signal. There are two ways to generate the pll lock. The first way is using the pll_lock from the pll module directly, the second way is using the counter to calcuate the cycle after enable the pll. For the second way, after enable the pll, the phy will start to count until the counter equal the reg_wait_cnt[15:0], the high the pll lock. The bit[15:8] of pll lock wait signal. There are two ways to generate the pll lock. The first way is using the pll_lock from the pll module directly, the second way is using the counter to calcuate the cycle after enable the pll. For the second way, after enable the pll, the phy will start to count until the counter equal the reg_wait_cnt[15:0], the high the pll lock. |

Offset: 0x8C

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [28:20] | reg_lp_stvalue | R/W | 0xff | The timing interval of the zqcalib training value changing when we do the zq calibration. Unit: dfi_clk1x |
| [18:10] | reg_lp_ackvalue | R/W | 0xff | The timing interaval of the zqcalib between the pull down training state to pull up training state. Unit:dfi_clk1x. |
| [9:8] | reg_wait_cnt | R/W | 0x0 | Reserved. |
| [7:6] | reg_lp_stvalue | R/W | 0x0 | Reserved. |
| [5:4] | reg_lp_ackvalue | R/W | 0x0 | Choose the current zqcalib mode when enable the zqcalib function. 00: Driver Pull-Down zqcalib mode. 01: Drive Pull-Up zqcalib mode. 10: ODT Pull-Down zqcalib mode. 11: ODT Pull-Up zqcalib mode. |
| [3] | reg_wait_cnt | R/W | 0x1 | The power down signal of zqcalib module when enable the zqcalib function. 1: Power down mode. 0: Normal work mode. |
| [2] | reg_lp_stvalue | R/W | 0x0 | The clean signal of zqcalib module. High is valid. |
| [1] | reg_lp_ackvalue | R/W | 0x0 | The bypass enable of zqcalib function. Active high. 1: The phy will use the register reg_drvlegn_zqcali/ reg_drvlegpb_zqcali/ reg_odtlegn_zqcali/ reg_odtlegpb_zqcali to control zqcalib. 0: It will use the zqcali module to control. |
| [0] | reg_wait_cnt | R/W | 0x0 | The enable signal of zqcalib function. |

Offset: 0x90

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [28:24] | reg_drvlegn_zqcali | R/W | 0xe | The drivier pull-down value when reg_zqcali_bypass(reg0x66[4]) set to 1'b1 to enable the zqcalib bypass function. |
| [20:16] | reg_drvlegpb_zqcali | R/W | 0xe | The drivier pull-up value when reg_zqcali_bypass(reg0x66[4]) set to 1'b1 to enable the zqcalib bypass function. |
| [12:8] | reg_odtlegn_zqcali | R/W | 0x7 | The odt pull-up value when reg_zqcali_bypass(reg0x66[4]) set to 1'b1 to enable the zqcalib bypass function. |
| [4:0] | reg_odtlegpb_zqcali | R/W | 0x7 | The odt pull up value when reg_zqcali_bypass(reg0x66[4]) set to 1'b1 to enable the zqcalib bypass function. |

Offset: 0x94

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [22:14] | reg_pd_interval | R/W | 0xff | The timing interaval of the zqcalib between the pull down training state to pull down training state. Unit:dfi_clk1x. |
| [12] | reg_rd_train_perdef_en | R/W | 0x0 | The enable signal of the read predef training, predefine Mode. 1: Enable the predef train of the read train. 0: Exit the predef train of the read train |
| [11] | reg_train_vref_en | R/W | 0x0 | The enable signal of the vref training. 1: Enable the vref train of the read train(predefine mode)/write train. 0: Exit the vref train of the read train(predefine mode)/write train. reuse for rd perdef training and wr training |
| [10] | reg_ddr4_dbi | R/W | 0x0 | When you enable rd DBI function of DDR4, and you want to do wr training with DBI open, you should set this register to 1’b1 at first. 1: support the DBI function during wr training. 0: Nonsupports the DBI function during wr training. |
| [9:8] | reg_rdtrain_cs_sel | R/W | 0x0 | Choose the rank of the read train. 2’b10: Choose Rank0. 2’b01: Choose Rank1. |
| [7] | reg_rx_vref_value_update | R/W | 0x0 | When You want to change the PHY’s vref value, you should config this register , at posedge of this register, the value of reg_a/b_l/r_vref1_margsel will be latched to PHY . |
| [6] | reg_rd_train_dqs_range_bypass | R/W | 0x0 | The enable signal to setting the DQS scan range using the register.(rd training) 1: Enable to use the register to set the DQS scan range. 0: Use the default DQS scan range[0~6’h3f]. |
| [5] | reg_bypass_rd_train_cmd_start_en | R/W | 0x0 | Generate the read train command when enable bypass read training mode. High pulse valid. |
| [4] | reg_bypass_rd_train_en | R/W | 0x0 | Enable the bypass read train function. 1: Enable the read train bypass mode. 0: Disable the read train bypass mode. |
| [3] | reg_rd_train_check_value_en | R/W | 0x0 | Choose the check value of the read train. 0: Use the fix mode. DDR3 = 8’b10101010 LPDDR3 = 16’hcc55 LPDDR4 = {MR#40,MR#32} DDR4 = 8’haa/8’hcc/8’hf0/8’h00 1: Use the register to configure. |
| [2] | reg_rd_train_freq_update | R/W | 0x0 | When You want to change the PHY’s rx per-bit skew value, you should config this register , at posedge of this signal, the value of reg_dqxx_invdelayselrx will be latched to PHY. |
| [1] | reg_dqs_rd_train_en | R/W | 0x0 | The enable signal of the DQS Scan.(rd training) 1: The auto read train will complete the DQ perbit skew train and DQS-DQ Eye train.(not suggest) 0: The auto read train will only complete the DQ perbit skew train. |
| [0] | reg_dq_rd_train_en | R/W | 0x0 | The enable signal of the read training Auto Mode. 1: Enable the auto train of the read train. 0: Exit the auto train of the read train. |

Offset: 0x98

Reserved for LPDDR4 parameter settting.

Offset: 0x9C

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [31:16] | reg_ddr4_mr4_value | R/W | 0x0 | The load mode value of MR4 when do the read train of DDR4. |
| [7:0] | reg_ddr4_mr3 | R/W | 0x0 | The load mode value of MR3 when do the read train of DDR4 |

Offset: 0xA0

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [22] | reg_wrtrain_lpddr4_vref_range | R/W | 0x1 | Choose the adjust range of SDRAM's vref , reference to JESD-LPDDR4 |
| [21:16] | reg_pbit_deskew_offset_for_lpddr4 | R/W | 0x0 | The tx delay line adjust signal of the DQS/DQSB after the write-leveling. The tx delay line value will equal the write-leveing result + reg_pbit_deskew_offset_for_lpddr4 and apply to the DQ/DM. |
| [9] | reg_dqs_wr_train_en | R/W | 0x0 | The enable signal of the DQS Scan. When this bit set to “1”, it will enable adjust the DQS to find the best window of the DQ. Only used for debug, because it will change the write-leveling result if the user enable the write leveling function. |
| [8] | reg_wrtrain_check_data_value_random_gen | R/W | 0x1 | 1’b1: PHY generate random data and write to SDRAM during write training. 1’b0: the user config dq0-7_train_check_data_value0-9 and PHY write it (10*BL8) to SDRAM during write training. |
| [7:6] | reg_wrtrain_cs_sel | R/W | 0x0 | Choose the rank of the write train. 2’b10: Choose Rank0. 2’b01: Choose Rank1. |
| [5] | reg_wr_train_rst | R/W | 0x0 | The wr train fsm clear signal. 1: Reset the write train fsm. 0: Keep the current state. |
| [4] | reg_wr_train_dqs_default_bypass | R/W | 0x0 | Choose the DQS default value when enable the write train (The DQS scan mode is disabled). 0: Use the write-leveling value. 1: Use the register to choose the default value. |
| [3] | reg_wr_train_dqs_range_bypass | R/W | 0x0 | The enable signal to setting the DQS scan range using the register.(wr training) 1: Enable to use the register to set the DQS scan range. 0: Use the default DQS scan range[0~6’h3f]. |
| [2] | reg_wr_train_freq_update | R/W | 0x0 | not used |
| [1] | reg_dq_wr_train_en | R/W | 0x0 | The enable signal of the write training Auto Mode. 1: Enable the auto train of the read train. 0: Exit the auto train of the read train. |
| [0] | reg_dq_wr_train_auto | R/W | 0x0 | Choose the mode of the write train. 1: Choose the write train auto mode,you should set it to 1'b1 before write train. |

Offset: 0xA4

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [30:28] | reg_wr_train_ba_addr | R/W | 0x0 | The bit[2:0] of bank address for write/read operation in DDR3/4 and LPDDR3 write training mode.](reuse for rd perdef training and wr training ) |
| [25:16] | reg_wr_train_col_addr | R/W | 0x0 | The bit[7:0] of col address for write/read operation in DDR3/4 and LPDDR3 write training mode.](reuse for rd perdef training and wr training ) The bit[9:8] of col address for write/read operation in DDR3/4 and LPDDR3 write training mode.(reuse for rd perdef training and wr training ) |
| [15:0] | reg_wr_train_row_addr | R/W | 0x0 | The bit[7:0] of row address for write/read operation in DDR3/4 and LPDDR3 write training mode.(reuse for rd perdef training and wr training ) The bit[15:8] of row address for write/read operation in DDR3/4 and LPDDR3 write training mode.(reuse for rd perdef training and wr training ) |

Offset: 0xA8

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [31:18] | reg_phy_trefi | R/W | 0x960 | The value is Trefi/dfi_clk1x |
| [17:8] | reg_phy_trfc | R/W | 0x8c | The register is used to control the auto-refresh interval when the phy do the training. The reg_phy_trfc[9:0] = {reg0x4e[7:6],reg0x57[7:0]}. The auto-refresh interval = reg_phy_trfc *Tdfi_clk1x. The bit9 and bit 8 of the reg_phy_trfc. The reg_phy_trfc is used to control the interval of the auto-refresh. The PHY will send the auto-refresh when it is in the training mode. Auto-refresh interval = reg_phy_trfc* Tdfi_clk1x |
| [7:4] | reg_max_refi_cnt | R/W | 0x8 | this register used in wr/rd training,if you set reg_phy_refresh_en to 1’b1. this register means how many auto refresh cmd can be accumulated at one time.the maximum is 8,and the larger the value ,the less time training takes . |
| [0] | reg_phy_refresh_en | R/W | 0x0 | When you want to enable phy auto refresh function in training, you should set this reg to 1 at frist. |

Offset: 0xAC

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [19:12] | reg_ddrphy_trp | R/W | 0xf | This register is used to control the timing parameter tRP which means the delay between the Prechage command to the Next Command. |
| [11:6] | reg_data_path_clk_gate_dly | R/W | 0x0 |  |
| [5] | reg_data_path_clk_gate_dly_bp | R/W | 0x0 |  |
| [4] | reg_freq_choose_bypass | R/W | 0x0 | The bypass signal of the freq point choose. Active High. 1: It will use the reg_freq_choose_t to choose the freq point. 0: The phy will choose the freq point based on the dfi_freqeuncy at the phy initialization and fast frequency change. |
| [3] | reg_all_freq_train_finish | R/W | 0x0 | keep default |
| [2] | reg_pll_lock_bypass | R/W | 0x0 | Choose the way to wait for the pll lock after the pll enable. 0: Wait the pll lock from the pll module. 1: Wait the pll lock by enable the pll lock counter after the pll enable. |
| [1] | reg_pllpd_bypass | R/W | 0x0 | Choose the way to control the pll pd signal of the pll module. 1: Only can use the register to control. 0: Automatically control in the dfi low power mode. |
| [0] | reg_lpddr4_write_postamble_sel | R/W | 0x0 | The write postamble control signal of the LPDDR4 mode. Please keep the same setting with the LPDDR4 MR1[7]. 1:1.5*tCK. 0:0.5*tCK. |

Offset: 0xB0

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [27:22] | reg_wl_dqs_lock_point | R/W | 0x1b | write leveling's dqs tx delay lock point |
| [21:16] | reg_wl_dqs_start_point | R/W | 0x0 | write leveling's dqs tx delay start point |
| [13:8] | reg_rd_train_dqs_scan_max | R/W | 0x3e | the max rx dqs delay when do rd training dqs scan. |
| [5:0] | reg_rd_train_dq_scan_max | R/W | 0x3e | control the scan range of dq's rd training |

Offset: 0xF0

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [16] | reg_cmd_drv_zqcalib_en | R/W | 0x0 | The comand driver control signal. 1: Choose the zqcalibration result as the control signal. 0: Choose the register as the conrol signal. |
| [15] | reg_cmd_fbsel_reg | R/W | 0x0 | The phy bist command path seletc signal. 0: Choose to arrive the pad. 1: Choose the interal as the end. |
| [14] | reg_cmd_fben_reg | R/W | 0x0 | The phy bist cmmand feebback enable signal. Active high. |
| [13] | reg_cmd_abutweakpub_reg | R/W | 0x1 | The enable signal of CMD PAD weak pull up. Active Low. |
| [12:8] | reg_cmd_abutslewpu_reg | R/W | 0x0 | CMD/CK rising edge slew rate control, larger value means larger risign slew rate. |
| [5] | reg_cmd_abutweakpd_reg | R/W | 0x0 | The enable signal of CMD PAD weak pull down. Active high. |
| [4:0] | reg_cmd_abutslewpd_reg | R/W | 0x0 | CMD/CK falling edge slew rate control, larger value means larger falling slew rate. |

Offset: 0xF4

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [28:24] | reg_cmd_abutnrcomp_reg | R/W | 0xE | The pull-down resistance of CMD except CK. |
| [20:16] | reg_cmd_abutprcomp_reg | R/W | 0xE | The pull-up resistance of CMD except CK. |
| [12:8] | reg_cmd_abutnrcomp_ck0_reg | R/W | 0xE | The pull-down resistance of CK and CKB. |
| [4:0] | reg_cmd_abutprcomp_ck0_reg | R/W | 0xE | The pull-up resistance of CK and CKB. |

Offset: 0xF8

Reserved

Offset: 0x104~0x120 (8 registers)

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [28:24] | reg_XX3_invdelaysel_bp | R/W | 0x7 | tx delay line of XX3 signal.Each step is 4UI/256. |
| [20:16] | reg_XX2_invdelaysel_bp | R/W | 0x7 | tx delay line of XX2 signal.Each step is 4UI/256. |
| [12:8] | reg_XX1_invdelaysel_bp | R/W | 0x7 | tx delay line of XX1 signal.Each step is 4UI/256. |
| [4:0] | reg_XX0_invdelaysel_bp | R/W | 0x7 | tx delay line of XX0 signal.Each step is 4UI/256. |

4 entries (MSB to LSB) in 8 registers sum up to 32 entries, coresponding to the following list of signals:

a0, … a17, ba0, ba1, bg0, bg1, cke0, cke1, ckb, ck, odt0, odt1, csb0, csb1, resetn, actn

Offset: 0x150~0x1AC (24 registers)

| Reg | Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- | --- |
| 0x150 | [31:24] | dq0_train_check_data_value0 | R/W | 0x5a | burst8 for A_DQ0/A_DQ8/B_DQ0/B_DQ8 |
| 0x150 | [23:16] | dq0_train_check_data_value1 | R/W | 0xa5 | burst8 for A_DQ0/A_DQ8/B_DQ0/B_DQ8 |
| 0x150 | [15:8] | dq0_train_check_data_value2 | R/W | 0xc3 | burst8 for A_DQ0/A_DQ8/B_DQ0/B_DQ8 |
| 0x150 | [7:0] | dq0_train_check_data_value3 | R/W | 0x3c | burst8 for A_DQ0/A_DQ8/B_DQ0/B_DQ8 |
| 0x154 | [31:24] | dq0_train_check_data_value4 | R/W | 0xff | burst8 for A_DQ0/A_DQ8/B_DQ0/B_DQ8 |
| 0x154 | [23:16] | dq0_train_check_data_value5 | R/W | 0x55 | burst8 for A_DQ0/A_DQ8/B_DQ0/B_DQ8 |
| 0x154 | [15:8] | dq0_train_check_data_value6 | R/W | 0x00 | burst8 for A_DQ0/A_DQ8/B_DQ0/B_DQ8 |
| 0x154 | [7:0] | dq0_train_check_data_value7 | R/W | 0xaa | burst8 for A_DQ0/A_DQ8/B_DQ0/B_DQ8 |
| 0x158 | [15:8] | dq0_train_check_data_value8 | R/W | 0xf0 | burst8 for A_DQ0/A_DQ8/B_DQ0/B_DQ8 |
| 0x158 | [7:0] | dq0_train_check_data_value9 | R/W | 0x0f | burst8 for A_DQ0/A_DQ8/B_DQ0/B_DQ8 |
| 0x15C | [31:24] | dq1_train_check_data_value0 | R/W | 0x5a | burst8 for A_DQ1/A_DQ9/B_DQ1/B_DQ9 |
| 0x15C | [23:16] | dq1_train_check_data_value1 | R/W | 0xa5 | burst8 for A_DQ1/A_DQ9/B_DQ1/B_DQ9 |
| 0x15C | [15:8] | dq1_train_check_data_value2 | R/W | 0xc3 | burst8 for A_DQ1/A_DQ9/B_DQ1/B_DQ9 |
| 0x15C | [7:0] | dq1_train_check_data_value3 | R/W | 0x3c | burst8 for A_DQ1/A_DQ9/B_DQ1/B_DQ9 |
| 0x160 | [31:24] | dq1_train_check_data_value4 | R/W | 0xff | burst8 for A_DQ1/A_DQ9/B_DQ1/B_DQ9 |
| 0x160 | [23:16] | dq1_train_check_data_value5 | R/W | 0x55 | burst8 for A_DQ1/A_DQ9/B_DQ1/B_DQ9 |
| 0x160 | [15:8] | dq1_train_check_data_value6 | R/W | 0x00 | burst8 for A_DQ1/A_DQ9/B_DQ1/B_DQ9 |
| 0x160 | [7:0] | dq1_train_check_data_value7 | R/W | 0xaa | burst8 for A_DQ1/A_DQ9/B_DQ1/B_DQ9 |
| 0x164 | [15:8] | dq1_train_check_data_value8 | R/W | 0xf0 | burst8 for A_DQ1/A_DQ9/B_DQ1/B_DQ9 |
| 0x164 | [7:0] | dq1_train_check_data_value9 | R/W | 0x0f | burst8 for A_DQ1/A_DQ9/B_DQ1/B_DQ9 |
| 0x168 | [31:24] | dq2_train_check_data_value0 | R/W | 0x5a | burst8 for A_DQ2/A_DQ10/B_DQ2/B_DQ10 |
| 0x168 | [23:16] | dq2_train_check_data_value1 | R/W | 0xa5 | burst8 for A_DQ2/A_DQ10/B_DQ2/B_DQ10 |
| 0x168 | [15:8] | dq2_train_check_data_value2 | R/W | 0xc3 | burst8 for A_DQ2/A_DQ10/B_DQ2/B_DQ10 |
| 0x168 | [7:0] | dq2_train_check_data_value3 | R/W | 0x3c | burst8 for A_DQ2/A_DQ10/B_DQ2/B_DQ10 |
| 0x16C | [31:24] | dq2_train_check_data_value4 | R/W | 0xff | burst8 for A_DQ2/A_DQ10/B_DQ2/B_DQ10 |
| 0x16C | [23:16] | dq2_train_check_data_value5 | R/W | 0x55 | burst8 for A_DQ2/A_DQ10/B_DQ2/B_DQ10 |
| 0x16C | [15:8] | dq2_train_check_data_value6 | R/W | 0x00 | burst8 for A_DQ2/A_DQ10/B_DQ2/B_DQ10 |
| 0x16C | [7:0] | dq2_train_check_data_value7 | R/W | 0xaa | burst8 for A_DQ2/A_DQ10/B_DQ2/B_DQ10 |
| 0x170 | [15:8] | dq2_train_check_data_value8 | R/W | 0xf0 | burst8 for A_DQ2/A_DQ10/B_DQ2/B_DQ10 |
| 0x170 | [7:0] | dq2_train_check_data_value9 | R/W | 0x0f | burst8 for A_DQ2/A_DQ10/B_DQ2/B_DQ10 |
| 0x174 | [31:24] | dq3_train_check_data_value0 | R/W | 0x5a | burst8 for A_DQ3/A_DQ11/B_DQ3/B_DQ11 |
| 0x174 | [23:16] | dq3_train_check_data_value1 | R/W | 0xa5 | burst8 for A_DQ3/A_DQ11/B_DQ3/B_DQ11 |
| 0x174 | [15:8] | dq3_train_check_data_value2 | R/W | 0xc3 | burst8 for A_DQ3/A_DQ11/B_DQ3/B_DQ11 |
| 0x174 | [7:0] | dq3_train_check_data_value3 | R/W | 0x3c | burst8 for A_DQ3/A_DQ11/B_DQ3/B_DQ11 |
| 0x178 | [31:24] | dq3_train_check_data_value4 | R/W | 0xff | burst8 for A_DQ3/A_DQ11/B_DQ3/B_DQ11 |
| 0x178 | [23:16] | dq3_train_check_data_value5 | R/W | 0x55 | burst8 for A_DQ3/A_DQ11/B_DQ3/B_DQ11 |
| 0x178 | [15:8] | dq3_train_check_data_value6 | R/W | 0x00 | burst8 for A_DQ3/A_DQ11/B_DQ3/B_DQ11 |
| 0x178 | [7:0] | dq3_train_check_data_value7 | R/W | 0xaa | burst8 for A_DQ3/A_DQ11/B_DQ3/B_DQ11 |
| 0x17C | [15:8] | dq3_train_check_data_value8 | R/W | 0xf0 | burst8 for A_DQ3/A_DQ11/B_DQ3/B_DQ11 |
| 0x17C | [7:0] | dq3_train_check_data_value9 | R/W | 0x0f | burst8 for A_DQ3/A_DQ11/B_DQ3/B_DQ11 |

| Reg | Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- | --- |
| 0x180 | [31:24] | dq4_train_check_data_value0 | R/W | 0x5a | burst8 for A_DQ4/A_DQ12/B_DQ4/B_DQ12 |
| 0x180 | [23:16] | dq4_train_check_data_value1 | R/W | 0xa5 | burst8 for A_DQ4/A_DQ12/B_DQ4/B_DQ12 |
| 0x180 | [15:8] | dq4_train_check_data_value2 | R/W | 0xc3 | burst8 for A_DQ4/A_DQ12/B_DQ4/B_DQ12 |
| 0x180 | [7:0] | dq4_train_check_data_value3 | R/W | 0x3c | burst8 for A_DQ4/A_DQ12/B_DQ4/B_DQ12 |
| 0x184 | [31:24] | dq4_train_check_data_value4 | R/W | 0xff | burst8 for A_DQ4/A_DQ12/B_DQ4/B_DQ12 |
| 0x184 | [23:16] | dq4_train_check_data_value5 | R/W | 0x55 | burst8 for A_DQ4/A_DQ12/B_DQ4/B_DQ12 |
| 0x184 | [15:8] | dq4_train_check_data_value6 | R/W | 0x00 | burst8 for A_DQ4/A_DQ12/B_DQ4/B_DQ12 |
| 0x184 | [7:0] | dq4_train_check_data_value7 | R/W | 0xaa | burst8 for A_DQ4/A_DQ12/B_DQ4/B_DQ12 |
| 0x188 | [15:8] | dq4_train_check_data_value8 | R/W | 0xf0 | burst8 for A_DQ4/A_DQ12/B_DQ4/B_DQ12 |
| 0x188 | [7:0] | dq4_train_check_data_value9 | R/W | 0x0f | burst8 for A_DQ4/A_DQ12/B_DQ4/B_DQ12 |
| 0x18C | [31:24] | dq5_train_check_data_value0 | R/W | 0x5a | burst8 for A_DQ5/A_DQ13/B_DQ5/B_DQ13 |
| 0x18C | [23:16] | dq5_train_check_data_value1 | R/W | 0xa5 | burst8 for A_DQ5/A_DQ13/B_DQ5/B_DQ13 |
| 0x18C | [15:8] | dq5_train_check_data_value2 | R/W | 0xc3 | burst8 for A_DQ5/A_DQ13/B_DQ5/B_DQ13 |
| 0x18C | [7:0] | dq5_train_check_data_value3 | R/W | 0x3c | burst8 for A_DQ5/A_DQ13/B_DQ5/B_DQ13 |
| 0x190 | [31:24] | dq5_train_check_data_value4 | R/W | 0xff | burst8 for A_DQ5/A_DQ13/B_DQ5/B_DQ13 |
| 0x190 | [23:16] | dq5_train_check_data_value5 | R/W | 0x55 | burst8 for A_DQ5/A_DQ13/B_DQ5/B_DQ13 |
| 0x190 | [15:8] | dq5_train_check_data_value6 | R/W | 0x00 | burst8 for A_DQ5/A_DQ13/B_DQ5/B_DQ13 |
| 0x190 | [7:0] | dq5_train_check_data_value7 | R/W | 0xaa | burst8 for A_DQ5/A_DQ13/B_DQ5/B_DQ13 |
| 0x194 | [15:8] | dq5_train_check_data_value8 | R/W | 0xf0 | burst8 for A_DQ5/A_DQ13/B_DQ5/B_DQ13 |
| 0x194 | [7:0] | dq5_train_check_data_value9 | R/W | 0x0f | burst8 for A_DQ5/A_DQ13/B_DQ5/B_DQ13 |
| 0x198 | [31:24] | dq6_train_check_data_value0 | R/W | 0x5a | burst8 for A_DQ6/A_DQ14/B_DQ6/B_DQ14 |
| 0x198 | [23:16] | dq6_train_check_data_value1 | R/W | 0xa5 | burst8 for A_DQ6/A_DQ14/B_DQ6/B_DQ14 |
| 0x198 | [15:8] | dq6_train_check_data_value2 | R/W | 0xc3 | burst8 for A_DQ6/A_DQ14/B_DQ6/B_DQ14 |
| 0x198 | [7:0] | dq6_train_check_data_value3 | R/W | 0x3c | burst8 for A_DQ6/A_DQ14/B_DQ6/B_DQ14 |
| 0x19C | [31:24] | dq6_train_check_data_value4 | R/W | 0xff | burst8 for A_DQ6/A_DQ14/B_DQ6/B_DQ14 |
| 0x19C | [23:16] | dq6_train_check_data_value5 | R/W | 0x55 | burst8 for A_DQ6/A_DQ14/B_DQ6/B_DQ14 |
| 0x19C | [15:8] | dq6_train_check_data_value6 | R/W | 0x00 | burst8 for A_DQ6/A_DQ14/B_DQ6/B_DQ14 |
| 0x19C | [7:0] | dq6_train_check_data_value7 | R/W | 0xaa | burst8 for A_DQ6/A_DQ14/B_DQ6/B_DQ14 |
| 0x1A0 | [15:8] | dq6_train_check_data_value8 | R/W | 0xf0 | burst8 for A_DQ6/A_DQ14/B_DQ6/B_DQ14 |
| 0x1A0 | [7:0] | dq6_train_check_data_value9 | R/W | 0x0f | burst8 for A_DQ6/A_DQ14/B_DQ6/B_DQ14 |
| 0x1A4 | [31:24] | dq7_train_check_data_value0 | R/W | 0x5a | burst8 for A_DQ7/A_DQ15/B_DQ7/B_DQ15 |
| 0x1A4 | [23:16] | dq7_train_check_data_value1 | R/W | 0xa5 | burst8 for A_DQ7/A_DQ15/B_DQ7/B_DQ15 |
| 0x1A4 | [15:8] | dq7_train_check_data_value2 | R/W | 0xc3 | burst8 for A_DQ7/A_DQ15/B_DQ7/B_DQ15 |
| 0x1A4 | [7:0] | dq7_train_check_data_value3 | R/W | 0x3c | burst8 for A_DQ7/A_DQ15/B_DQ7/B_DQ15 |
| 0x1A8 | [31:24] | dq7_train_check_data_value4 | R/W | 0xff | burst8 for A_DQ7/A_DQ15/B_DQ7/B_DQ15 |
| 0x1A8 | [23:16] | dq7_train_check_data_value5 | R/W | 0x55 | burst8 for A_DQ7/A_DQ15/B_DQ7/B_DQ15 |
| 0x1A8 | [15:8] | dq7_train_check_data_value6 | R/W | 0x00 | burst8 for A_DQ7/A_DQ15/B_DQ7/B_DQ15 |
| 0x1A8 | [7:0] | dq7_train_check_data_value7 | R/W | 0xaa | burst8 for A_DQ7/A_DQ15/B_DQ7/B_DQ15 |
| 0x1AC | [15:8] | dq7_train_check_data_value8 | R/W | 0xf0 | burst8 for A_DQ7/A_DQ15/B_DQ7/B_DQ15 |
| 0x1AC | [7:0] | dq7_train_check_data_value9 | R/W | 0x0f | burst8 for A_DQ7/A_DQ15/B_DQ7/B_DQ15 |

Offset: 0x1AC

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [19] | reg_wrtrain_odt_advance | R/W | 0x0 | the register is used in write train,let the odt open advance when do write operation ,you can keep the default |
| [18:16] | reg_wrtrain_odt_keep | R/W | 0x1 | the register is used in write train,let the odt open keep when do wirte operation, you can keep the default |

Offset: 0x1B0

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [31:24] | reg_wrtrain_vref_wait_vref_cnt_50ns | R/W | 0x14 | the register is used in write train, if you enable tx vref scan function, you need to configure this register.When set a new tx vref value to sdram , we should wait 100-250ns ,then it works, you should set this register tell PHY how long is 50ns,then the phy have a reference.So the reg’s value = 50ns/dfi_1xclk |
| [23:21] | reg_train_vref_step_min | R/W | 0x1 | the register is used in read predef train or write train, if you enable vref scan function, you can configure this register.this register means How many step at a time, when doing A fine vref scanning. |
| [20:16] | reg_train_vref_step_max | R/W | 0x8 | the register is used in read predef train or write train, if you enable vref scan function, you can configure this register.this register means How many step at a time, when doing A rough vref scanning. |
| [15:10] | reg_cmd_invdelaysel_sel | R/W | 0x0 | The command tx delay line value obs signal. The user can use this register to get the current control value of each command pad. 0:A0 1:A1 |
| [9:0] | reg_rdtrain_wait_vref_valid_cnt | R/W | 0x140 | the register is used in read predef train, if you enable rx vref scan function, you need to configure this register. When set a new rx vref value ,we should wait 800ns ,then it works.So the reg’s value = 800ns/dfi_1xclk |

Offset: 0x1F0

| Bits | Name | Access | Definition |
| --- | --- | --- | --- |
| [7] | train_all_step_done | RO | The complete signal of the write training/rd perdef training . Active high. The user need to wait this bit change to high after enable the write training. (reuse for rd perdef training and wr training ) |
| [6] | train_step1_delay_done | RO | The compelte signal of the delay scan before the vref scan when write training/rd perdef training. Active high.(reuse for rd perdef training and wr training ) |
| [5] | train_step2_vref_done | RO |  |
| [4] | train_step3_delay_done | RO | The compelte signal fo the delay scan after the vref scan when write training/rd perdef training. Active high. (reuse for rd perdef training and wr training ) |
| [3] | train_step1_error | RO | when train done ,you need to check this register.this reg’s value equal to 1’b1 means step1 have error in rd perdef training or write training.(reuse for rd perdef training and wr training ) . The step of training can refer to read train and write train section. |
| [2] | train_step2_error | RO | when train done ,you need to check this register.this reg’s value equal to 1’b1 means step2 have error in rd perdef training or write training.(reuse for rd perdef training and wr training ) |
| [1] | train_step3_error | RO | when train done ,you need to check this register.this reg’s value equal to 1’b1 means step3 have error in rd perdef training or write training.(reuse for rd perdef training and wr training ) |
| [0] | train_true_done | RO | The complete signal of the read training(mpr/mpc mode). Active high. |

Offset: 0x1F4

| Bits | Name | Access | Definition |
| --- | --- | --- | --- |
| [4] | user_load_mode_busy | RO |  |
| [3] | pwrok_reg_po | RO | Reserved. |
| [2] | pwrokcore | RO |  |
| [1] | lock_mpll | RO |  |
| [0] | lock_pll_dqcmd | RO | The lock signal of the pll. Active high. The pll will be locked in 5000 reference clock cycles after the pd operation. |

Offset: 0x1F8

| Bits | Name | Access | Definition |
| --- | --- | --- | --- |
| [28:24] | drvlegn_zqcali_2reg[4:0] | RO | The zqclibration result of driver pull down after the zqcalib training. |
| [20:16] | drvlegpb_zqcali_2reg[4:0] | RO | The zqclibration result of driver pull up after the zqcalib training. |
| [12:8] | odtlegn_zqcali_2reg[4:0] | RO | The zqclibration result of odt pull down after the zqcalib training. |
| [4:0] | odtlegpb_zqcali_2reg[4:0] | RO | The zqclibration result of odt pull up after the zqcalib training. |

Offset: 0x1FC

| Bits | Name | Access | Definition |
| --- | --- | --- | --- |
| [18:8] | reg_x1clk_ndiv_cnt[10:0] | RO |  |
| [4] | reg_zqcali_done | RO | The complete signal of the zacalib training. Active high. |
| [3] | reg_drvpd_overflow | RO | The fail flag of driver pull down zacalib trraining. Active high. |
| [2] | reg_drvpu_overflow | RO | The fail flag of driver pull up zacalib trraining. Active high. |
| [1] | reg_odtpd_overflow | RO | The fail flag of odt pull down zacalib trraining. Active high. |
| [0] | reg_odtpu_overflow | RO | The fail flag of odt pull up zacalib trraining. Active high. |

Offset: 0x200~0x208

Reserved read-only registers for PHY BIST.

Offset: 0x20C

| Bits | Name | Access | Definition |
| --- | --- | --- | --- |
| [11:8] | wl_done_byte[3:0] | RO | The write leveling complete signal of each channel. Active high. [0]: The write leveling complete of byte0. [1]: The write leveling complete of byte1. [2]: The write leveling complete of byte2. [3]: The write leveling compelte of byte3. |
| [5] | calib_end | RO | The rx dqs calib complete signal. Active high. Only change to 1'b1 when all open channel have compelte the rx dqs calibration. |
| [4] | calib_error | RO | The error flag of the rx dqs calib. Active high. |
| [3:0] | calib_done_byte[3:0] | RO | The rx dqs calib complete signal of each channel. Active high. [0]: The rx dqs calib complete of byte0. [1]: The rx dqs calib complete of byte1. [2]: The rx dqs calib complete of byte2. [3]: The rx dqs calib compelte of byte3. |

Offset: 0x210

| Bits | Name | Access | Definition |
| --- | --- | --- | --- |
| [19:18] | cha_rank_cat_bp_cmd_send_rdy[1:0] | RO | [0]:The reay signal of the command bus training bypass mode of channel b RANK0. Active high. After the user enable the command bus training bypass mode, you need to wait this bit change to high. When this bit change to to high, it means the sdram has already go into the command bus training mode. Then the user can use the register to send the CBT command to do the command bus training mode. [1]:The reay signal of the command bus training bypass mode of channel a RANK1. Active high. After the user enable the command bus training bypass mode, you need to wait this bit change to high. When this bit change to to high, it means the sdram has already go into the command bus training mode. Then the user can use the register to send the CBT command to do the command bus training mode. |
| [17:16] | chb_rank_cat_bp_cmd_send_rdy[1:0] | RO | [0]:The reay signal of the command bus training bypass mode of channel b RANK0. Active high. After the user enable the command bus training bypass mode, you need to wait this bit change to high. When this bit change to to high, it means the sdram has already go into the command bus training mode. Then the user can use the register to send the CBT command to do the command bus training mode. [1]:The reay signal of the command bus training bypass mode of channel a RANK0. Active high. After the user enable the command bus training bypass mode, you need to wait this bit change to high. When this bit change to to high, it means the sdram has already go into the command bus training mode. Then the user can use the register to send the CBT command to do the command bus training mode. |
| [15:14] | cha_rank_cat_bp_done[1:0] | RO | [0]:The command bus training bypass mode compelte signal of channel b RANK1. Actve high. After the user enable the command bus training bypass mode and use the register to compelte the command bus training, the user can stop the bypass mode by set the reg_cat_bp_start to low, then the phy will exit the command bus train mode and set to this bit to "1". Then the user can set the reg_cat_bp_en to exit the command bust train mode. [1]:The command bus training bypass mode compelte signal of channel a RANK1. Actve high. After the user enable the command bus training bypass mode and use the register to compelte the command bus training, the user can stop the bypass mode by set the reg_cat_bp_start to low, then the phy will exit the command bus train mode and set to this bit to "1". Then the user can set the reg_cat_bp_en to exit the command bust train mode. |
| [13:12] | chb_rank_cat_bp_done[1:0] | RO | [0]:The command bus training bypass mode compelte signal of channel b RANK0. Actve high. After the user enable the command bus training bypass mode and use the register to compelte the command bus training, the user can stop the bypass mode by set the reg_cat_bp_start to low, then the phy will exit the command bus train mode and set to this bit to "1". Then the user can set the reg_cat_bp_en to exit the command bust train mode. [1]:The command bus training bypass mode compelte signal of channel a RANK0. Actve high. After the user enable the command bus training bypass mode and use the register to compelte the command bus training, the user can stop the bypass mode by set the reg_cat_bp_start to low, then the phy will exit the command bus train mode and set to this bit to "1". Then the user can set the reg_cat_bp_en to exit the command bust train mode. |
| [11:0] | ca_check_value[11:0] | RO | [5:0]:The command bus train read back data from the dq[5:0] of channel a when enable the command bus train bypass mode. When enable the command bus train bypass mode and go into the comand bus train mode, if the phy send the CBT command, the dq will read back the data on the command bus through, the read back data will be stored in this register. The user can use this register to get the read back data and judge the curren tx delay line is right or not. [6:11]:The command bus train read back data from the dq[5:0] of channel b when enable the command bus train bypass mode. When enable the command bus train bypass mode and go into the comand bus train mode, if the phy send the CBT command, the dq will read back the data on the command bus through, the read back data will be stored in this register. The user can use this register to get the read back data and judge the curren tx delay line is right or not. |

Offset: 0x214

| Bits | Name | Access | Definition |
| --- | --- | --- | --- |
| [23:16] | cha_cat_auto_cs_train_err[7:0] | RO | The auto command bus cs train error flag of channel a. [0]: The training result of csb for rank0 is abnormal if enable the cs train. [1]: The training result of csb for rank1 is abnormal if enable the cs train. [2]: Can't find the vref pass window of channel a csb for rank0 if enable the cs train. [3]: Can't find the vref pass window of channel a csb for rank1 if enable the cs train. [4]: Can't find the tx delay line pass window of channel a csb for rank0 if enable the cs train. [5]: Can't find the tx delay line pass window of channel a csb for rank1 if enable the cs train. [6]: The start point of the channel a CSB0 training is abnormal if enable the cs train. The training will be fail. [7]: The start point of the channel a CSB1 training is abnormal if enable the cs train. The training will be fail. |
| [15:8] | chb_cat_auto_cs_train_err[7:0] | RO | The auto command bus cs train error flag of channel b. [0]: The training result of csb for rank0 is abnormal if enable the cs train. [1]: The training result of csb for rank1 is abnormal if enable the cs train. [2]: Can't find the vref pass window of channel b csb for rank0 if enable the cs train. [3]: Can't find the vref pass window of channel b csb for rank1 if enable the cs train. [4]: Can't find the tx delay line pass window of channel b csb for rank0 if enable the cs train. [5]: Can't find the tx delay line pass window of channel b csb for rank1 if enable the cs train. [6]: The start point of the channel b CSB0 training is abnormal if enable the cs train. The training will be fail. [7]: The start point of the channel b CSB1 training is abnormal if enable the cs train. The training will be fail. |
| [3] | cha_cat_done | RO | The auto command bus train complete signal of channel a. Active high. |
| [2] | chb_cat_done | RO | The auto command bus train complete signal of channel b. Active high. |
| [1] | cha_cat_bp_cmd_send_done | RO | The flag of channel b that the phy compelte to send the CBT command when enable the command bus training bypass mode. Active high. The user needs to wait this bit change to 1'b1 when it prepares to send the next CBT command. |
| [0] | chb_cat_bp_cmd_send_done | RO | The flag of channel a that the phy compelte to send the CBT command when enable the command bus training bypass mode. Active high. The user needs to wait this bit change to 1'b1 when it prepares to send the next CBT command. |

Offset: 0x230

| Bits | Name | Access | Definition |
| --- | --- | --- | --- |
| [21:16] | reg_cmd_invdelaysel[5:0] | RO | The command tx delay line value obs signal. The user can use this register to get the current control value of each command pad. 0:A0 1:A1 |
| [13:8] | reg_wrtrain_vref_max_value[5:0] | RO | The max pass point of the tx SDRAM's DQ vref for dq . |
| [5:0] | reg_wrtrain_vref_min_value[5:0] | RO | The max pass point of the tx SDRAM's DQ vref for dq . |

Offset: 0x234~0x270

| Reg | Bits | Name | Definition |
| --- | --- | --- | --- |
| 0x234 | [29:24] | cha_rank0_ca0_min_perbit_skew_pass | Min pass point of tx delay for rank0 channel A ca0. |
| 0x234 | [21:16] | cha_rank0_ca1_min_perbit_skew_pass | Min pass point of tx delay for rank0 channel A ca1. |
| 0x234 | [13:8] | cha_rank0_ca2_min_perbit_skew_pass | Min pass point of tx delay for rank0 channel A ca2. |
| 0x234 | [5:0] | cha_rank0_ca3_min_perbit_skew_pass | Min pass point of tx delay for rank0 channel A ca3. |
| 0x238 | [29:24] | cha_rank0_ca4_min_perbit_skew_pass | Min pass point of tx delay for rank0 channel A ca4. |
| 0x238 | [21:16] | cha_rank0_ca5_min_perbit_skew_pass | Min pass point of tx delay for rank0 channel A ca5. |
| 0x238 | [13:8] | cha_rank1_ca0_min_perbit_skew_pass | Min pass point of tx delay for rank1 channel A ca0. |
| 0x238 | [5:0] | cha_rank1_ca1_min_perbit_skew_pass | Min pass point of tx delay for rank1 channel A ca1. |
| 0x23C | [29:24] | cha_rank1_ca2_min_perbit_skew_pass | Min pass point of tx delay for rank1 channel A ca2. |
| 0x23C | [21:16] | cha_rank1_ca3_min_perbit_skew_pass | Min pass point of tx delay for rank1 channel A ca3 |
| 0x23C | [13:8] | cha_rank1_ca4_min_perbit_skew_pass | Min pass point of tx delay for rank1 channel A ca4. |
| 0x23C | [5:0] | cha_rank1_ca5_min_perbit_skew_pass | Min pass point of tx delay for rank1 channel A ca5. |
| 0x240 | [29:24] | cha_rank0_min_cs_perbit_skew_pass | Min pass point of tx delay for rank0 channel A cs. |
| 0x240 | [21:16] | cha_rank1_min_cs_perbit_skew_pass | Min pass point of tx delay for rank1 channel A cs |
| 0x240 | [13:8] | cha_rank0_max_cs_perbit_skew_pass | Max pass point of tx delay for rank0 channel A cs. |
| 0x240 | [5:0] | cha_rank1_max_cs_perbit_skew_pass | Max pass point of tx delay for rank1 channel A cs. |
| 0x244 | [29:24] | cha_rank0_ca0_min_perbit_skew_pass | Max pass point of tx delay for rank0 channel A ca0. |
| 0x244 | [21:16] | cha_rank0_ca1_min_perbit_skew_pass | Max pass point of tx delay for rank0 channel A ca1. |
| 0x244 | [13:8] | cha_rank0_ca2_min_perbit_skew_pass | Max pass point of tx delay for rank0 channel A ca2. |
| 0x244 | [5:0] | cha_rank0_ca3_min_perbit_skew_pass | Max pass point of tx delay for rank0 channel A ca3. |
| 0x248 | [29:24] | cha_rank0_ca4_min_perbit_skew_pass | Max pass point of tx delay for rank0 channel A ca4. |
| 0x248 | [21:16] | cha_rank0_ca5_min_perbit_skew_pass | Max pass point of tx delay for rank0 channel A ca5. |
| 0x248 | [13:8] | cha_rank1_ca0_min_perbit_skew_pass | Max pass point of tx delay for rank1 channel A ca0. |
| 0x248 | [5:0] | cha_rank1_ca1_min_perbit_skew_pass | Max pass point of tx delay for rank1 channel A ca1. |
| 0x24C | [29:24] | cha_rank1_ca2_min_perbit_skew_pass | Max pass point of tx delay for rank1 channel A ca2. |
| 0x24C | [21:16] | cha_rank1_ca3_min_perbit_skew_pass | Max pass point of tx delay for rank1 channel A ca3 |
| 0x24C | [13:8] | cha_rank1_ca4_min_perbit_skew_pass | Max pass point of tx delay for rank1 channel A ca4. |
| 0x24C | [5:0] | cha_rank1_ca5_min_perbit_skew_pass | Max pass point of tx delay for rank1 channel A ca5. |
| 0x250 | [29:24] | chb_rank0_ca0_min_perbit_skew_pass | Min pass point of tx delay for rank0 channel B ca0. |
| 0x250 | [21:16] | chb_rank0_ca1_min_perbit_skew_pass | Min pass point of tx delay for rank0 channel B ca1. |
| 0x250 | [13:8] | chb_rank0_ca2_min_perbit_skew_pass | Min pass point of tx delay for rank0 channel B ca2. |
| 0x250 | [5:0] | chb_rank0_ca3_min_perbit_skew_pass | Min pass point of tx delay for rank0 channel B ca3. |
| 0x254 | [29:24] | chb_rank0_ca4_min_perbit_skew_pass | Min pass point of tx delay for rank0 channel B ca4. |
| 0x254 | [21:16] | chb_rank0_ca5_min_perbit_skew_pass | Min pass point of tx delay for rank0 channel B ca5. |
| 0x254 | [13:8] | chb_rank1_ca0_min_perbit_skew_pass | Min pass point of tx delay for rank1 channel B ca0. |
| 0x254 | [5:0] | chb_rank1_ca1_min_perbit_skew_pass | Min pass point of tx delay for rank1 channel B ca1. |
| 0x258 | [29:24] | chb_rank1_ca2_min_perbit_skew_pass | Min pass point of tx delay for rank1 channel B ca2. |
| 0x258 | [21:16] | chb_rank1_ca3_min_perbit_skew_pass | Min pass point of tx delay for rank1 channel B ca3 |
| 0x258 | [13:8] | chb_rank1_ca4_min_perbit_skew_pass | Min pass point of tx delay for rank1 channel B ca4. |
| 0x258 | [5:0] | chb_rank1_ca5_min_perbit_skew_pass | Min pass point of tx delay for rank1 channel B ca5. |
| 0x25C | [29:24] | chb_rank0_min_cs_perbit_skew_pass | Min pass point of tx delay for rank0 channel B cs. |
| 0x25C | [21:16] | chb_rank1_min_cs_perbit_skew_pass | Min pass point of tx delay for rank1 channel B cs |
| 0x25C | [13:8] | chb_rank0_max_cs_perbit_skew_pass | Max pass point of tx delay for rank0 channel B cs. |
| 0x25C | [5:0] | chb_rank1_max_cs_perbit_skew_pass | Max pass point of tx delay for rank1 channel B cs. |
| 0x260 | [29:24] | chb_rank0_ca0_min_perbit_skew_pass | Max pass point of tx delay for rank0 channel B ca0. |
| 0x260 | [21:16] | chb_rank0_ca1_min_perbit_skew_pass | Max pass point of tx delay for rank0 channel B ca1. |
| 0x260 | [13:8] | chb_rank0_ca2_min_perbit_skew_pass | Max pass point of tx delay for rank0 channel B ca2. |
| 0x260 | [5:0] | chb_rank0_ca3_min_perbit_skew_pass | Max pass point of tx delay for rank0 channel B ca3. |
| 0x264 | [29:24] | chb_rank0_ca4_min_perbit_skew_pass | Max pass point of tx delay for rank0 channel B ca4. |
| 0x264 | [21:16] | chb_rank0_ca5_min_perbit_skew_pass | Max pass point of tx delay for rank0 channel B ca5. |
| 0x264 | [13:8] | chb_rank1_ca0_min_perbit_skew_pass | Max pass point of tx delay for rank1 channel B ca0. |
| 0x264 | [5:0] | chb_rank1_ca1_min_perbit_skew_pass | Max pass point of tx delay for rank1 channel B ca1. |
| 0x268 | [29:24] | chb_rank1_ca2_min_perbit_skew_pass | Max pass point of tx delay for rank1 channel B ca2. |
| 0x268 | [21:16] | chb_rank1_ca3_min_perbit_skew_pass | Max pass point of tx delay for rank1 channel B ca3 |
| 0x268 | [13:8] | chb_rank1_ca4_min_perbit_skew_pass | Max pass point of tx delay for rank1 channel B ca4. |
| 0x268 | [5:0] | chb_rank1_ca5_min_perbit_skew_pass | Max pass point of tx delay for rank1 channel B ca5. |
| 0x26C | [29:24] | cha_rank0_min_cs_vref_pass | Min pass point of tx delay for rank0 channel A cs. |
| 0x26C | [21:16] | cha_rank0_max_cs_vref _pass | Max pass point of tx delay for rank0 channel A cs |
| 0x26C | [13:8] | cha_rank1_min_cs_vref _pass | Min pass point of tx delay for rank1 channel A cs. |
| 0x26C | [5:0] | cha_rank1_max_cs_vref _pass | Max pass point of tx delay for rank1 channel A cs. |
| 0x270 | [29:24] | chb_rank0_min_cs_vref _pass | Min pass point of tx delay for rank0 channel B cs. |
| 0x270 | [21:16] | chb_rank0_max_cs_vref _pass | Max pass point of tx delay for rank0 channel B cs |
| 0x270 | [13:8] | chb_rank1_min_cs_vref _pass | Min pass point of tx delay for rank1 channel B cs. |
| 0x270 | [5:0] | chb_rank1_max_cs_vref _pass | Max pass point of tx delay for rank1 channel B cs. |

Offset: 0x274

| Bits | Name | Access | Definition |
| --- | --- | --- | --- |
| [19:16] | reg_rd_train_readback_data_valid_byte[3:0] | RO | The readback data valid signal when enable the read training bypass mode. Active High. When enable the read training bpass mode, after send out the read command and the read back data will be received by the phy and store in the following registers. When this bit is high, it means the read back data is valid. [0]: The read back data valid signal of byte0. [1]: The read back data valid signal of byte1. [2]: The read back data valid signal of byte2. [3]: The read back data valid signal of byte3. |
| [15:12] | reg_train_done_for_rd_to_reg[3:0] | RO | The done signal of read train for byte0 to byte3. Active high. [0]: The done signal of the read train for byte0. [1]: The done signal of the read train for byte1. [2]: The done signal of the read train for byte2. [3]: The done signal of the read train for byte3. |
| [11:8] | reg_train_error_for_rd_byte[3:0] | RO | The error signal of read train for byte0 to byte3. Active high. [0]: The error signal of the read train for byte0. [1]: The error signal of the read train for byte1. [2]: The error signal of the read train for byte2. [3]: The error signal of the read train for byte3. |
| [7:4] | reg_wr_train_done_byte[3:0] | RO | The done flag of the write training for byte0 to byte3. High means done. [0]: The done flag of the write training for byte0. [1]: The done flag of the write training for byte1. [2]: The done flag of the write training for byte2. [3]: The done flag of the write training for byte3. |
| [3:0] | reg_wr_train_error_byte[3:0] | RO | The error flag of the write training for byte0 to byte3. High means error. [0]: The error flag of the write training for byte0. [1]: The error flag of the write training for byte1. [2]: The error flag of the write training for byte2. [3]: The error flag of the write training for byte3. |

3.2 Detailed definition of DDR Control register

BASEADDR：0x4003 0000 (DDR0_Ctrl)   0x4004 0000 (DDR1_Ctrl)

Offset: 0x0

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [31:23] | reg_mem_bist_channel_en | R/W | 0xff | The byte enable signal of the Controller MEM-BIST function. [0]: The enable signal of the BYTE0. [1]: The enable signal of the BYTE1. … [8]: The enable signal of the BYTE8. When choose the DQ72 mode, this register should set to 9'h1ff. When choose the DQ64 mode, this register should set to 8'hff. When choose the DQ32 mode, this register should set to 8'hf. When choose the DQ16 mode, this register should set to 8'h3. When choose the DQ8 mode, this register should set to 8'h1. |
| [15] | reg_ddrc_core_clock_gate_en | R/W | 0x0 | The clock gate enable signal of the core. 1: When there is no pending write operation, read operation and all read data has already back. The unused logic will be keep the current state and go to the clock gating state. 0: Disable the clock gate function. |
| [14] | reg_aclk_cg_en | R/W | 0x1 | The axi clock gate enable signal. 1: The axi slave will disable the axi clock automatically to save powerafter all axi transfter has completed and no any new axi transfer preprepare to deal with. 0: The axi slave will not disable the axi clock. |
| [13] | reg_ddrc_2t_mode | R/W | 0x0 | The enable signal of the 2T/2N mode for DDR3/DDR4. Acitvie High. This bit only can set to 1'b1 when the controller chooses the DDR3/DDR4 mode. |
| [12] | reg_out_of_order_dis | R/W | 0x0 | Disable the out of order function of the AXI Read Data Channel. Active High. 1: Disable the out of order function of the read channel. The read back data will send back to the AXI Master according to the order that the read commands accept on the AXI Read Address Channel. So the rid will send back to the AXI Master as the same order that the AXI Read Command are accpeted on the AXI Read Address Channel. 0: Enable the out of order function of the read channel. For the different rid, the controller will base on the first ready first back strategy, but for the same rid, the controller will keep the same order that the AXI Read Command are accepted on the AXI Read Address Channel. Becuase that the controller has the ability to reorder the read command, so this strategy can make the read back datas which have different rid can send back to the AXI master firstly if it comes back early to decrease the latency and improve the bandwidth. |
| [11] | reg_ctrl_ddrc_clk_disable | R/W | 0x0 | Used to control the dfi_dram_clk_disable signal of the dfi interface. The dfi_dram_clk_disable will the same logic state with this signal and have 3 core_clock delay. |
| [10:9] | reg_sdram_dq_width | R/W | 0x2 | The register use to choose the DQ width that the controller needs to support. 2'b00: DQ8 2'b01: DQ16 2'b10: DQ32 2'b11: DQ64 If Eable the sideband ecc funciton. 2'b00: DQ16 2'b01: DQ24 2'b10: DQ40 2'b11: DQ72 |
| [8:7] | reg_sdram_burst_length | R/W | 0x1 | The register used to choose the burst length that the controller needs to support. 2'b01: Burst 8 2'b10: Burst 16. |
| [6] | reg_disable_imc_if | R/W | 0x0 | The register used to disable the core to accept the command from the slave interface which can cause the back pressure to the AXI Master. Active High. There are two mechanisms to make the core of the controller to stop to accept the command from the slave interface: 1) When the core of the controller is in the self-refresh state or the power down state. If the self-refresh state or power down state is triggered automatically because of the lack of axi transaction, when the core of the controller detects the axi transaction in the axi slave, it will start to exit the self-refresh or power down. After the core of the controller exit the above state, it will start to enable to accept the command from the slave interface. 2) When reg_disable_imc_if change to high. If the user wants to make the controller,phy and sdram goes into the low power state, it can active this register firstly. Then the core will wait all command pending in the core have been executed and prepare to go into the low power state. |
| [5] | reg_ddrc_skip_init_start | R/W | 0x0 | The enable signal to skip the sdram initial flow. Active High. 1: The controller will skip the sdram initial flow and set the core to the sdram initial complete state. Used in the LPDDR4 PHY command bus training mode. 0: Keep the current state. |
| [4] | reg_phy_init_start | R/W | 0x0 | The signal used to control the phy initial and frequency change flow. Active High. This signal will control the dfi_init_start signal of the DFI Interface. When this register set to high, then the dfi_init_start will change to high after 3 core clock cycle. |
| [3] | reg_ddrc_init_start | R/W | 0x0 | The sdram initialization enable signal. Active High. |
| [2:0] | reg_ddrc_mem_choose | R/W | 0x0 | Choose the memory type that the controller needs to support: 000:DDR2 010:DDR3 100:DDR4 001:LPDDR2 011:LPDDR3 101:LPDDR4 111:LPDDR5 |

Offset: 0x4

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [31:16] | reg_trcd | R/W | 0xf | tRCD: Indicates the minimum time from activate to read or write command to same bank. tRCD = {reg_trcd[7:1],1'b1}*Tck. Eg: When reg_trcd = 8'h4/8'h5, the tRCD = 5Tck. |
| [15:0] | reg_trp | R/W | 0xf | tRP: Indicates the minimum time from single-bank precharge to activate of same bank. Only can support the even delay. tRP = reg_trp * Tck Note: reg_trp >= 6. Note: The controller can only support the even number of Tck delay, even the reg_tpd is odd. The controller will ignore the bit[0]. |

Offset: 0x8

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [15:0] | reg_tras | R/W | 0x3fff | Used to control the timing parameter of tRAS(MAX): The page need to be close after the tRAS time. tRAS=reg_tras * Tclk |

Offset: 0xC

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [31:16] | reg_trc | R/W | 0xf | tRC - Specifies the minimum time between activates to same bank tRC = reg_trc* Tck Note: reg_trc >=6. |
| [15:0] | reg_tras_min | R/W | 0xf | tRAS(min) - Specifies the minimum time between activate and precharge to the same bank. tRAS(min)= reg_tras_min*Tck |

Offset: 0x10

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [31:16] | reg_trrd_l | R/W | 0xf | DDR4: tRRD_L: This is the minimum time between activates from bank "a" to bank "b" for same bank group. tRRD_L = reg_trrd_l *Tck |
| [15:0] | reg_trrd_s | R/W | 0xf | tRRD_S: This is the minimum time between activates from bank "a" to bank "b" for different bank group. tRRD_S = reg_trrd_s *Tck |

Offset: 0x14

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [31:24] | reg_tfaw | R/W | 0xf | tFAW - valid only when 8 or more banks(or banks x bank groups) are present.In 8-bank design, at most 4 banks must be activated in a rolling window of tFAW cycles. tFAW= reg_tfaw*Tck |
| [23:16] | reg_twr | R/W | 0xf | tWR: Write recovery time. Only can support the even delay. tWR = reg_twr *Tck |
| [15:8] | reg_twtr_s | R/W | 0xf | tWTR_S: Internal write to read command delay for different bank group tWTR_S = reg_twtr_s *Tck |
| [7:0] | reg_twtr_l | R/W | 0xf | tWTR_L: Internal write to read command delay for same bank group tWTR_L = reg_twtr_l *Tck |

Offset: 0x18

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [31:24] | reg_trtw | R/W | 0x0 | The register is used to control the delay between the read command to the write command. |
| [23:16] | reg_trtp | R/W | 0xf | tRTP - Specifies the minimum time from read to precharge of same bank. Only can support the odd delay. tRTP = reg_trtp * Tck |
| [15:8] | reg_tccd_s | R/W | 0xf | For DDR3 mode, this parameter is used to control the timing delay between the Cas to Cas command delay of the same bank. For DDR4 mode, this parameter is used to control the timing delay between the Cas to CAS command delay of the different bank group. tCCD_S = reg_tccd_s *Tck |
| [7:0] | reg_tccd_l | R/W | 0xf | DDR4: tCCD_L: This is the minimum time between two reads or two writes for same bank group. tCCD_L = reg_tccd_l *Tck |

Offset: 0x1C

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [31:24] | reg_ddrc_tmrd | R/W | 0x0 | tMRD- Indicates the number of cycles to wait after a mode register write or read tMRD = reg_drrc_tmrd *Tclk |
| [23:16] | reg_ddrc_tmod | R/W | 0x0 | tMOD - Parameter used only in DDR3 and DDR4.Indicates the number of cycles between load mode command and following non-load mode command tMOD = reg_ddrc_tmod *Tclk |
| [15:8] | reg_ddrc_tmrw | R/W | 0x0 | tMRW - Parameter used only in DDR3 and DDR4.Indicates the number of cycles between load mode command and following load mode command tMRW = reg_ddrc_tmrw *Tclk |
| [7:0] | reg_ddrc_tmrr | R/W | 0x0 | tMRR - Parameter used only in LPDDR4.Indicates the number of cycles between load mode read command and following valid command tMRR = reg_ddrc_tmrr *Tclk |

Offset: 0x20

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [31:24] | reg_ddrc_tmrr2mrw | R/W | 0x0 | tMRR2MRW: This is the minimus time the mode register read to mode register write operation. tMRR2MRW = reg_ddrc_tmrr2mrw * Tck. |
| [22:13] | reg_ddrc_trfc_min | R/W | 0x11f | Used to control the timing parameter of tRFC: The minium timer form refresh to other commands. tRFC= reg_ddrc_t_rfc_min *Tck |
| [12:0] | reg_ddrc_trefi_x32 | R/W | 0x7f | Used to control the timing parameter of tREFI: The max timer between the auto refresh. In the idle conditional, the controller will generate the Auto-Refresh command according to the tREFI, but when the row is open at the same time, the controller needs to wait to close that pgae for tRAS_MIN which is controlled by the reg_tras_min. So when you configure this register, the user needs to provide some lead. eg: For 800Mbps of DDR3, when the tREFI is 7.8us. The reg_ddrc_t_refx_x32 = ((7800/2.5)>>1)/32-reg_tras_min. tREFI= reg_ddrc_t_refi_x32 *32*Tcore_clk |

Offset: 0x24

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [28:24] | reg_ddrc_tmrr2mrw | R/W | 0x5 | The register is used to control the timing parameter tPD which means the Power Down Entry to Exit Timing. tPD = reg_tpd *Tclk. |
| [20:16] | reg_ddrc_trfc_min | R/W | 0x5 | This register is used to control the timing parameter tCKE which means the minimum pulse width of the CKE. tCKE = reg_tcke * tCK. Note: The controller can only support the even number of Tck delay, even the reg_tpd is odd. The controller will ignore the bit[0]. |
| [11:0] | reg_ddrc_trefi_x32 | R/W | 0x0 | The register related to the DDR4/DDR3 initialization. Used to control the timing parameter tXPR which means the gap when the dfi_cke changes to high to the following command. tXPR = reg_ddrc_txpr*Tclk |

Offset: 0x28

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [31:24] | reg_tccd_mw | R/W | 0x20 | The register is used to control the timing parameter tCCDMW timing parameter when the Controller Choose the LPDDR4 mode. This is the minimum time from write or masked write to masked write command for same bank. tCCDMW = reg_tccd_mw*Tclk |
| [23:16] | reg_tppd | R/W | 0x4 | The register is used to control the timing of the tPPD timing parameter when the Controller choose the LPDDR4 mode. This is the minimum time from precharge to precharge command. tPPD = reg_tppd * Tclk |
| [15:8] | reg_ddrc_tdllk | R/W | 0x0 | The register related to the initialization. Used to control the timing parameter tDLLK which means the timing between the last load mode command to the DES command after the ZQCL. tDLLK = reg_ddrc_tdllk * Tclk. |
| [7:0] | reg_ddrc_trp | R/W | 0x1 | This register is used to control the timing parameter tRP which means the timing between the precharge command to the active command for the same bank. |

Offset: 0x2C

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [27:24] | reg_tckesr | R/W | 0x5 | This register is used to control the timing parameters of tCKESR which means the minimum pulse of the CKE between the self-refrsh entry to self-refresh exit. tCKESR = reg_tckesr * Tck. Note: The controller can only support the even number of Tck delay, even the reg_tpd is odd. The controller will ignore the bit[0]. |
| [19:16] | reg_ddrc_tescke | R/W | 0x0 | The register is used to control the delay from self refresh command to CKE input low. Delay Unit = tCK. |
| [15:8] | reg_ddrc_trpa | R/W | 0x0 | Reserved. |
| [7:0] | reg_ddrc_txp | R/W | 0x8 | The register is used to control the gap between the powern exit command to next valid command. Delay Unit = tCK. |

Offset: 0x30

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [29:16] | reg_ddrc_txs | R/W | 0x200 | The register is used to control the gap between the sref-refresh exit command to next valid command. Delay Unit = tCK. Note: The inno ddr controller will ignore the next valid command needs the DLL locked or not. The user should choose the max( txs,txsdll,txs_ABOUT,txs_fast). |
| [13:0] | reg_ddrc_tzqoper | R/W | 0x200 | Normal operation Full calibration time. After the self-refresh operation, the controller will send the zacalib operation and wait for reg_ddrc_tzqoper to allower to generate the following load mode command when reg_ddrc_zqcal_after_sref_en = 1'b1. |

Offset: 0x34

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [31:16] | reg_derated_trcd | R/W | 0xf | tRCD: Indicates the minimum time from activate to read or write command to same bank for lpddr4/5 derated mode. tRCD = {reg_derated_trcd[7:1],1'b1}*Tck. Eg: When reg_derated_trcd = 8'h4/8'h5, the tRCD = 5Tck. |
| [15:0] | reg_derated_trp | R/W | 0xf | tRP: Indicates the minimum time from single-bank precharge to activate of same bank. Only can support the even delay for lpddr4/5 derated mode. tRP = reg_derated_trp * Tck Note: reg_derated_trp >= 6. Note: The controller can only support the even number of Tck delay, even the reg_tpd is odd. The controller will ignore the bit[0]. |

Offset: 0x38

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [31:16] | reg_derated_trc | R/W | 0xf | tRC - Specifies the minimum time between activates to same bank for lpddr4/5 derated mode. tRC = reg_derated_trc* Tck Note: reg_trc >=6. |
| [15:0] | reg_derated_tras_min | R/W | 0xf | tRAS(min) - Specifies the minimum time between activate and precharge to the same bank for lpddr4/5 derated mode. tRAS(min)= reg_derated_tras_min*Tck |

Offset: 0x3C

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [31:16] | reg_derated_trrd_l | R/W | 0xf | DDR4/LPDDR5: tRRD_L: This is the minimum time between activates from bank "a" to bank "b" for same bank group for lpddr4/5 derated mode. tRRD_L = reg_derated_trrd_l *Tck |
| [15:0] | reg_derated_trrd_s | R/W | 0xf | tRRD_S: This is the minimum time between activates from bank "a" to bank "b" for different bank group for lpddr4/5 derated mode. tRRD_S = reg_derated_trrd_s *Tck |

Offset: 0x40

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [31:24] | reg_ddrc_init_treset_l_x1024 | R/W | 0x0 | The register is related to the sdram initialization. It is used to control the timing that the dfi_resetn should keep low when the controller is in sdram initialization state. For DDR3, it should keep at least 200us. The delay = reg_ddrc_treset_l_x1024 * 1024 * Tclk. |
| [23:11] | reg_ddrc_init_treset_h_x1024 | R/W | 0x0 | The register is related to the sdram initialization. It is used to control the timing that the dfi_resetn changes to high to the dfi_cke chagne to high when the controller is in sdram initialization state. For DDR3, it should keep at least 500us. The delay = reg_ddrc_treset_h_x1024 * 1024 * Tclk. |
| [10:0] | reg_ddrc_init_tzqinit | R/W | 0x0 | The register related to the initialization. Used to control the timing parameter tzqinit which means the timing between the ZQCL to the following command. tZQINIT = reg_ddrc_tzqinit * Tclk. |

Offset: 0x44

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [15:8] | reg_tinit1_100ns | R/W | 0x0 | Used to contol the timing parameter of tinit1 100ns for lpddr23. |

Offset: 0x48

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [15:8] | reg_ddrc_tsr | R/W | 0xf | The reigster used to control the timing paramter tSR: Minimum self refresh time (entry to exit). Only Used for LPDDR4. Please set to 0 for other sdram types. |
| [7:0] | reg_ddrc_txsr | R/W | 0xf | The reigster used to control the timing paramter tXSR: .Self refresh exit to next valid command delay. Only Used for LPDDR4. Please set to 0 for other sdram types. |

Offset: 0x4C

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [16] | reg_ddrc_zqcal_after_sref_en | R/W | 0x1 | The enable signal to generate the zqcali operation after the exit self-refresh operation. Active High. |
| [15:8] | reg_slm_cmd_idle_cnt_x32_ppd | R/W | 0x40 | The idle numbers that will triger the controller to generate the precharge power down command when reg_srm_dis_auto_ppd = 1'b0. The idle_numbers = reg_srm_cmd_idle_cnt_x32_pd * 32. |
| [7:0] | reg_slm_cmd_idle_cnt_x32_sref | R/W | 0x40 | The idle numbers that will triger the controller to generate the self-refresh command when reg_srm_dis_auto_sref = 1'b0. The idle_numbers = reg_srm_cmd_idle_cnt_x32_sref * 32. |

Offset: 0x50

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [25] | reg_ddrc_lpddrn_enter_ppd_at_self_refresh | R/W | 0x1 | The enable signal to make the controller go into the power down mode after enter the self-refresh mode automatically during the LPDDR4 and LPDDR5 mode. Active High. |
| [24] | reg_ddrc_lpddrn_exit_ppd_at_self_refresh | R/W | 0x1 | The enable signal to make the controller exit into the power down mode when prepare to exit the self-refresh mode automatically during the LPDDR4 and LPDDR5 mode. Active High. |
| [23:16] | reg_ddrc_sref2dfilp_gap | R/W | 0x8 | The register is used to control the gap between the sref-refresh enter command on the sdram pad to active the dfi_lp_req. This register valid only when the reg_ddrc_dfilp_en = 1'b1. Delay Unit = tCK. |
| [15:8] | reg_ddrc_dfilp2sref_gap | R/W | 0x8 | The register is used to control the gap between the low dfi_lp_ack to sref-refresh exit command on the sdram pad. This register valid only when the reg_ddrc_dfilp_en = 1'b1. Delay Unit = tCK. |
| [7] | reg_dfi_lp_state_clean | R/W | 0x0 | The error staus( reg_dfi_lp_entry_handshake_fail and reg_dfi_lp_exit_handshake_timeout) clean signal. Active high. When this bit set to high, the error staus will be clean to zero then to lock the new staus of the dfi low power. |
| [6] | reg_ddrc_dfilp_en | R/W | 0x1 | The enable singal of the dfi low power function. When enable this function, the controller will use the dfi low power interface to make the PHY go into the low power state after it generates the self-refresh command. 1: Enable the dfi low power function when the controller goes into the low power flow. 0: Disable the dfi low power function. |
| [5] | reg_lpddr4_self_ppd_en | R/W | 0x0 | The enable singal of the self refresh power down function. |
| [4] | reg_slm_sref_ppd_req | R/W | 0x0 | The request signal to use the controller to make the SDRAM go into the self-refresh power down state. |
| [3] | reg_slm_dis_auto_ppd | R/W | 0x1 | The disable signal of the auto generate precharge power down function. Active high. 0: Enable auto generate precharge power down function. After enable this function, the controller will monitor the idle state of the core, when the idle numbers arrive the number setting by the reg_srm_cmd_idle_cnt_x32_pd, the controller will generate the precharge power downcommand automatically. 1: Disable auto generate precharge power down function. |
| [2] | reg_slm_dis_auto_sref | R/W | 0x1 | The disable signal of the auto generate self-refresh function. Active high. 0: Enable auto generate self-refresh function. After enable this function, the controller will monitor the idle state of the core, when the idle numbers arrive the number setting by the reg_srm_cmd_idle_cnt_x32_sref, the controller will generate the self-refresh command automatically. 1: Disable auto generate self-refresh function. |
| [1] | reg_slm_ppd_req | R/W | 0x0 | The request signal to use the controller to make the SDRAM go into the precharge power down state. Posedge is valid. When the controller detects this signal changes from low to high, it will stall the command go into the core of the controller from the slave interface and wait for all the command in the queue have been executed safety, then it will generate the power-down command and send to the SDRAM. |
| [0] | reg_slm_sref_req | R/W | 0x0 | The request signal to use the controller to make the SDRAM go into the self-refresh state. Posedge is valid. When the controller detects this signal changes from low to high, it will stall the command go into the core of the controller from the slave interface and wait for all the command in the queue have been executed safety, then it will generate the slef-refresh command and send to the SDRAM. |

Offset: 0x54

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [31:24] | reg_ddrc_dfilp2pd_gap | R/W | 0x8 | The register is used to control the gap between the low dfi_lp_ack to pd-refresh exit command on the sdram pad. This register valid only when the reg_ddrc_dfilp_en = 1'b1. Delay Unit = tCK. |
| [23:16] | reg_ddrc_pd2dfilp_gap | R/W | 0x8 | The register is used to control the gap between the power down enter command on the sdram pad to active the dfi_lp_req. This register valid only when the reg_ddrc_dfilp_en = 1'b1. Delay Unit = tCK. |
| [15:8] | reg_dfi_lp_entry_wait_time | R/W | 0x9 | The register is used to set the waiting time of the assert of the dfi_lp_ack after the controller assert the dfi_lp_req. The delay unit is core_clk. When the controller can't wait for the assert of the dfi_lp_ack, the reg_dfi_lp_entry_handshake signal will change to high and keep to 1'b1 until to the next dfi low power operation. |
| [7:4] | reg_dfi_lp_wakeup_sref | R/W | 0x0 | The register is used to conrol the dfi_lp_wakeup value of the dfi low power interface when the controller is deal with the low power request. When the controller goes into the low power of the self-refresh state and the reg_ddrc_dfilp_en = 1'b1, the controller will also use the dfi low power interface to make the phy goes into different low power state through the dfi_lp_wakeup. And the dfi_lp_wakeup will output the value setting by this register when assert the dfi_lp_req. |
| [3:0] | reg_dfi_lp_wakeup_pd | R/W | 0x0 | The register is used to conrol the dfi_lp_wakeup value of the dfi low power interface when the controller is deal with the low power request. When the controller goes into the low power of the precharge power down state and the reg_ddrc_dfilp_en = 1'b1, the controller will also use the dfi low power interface to make the phy goes into different low power state through the dfi_lp_wakeup. And the dfi_lp_wakeup will output the value setting by this register when assert the dfi_lp_req. |

Offset: 0x58

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [31:28] | reg_ddrc_wr_gap_of_diff_rank | R/W | 0xf | When the controller is in mutl-rank configurations, this register is used to control the number of clocks of gap when performing consecutive writes to different banks. |
| [27:24] | reg_ddrc_rd_gap_of_diff_rank | R/W | 0xf | When the controller is in mutl-rank configurations, this register is used to control the number of clocks of gap when performing consecutive reads to different banks. |
| [23:16] | reg_two_rank_auto_ref_diff_r1 | R/W | 0x1f | The auto refresh counter start value of rank1 will decrease this value as the start value of the auto refresh counter. It will have the influence to the auto refresh interval of the two rank. The ¦reg_two_rank_auto_ref_diff_r1 - reg_two_rank_auto_ref_diff_r0¦ will control the interval of the auto refresh operation between the two ranks. This value shoule be smaller then the reg_ddrc_t_refi_x32. |
| [15:8] | reg_two_rank_auto_ref_diff_r0 | R/W | 0x0 | The auto refresh counter start value of rank0 will decrease this value as the start value of the auto refresh counter. It will have the influence to the auto refresh interval of the two rank. The ¦reg_two_rank_auto_ref_diff_r1 - reg_two_rank_auto_ref_diff_r0¦ will control the interval of the auto refresh operation between the two ranks. This value shoule be smaller then the reg_ddrc_t_refi_x32. |
| [7:0] | reg_ddrc_rd2wr_of_diff_rank | R/W | 0x7 | The register is used to control the timing gap of the read to write operation between two ranks. For LPDDR3: reg_ddrc_rd2wr_of_diff_rank >= (RL+4-WL)/2+1 |

Offset: 0x5C, 0x60, 0x64, 0x68, 0x6C, 0x70, 0x74, 0x78

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [25:0] | reg_user_load_mode_value_mN_r0 (N=0…7) | R/W | 0x0 | This register is used to control the N+1th load mode register and load mode value of rank 0 when enable the user load mode function. (N= 0…7) The bit[24] is used to means the valid of this load mode value. For DDR3 mode, the bit[15:14] is used to control the load mode address and bit[13:0] is used to control the load mode value. For DDR4 mode, the bit[20:18] is used to control the load mode address and bit[17:0] is used to control the load mode value. Note: The minimum gap between the two load mode command of same rank is 6*Tck when enable the user load mode function. So the register reg_ddrc_tmrd can be valid only when reg_ddrc_tmrd > 6Tck. |

Offset: 0x7C, 0x80, 0x84, 0x88, 0x8C, 0x90, 0x94, 0x98

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [25:0] | reg_user_load_mode_value_mN_r1 (N=0…7) | R/W | 0x0 | This register is used to control the N+1th load mode register and load mode value of rank 1 when enable the user load mode function. (N= 0…7) The bit[24] is used to means the valid of this load mode value. For DDR3 mode, the bit[15:14] is used to control the load mode address and bit[13:0] is used to control the load mode value. For DDR4 mode, the bit[20:18] is used to control the load mode address and bit[17:0] is used to control the load mode value. Note: The minimum gap between the two load mode command of same rank is 6*Tck when enable the user load mode function. So the register reg_ddrc_tmrd can be valid only when reg_ddrc_tmrd > 6Tck. |

Offset: 0x9C

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [4] | reg_user_load_mode_en | R/W | 0x0 | When this bit change from low to high, the controller will trigger the user load mode function and generate the load mode command. The load mode address and value is controlled using the register reg_user_load_mode_value(reg0x24[23:0]). |
| [3] | reg_init_lpddr4_mrr0_mrw1 | R/W | 0x0 | Reserved |
| [2] | reg_init_lpddr4_mrr_en | R/W | 0x0 | Reserved |
| [1] | reg_init_lpddr4_mrw_1_en | R/W | 0x0 | Reserved |
| [0] | reg_init_lpddr4_mrw_2_en | R/W | 0x0 | Reserved |

Offset: 0xA0

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [25] | reg_ddrc_ecc_en | R/W | 0x0 | The enable signal of the Sideband ECC function. Active High. 1: Enable the Sideband ECC function. 0: Disable the Sideband ECC function. |
| [24] | reg_ddrc_ecc_dis_scrub | R/W | 0x0 | The disable signal of the scrub funtion when the reg_ddrc_ecc_en set to 1'b1. Active High. 1: When the IMC detected the corrected error, it will only send the corrected data to the master. 0: When the IMC detected the corrected error, it will not only send the corrected data to the master but also use the RMW to correct the data in the SDRAM. |
| [19] | reg_ddrc_ecc_uncorr_err_cnt_clr | R/W | 0x0 | When this register set to 1, it will clear the reg_ddrc_ecc_uncorr_err_cnt, reg_ddrc_ecc_uncorr_bit_num and reg_ddrc_ecc_uncorr_err. |
| [18] | reg_ddrc_ecc_corr_err_cnt_clr | R/W | 0x0 | When this register set to 1, it will clear the reg_ddrc_ecc_corr_err_cnt, reg_ddrc_ecc_corr_bit_num, rand reg_ddrc_ecc_corr_err. |
| [17] | reg_ddrc_ecc_uncorr_err_info_clr | R/W | 0x0 | When this register set to 1, it will clean the reg_ddrc_ecc_uncorr_err_rank, reg_ddrc_ecc_uncorr_err_bank,reg_ddrc_ecc_uncorr_err_row, reg_ddrc_ecc_uncorr_exp_data, reg_ddrc_ecc_uncorr_act_data and reg_ddrc_ecc_uncorr_err_col. |
| [16] | reg_ddrc_ecc_corr_err_info_clr | R/W | 0x0 | When this register set to 1, it will clean the reg_ddrc_ecc_corr_err_rank, reg_ddrc_ecc_corr_err_bank,reg_ddrc_ecc_corr_err_row, reg_ddrc_ecc_corr_exp_data, reg_ddrc_ecc_corr_act_data and reg_ddrc_ecc_corr_err_col. |
| [14] | reg_ddrc_ecc_poison_en | R/W | 0x0 | Enable the data poison mode. Active High. 0: Disable the data poison mode. 1: Enable the data poison mode. |
| [13] | reg_ddrc_ecc_poison_data_mode | R/W | 0x0 | Select the data poison mode when enable the data poison mode of the ECC function. 0: Poision 1 bits which can be correctable. 1: Poision 2 bits which is uncorrectable. |
| [12:8] | reg_ddrc_ecc_poison_bit_num | R/W | 0x0 | Select the bit number when enable the data poison mode of the ECC function. When the reg_ddrc_ecc_poison_data_mode set to 1, the next error bit number is reg_ddrc_ecc_poison_bit_num + 1. |
| [6:5] | reg_ddrc_ecc_poison_sdram_beat | R/W | 0x0 | Select the sdram beat when enable the data poison mode of the ECC function. |
| [4] | reg_ddrc_ecc_poison_rank | R/W | 0x0 | The rank number of the read address which chooses for the ECC poison. |
| [3:0] | reg_ddrc_ecc_poison_bank | R/W | 0x0 | The bank number of the read address which chooses for the ECC poison. When the IMC chooses DDR3 mode, the bank[2:0] means the bank number. When the IMC chooses DDR4 mode, the bank[1:0] means the bank number, the bank[3:2] means the bank group number. |

Offset: 0xA4

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [28:12] | reg_ddrc_ecc_poison_row | R/W | 0x0 | The row number of the read address which chooses for the ECC poison. |
| [11:0] | reg_ddrc_ecc_poison_col | R/W | 0x0 | The col number of the read address which chooses for the ECC poison. |

Offset: 0xA8

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [3:1] | reg_cal_efficiency_number_cnt | R/W | 0x0 | This register is used to controler the ddr controller efficiency calculate times.eg. When this register is set to 7, it will calculate 7 times for the clock cycle defined by the reg_ddrc_efficiency_cal_cnt. |
| [0] | reg_ddrc_efficiency_cal_en | R/W | 0x0 | The ddr controller efficiency calculate enable signal. When this bit set to 1, it will start to calculate the controller efficency in the clock cycle defined by the reg_ddrc_effieiency_cal_ cnt. |

Offset: 0xAC

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [30:0] | reg_ddrc_efficiency_cal_cnt | R/W | 0x7fffffff | This register is used to control the ddr controller efficiency calculate cycle. |

Offset: 0xB0, 0xB4, 0xB8, 0xBC, 0xC0, 0xC4, 0xC8, 0xCC

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [29:20] | reg_wr_priorityN (N=0…7) | R/W | 0x123 | The initial load value of write aging counters for axi slave portN (N=0…7). The value will be load after the system reset or after write operation grant to the axi slave port N (N=0…7). The aging coutners decreas every clock cycle when the port N (N=0…7) has write request but not granted. The port's priority increases when the aging counter starts to decrease. When the aging counter becomes 0, the corresponding port has the highest priority level. For signle port, the aging counters are only used when the timout change to 0 to force read-write direction switching. |
| [17] | reg_wr_port_aging_enN (N=0…7) | R/W | 0x1 | The enable signal of aging function for the write opdration of axi slave port N (N=0…7). Active high. |
| [16] | reg_wr_port_urgent_enN (N=0…7) | R/W | 0x0 | Reserved. |
| [13:4] | reg_rd_port_priorityN (N=0…7) | R/W | 0x123 | The initial load value of read aging counters for axi slave port N(N=0…7). The value will be load after the system reset or after write operation grant to the axi slave port N (N=0…7). The aging coutners decreas every clock cycle when the port N (N=0…7) has write request but not granted. The port's priority increases when the aging counter starts to decrease. When the aging counter becomes 0, the corresponding port has the highest priority level. For signle port, the aging counters are only used when the timout change to 0 to force read-write direction switching. |
| [1] | reg_rd_port_aging_enN (N=0…7) | R/W | 0x1 | The enable signal of aging function for the read operation of axi slave port N (N=0…7). Active high. |
| [0] | reg_rd_port_urgent_enN (N=0…7) | R/W | 0x0 | Reserved. |

Offset: 0xD0

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [2] | reg_go2critical_en | R/W | 0x0 | Reserved. |
| [1] | reg_pagematch_limit | R/W | 0x1 | The enable signal of the pagamatch limit function. 1: The arbiter will keep to delay with current port for 4 request if the following 4 request is pagematch request. 0: Disable this function. |
| [0] | reg_prefer_write | R/W | 0x1 | If the register set to high, when the write and read request has the same priority, the write has high priroty. |

Offset: 0xD4

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [31:24] | reg_wr_starve_time | R/W | 0x1f | Used to control the number of core clocks that the write request changes to the starve which will have highest priority. The minimum valid value is 0x1. When the value set to 0x0, it will disable the starvation functions. |
| [23:16] | reg_rd_starve_time | R/W | 0x1f | Used to control the number of core clocks that the read request changes to the starve which will have highest priority. The minimum valid value is 0x1. When the value set to 0x0, it will disable the starvation functions. |
| [15:8] | reg_rd2wr_switch_threhold | R/W | 0x1a | Indicates the max transaction that the write operation can be dealed with after the grant to the write request from the starvation functions. |
| [7:0] | reg_wr2rd_switch_threhold | R/W | 0x5 | Indicates the max transaction that the read operation can be dealed with after the grant to the read request from the starvation functions. |

Offset: 0xD8

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [21:16] | reg_ddrc_wodt_delay | R/W | 0x2 | The register used to control the delay between the write command and odt. So it will decide the start point of the ODT. The delay unit is tCK. The min supoort value is 2. |
| [12:8] | reg_ddrc_wodt_length | R/W | 0x6 | The register used to control the legnth of the write odt. Length unit: tCK. The min support value is 2. Eg. When this register set to 4'h6, the the write odt will keep 6 tCK. This value only can be set to the even number. |
| [2] | reg_ddrc_wodt_disable | R/W | 0x0 | The wodt enable signal. Active Low. 0: The dfi_odt will generate based on the reg_ddrc_wodt_length and reg_ddrc_wodt_delay at the write operation. 1: The dfi_odt will output the 4'h0 and keep unchanged. The reg_ddrc_wodt_always_open will have higer priority. |
| [1] | reg_ddrc_wodt_always_open | R/W | 0x0 | The enable signal to set the dfi_odt to 4'hf and keep unchaged. Active High. 1: The dfi_odt will always output 4'hf to make the ODT keep high. 0: The dfi_odt is controllered by other signals. |
| [0] | reg_ddrc_wodt_st_bp | R/W | 0x0 | The bypass enable signal to control the delay between the write command to the dfi odt. 1: Choose to use the reg_ddrc_wodt_delay to control. 0: According to use the CWL to control the delay automatically. |

Offset: 0xDC

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [28:24] | reg_al_bp | R/W | 0x0 | The bypass al value when the reg_cwl_al_cl_bp = 1'b1. |
| [21:16] | reg_cwl_bp | R/W | 0x8 | The bypass cwl value when the reg_cwl_al_cl_bp = 1'b1. |
| [13:8] | reg_cl_bp | R/W | 0x7 | The bypass cl value when the reg_cwl_al_cl_bp = 1'b1. |
| [0] | reg_cwl_al_cl_bp | R/W | 0x0 | The CWL/AL/CL parameter configure byass enable signal. Active. 1: Using the reg_al_bp/reg_cwl_bp/reg_cl_bp to control the value of the CWL/ACL/CL in the controller which will control the dfi interface timing. 0: The controller will automatically update the CWL/AL/CL value from the load mode operation. |

Offset: 0xE0

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [28:24] | reg_dfi_tphy_wrlat | R/W | 0x0 | The register is used to control the timing parameters tPHY_WRLAT when the reg_dfi_tphy_wralt_bp set to 1'b1. |
| [23:16] | reg_ddrc_tctrl_delay | R/W | 0x6 | It is the delay of the command from the dfi interface to command pad of the PHY that the controller will connect. It is the timing parameters of the PHY that the controller should know. The users need to check the timing parameters(tctrl_delay) of the PHY and configure the same value to this register. |
| [12:8] | reg_dfi_trdata_en | R/W | 0x0 | The register is used to control the delaythe timing parameters tPHY_RDDATA_EN when the reg_dfi_trdata_en_bp set to 1'b1. |
| [1] | reg_dfi_tphy_wrlat_bp | R/W | 0x0 | Enable the bypass mode to control the timing parameters tPHY_WRLAT which means the delay between the dfi write command to dfi_wrdata/dfi_wrdata_en. Active High. 0: The controller will use the CWL/AL/WL to control the timing parameters tPHY_WRLAT. 1: The controller will use the register reg_dfi_tphy_wrlat to control the timing parameters tPHY_WRLAT. |
| [0] | reg_dfi_trdata_en_bp | R/W | 0x0 | Enable the bypass mode to control the timing parameters tPHY_RDDATA_EN which means the delay between the dfi read command to dfi_rddata_en. Active High. 0: The controller will use the AL/CL/RL to control the timing parameters tPHY_RDDATA_EN. 1: The controller will use the reg_dfi_tphy_wrlat to control the delay between the dfi read command to dfi_rddata_en. |

Offset: 0xE4

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [21:16] | reg_hp_rqueue_max_dep | R/W | 0x0 | Reserved. |
| [13:8] | reg_lp_rqueue_max_dep | R/W | 0x20 | This register is used to control the read command queue depth which used for read command reorder. The max depth is 32. When enable the ECC function, this register should set to 'd31. When disable the ECC function, this register should set to 'd32. |
| [5:0] | reg_wqueue_max_dep | R/W | 0x20 | This register is used to control the write command queue depth which used for write command reorder. The max depth is 32. When enable the ECC function, this register should set to 'd31. When disable the ECC function, this register should set to 'd32. |

Offset: 0xE8 ~ 0x1D4

These are MEM BIST related registers and reserved for internal use.

Offset: 0x1D8

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [14] | reg_sdram_rank_size | R/W | 0x1 | Choose the size of the rank that the Controller needs to support. 0x0: Support 1 RANK. 0x1: Support 2 RANKs. The user needs to accord to the system request to choose the rank number that the controller needs to support. |
| [11:9] | reg_sdram_bank_size | R/W | 0x4 | Choose the size of the bank. 3'h3: Support 8 banks. 3'h4: Support 16 banks. The user needs to accord the sdram capacity to choose the bank size. |
| [8:4] | reg_sdram_row_size | R/W | 0xf | Choose the size of the row. 4'hf: Choose the row bits width is 15. 4'he: Choose the row bits widths is 14. The user needs to accord the sdram capacity to choose the row size. |
| [3:0] | reg_sdram_col_size | R/W | 0xa | Choose the size of the column. 4'ha: Choose the row bits width is 10. 4'h9: Choose the col bits width is 9. The user needs to accord the sdram capacity to choose the col size. |

Offset: 0x1DC

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [29:24] | reg_ddrc_col_offset_bit3 | R/W | 0x3 | This register is used to control the map between the col[3] and DDR_ADDR. col[3] = DDR_ADDR[reg_ddrc_col_offset_bit3] |
| [21:16] | reg_ddrc_col_offset_bit2 | R/W | 0x2 | This register is used to control the map between the col[2] and DDR_ADDR. col[2] = DDR_ADDR[reg_ddrc_col_offset_bit2] |
| [13:8] | reg_ddrc_col_offset_bit1 | R/W | 0x1 | This register is used to control the map between the col[1] and DDR_ADDR. col[1] = DDR_ADDR[reg_ddrc_col_offset_bit1] |
| [5:0] | reg_ddrc_col_offset_bit0 | R/W | 0x0 | This register is used to control the map between the col[0] and DDR_ADDR. col[0] = DDR_ADDR[reg_ddrc_col_offset_bit0] |

Offset: 0x1E0

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [29:24] | reg_ddrc_col_offset_bit7 | R/W | 0x7 | This register is used to control the map between the col[7] and DDR_ADDR. col[7] = DDR_ADDR[reg_ddrc_col_offset_bit7] |
| [21:16] | reg_ddrc_col_offset_bit6 | R/W | 0x6 | This register is used to control the map between the col[6] and DDR_ADDR. col[6] = DDR_ADDR[reg_ddrc_col_offset_bit6] |
| [13:8] | reg_ddrc_col_offset_bit5 | R/W | 0x5 | This register is used to control the map between the col[5] and DDR_ADDR. col[5] = DDR_ADDR[reg_ddrc_col_offset_bit5] |
| [5:0] | reg_ddrc_col_offset_bit4 | R/W | 0x4 | This register is used to control the map between the col[4] and DDR_ADDR. col[4] = DDR_ADDR[reg_ddrc_col_offset_bit4] |

Offset: 0x1E4

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [29:24] | reg_ddrc_col_offset_bit11 | R/W | 0x0 | This register is used to control the map between the col[11] and DDR_ADDR. col[11] = DDR_ADDR[reg_ddrc_col_offset_bit11] |
| [21:16] | reg_ddrc_col_offset_bit10 | R/W | 0x0 | This register is used to control the map between the col[10] and DDR_ADDR. col[10] = DDR_ADDR[reg_ddrc_col_offset_bit10] |
| [13:8] | reg_ddrc_col_offset_bit9 | R/W | 0x9 | This register is used to control the map between the col[9] and DDR_ADDR. col[9] = DDR_ADDR[reg_ddrc_col_offset_bit9] |
| [5:0] | reg_ddrc_col_offset_bit8 | R/W | 0x8 | This register is used to control the map between the col[8] and DDR_ADDR. col[8] = DDR_ADDR[reg_ddrc_col_offset_bit8] |

Offset: 0x1E8

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [29:24] | reg_ddrc_row_offset_bit3 | R/W | 0xd | This register is used to control the map between the row[3] and DDR_ADDR. row[3] = DDR_ADDR[reg_ddrc_row_offset_bit3] |
| [21:16] | reg_ddrc_row_offset_bit2 | R/W | 0xc | This register is used to control the map between the row[2] and DDR_ADDR. row[2] = DDR_ADDR[reg_ddrc_row_offset_bit2] |
| [13:8] | reg_ddrc_row_offset_bit1 | R/W | 0xb | This register is used to control the map between the row[1] and DDR_ADDR. row[1] = DDR_ADDR[reg_ddrc_row_offset_bit1] |
| [5:0] | reg_ddrc_row_offset_bit0 | R/W | 0xa | This register is used to control the map between the row[0] and DDR_ADDR. row[0] = DDR_ADDR[reg_ddrc_row_offset_bit0] |

Offset: 0x1EC

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [29:24] | reg_ddrc_row_offset_bit7 | R/W | 0x11 | This register is used to control the map between the row[7] and DDR_ADDR. row[7] = DDR_ADDR[reg_ddrc_row_offset_bit7] |
| [21:16] | reg_ddrc_row_offset_bit6 | R/W | 0x10 | This register is used to control the map between the row[6] and DDR_ADDR. row[6] = DDR_ADDR[reg_ddrc_row_offset_bit6] |
| [13:8] | reg_ddrc_row_offset_bit5 | R/W | 0xf | This register is used to control the map between the row[5] and DDR_ADDR. row[5] = DDR_ADDR[reg_ddrc_row_offset_bit5] |
| [5:0] | reg_ddrc_row_offset_bit4 | R/W | 0xe | This register is used to control the map between the row[4] and DDR_ADDR. row[4] = DDR_ADDR[reg_ddrc_row_offset_bit4] |

Offset: 0x1F0

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [29:24] | reg_ddrc_row_offset_bit11 | R/W | 0x15 | This register is used to control the map between the row[11] and DDR_ADDR. row[11] = DDR_ADDR[reg_ddrc_row_offset_bit11] |
| [21:16] | reg_ddrc_row_offset_bit10 | R/W | 0x14 | This register is used to control the map between the row[10] and DDR_ADDR. row[10] = DDR_ADDR[reg_ddrc_row_offset_bit10] |
| [13:8] | reg_ddrc_row_offset_bit9 | R/W | 0x13 | This register is used to control the map between the row[9] and DDR_ADDR. row[9] = DDR_ADDR[reg_ddrc_row_offset_bit9] |
| [5:0] | reg_ddrc_row_offset_bit8 | R/W | 0x12 | This register is used to control the map between the row[8] and DDR_ADDR. row[8] = DDR_ADDR[reg_ddrc_row_offset_bit8] |

Offset: 0x1F4

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [29:24] | reg_ddrc_row_offset_bit15 | R/W | 0x19 | This register is used to control the map between the row[15] and DDR_ADDR. row[15] = DDR_ADDR[reg_ddrc_row_offset_bit15] |
| [21:16] | reg_ddrc_row_offset_bit14 | R/W | 0x18 | This register is used to control the map between the row[14] and DDR_ADDR. row[14] = DDR_ADDR[reg_ddrc_row_offset_bit14] |
| [13:8] | reg_ddrc_row_offset_bit13 | R/W | 0x17 | This register is used to control the map between the row[13] and DDR_ADDR. row[13] = DDR_ADDR[reg_ddrc_row_offset_bit13] |
| [5:0] | reg_ddrc_row_offset_bit12 | R/W | 0x16 | This register is used to control the map between the row[12] and DDR_ADDR. row[12] = DDR_ADDR[reg_ddrc_row_offset_bit12] |

Offset: 0x1F8

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [13:8] | reg_ddrc_row_offset_bit17 | R/W | 0x0 | This register is used to control the map between the row[17] and DDR_ADDR. row[17] = DDR_ADDR[reg_ddrc_row_offset_bit17] |
| [5:0] | reg_ddrc_row_offset_bit16 | R/W | 0x0 | This register is used to control the map between the row[16] and DDR_ADDR. row[16] = DDR_ADDR[reg_ddrc_row_offset_bit16] |

Offset: 0x1FC

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [29:24] | reg_ddrc_ba_offset_bit3 | R/W | 0x0 | This register is used to control the map between the bank[3] and DDR_ADDR. bank[3] = DDR_ADDR[reg_ddrc_bank_offset_bit3] For DDR4, This register is used to control the map between the bg[1] and DDR_ADDR. bg[1] = DDR_ADDR[reg_ddrc_bank_offset_bit3] |
| [21:16] | reg_ddrc_ba_offset_bit2 | R/W | 0x1c | This register is used to control the map between the bank[2] and DDR_ADDR. bank[2] = DDR_ADDR[reg_ddrc_bank_offset_bit2] For DDR4, This register is used to control the map between the bg[0] and DDR_ADDR. bg[0] = DDR_ADDR[reg_ddrc_bank_offset_bit2] |
| [13:8] | reg_ddrc_ba_offset_bit1 | R/W | 0x1b | This register is used to control the map between the bank[1] and DDR_ADDR. bank[1] = DDR_ADDR[reg_ddrc_bank_offset_bit1] |
| [5:0] | reg_ddrc_ba_offset_bit0 | R/W | 0x1a | This register is used to control the map between the bank[0] and DDR_ADDR. bank[0] = DDR_ADDR[reg_ddrc_bank_offset_bit0] |

Offset: 0x200

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [13:8] | reg_ddrc_rank_offset_bit1 | R/W | 0x1e | This register is used to control the map between the rank[1] and DDR_ADDR. rank[1] = DDR_ADDR[reg_ddrc_rank_offset_bit1] |
| [5:0] | reg_ddrc_rank_offset_bit0 | R/W | 0x1d | This register is used to control the map between the rank[0] and DDR_ADDR. rank[0] = DDR_ADDR[reg_ddrc_rank_offset_bit0] |

Offset: 0x204

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [0] | reg_ddrc_addrmap_hash_en | R/W | 0x0 | Reserved. |

Offset: 0x208

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [5:2] | reg_ddrc_valid_rank_num | R/W | 0x1 | Reserved. |
| [1:0] | reg_ddrc_valid_rank_channel | R/W | 0x1 | Used to control the valid rank channel. 2'b01: When the controller needs to support only 1 rank. 2'b11: When the controller needs to support 2 ranks. |

Offset: 0x20C

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [17] | reg_ddr4_rdbi_en | R/W | 0x0 | The read dbi enable signal of the DDR4. Active High. When the user enable the RDBI function of the DDR4 SDRAM by setting the MR5[12] to 1'b1, this bit should set to 1'b1. |
| [16] | reg_wdbi_en | R/W | 0x0 | The write DBI enable signal of the controller of the DDR4 or LPDDR4. 0:Dsiable the write DBI function. 1:Enable the write DBI function of the DDR4. When choose the DDR4 mode and enable the DBI mode: 1) The data of each byte and the corresponding dm signal will equal or more than 5 bits "1" 2) Can't support the data mask function directly, the controller will use the RMW operation to complete the data mask operation. |
| [15:8] | reg_pageclose_timer | R/W | 0x0 | When the reg_pageclose_en = 1'b1 && reg_pageclose_timer == 8'h0, the controller generates the auto-precharge write or auto precharge read command at the last page open write or read operation in the command queue to close that open bank. When the reg_pageclse_en = 1'b1 && reg_pageclse_timer > 8'h0, the controller will wait for the delay setting by the reg_pageclose_timer after to last page open write or read operation in the command queue to generate the precharge command to close that open bank. When the delay setting is smaller than the (reg_twtp/2) or (reg_trtp/2), the controller will accord to the reg_twtp and reg_trtp to generate the precharge to close the open bank. Delay Unit: Tcore_clk Note: When the reg_pageclose_timer != 8'h0 and disable the auto precharge function, the reg_page_close_timer > (reg_tccd/2), or it will influence the efficiency of the SDRAM. |
| [0] | reg_pageclose_en | R/W | 0x1 | When the reg_pageclose_en = 1'b1 && reg_pageclose_timer == 8'h0, the controller generates the auto-precharge write or auto precharge read command at the last page open write or read operation in the command queue to close that open bank. When the reg_pageclse_en = 1'b1 && reg_pageclse_timer > 8'h0, the controller will wait for the delay setting by the reg_pageclose_timer after to last page open write or read operation in the command queue to generate the precharge command to close that open bank. When the delay setting is smaller than the (reg_twtp/2) or (reg_trtp/2), the controller will accord to the reg_twtp and reg_trtp to generate the precharge to close the open bank. Delay Unit: Tcore_clk Note: When the reg_pageclose_timer != 8'h0 and disable the auto precharge function, the reg_page_close_timer > (reg_tccd/2), or it will influence the efficiency of the SDRAM. |

Offset: 0x210

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [18:16] | reg_ddrc_auto_ref_burst | R/W | 0x3 | Used to control the burst number of the auto refresh when the reg_ddrc_auto_ref_mode = 1'b1. |
| [15:8] | reg_ddrc_auto_ref_pre_margin | R/W | 0x1f | Reserved. |
| [2] | reg_ddrc_auto_ref_dis | R/W | 0x0 | The disable signal of the auto refresh. Active High. 1 Disable the auto refresh function, then the controller will stop to send the auto refresh. 0: Enable the auto frefresh function, then the controller will auto generate the auto refresh based on the sdram specification. |
| [1] | reg_ddrc_auto_ref_mode | R/W | 0x1 | Choose the auto refresh mode. 1: Gnerate the auto refresh each tREFI time interval. 0: Enable the burst mode. The controler will accord to the setting of the reg_ddrc_auto_ref_burst to keep the auto-refresh command, when the number of the auto-refresh number equals to the reg_ddrc_auto_ref_burst, it will use the burst to send the auto-refresh successively. |
| [0] | reg_auto_ref_gen | R/W | 0x0 | The generate signal of the auto refresh. Posedge will be valid. |

Offset: 0x214

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [31:16] | reg_lpddrn_ddrn_load_mode00 | R/W | 0x0 | The register related to the lpddrn/ddrn initialization. This register is used to control the value which will be written to the MR of LPDDRn/DDRn at sdram initialization state. DDR4 corresponding to MR3 ; |
| [15:0] | reg_lpddrn_ddrn_load_mode01 | R/W | 0x0 | The register related to the lpddrn/ddrn initialization. This register is used to control the value which will be written to the MR of LPDDRn/DDRn at sdram initialization state. DDR4 corresponding to MR6 |

Offset: 0x218

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [31:16] | reg_lpddrn_ddrn_load_mode02 | R/W | 0x0 | The register related to the lpddrn/ddrn initialization. This register is used to control the value which will be written to the MR of LPDDRn/DDRn at sdram initialization state. DDR4 corresponding to MR5 ; |
| [15:0] | reg_lpddrn_ddrn_load_mode03 | R/W | 0x0 | The register related to the lpddrn/ddrn initialization. This register is used to control the value which will be written to the MR of LPDDRn/DDRn at sdram initialization state. DDR4 corresponding to MR4 |

Offset: 0x21C

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [31:16] | reg_lpddrn_ddrn_load_mode04 | R/W | 0x0 | The register related to the lpddrn/ddrn initialization. This register is used to control the value which will be written to the MR of LPDDRn/DDRn at sdram initialization state. DDR4 corresponding to MR2 ; |
| [15:0] | reg_lpddrn_ddrn_load_mode05 | R/W | 0x0 | The register related to the lpddrn/ddrn initialization. This register is used to control the value which will be written to the MR of LPDDRn/DDRn at sdram initialization state. DDR4 corresponding to MR1 |

Offset: 0x220

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [31:16] | reg_lpddrn_ddrn_load_mode06 | R/W | 0x0 | The register related to the lpddrn/ddrn initialization. This register is used to control the value which will be written to the MR of LPDDRn/DDRn at sdram initialization state. DDR4 corresponding to MR0 ; |
| [15:0] | reg_lpddrn_ddrn_load_mode07 | R/W | 0x0 | Reserved. |

Offset: 0x224 ~ 0x2C0

These registeres holds the register settings for LPDDR4. These registers are reserved for DDR4.

Offset: 0x2C4

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [30] | reg_dfi_lp_entry_handshake_fail | RO | - | The flag that the entry of the dfi low power interface is fail. When this signal change to high, it means that the controller can't wait the dfi_lp_ack change to high after the core_clk cycles setting by the reg_dfi_lp_entry_wait_time when the dfi_lp_req is active. There are two conditions: 1) The phy can't support the dfi low power interface. 2) The handshake can't meet the timing of the DFI specification. The status of this register can be clean when reg_dfi_lp_state_clean set to high. |
| [29] | reg_dfi_lp_exit_handshake_timeout | RO | - | The flag that the exit of the dfi low power interface is timeout. When this signal change to high, it means that the controller can't wait the dfi_lp_ack change to low after the core_clk cycles setting by the reg_dfi_lp_wakeup_sref in self-refresh low power state or the reg_dfi_lp_wakeup_pd in power down low power state when the dfi_lp_req is dessert. There are two conditions: 1) The phy can't support the dfi low power interface. 2) The handshake can't meet the timing of the DFI specification. The status of this register can be clean when reg_dfi_lp_state_clean set to high. |
| [2] | ddrc_init_act | RO | - | The sdram initilization state signal. 1: The sdram is in initialization state. 0: The sdram compeltes that initialization. |
| [1] | sdram_init_done_to_reg | RO | - | The done signal of the sdram initialization after set the reg_ddrc_init_start to high. Active high. When this bit changes to high after set the reg_ddrc_init_start to high, it means that the controller completes the initialization of the SDRAM. |
| [0] | phy_init_complete_to_reg | RO | - | The done signal of the phy initialization after ther reg_phy_init_start to high. Active high. When this bit changes to high after the set the reg_phy_init_start to high, it means that the controller compeltes the initialization of the SDRAM. |

Offset: 0x2C8 ~ 0x34C

These are MEM BIST related registers and reserved for internal use.

Offset: 0x350 ~ 0x414

These are read-only registers for debug purpose. Reserved for internal use.

4 Program Guide

- Call ddr_init() to complete DDR initialization.

- Call ddr_test() for DDR read and write testing.

Chapter 18 LISoC_APB_MIPI_RX

1 Function list

MIPI_RX module is used to receive input MIPI CSI-2 video data in preparation for further processing inside the LISOC.

2 Register list

Base address: 0x4005 0000

| Offset | Name | Access | Reset | Width | Description |
| --- | --- | --- | --- | --- | --- |
| 0x00 | REG_0x000 | RW | 0x00 | 32 |  |
| 0x0 4 | REG_0x004 | RW | 0x02 | 32 |  |
| 0x0 c | REG_0x00c | RW | 0x02 | 32 |  |
| 0x 1 0 | REG_0x010 | RW | 0x00 | 32 |  |
| 0x14 | REG_0x014 | RW | 0x00 | 32 |  |
| 0x18 | REG_0x018 | RW | 0x00 | 32 |  |
| 0x1c | REG_0x01c | RW | 0x00 | 32 |  |
| 0x 2 0 | REG_0x020 | RW | 0x00 | 32 |  |
| 0x24 | REG_0x024 | RW | 0x00 | 32 |  |
| 0x28 | REG_0x028 | RW | 0x00 | 32 |  |
| 0x34 | REG_0x034 | RW | 0x08 | 32 |  |
| 0x 6 0 | REG_0x060 | RW | 0x00 | 32 |  |
| 0x88 | REG_0x088 | RW | 0x00 | 32 |  |
| 0x a8 | REG_0x0a8 | RW | 0x00 | 32 |  |
| 0xac | REG_0x0ac | RW | 0x00 | 32 |  |
| 0x118 | REG_0x118 | RW | 0x01000000 | 32 |  |
| 0x120 | REG_0x120 | RW | 0x3210 | 32 |  |
| 0x164 | REG_0x164 | RW | 0xa0000 | 32 |  |
| 0x184 | REG_0x184 | RW | 0x00 | 32 |  |
| 0x188 | REG_0x188 | RO | 0x00 | 32 |  |
| 0x18c | REG_0x18c | RO | 0x00 | 32 |  |
| 0x 19 0 | REG_0x190 | RO | 0x00 | 32 |  |
| 0x 2a 0 | REG_0x2a0 | RW | 0x00 | 32 |  |
| 0x 2a4 | REG_0x2a4 | RW | 0x00 | 32 |  |
| 0xd0c | REG_0xd0c | RO | 0x00 | 32 |  |
| 0xd4c | REG_0xd4c | RO | 0x00 | 32 |  |
| 0xd54 | REG_0xd54 | RW | 0x00 | 32 |  |
| 0xf00 | REG_0xf00 | RW | 0x00 | 32 |  |
| 0xfc0 | REG_0xfc0 | RW | 0x00 | 32 |  |

3 Detailed definition of registers

REG_0x000

- Size：32bits

- Address offset：0x00

- Read/Write access：read/write

- Reset Value: 0x0

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [8] | LINK_P0_RST_N | RW | 0x00 | LINK port0 RESET 0:Reset 1:Active |
| [0] | PHYA_RX_PDATA_RESET_N | RW | 0x00 | PHY Logic Circuit RESET 0:Reset 1:Active |

REG_0x004

- Size：32bits

- Address offset：0x04

- Read/Write access：read/write

- Reset Value: 0x2

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [2:0] | MODE_P0_OPERATION | RW | 0x02 | Operation Mode Control Link Port0 0x0: Not Use 0x1: Not Use 0x2: D-PHY 0x3: subLVDS 0x4: CMOS 0x5-0x7: Not Use |

REG_0x00c

- Size：32bits

- Address offset：0x0c

- Read/Write access：read/write

- Reset Value: 0x2

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [2:0] | PHYA_RX_MODE | RW | 0x02 | Operation Mode Control Link of PHY 0x0: Not Use 0x1: Not Use 0x2: D-PHY 0x3: subLVDS 0x4: CMOS 0x5-0x7: Not Use |

REG_0x010

- Size：32bits

- Address offset：0x10

- Read/Write access：read/write

- Reset Value: 0x0

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [1:0] | MODE_P0_PIX_PARA | RW | 0x00 | Link Port0: the number of parallel pixel-data output to Application Layer 0: 1 Pixel Out 1: 2 Pixel Out 2: 4 Pixel Out 3: 8 Pixel Out |

REG_0x014

- Size：32bits

- Address offset：0x14

- Read/Write access：read/write

- Reset Value: 0x0

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [0] | PHYA_RX_IREF_POW_EN | RW | 0x00 | PHY Analog Enable 0: PowerDown 1: Active |

REG_0x018

- Size：32bits

- Address offset：0x18

- Read/Write access：read/write

- Reset Value: 0x0

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [16] | PHYA_RX_C0_LP_AMP_POW_EN | RW | 0x00 | PHY Clock Port0 LowPower Amp Power Enable 0: PowerDown 1: Active |
| [0] | PHYA_RX_C0_HS_AMP_POW_EN | RW | 0x00 | PHY Clock Port0 HighSpeed Amp Power Enable 0: PowerDown 1: Active |

REG_0x01c

- Size：32bits

- Address offset：0x1c

- Read/Write access：read/write

- Reset Value: 0x0

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [3] | PHYA_RX_D3_LP_AMP_POW_EN | RW | 0x00 | PHY DataLane3 LowPower Amp Power Enable 0:PowerDown 1:Active |
| [2] | PHYA_RX_D2_LP_AMP_POW_EN | RW | 0x00 | PHY DataLane2 LowPower Amp Power Enable 0:PowerDown 1:Active |
| [1] | PHYA_RX_D1_LP_AMP_POW_EN | RW | 0x00 | PHY DataLane1 LowPower Amp Power Enable 0:PowerDown 1:Active |
| [0] | PHYA_RX_D0_LP_AMP_POW_EN | RW | 0x00 | PHY DataLane0 LowPower Amp Power Enable 0:PowerDown 1:Active |

REG_0x020

- Size：32bits

- Address offset：0x20

- Read/Write access：read/write

- Reset Value: 0x0

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [3] | PHYA_RX_D3_HS_AMP_POW_EN | RW | 0x00 | PHY DataLane3 HighSpeed Amp Power Enable 0:PowerDown 1:Active |
| [2] | PHYA_RX_D2_HS_AMP_POW_EN | RW | 0x00 | PHY DataLane2 HighSpeed Amp Power Enable 0:PowerDown 1:Active |
| [1] | PHYA_RX_D1_HS_AMP_POW_EN | RW | 0x00 | PHY DataLane1 HighSpeed Amp Power Enable 0:PowerDown 1:Active |
| [0] | PHYA_RX_D0_HS_AMP_POW_EN | RW | 0x00 | PHY DataLane0 HighSpeed Amp Power Enable 0:PowerDown 1:Active |

REG_0x024

- Size：32bits

- Address offset：0x24

- Read/Write access：read/write

- Reset Value: 0x0

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [0] | PHYA_RX_C0_RES_EN | RW | 0x00 | PHY Clock Port0 Termination Resistance Enable 0:OFF 1:ON |

REG_0x028

- Size：32bits

- Address offset：0x28

- Read/Write access：read/write

- Reset Value: 0x0

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [3] | PHYA_RX_D3_RES_EN | RW | 0x00 | PHY DataLane3 Termination Resistance Enable 0:OFF 1:ON |
| [2] | PHYA_RX_D2_RES_EN | RW | 0x00 | PHY DataLane2 Termination Resistance Enable 0:OFF 1:ON |
| [1] | PHYA_RX_D1_RES_EN | RW | 0x00 | PHY DataLane1 Termination Resistance Enable 0:OFF 1:ON |
| [0] | PHYA_RX_D0_RES_EN | RW | 0x00 | PHY DataLane0 Termination Resistance Enable 0:OFF 1:ON |

REG_0x034

- Size：32bits

- Address offset：0x34

- Read/Write access：read/write

- Reset Value: 0x8

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [3:0] | PHYA_RX_RESCTL | RW | 0x08 | PHY Termination Resistance Setting 0x0: OFF 0x1: max value ... 0x7: 97ohm (recommend setting for subLVDS mode) 0x8: 98ohm (recommend setting for DPHY mode) 0xA: min value 0xB-0xF: setting prohibited |

REG_0x060

- Size：32bits

- Address offset：0x60

- Read/Write access：read/write

- Reset Value: 0x0

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [2:0] | PHYA_RX_BUS_MODE | RW | 0x00 | PHY de-serializer setting 0x0: 8bit 0x1-0x7: setting prohibited |

REG_0x088

- Size：32bits

- Address offset：0x88

- Read/Write access：read/write

- Reset Value: 0x0

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [4] | PHYA_RX_IRMSEL | RW | 0x00 | PHY Bias Current Circult Selectable 0: BGR Circult 1: Resistance Split |
| [0] | PHYA_RX_VRMSEL | RW | 0x00 | PHY Bias Voltage Circult Selectable 0: BGR Circult 1: Resistance Split |

REG_0x0a8

- Size：32bits

- Address offset：0xa8

- Read/Write access：read/write

- Reset Value: 0x0

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [21:20] | PHYA_RX_LP_HL | RW | 0x00 | PHY LPRX rise side Threshold Adjustment |
| [17:16] | PHYA_RX_LP_LH | RW | 0x00 | PHY LPRX fall side Threshold Adjustment |
| [9:8] | PHYA_RX_OFFSET | RW | 0x00 | PHY Differential Hi-Z input offset |

REG_0x0ac

- Size：32bits

- Address offset：0xac

- Read/Write access：read/write

- Reset Value: 0x0

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [23:20] | MODE_P0_LBL_FIDMAP_SEL_3 | RW | 0x00 | FID3 Bit Position in SYNC CODE 4th Port0 |
| [19:16] | MODE_P0_LBL_FIDMAP_SEL_2 | RW | 0x00 | FID2 Bit Position in SYNC CODE 4th Port0 |
| [15:12] | MODE_P0_LBL_FIDMAP_SEL_1 | RW | 0x00 | FID1 Bit Position in SYNC CODE 4th Port0 |
| [11:8] | MODE_P0_LBL_FIDMAP_SEL_0 | RW | 0x00 | FID0 Bit Position in SYNC CODE 4th Port0 |
| [7:4] | MODE_P0_LBL_FIDMAP_SEL_EN | RW | 0x00 | FID Output Select/Enable Port0 Bit [0]: control p0_fid_out[0] 0: MIPI-CSI2 VC[0] 1: Output Signal selected by MODE_P0_LBL_FIDMAP_SEL_0 Bit [1]: control p0_fid_out[1] 0: MIPI-CSI2 VC[1] 1: Output Signal selected by MODE_P0_LBL_FIDMAP_SEL_1 Bit [2]: control p0_fid_out[2] 0: MIPI-CSI2 VC[2] 1: Output Signal selected by MODE_P0_LBL_FIDMAP_SEL_2 Bit [3]: control p0_fid_out[3] 0: MIPI-CSI2 VC[3] 1: Output Signal selected by MODE_P0_LBL_FIDMAP_SEL_3 |
| [1:0] | MODE_P0_LBL | RW | 0x00 | Line by Line HDR/WDR Mode Enable Port0 0: Normal Operation (Disable HDR/WDR) 1: FID embeded on SYNC CODE , or VC CODE 2: FID embeded on Pixel Data 3: Not Use |

REG_0x118

- Size：32bits

- Address offset：0x118

- Read/Write access：read/write

- Reset Value: 0x1000000

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [28] | DPHY_P0_MODE_LPOP_OFF | RW | 0x00 | D-PHY LP Operation Mode Port0 0: LP Operation ON 1: LP Operation OFF |
| [24] | DPHY_P0_MODE_MSB_FIRST | RW | 0x01 | D-PHY Port 0 Reverse Parallel Data alignment (bit order) from PHY analog 0: Reverse 1: Not reverse |
| [20] | DPHY_P0_MODE_HSDM_OFF | RW | 0x00 | D-PHY HS_Data Masking Port0 0: HS_Data masked to "0" when HS_EN = "0" 1: HS_Data not masked. |
| [16] | DPHY_P0_LP_SYNC_OFF | RW | 0x00 | D-PHY LP Syncroizer Port0 0: LP Syncronizer ON 1: LP Syncronizer OFF |
| [2:0] | DPHY_P0_MODE_LANENUM | RW | 0x00 | D-PHY the Number of Data Lane Port0 0: 1 Lane Mode 1: 2 Lane Mode 2: 3 Lane Mode 3: 4 Lane Mode 4: setting forbidden 5: setting forbidden 6: setting forbidden 7: 8 Lane Mode |

REG_0x120

- Size：32bits

- Address offset：0x120

- Read/Write access：read/write

- Reset Value: 0x3210

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [14:12] | DPHY_P0_PREG_DLM_3 | RW | 0x03 | D-PHY Data Lane Mapping for Link Port0 0：Input Data Lane 0 is connected to Internal Lane 3 1：Input Data Lane 1 is connected to Internal Lane 3 2：Input Data Lane 2 is connected to Internal Lane 3 3：Input Data Lane 3 is connected to Internal Lane 3 4：Input Data Lane 4 is connected to Internal Lane 3 5：Input Data Lane 5 is connected to Internal Lane 3 6：Input Data Lane 6 is connected to Internal Lane 3 7：Input Data Lane 7 is connected to Internal Lane 3 |
| [10:8] | DPHY_P0_PREG_DLM_2 | RW | 0x02 | D-PHY Data Lane Mapping for Link Port0 0：Input Data Lane 0 is connected to Internal Lane 2 1：Input Data Lane 1 is connected to Internal Lane 2 2：Input Data Lane 2 is connected to Internal Lane 2 3：Input Data Lane 3 is connected to Internal Lane 2 4：Input Data Lane 4 is connected to Internal Lane 2 5：Input Data Lane 5 is connected to Internal Lane 2 6：Input Data Lane 6 is connected to Internal Lane 2 7：Input Data Lane 7 is connected to Internal Lane 2 |
| [6:4] | DPHY_P0_PREG_DLM_1 | RW | 0x01 | D-PHY Data Lane Mapping for Link Port0 0：Input Data Lane 0 is connected to Internal Lane 1 1：Input Data Lane 1 is connected to Internal Lane 1 2：Input Data Lane 2 is connected to Internal Lane 1 3：Input Data Lane 3 is connected to Internal Lane 1 4：Input Data Lane 4 is connected to Internal Lane 1 5：Input Data Lane 5 is connected to Internal Lane 1 6：Input Data Lane 6 is connected to Internal Lane 1 7：Input Data Lane 7 is connected to Internal Lane 1 |
| [2:0] | DPHY_P0_PREG_DLM_0 | RW | 0x00 | D-PHY Data Lane Mapping for Link Port0 0：Input Data Lane 0 is connected to Internal Lane 0 1：Input Data Lane 1 is connected to Internal Lane 0 2：Input Data Lane 2 is connected to Internal Lane 0 3：Input Data Lane 3 is connected to Internal Lane 0 4：Input Data Lane 4 is connected to Internal Lane 0 5：Input Data Lane 5 is connected to Internal Lane 0 6：Input Data Lane 6 is connected to Internal Lane 0 7：Input Data Lane 7 is connected to Internal Lane 0 |

REG_0x164

- Size：32bits

- Address offset：0x164

- Read/Write access：read/write

- Reset Value: 0xa0000

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [31:16] | DPHY_P0_TIME_RES_ON | RW | 0x0a | D-PHY adjustment value of output timing of HS_EN Port0 |
| [15:0] | DPHY_P0_TIME_HS_ON | RW | 0x00 | D-PHY adjustment value of output timing of TE_xx_EN Port0 0xFFFF : TE_XX_EN is fixed to "0"(DISABLE) except 0xFFFF ： TE_xx_EN is automatically controlled |

REG_0x184

- Size：32bits

- Address offset：0x184

- Read/Write access：read only

- Reset Value: 0x00

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [2:0] | CSI2_P0_STATE | RO | 0x00 | CSI-2 Payload Status Port0 0: HS Code Detect Wait 1: Packet Header Analisys 2: Calcuration Line Length 3: Burst Transmit 4: Packet Done |

REG_0x188

- Size：32bits

- Address offset：0x188

- Read/Write access：read only

- Reset Value: 0x00

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [31:24] | CSI2_P0_ECC | RO | 0x00 | CSI-2 Packet ECC Value Port0 |
| [23:8] | CSI2_P0_WC | RO | 0x00 | CSI-2 Packet Word Count Port0 |
| [7:0] | CSI2_P0_DATA_ID | RO | 0x00 | CSI-2 Packet Data ID Port0 |

REG_0x18c

- Size：32bits

- Address offset：0x18c

- Read/Write access：read only

- Reset Value: 0x00

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [31:16] | CSI2_P0_PKT_COUNT | RO | 0x00 | CSI-2 Packet Recive Count Port0 |
| [15:0] | CSI2_P0_LINE_LENGTH | RO | 0x00 | CSI-2 Packet Line Length Port0 |

REG_0x190

- Size：32bits

- Address offset：0x190

- Read/Write access：read only

- Reset Value: 0x00

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [31:16] | CSI2_P0_ERR_CS | RO | 0x00 | CSI-2 Packet Checksum Error Count Port0 |
| [15:0] | CSI2_P0_ERR_ECC | RO | 0x00 | CSI-2 Packet ECC Error Count Port0 |

REG_0x2a0

- Size：32bits

- Address offset：0x2a0

- Read/Write access：read/write

- Reset Value: 0x00

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [1] | APP_P0_CHK_VENA | RW | 0x00 | V-Size check enable Port 0 0: Disable 1: Enable |
| [0] | APP_P0_CHK_HENA | RW | 0x00 | H-size check enable Port 0 0: Disable 1: Enable |

REG_0x2a4

- Size：32bits

- Address offset：0x2a4

- Read/Write access：read/write

- Reset Value: 0x00

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [31:16] | APP_P0_CHK_VSIZE | RW | 0x00 | Number of V-size for V-size check Port 0 |
| [15:0] | APP_P0_CHK_HSIZE | RW | 0x00 | Number of H-size for H-size check Port 0 |

REG_0xd0c

- Size：32bits

- Address offset：0xd0c

- Read/Write access：read only

- Reset Value: 0x00

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [31:24] | DPHY_P0_ERR_DET_SC_3 | RO | 0x00 | DPHY Sync code error detect Port0 |
| [23:16] | DPHY_P0_ERR_DET_SC_2 | RO | 0x00 | DPHY Sync code error detect Port0 |
| [15:8] | DPHY_P0_ERR_DET_SC_1 | RO | 0x00 | DPHY Sync code error detect Port0 |
| [7:0] | DPHY_P0_ERR_DET_SC_0 | RO | 0x00 | DPHY Sync code error detect Port0 |

REG_0xd4c

- Size：32bits

- Address offset：0xd4c

- Read/Write access：read only

- Reset Value: 0x00

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [7:0] | DPHY_P0_ERR_DET_SC_AL | RO | 0x00 | DPHY Sync code error detect Port0 |

REG_0xd54

- Size：32bits

- Address offset：0xd54

- Read/Write access：read/write

- Reset Value: 0x00

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [23:0] | CSI2_P0_DATA_TYPE_USER | RW | 0x00 | CSI2 Data type user define setting port0 |

REG_0xf00

- Size：32bits

- Address offset：0xf00

- Read/Write access：read/write

- Reset Value: 0x00

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [13] | RX_PCLK_DIV_EN | RW | 0x00 | Test Mode Register |
| [10:8] | RX_PDATA_OUT_SEL | RW | 0x00 | Test Mode Register |
| [1:0] | REG_RDA_SEL | RW | 0x00 | TestMode RDA Read data bit select |

REG_0xfc0

- Size：32bits

- Address offset：0xfc0

- Read/Write access：read/write

- Reset Value: 0x00

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [31:0] | USER_CTRL | RW | 0x00 | USER Control Register, Currently not in use. |

4  Program Guide

Chapter 19 LISoC_APB_MIPI_TX

1 Function list

MIPI_TX module is used to output the processed video data in MIPI CSI-2 format.

2 Register list

Base address: 0x4006 0000

| Offset | Name | Access | Reset | Width | Description |
| --- | --- | --- | --- | --- | --- |
| 0x00 | REG_0x000 | RW | 0x00 | 32 |  |
| 0x0 4 | REG_0x004 | RW | 0x00 | 32 |  |
| 0x0 8 | REG_0x008 | RW | 0x00 | 32 |  |
| 0x0c | REG_0x00c | RW | 0x00 | 32 |  |
| 0x 1 0 | REG_0x010 | RW | 0x00 | 32 |  |
| 0x14 | REG_0x014 | RW | 0xF10000F1 | 32 |  |
| 0x18 | REG_0x018 | RW | 0x01 | 32 |  |
| 0x 2 0 | REG_0x020 | RW | 0x00 | 32 |  |
| 0x24 | REG_0x024 | RO | 0x00 | 32 |  |
| 0x30 | REG_0x030 | RW | 0x32100003 | 32 |  |
| 0x34 | REG_0x034 | RW | 0x0f0f0507 | 32 |  |
| 0x38 | REG_0x038 | RW | 0x08080807 | 32 |  |
| 0x3c | REG_0x03c | RW | 0x0909 | 32 |  |
| 0x 4 0 | REG_0x040 | RW | 0x00 | 32 |  |
| 0x44 | REG_0x044 | RW | 0x00 | 32 |  |
| 0x 4c | REG_0x04c | RO | 0x00 | 32 |  |
| 0x54 | REG_0x054 | RW | 0x00 | 32 |  |
| 0xf00 | REG_0xf00 | RW | 0x00 | 32 |  |
| 0xfc0 | REG_0xfc0 | RW | 0x00 | 32 |  |

3 Detailed definition of registers

REG_0x000

- Size：32bits

- Address offset：0x00

- Read/Write access：read/write

- Reset Value: 0x0

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [24] | PHYA_PLL_RESETN | RW | 0x00 | PLL reset (active low) |
| [20] | PHYA_PLL_ANA_EN | RW | 0x00 | Power up control of PLL |
| [16] | PHYA_CNT_RST_N | RW | 0x00 | Count reset (active low) |
| [12] | PHYA_RST_N | RW | 0x00 | Reset (active low) |
| [8] | PHYA_EN | RW | 0x00 | Output enable |
| [4] | PHYA_TX_POW_EN | RW | 0x00 | Power up control of TX |
| [0] | PHYA_REF_POW_EN | RW | 0x00 | Power up control of BGR/BIASgen |

REG_0x004

- Size：32bits

- Address offset：0x00

- Read/Write access：read/write

- Reset Value: 0x0

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [30:24] | PHYA_PLL_K3DIV | RW | 0x00 | Output clock divider |
| [21:20] | PHYA_PLL_K2DIV | RW | 0x00 | Output clock divider |
| [17:16] | PHYA_PLL_K1DIV | RW | 0x00 | Output clock divider |
| [14:8] | PHYA_PLL_N2DIV | RW | 0x00 | Feedback clock divider |
| [5:4] | PHYA_PLL_N1DIV | RW | 0x00 | High speed Feedback clock divider |
| [2:0] | PHYA_PLL_PDIV | RW | 0x00 | Input mreference clock divider |

ss

REG_0x008

- Size：32bits

- Address offset：0x08

- Read/Write access：read/write

- Reset Value: 0x0

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [27:24] | PHYA_PLL_VCO_IGAIN | RW | 0x00 | PLL/BGR test select |
| [20] | PHYA_PLL_OSC_CLMP_DIS | RW | 0x00 | PLL refencence clock through mode |
| [17:16] | PHYA_PLL_V2I_OFFSET | RW | 0x00 | Test mode |
| [13:12] | PHYA_PLL_OSC_SEL | RW | 0x00 | Reference voltage switching |
| [10:8] | PHYA_PLL_KVCO_TRIM | RW | 0x00 | Reference voltage switching |
| [5:0] | PHYA_PLL_CHP_TRIM | RW | 0x00 | Output impedance adjustment signal |

REG_0x00c

- Size：32bits

- Address offset：0x0c

- Read/Write access：read/write

- Reset Value: 0x0

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [24] | PHYA_PLL_LPE_SEL_3ORD | RW | 0x00 | Refer to PHYA specification |
| [20] | PHYA_PLL_LPF_RSEL | RW | 0x00 | Refer to PHYA specification |
| [16] | PHYA_PLL_LPF_CSEL | RW | 0x00 | Refer to PHYA specification |
| [12] | PHYA_CK_EDG | RW | 0x00 | SCO rise/fall edge switching |
| [8] | PHYA_POL_SCK | RW | 0x00 | Serial clock inversion |
| [5] | PHYA_IRM | RW | 0x00 | Reference voltage switching |
| [4] | PHYA_VRM | RW | 0x00 | Reference voltage switching |
| [1] | PHYA_ZDN | RW | 0x00 | Output impedance adjustment signal |
| [0] | PHYA_ZUP | RW | 0x00 | Output impedance adjustment signal |

REG_0x010

- Size：32bits

- Address offset：0x10

- Read/Write access：read/write

- Reset Value: 0x0

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [31] | PHYA_D3_INITDATA | RW | 0x00 | Shift register initial value setting |
| [30] | PHYA_D2_INITDATA | RW | 0x00 | Shift register initial value setting |
| [29] | PHYA_D1_INITDATA | RW | 0x00 | Shift register initial value setting |
| [28] | PHYA_D0_INITDATA | RW | 0x00 | Shift register initial value setting |
| [23] | PHYA_D3_PN | RW | 0x00 | Serial output polarity reversal |
| [22] | PHYA_D2_PN | RW | 0x00 | Serial output polarity reversal |
| [21] | PHYA_D1_PN | RW | 0x00 | Serial output polarity reversal |
| [20] | PHYA_D0_PN | RW | 0x00 | Serial output polarity reversal |
| [16] | PHYA_CK_PN | RW | 0x00 | Serial output polarity reversal |
| [15] | PHYA_D3_PLDN | RW | 0x00 | Serial Output pull-down |
| [14] | PHYA_D2_PLDN | RW | 0x00 | Serial Output pull-down |
| [13] | PHYA_D1_PLDN | RW | 0x00 | Serial Output pull-down |
| [12] | PHYA_D0_PLDN | RW | 0x00 | Serial Output pull-down |
| [8] | PHYA_CK_PLDN | RW | 0x00 | Serial Output pull-down |
| [7] | PHYA_D3_POW_EN | RW | 0x00 | Serial Output power-up |
| [6] | PHYA_D2_POW_EN | RW | 0x00 | Serial Output power-up |
| [5] | PHYA_D1_POW_EN | RW | 0x00 | Serial Output power-up |
| [4] | PHYA_D0_POW_EN | RW | 0x00 | Serial Output power-up |
| [0] | PHYA_CK_POW_EN | RW | 0x00 | Serial Output power-up |

REG_0x014

- Size：32bits

- Address offset：0x14

- Read/Write access：read/write

- Reset Value: 0xF10000F1

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [31] | PHYA_D3_LPTX_EN_RST_N | RW | 0x1 | FF reset for LP-TX enable (active low) |
| [30] | PHYA_D2_LPTX_EN_RST_N | RW | 0x1 | FF reset for LP-TX enable (active low) |
| [29] | PHYA_D1_LPTX_EN_RST_N | RW | 0x1 | FF reset for LP-TX enable (active low) |
| [28] | PHYA_D0_LPTX_EN_RST_N | RW | 0x1 | FF reset for LP-TX enable (active low) |
| [24] | PHYA_CK_LPTX_EN_RST_N | RW | 0x1 | FF reset for LP-TX enable (active low) |
| [23] | PHYA_D3_LPTX_EN_SET_N | RW | 0x0 | FF set for LP-TX enable (active low) |
| [22] | PHYA_D2_LPTX_EN_SET_N | RW | 0x0 | FF set for LP-TX enable (active low) |
| [21] | PHYA_D1_LPTX_EN_SET_N | RW | 0x0 | FF set for LP-TX enable (active low) |
| [20] | PHYA_D0_LPTX_EN_SET_N | RW | 0x0 | FF set for LP-TX enable (active low) |
| [16] | PHYA_CK_LPTX_EN_SET_N | RW | 0x0 | FF set for LP-TX enable (active low) |
| [15] | PHYA_D3_HSTX_EN_RST_N | RW | 0x0 | FF reset for HS-TX enable (active low) |
| [14] | PHYA_D2_HSTX_EN_RST_N | RW | 0x0 | FF reset for HS-TX enable (active low) |
| [13] | PHYA_D1_HSTX_EN_RST_N | RW | 0x0 | FF reset for HS-TX enable (active low) |
| [12] | PHYA_D0_HSTX_EN_RST_N | RW | 0x0 | FF reset for HS-TX enable (active low) |
| [8] | PHYA_CK_HSTX_EN_RST_N | RW | 0x0 | FF reset for HS-TX enable (active low) |
| [7] | PHYA_D3_HSTX_EN_SET_N | RW | 0x1 | FF set for HS-TX enable (active low) |
| [6] | PHYA_D2_HSTX_EN_SET_N | RW | 0x1 | FF set for HS-TX enable (active low) |
| [5] | PHYA_D1_HSTX_EN_SET_N | RW | 0x1 | FF set for HS-TX enable (active low) |
| [4] | PHYA_D0_HSTX_EN_SET_N | RW | 0x1 | FF set for HS-TX enable (active low) |
| [0] | PHYA_CK_HSTX_EN_SET_N | RW | 0x1 | FF reset for HS-TX enable (active low) |

REG_0x018

- Size：32bits

- Address offset：0x18

- Read/Write access：read/write

- Reset Value: 0x01

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [29:28] | PHYA_D3_DLY | RW | 0x0 | Delay time setting |
| [25:24] | PHYA_D2_DLY | RW | 0x0 | Delay time setting |
| [21:20] | PHYA_D1_DLY | RW | 0x0 | Delay time setting |
| [17:16] | PHYA_D0_DLY | RW | 0x0 | Delay time setting |
| [1:0] | PHYA_CK_DLY | RW | 0x1 | Delay time setting |

REG_0x020

- Size：32bits

- Address offset：0x20

- Read/Write access：read/write

- Reset Value: 0x0

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [23] | PHYA_D3_TST_PSI | RW | 0x00 | Serial output HS test input |
| [22] | PHYA_D2_TST_PSI | RW | 0x00 | Serial output HS test input |
| [21] | PHYA_D1_TST_PSI | RW | 0x00 | Serial output HS test input |
| [20] | PHYA_D0_TST_PSI | RW | 0x00 | Serial output HS test input |
| [16] | PHYA_CK_TST_PSI | RW | 0x00 | Serial output HS test input |
| [15] | PHYA_D3_TSTIN_N | RW | 0x00 | Serial output LP test input (N) |
| [14] | PHYA_D3_TSTIN_P | RW | 0x00 | Serial output LP test input (P) |
| [13] | PHYA_D2_TSTIN_N | RW | 0x00 | Serial output LP test input (N) |
| [12] | PHYA_D2_TSTIN_P | RW | 0x00 | Serial output LP test input (P) |
| [11] | PHYA_D1_TSTIN_N | RW | 0x00 | Serial output LP test input (N) |
| [10] | PHYA_D1_TSTIN_P | RW | 0x00 | Serial output LP test input (P) |
| [9] | PHYA_D0_TSTIN_N | RW | 0x00 | Serial output LP test input (N) |
| [8] | PHYA_D0_TSTIN_P | RW | 0x00 | Serial output LP test input (P) |
| [1] | PHYA_CK_TSTIN_N | RW | 0x00 | Serial output LP test input (N) |
| [0] | PHYA_CK_TSTIN_P | RW | 0x00 | Serial output LP test input (P) |

REG_0x024

- Size：32bits

- Address offset：0x24

- Read/Write access：read only

- Reset Value: 0x0

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [15] | PHYA_D3_TST_PSO | RO | 0x00 | Serial output HS test output |
| [14] | PHYA_D2_TST_PSO | RO | 0x00 | Serial output HS test output |
| [13] | PHYA_D1_TST_PSO | RO | 0x00 | Serial output HS test output |
| [12] | PHYA_D0_TST_PSO | RO | 0x00 | Serial output HS test output |
| [8] | PHYA_CK_TST_PSO | RO | 0x00 | Serial output HS test output |
| [7] | PHYA_D3_TST_DLO | RO | 0x00 | Serial output HS test output |
| [6] | PHYA_D2_TST_DLO | RO | 0x00 | Serial output HS test output |
| [5] | PHYA_D1_TST_DLO | RO | 0x00 | Serial output HS test output |
| [4] | PHYA_D0_TST_DLO | RO | 0x00 | Serial output HS test output |
| [0] | PHYA_CK_TST_DLO | RO | 0x00 | Serial output HS test output |

REG_0x030

- Size：32bits

- Address offset：0x30

- Read/Write access：read/write

- Reset Value: 0x32100003

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [29:28] | DPHY_P0_TX_LP11 | RW | 0x3 | LP-11 configuration |
| [25:24] | DPHY_P0_TX_LP10 | RW | 0x2 | LP-10 configuration |
| [21:20] | DPHY_P0_TX_LP01 | RW | 0x1 | LP-01 configuration |
| [17:16] | DPHY_P0_TX_LP00 | RW | 0x0 | LP-01 configuration |
| [12] | CSI2_P0_TX_TC | RW | 0x0 | Clock Lane control in normal mode |
| [8] | CSI2_P0_TX_TS | RW | 0x0 | Packet transmission timing |
| [2:0] | MODE_LANENUM | RW | 0x3 | Output lane number |

REG_0x034

- Size：32bits

- Address offset：0x34

- Read/Write access：read/write

- Reset Value: 0x0f0f0507

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [29:24] | DPHY_P0_TX_TIME_T_CLK_PRE | RW | 0xf | Refer to DPHY_Timing_Adjustment_rxxx.xlsx |
| [22:16] | DPHY_P0_TX_TIME_T_CLK_ZERO | RW | 0xf | Refer to DPHY_Timing_Adjustment_rxxx.xlsx |
| [13:8] | DPHY_P0_TX_TIME_T_CLK_PREPARE | RW | 0x5 | Refer to DPHY_Timing_Adjustment_rxxx.xlsx |
| [5:0] | DPHY_P0_TX_TIME_T_CLK_LPX | RW | 0x7 | Refer to DPHY_Timing_Adjustment_rxxx.xlsx |

REG_0x038

- Size：32bits

- Address offset：0x38

- Read/Write access：read/write

- Reset Value: 0x08080807

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [29:24] | DPHY_P0_TX_TIME_T_HS_EOT | RW | 0x8 | Refer to DPHY_Timing_Adjustment_rxxx.xlsx |
| [21:16] | DPHY_P0_TX_TIME_T_HS_SOT | RW | 0x8 | Refer to DPHY_Timing_Adjustment_rxxx.xlsx |
| [13:8] | DPHY_P0_TX_TIME_T_HS_ZERO | RW | 0x8 | Refer to DPHY_Timing_Adjustment_rxxx.xlsx |
| [5:0] | DPHY_P0_TX_TIME_T_HS_PREPARE | RW | 0x7 | Refer to DPHY_Timing_Adjustment_rxxx.xlsx |

REG_0x03c

- Size：32bits

- Address offset：0x3c

- Read/Write access：read/write

- Reset Value: 0x0909

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [13:8] | DPHY_P0_TX_TIME_T_CLK_EOT | RW | 0x9 | Refer to DPHY_Timing_Adjustment_rxxx.xlsx |
| [5:0] | DPHY_P0_TX_TIME_T_CLK_POST | RW | 0x9 | Refer to DPHY_Timing_Adjustment_rxxx.xlsx |

REG_0x040

- Size：32bits

- Address offset：0x40

- Read/Write access：read/write

- Reset Value: 0x00

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [31:16] | DPHY_P0_TX_DESKEW_LENGTH | RW | 0x0 | Periode skew calibration cycles |
| [0] | DPHY_P0_TX_DESKEW_AUTO | RW | 0x0 | Periode skew calibration mode |

REG_0x044

- Size：32bits

- Address offset：0x44

- Read/Write access：read/write

- Reset Value: 0x00

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [31:16] | CSI2_P0_TX_HD_DLY | RW | 0x0 | Long Packet transmission timiing adjustment (delay) value |
| [15:0] | CSI2_P0_TX_SP_DLY | RW | 0x0 | Short Packet transmission timiing adjustment (delay) value |

REG_0x04c

- Size：32bits

- Address offset：0x4c

- Read/Write access：read only

- Reset Value: 0x00

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [12:8] | DPHY_P0_TX_FSM_PHYD | RO | 0x0 | Indicates state of PHY Digital Layer |
| [7:4] | CSI2_P0_TX_FSM_LINK | RO | 0x0 | Indicates state of Link Layer |
| [3:0] | CSI2_P0_TX_FSM_APP | RO | 0x0 | Indicates state of Application Layer |

REG_0x054

- Size：32bits

- Address offset：0x54

- Read/Write access：read/write

- Reset Value: 0x00

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [23:0] | CSI2_P0_TX_DATA_TYPE_USER | RW | 0x0 | CSI2 Data Type user define setting |

REG_0xf00

- Size：32bits

- Address offset：0xf00

- Read/Write access：read/write

- Reset Value: 0x00

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [26:24] | PHYA_PLL_ANA_MON_SEL | RW | 0x00 | Refer to PHYA specification |
| [16] | PHYA_PLL_ANA_TSTPD_EN | RW | 0x00 | Enable of Phase Detector input clocks from input divider and feedback divider |
| [10:8] | PHYA_TSTTX | RW | 0x00 | Test mode |
| [1:0] | REG_RDA_SEL | RW | 0x00 | RDA read data bit select |

REG_0xfc0

- Size：32bits

- Address offset：0xfc0

- Read/Write access：read/write

- Reset Value: 0x00

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [31:0] | USER_CTRL | RW | 0x00 | USER Control Register. |

4 Program Guide

Chapter 20 LISoC_APB_CCHG

1 Function list

CCHG module is used to adapt input video path clock rate to subsequent on-chip IPs (ISP and LISr).

2 Register list

Base address：0x400A 0000

| Offset | Name | Access | Reset | Width | Description |
| --- | --- | --- | --- | --- | --- |
| 0x00 | SFTRST | WO | 0x00 | 32 | Software Reset |
| 0x04 | HV_SZ | R/W | 0x08CA1130 | 32 | Horizontal- Vertical Size |
| 0x08 | HV_VALID_SZ | R/W | 0x08820F18 | 32 | Horizontal- Vertical Valid Size |
| 0x0c | W_OUT | R/W | 0x00 | 32 | Write Output |
| 0x10 | BANK_CYC | R/W | 0x64 | 32 | Bank Cycle |

3 Detailed definition of registers

SFTRST

- Name: Software Reset

- Size：32bits

- Addr：0x400A 0000

- Read/Write access：write only

- Reset Value: 0x0

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [31:1] | Reserved | RO | 0x00 | Reserved |
| [0] | SFTRST | WO | 0x00 | Software reset to CCHG module. Reading has no effect Writing 1 resets the CCHG module Writing 0 releases the reset to CCHG module. |

HV_SZ

- Name: Horizontal- Vertical Size

- Size：32bits

- Addr：0x400A 0004

- Read/Write access：read/write

- Reset Value: 0x08CA1130

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [27:16] | V_SIZE | R/W | 0x08CA | Vertical Size |
| [13:0] | H_SIZE | R/W | 0x1130 | Horizontal size |

HV_VALID_SZ

- Name: Horizontal- Vertical Valid Size

- Size：32bits

- Addr：0x400A 0008

- Read/Write access：read/write

- Reset Value: 0x08CA1130

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [27:16] | V_VALID_SIZE | R/W | 0x0882 | Vertical Size |
| [13:0] | H_VALID_SIZE | R/W | 0x0F18 | Horizontal size |

W_OUT

- Name: Write Output

- Size：32bits

- Addr：0x400A 000C

- Read/Write access：read/write

- Reset Value: 0x00

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [31:1] | Reserved | RO | 0x00 | Reserved. Read as zero. |
| [0] | W_OUT | R/W | 0x00 | Write Output. Write 1 to clear. |

BANK_CYC

- Name: Bank Cycle

- Size：32bits

- Addr：0x400A 0010

- Read/Write access：read/write

- Reset Value: 0x00

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [31:11] | Reserved | RO | 0x00 | Reserved. Read as zero. |
| [10:0] | BANK_CYCLE | R/W | 0x64 | Bank Cycle. |

4 Program Guide

Chapter 21 LISoC_APB_ISP

1 Function list

LISoC_APB_ISP module is used to do advanced Image Signal Processing such as Black Level Correction, Gain and white balance control, YC separation, etc.

2 Register list

Base address: 0x4007 0000

| Offset | Name | Access | Reset | Width | Description |
| --- | --- | --- | --- | --- | --- |
| 0x00 | REG_0x00 | R/W | 0x08800F18 | 32 |  |
| 0x04 | REG_0x04 | R/W | 0x00 | 32 |  |
| 0x08 | REG_0x08 | R/W | 0x00 | 32 |  |
| 0x0c | REG_0x0c | R/W | 0x40 | 32 |  |
| 0x10 | REG_0x10 | R/W | 0x059904cc | 32 |  |
| 0x14 | REG_0x14 | R/W | 0x00 | 32 |  |
| 0x18 | REG_0x18 | R/W | 0x40400000 | 32 |  |
| 0x1c | REG_0x1c | R/W | 0x40000040 | 32 |  |
| 0x20 | REG_0x20 | R/W | 0x40 | 32 |  |
| 0x24 | REG_0x24 | R/W | 0x80 | 32 |  |
| 0x28 | REG_0x28 | R/W | 0x1E280064 | 32 |  |
| 0x2c | REG_0x2c | R/W | 0x0301 | 32 |  |
| 0x30 | REG_0x30 | R/W | 0x05160808 | 32 |  |
| 0x34 | REG_0x34 | R/W | 0x08CA0898 | 32 |  |
| 0x38 | REG_0x38 | R/W | 0x08700780 | 32 |  |
| 0x3c | REG_0x3c | R/W | 0x00290060 | 32 |  |

3 Detailed definition of registers

REG_0x00

- Size：32bits

- Addr：0x4007 0000

- Read/Write access：read/write

- Reset Value: 0x08800F18

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [31:28] | NUM_R1 | RO | 0x00 |  |
| [27:16] | NUM_ROW | R/W | 0x880 |  |
| [15:13] | NUM_R0 | RO | 0x00 |  |
| [12:0] | NUM_COL | R/W | 0xF18 |  |

REG_0x04

- Size：32bits

- Addr：0x4007 0004

- Read/Write access：read/write

- Reset Value: 0x00

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [31:18] | BAYER_R2 | RO | 0x00 |  |
| [17] | BIT_MODE | RW | 0x00 |  |
| [16] | THROUGH_MODE | RW | 0x00 |  |
| [15:2] | BAYER_R1 | RO | 0x00 |  |
| [1:0] | BAYER_PATTERN | RW | 0x00 |  |

REG_0x08

- Size：32bits

- Addr：0x4007 0008

- Read/Write access：read/write

- Reset Value: 0x00

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [31:24] | OB_B_SUB | RW | 0x00 |  |
| [23:16] | OB_GB_SUB | RW | 0x00 |  |
| [15:8] | OB_GR_SUB | RW | 0x00 |  |
| [7:0] | OB_R_SUB | RW | 0x00 |  |

REG_0x0c

- Size：32bits

- Addr：0x4007 000c

- Read/Write access：read/write

- Reset Value: 0x40

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [31:8] | TG_R | RO | 0x00 |  |
| [7:0] | TOTAL_GAIN | RW | 0x40 |  |

REG_0x10

- Size：32bits

- Addr：0x4007 0010

- Read/Write access：read/write

- Reset Value: 0x059904cc

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [31:28] | WBG_R1 | RO | 0x00 |  |
| [27:16] | WB_B_GAIN | RW | 0X599 |  |
| [15:12] | WBG_R0 | RO | 0x00 |  |
| [11:0] | WB_R_GAIN | RW | 0x4CC |  |

REG_0x14

- Size：32bits

- Addr：0x4007 0014

- Read/Write access：read/write

- Reset Value: 0x00

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [31:3] | GF_R | RO | 0x00 |  |
| [2:0] | GAMMA_FACTOR | RW | 0x00 |  |

REG_0x18

- Size：32bits

- Addr：0x4007 0018

- Read/Write access：read/write

- Reset Value: 0x40400000

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [31:24] | APERTURE_V | RW | 0x40 |  |
| [23:16] | APERTURE_H | RW | 0x40 |  |
| [15:12] | APERTURE_RESERVE | RO | 0x00 |  |
| [11:0] | APERTURE_TH | RW | 0x00 |  |

REG_0x1c

- Size：32bits

- Addr：0x4007 001c

- Read/Write access：read/write

- Reset Value: 0x40000040

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [31:24] | MATRIX_B_Y_G2 | RW | 0x40 |  |
| [23:16] | MATRIX_B_Y_G1 | RW | 0x00 |  |
| [15:8] | MATRIX_R_Y_G2 | RW | 0x00 |  |
| [7:0] | MATRIX_R_Y_G1 | RW | 0x40 |  |

REG_0x20

- Size：32bits

- Addr：0x4007 0020

- Read/Write access：read/write

- Reset Value: 0x40

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [31:16] | Y_OUT_R | RO | 0x00 |  |
| [15:8] | Y_OUT_OFFSET | RW | 0x00 |  |
| [7:0] | Y_OUT_GAIN | RW | 0x40 |  |

REG_0x24

- Size：32bits

- Addr：0x4007 0024

- Read/Write access：read/write

- Reset Value: 0x80

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [31:8] | PSEUDO_R | RO | 0x00 |  |
| [7:0] | PSEUDO_SUPPRESS | RW | 0x80 |  |

REG_0x28

- Size：32bits

- Addr：0x4007 0028

- Read/Write access：read/write

- Reset Value: 0x1E280064

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [31:30] | AE_AWB_R3 | RO | 0x00 |  |
| [29:24] | AE_AWB_ROW | RW | 0x1E |  |
| [23:22] | AE_AWB_R2 | RO | 0x00 |  |
| [21:16] | AE_AWB_COL | RW | 0x28 |  |
| [15:7] | AE_AWB_R1 | RO | 0x00 |  |
| [6:0] | AE_AWB_UNIT | RW | 0x64 |  |

REG_0x2c

- Size：32bits

- Addr：0x4007 002c

- Read/Write access：read/write

- Reset Value: 0x0301

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [31:16] | IN_OFFSET_R | RO | 0x00 |  |
| [15:8] | V_IN_OFFSET | RW | 0x03 |  |
| [7:0] | H_IN_OFFSET | RW | 0x01 |  |

REG_0x30

- Size：32bits

- Addr：0x4007 0030

- Read/Write access：read/write

- Reset Value: 0x05160808

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [31:24] | VS_WIDTH | RW | 0x05 |  |
| [23:16] | HS_WIDTH | RW | 0x16 |  |
| [15:8] | V_OUT_OFFSET | RW | 0x08 |  |
| [7:0] | H_OUT_OFFSET | RW | 0x08 |  |

REG_0x34

- Size：32bits

- Addr：0x4007 0034

- Read/Write access：read/write

- Reset Value: 0x08CA0898

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [31:28] | SIZE_R1 | RO | 0x00 |  |
| [27:16] | V_SIZE | RW | 0x8CA |  |
| [15:13] | SIZE_R0 | RO | 0x00 |  |
| [12:0] | H_SIZE | RW | 0x898 |  |

REG_0x38

- Size：32bits

- Addr：0x4007 0038

- Read/Write access：read/write

- Reset Value: 0x08700780

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [31:28] | OUT_SIZE_R1 | RO | 0x00 |  |
| [27:16] | V_OUT_SIZE | RW | 0x870 |  |
| [15:13] | OUT_SIZE_R0 | RO | 0x00 |  |
| [12:0] | H_OUT_SIZE | RW | 0x780 |  |

REG_0x3c

- Size：32bits

- Addr：0x4007 003c

- Read/Write access：read/write

- Reset Value: 0x00290060

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [31:28] | FR_BLK_R1 | RO | 0x00 |  |
| [27:16] | V_FR_BLK | RW | 0x29 |  |
| [15:13] | FR_BLK_R0 | RO | 0x00 |  |
| [12:0] | H_FR_BLK | RW | 0x60 |  |

4 Program Guide

Chapter 22 LISoC_APB_LISr

1 Function list

LISoC_APB_ LISr module is used to do advanced Image Sharpening . LISr ® is a proprietary technology developed by Logic and Design Inc.

2 Register list

Base address: 0x4008 0000

| Offset | Name | Access | Reset | Width | Description |
| --- | --- | --- | --- | --- | --- |
| 0x00 | REG_0x00 | R/W | 0x10000000 | 32 |  |
| 0x04 | REG_0x04 | R/W | 0x00 | 32 |  |
| 0x08 | REG_0x08 | R/W | 0x00 | 32 |  |
| 0x0c | REG_0x0c | R/W | 0x00 | 32 |  |
| 0x10 | REG_0x10 | R/W | 0x01 | 32 |  |
| 0x14 | REG_0x14 | R/W | 0x00 | 32 |  |
| 0x18 | REG_0x18 | R/W | 0x00 | 32 |  |
| 0x1c | REG_0x1c | R/W | 0x00 | 32 |  |
| 0x20 | REG_0x20 | R/W | 0x00 | 32 |  |
| 0x24 | REG_0x24 | R/W | 0x00 | 32 |  |
| 0x28 | REG_0x28 | R/W | 0x00 | 32 |  |
| 0x34 | REG_0x34 | R/W | 0x01 | 32 |  |
| 0x38 | REG_0x38 | R/W | 0x00 | 32 |  |
| 0x3c | REG_0x3c | R/W | 0x00 | 32 |  |
| 0x40 | REG_0x40 | R/W | 0x00 | 32 |  |
| 0x44 | REG_0x44 | R/W | 0x00 | 32 |  |
| 0x48 | REG_0x48 | R/W | 0x4000 | 32 |  |
| 0x 4c | REG_0x4c | R/W | 0x00 | 32 |  |
| 0x50 | REG_0x50 | R/W | 0x08 | 32 |  |
| 0x54 | REG_0x54 | R/W | 0xBF3F | 32 |  |
| 0x58 | REG_0x58 | R/W | 0xBF3F | 32 |  |
| 0x60 | REG_0x60 | R/W | 0x10000 | 32 |  |
| 0x64 | REG_0x64 | R/W | 0x10000 | 32 |  |
| 0x68 | REG_0x68 | R/W | 0x10000 | 32 |  |
| 0x 6c | REG_0x6c | R/W | 0x10000 | 32 |  |
| 0x7c | REG_0x7c | R/W | 0x00 | 32 |  |
| 0x80 | REG_0x80 | R/W | 0x00 | 32 |  |
| 0x84 | REG_0x84 | R/W | 0x00 | 32 |  |
| 0x88 | REG_0x88 | R/W | 0x00 | 32 |  |
| 0x8c | REG_0x8c | R/W | 0x00 | 32 |  |
| 0x90 | REG_0x90 | R/W | 0x00 | 32 |  |
| 0x 94 | REG_0x94 | R/W | 0x00 | 32 |  |
| 0x98 | REG_0x98 | R/W | 0x00 | 32 |  |
| 0x 9c | REG_0x9c | R/W | 0x00 | 32 |  |
| 0x a0 | REG_0xa0 | RO | 0x00 | 32 |  |
| 0x a4 | REG_0xa4 | RO | 0x00 | 32 |  |
| 0x a8 | REG_0xa8 | RO | 0x00 | 32 |  |
| 0xac | REG_0xac | RO | 0x00 | 32 |  |
| 0x b0 | REG_0xb0 | RO | 0x00 | 32 |  |
| 0x b4 | REG_0xb4 | RO | 0x00 | 32 |  |
| 0x b8 | REG_0xb8 | RO | 0x00 | 32 |  |
| 0x bc | REG_0xbc | RO | 0x00 | 32 |  |
| 0xc0 | REG_0xc0 | RO | 0x00 | 32 |  |
| 0x c4 | REG_0xc4 | RO | 0x00 | 32 |  |
| 0xc8 | REG_0xc8 | RO | 0x00 | 32 |  |
| 0xcc | REG_0xcc | RO | 0x00 | 32 |  |
| 0xd0 | REG_0xd0 | R/W | 0x10 | 32 |  |
| 0xd4 | REG_0xd4 | RO | 0x400000 | 32 |  |
| 0xec | REG_0xec | RO | 0x00 | 32 |  |
| 0xf0 | REG_0xf0 | R/W | 0x00 | 32 |  |
| 0xf4 | REG_0xf4 | R/W | 0x00 | 32 |  |
| 0xf8 | REG_0xf8 | R/W | 0x00 | 32 |  |
| 0xfc | REG_0xfc | RO | 0x00 | 32 |  |

3 Detailed definition of registers

REG_0x00

- Size：32bits

- Addr：0x4008 0000

- Read/Write access：read/write

- Reset Value: 0x10000000

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [31] | IN_SEL | RW | 0x00 |  |
| [29:28] | IN_FORM | RW | 0x01 |  |
| [26:24] | IN_FRATE | RW | 0x00 |  |
| [23:12] | IN_HSIZE | RW | 0x00 |  |
| [11:0] | IN_VSIZE | RW | 0x00 |  |

REG_0x04

- Size：32bits

- Addr：0x4008 0004

- Read/Write access：read/write

- Reset Value: 0x00

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [31] | INH_SD | RW | 0x00 |  |
| [30] | SD_MODE | RW | 0x00 |  |
| [27:16] | SD_HSIZE | RW | 0x00 |  |
| [11:0] | SD_VSIZE | RW | 0x00 |  |

REG_0x08

- Size：32bits

- Addr：0x4008 0008

- Read/Write access：read/write

- Reset Value: 0x00

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [27:16] | SD_HOFS | RW | 0x00 |  |
| [11:0] | SD_VOFS | RW | 0x00 |  |

REG_0x0c

- Size：32bits

- Addr：0x4008 000c

- Read/Write access：read/write

- Reset Value: 0x00

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [31:28] | OUT_RESO | RW | 0x00 |  |
| [27:25] | OUT_FRATE | RW | 0x00 |  |
| [24] | OUT_I_P | RW | 0x00 |  |
| [23:20] | AHE_BLKSIZ | RW | 0x00 |  |
| [17:8] | CROP_HOFF | RW | 0x00 |  |
| [7:0] | CROP_VOFF | RW | 0x00 |  |

REG_0x10

- Size：32bits

- Addr：0x4008 0010

- Read/Write access：read/write

- Reset Value: 0x01

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [7:6] | IN_RANGE | RW | 0x00 |  |
| [5:4] | OUT_RANGE | RW | 0x00 |  |
| [3] | T0444AVE | RW | 0x00 |  |
| [2] | T0422AVE | RW | 0x00 |  |
| [1] | Y8RND | RW | 0x00 |  |
| [0] | NR8MEM10 | RW | 0x01 |  |

REG_0x14

- Size：32bits

- Addr：0x4008 0014

- Read/Write access：read/write

- Reset Value: 0x00

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [20] | MED_FIL | RW | 0x00 |  |
| [19] | TMAP4 | RW | 0x00 |  |
| [18] | TMAP3 | RW | 0x00 |  |
| [17] | TMAP2 | RW | 0x00 |  |
| [16] | TMAP1 | RW | 0x00 |  |
| [14] | K_FIL4 | RW | 0x00 |  |
| [13] | K_ADDON4 | RW | 0x00 |  |
| [12] | K4_EXP | RW | 0x00 |  |
| [10] | K_FIL3 | RW | 0x00 |  |
| [9] | K_ADDON3 | RW | 0x00 |  |
| [8] | K3_EXP | RW | 0x00 |  |
| [6] | K_FIL2 | RW | 0x00 |  |
| [5] | K_ADDON2 | RW | 0x00 |  |
| [4] | K2_EXP | RW | 0x00 |  |
| [2] | K_FIL1 | RW | 0x00 |  |
| [1] | K_ADDON1 | RW | 0x00 |  |
| [0] | K1_EXP | RW | 0x00 |  |

REG_0x18

- Size：32bits

- Addr：0x4008 0018

- Read/Write access：read/write

- Reset Value: 0x00

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [9] | AHE_ON | RW | 0x00 |  |
| [8] | VIDEO_ON | RW | 0x00 |  |
| [6:4] | OUT_FORM | RW | 0x00 |  |
| [1] | OUT_THRU | RW | 0x00 |  |
| [0] | OUT_DISP_ON | RW | 0x00 |  |

REG_0x1c

- Size：32bits

- Addr：0x4008 001c

- Read/Write access：read/write

- Reset Value: 0x00

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [11:0] | AHE_HOFS | RW | 0x00 |  |

REG_0x20

- Size：32bits

- Addr：0x4008 0020

- Read/Write access：read/write

- Reset Value: 0x00

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [11:0] | AHE_HWIDH | RW | 0x00 |  |

REG_0x24

- Size：32bits

- Addr：0x4008 0024

- Read/Write access：read/write

- Reset Value: 0x00

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [11:0] | AHE_VOFS | RW | 0x00 |  |

REG_0x28

- Size：32bits

- Addr：0x4008 0028

- Read/Write access：read/write

- Reset Value: 0x00

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [11:0] | AHE_VWIDH | RW | 0x00 |  |

REG_0x34

- Size：32bits

- Addr：0x4008 0034

- Read/Write access：read/write

- Reset Value: 0x01

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [15:8] | CONTRAST | RW | 0x00 |  |
| [2] | T_MAP_SHAP | RW | 0x00 |  |
| [1] | AUTO_OFFSET | RW | 0x00 |  |
| [0] | PIXEL_CONV | RW | 0x01 |  |

REG_0x38

- Size：32bits

- Addr：0x4008 0038

- Read/Write access：read/write

- Reset Value: 0x00

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [15:8] | T_MAP_TH | RW | 0x00 |  |
| [7:0] | T_MAP_OFFSET | RW | 0x00 |  |

REG_0x3c

- Size：32bits

- Addr：0x4008 003c

- Read/Write access：read/write

- Reset Value: 0x00

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [15:8] | AOFF_VAL | RW | 0x00 |  |
| [5] | FCF_ON | RW | 0x00 |  |
| [4] | AHEF_ON | RW | 0x00 |  |
| [2] | UVENHON | RW | 0x00 |  |
| [1:0] | MOVESW | RW | 0x00 |  |

REG_0x40

- Size：32bits

- Addr：0x4008 0040

- Read/Write access：read/write

- Reset Value: 0x00

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [7:0] | ALPHA_AHE | RW | 0x00 |  |

REG_0x44

- Size：32bits

- Addr：0x4008 0044

- Read/Write access：read/write

- Reset Value: 0x00

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [7:0] | ALPHA_FC | RW | 0x00 |  |

REG_0x48

- Size：32bits

- Addr：0x4008 0048

- Read/Write access：read/write

- Reset Value: 0x4000

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [15:10] | CRATE | RW | 0x10 |  |
| [7:0] | CB_OFSET | RW | 0x00 |  |

REG_0x4c

- Size：32bits

- Addr：0x4008 004c

- Read/Write access：read/write

- Reset Value: 0x00

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [7:0] | CR_OFSET | RW | 0x00 |  |

REG_0x50

- Size：32bits

- Addr：0x4008 0050

- Read/Write access：read/write

- Reset Value: 0x08

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [11:10] | MDBLD | RW | 0x00 |  |
| [7:0] | MOVETHVAL | RW | 0x08 |  |

REG_0x54

- Size：32bits

- Addr：0x4008 0054

- Read/Write access：read/write

- Reset Value: 0xBF3F

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [15:8] | YBUF_BLD | RW | 0xBF |  |
| [7:0] | NEWY_BLD | RW | 0x3F |  |

REG_0x58

- Size：32bits

- Addr：0x4008 0058

- Read/Write access：read/write

- Reset Value: 0xBF3F

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [15:8] | UVBUF_BLD | RW | 0xBF |  |
| [7:0] | NEWUV_BLD | RW | 0x3F |  |

REG_0x60

- Size：32bits

- Addr：0x4008 0060

- Read/Write access：read/write

- Reset Value: 0x10000

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [23:16] | KVAL_1U | RW | 0x01 |  |
| [15:0] | KVAL_1L | RW | 0x00 |  |

REG_0x64

- Size：32bits

- Addr：0x4008 0064

- Read/Write access：read/write

- Reset Value: 0x10000

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [23:16] | KVAL_2U | RW | 0x01 |  |
| [15:0] | KVAL_2L | RW | 0x00 |  |

REG_0x68

- Size：32bits

- Addr：0x4008 0068

- Read/Write access：read/write

- Reset Value: 0x10000

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [23:16] | KVAL_3U | RW | 0x01 |  |
| [15:0] | KVAL_3L | RW | 0x00 |  |

REG_0x6c

- Size：32bits

- Addr：0x4008 006c

- Read/Write access：read/write

- Reset Value: 0x10000

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [23:16] | KVAL_4U | RW | 0x01 |  |
| [15:0] | KVAL_4L | RW | 0x00 |  |

REG_0x7c

- Size：32bits

- Addr：0x4008 007c

- Read/Write access：read/write

- Reset Value: 0x00

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [1] | HIST_ON | RW | 0x00 |  |
| [0] | AVE_YUV | RW | 0x00 |  |

REG_0x80

- Size：32bits

- Addr：0x4008 0080

- Read/Write access：read/write

- Reset Value: 0x00

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [11:0] | YAVE_XP | RW | 0x00 |  |

REG_0x84

- Size：32bits

- Addr：0x4008 0084

- Read/Write access：read/write

- Reset Value: 0x00

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [11:0] | YAVE_YP | RW | 0x00 |  |

REG_0x88

- Size：32bits

- Addr：0x4008 0088

- Read/Write access：read/write

- Reset Value: 0x00

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [11:0] | YAVE_HW | RW | 0x00 |  |

REG_0x8c

- Size：32bits

- Addr：0x4008 008c

- Read/Write access：read/write

- Reset Value: 0x00

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [11:0] | YAVE_VW | RW | 0x00 |  |

REG_0x90

- Size：32bits

- Addr：0x4008 0090

- Read/Write access：read/write

- Reset Value: 0x00

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [11:0] | CAVE_XP | RW | 0x00 |  |

REG_0x94

- Size：32bits

- Addr：0x4008 0094

- Read/Write access：read/write

- Reset Value: 0x00

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [11:0] | CAVE_YP | RW | 0x00 |  |

REG_0x98

- Size：32bits

- Addr：0x4008 0098

- Read/Write access：read/write

- Reset Value: 0x00

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [11:0] | CAVE_HW | RW | 0x00 |  |

REG_0x9c

- Size：32bits

- Addr：0x4008 009c

- Read/Write access：read/write

- Reset Value: 0x00

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [11:0] | CAVE_VW | RW | 0x00 |  |

REG_0xa0

- Size：32bits

- Addr：0x4008 00a0

- Read/Write access：read only

- Reset Value: 0x00

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [15:0] | YAVE_R1D | RO | 0x00 |  |

REG_0xa4

- Size：32bits

- Addr：0x4008 00a4

- Read/Write access：read only

- Reset Value: 0x00

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [15:0] | YAVE_R2D | RO | 0x00 |  |

REG_0xa8

- Size：32bits

- Addr：0x4008 00a8

- Read/Write access：read only

- Reset Value: 0x00

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [15:0] | YAVE_R3D | RO | 0x00 |  |

REG_0xac

- Size：32bits

- Addr：0x4008 00ac

- Read/Write access：read only

- Reset Value: 0x00

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [15:0] | YAVE_R4D | RO | 0x00 |  |

REG_0xb0

- Size：32bits

- Addr：0x4008 00b0

- Read/Write access：read only

- Reset Value: 0x00

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [15:0] | CBAVE_R1D | RO | 0x00 |  |

REG_0xb4

- Size：32bits

- Addr：0x4008 00b4

- Read/Write access：read only

- Reset Value: 0x00

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [15:0] | CBAVE_R2D | RO | 0x00 |  |

REG_0xb8

- Size：32bits

- Addr：0x4008 00b8

- Read/Write access：read only

- Reset Value: 0x00

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [15:0] | CBAVE_R3D | RO | 0x00 |  |

REG_0xbc

- Size：32bits

- Addr：0x4008 00bc

- Read/Write access：read only

- Reset Value: 0x00

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [15:0] | CBAVE_R4D | RO | 0x00 |  |

REG_0xc0

- Size：32bits

- Addr：0x4008 00c0

- Read/Write access：read only

- Reset Value: 0x00

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [15:0] | CRAVE_R1D | RO | 0x00 |  |

REG_0xc4

- Size：32bits

- Addr：0x4008 00c4

- Read/Write access：read only

- Reset Value: 0x00

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [15:0] | CRAVE_R2D | RO | 0x00 |  |

REG_0xc8

- Size：32bits

- Addr：0x4008 00c8

- Read/Write access：read only

- Reset Value: 0x00

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [15:0] | CRAVE_R3D | RO | 0x00 |  |

REG_0xcc

- Size：32bits

- Addr：0x4008 00cc

- Read/Write access：read only

- Reset Value: 0x00

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [15:0] | CRAVE_R4D | RO | 0x00 |  |

REG_0xd0

- Size：32bits

- Addr：0x4008 00d0

- Read/Write access：read/write

- Reset Value: 0x10

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [31:23] | PN_SW_LED1 | R/W | 0x00 |  |
| [4] | PN_SW1_ST | RO | 0x01 |  |
| [3:0] | PN_SW1_CN | RO | 0x00 |  |

REG_0xd4

- Size：32bits

- Addr：0x4008 00d4

- Read/Write access：read only

- Reset Value: 0x400000

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [22] | RE_PSW_ST | RO | 0x01 |  |
| [21] | RE_A_ST | RO | 0x00 |  |
| [20] | RE_B_ST | RO | 0x00 |  |
| [19:16] | PN_RE_PSW_CN | RO | 0x00 |  |
| [15:0] | PN_RESW_CN | RO | 0x00 |  |

REG_0xec

- Size：32bits

- Addr：0x4008 00ec

- Read/Write access：read/write

- Reset Value: 0x00

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [31] | FLG_CLR | WC | 0x00 |  |
| [18] | ORUN_SCALE | RO | 0x00 |  |
| [17] | URUN_SCALE | RO | 0x00 |  |
| [16] | URUN_LBUF_T | RO | 0x00 |  |
| [15] | ORUN_LBUF | RO | 0x00 |  |
| [14] | ORUN_LBUF_B | RO | 0x00 |  |
| [13] | ORUN_LBUF_A | RO | 0x00 |  |
| [12] | ORUN_MD | RO | 0x00 |  |
| [11] | URUN_MD | RO | 0x00 |  |
| [10] | ORUN_YNR | RO | 0x00 |  |
| [9] | URUN_YNR | RO | 0x00 |  |
| [8] | ORUN_UVNR | RO | 0x00 |  |
| [7] | URUN_UVNR | RO | 0x00 |  |

REG_0xf0

- Size：32bits

- Addr：0x4008 00f0

- Read/Write access：read/write

- Reset Value: 0x00

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [1] | SOFT_RST | R/W | 0x00 |  |
| [0] | SYS_RST | R/W | 0x00 |  |

REG_0xf4

- Size：32bits

- Addr：0x4008 00f4

- Read/Write access：read/write

- Reset Value: 0x00

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [4] | LEDC | R/W | 0x00 |  |
| [3:0] | LEDP | R/W | 0x00 |  |

REG_0xf8

- Size：32bits

- Addr：0x4008 00f8

- Read/Write access：read/write

- Reset Value: 0x00

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [3:0] | DIPSW | R/W | 0x00 |  |

REG_0xfc

- Size：32bits

- Addr：0x4008 00fc

- Read/Write access：read only

- Reset Value: 0x00

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [15:8] | PRODUCT | RO | 0x00 |  |
| [7:0] | REVISION | RO | 0x00 |  |

4 Program Guide

Chapter 23 LISoC_APB_AF

  - Function list

LISoC_APB_ AF module is used to do Image auto-focus.

  - Register list

| Offset | Name | Access | Reset | Width | Description |
| --- | --- | --- | --- | --- | --- |
| 0x00 | REG_0x00 | RO | 0x00 | 32 |  |
| 0x04 | REG_0x04 | R/W | 0x7980C00 | 32 |  |
| 0x08 | REG_0x08 | R/W | 0x6C0180 | 32 |  |
| 0x0c | REG_0x0c | R/W | 0x010001 | 32 |  |
| 0x10 | REG_0x10 | R/W | 0x10 | 32 |  |
| 0x14 | REG_0x14 | R/W | 0x00 | 32 |  |

  - Detailed definition of registers

REG_0x00

- Size：32bits

- Addr：0x400b 0000

- Read/Write access：write only

- Reset Value: 0x00

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [0] | SFTRST | WO | 0x00 |  |

REG_0x04

- Size：32bits

- Addr：0x400b 0004

- Read/Write access：read/write

- Reset Value: 0x7980C00

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [27:16] | V_P_AREA_SIZE | RW | 0x798 |  |
| [11:0] | H_P_AREA_SIZE | RW | 0xC00 |  |

REG_0x08

- Size：32bits

- Addr：0x400b 0008

- Read/Write access：read/write

- Reset Value: 0x6C0180

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [27:16] | V_OFFSET | RW | 0x6C |  |
| [11:0] | H_OFFSET | RW | 0x180 |  |

REG_0x0c

- Size：32bits

- Addr：0x400b 000c

- Read/Write access：read/write

- Reset Value: 0x10001

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [18:16] | V_DIV | RW | 0x01 |  |
| [2:0] | H_DIV | RW | 0x01 |  |

REG_0x10

- Size：32bits

- Addr：0x400b 0010

- Read/Write access：read/write

- Reset Value: 0x059904cc

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [31:28] | SSB | RO | 0x00 |  |

REG_0x14

- Size：32bits

- Addr：0x400b 0014

- Read/Write access：read/write

- Reset Value: 0x00

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [1:0] | STATUS_CLR_RES | RW | 0x00 |  |

4 Program Guide

Chapter 24 LISoC_APB_TPGEN

  - Function list

LISoC_APB_ TPGEN module is used to send Image data to test the function of LISoC datapath.

  - Register list

| Offset | Name | Access | Reset | Width | Description |
| --- | --- | --- | --- | --- | --- |
| 0x00 | REG_0x00 | R/W | 0x8CA0960 | 32 |  |
| 0x04 | REG_0x04 | R/W | 0x30000 | 32 |  |
| 0x08 | REG_0x08 | R/W | 0x00 | 32 |  |
| 0x0c | REG_0x0c | R/W | 0x880078C | 32 |  |
| 0x10 | REG_0x10 | R/W | 0x64C0096 | 32 |  |
| 0x14 | REG_0x14 | R/W | 0x11A008D | 32 |  |
| 0x18 | REG_0x18 | R/W | 0xFFF0000 | 32 |  |
| 0x1c | REG_0x1c | R/W | 0xFF0000 | 32 |  |
| 0x20 | REG_0x20 | R/W | 0xFF0000 | 32 |  |
| 0x24 | REG_0x24 | R/W | 0x00 | 32 |  |
| 0x28 | REG_0x28 | R/W | 0x00 | 32 |  |
| 0x30 | REG_0x30 | R/W | 0x8CA0898 | 32 |  |
| 0x34 | REG_0x34 | R/W | 0x30000 | 32 |  |
| 0x38 | REG_0x38 | R/W | 0x00 | 32 |  |
| 0x3c | REG_0x3c | R/W | 0x880078C | 32 |  |
| 0x40 | REG_0x40 | R/W | 0x64C0096 | 32 |  |
| 0x44 | REG_0x44 | R/W | 0x11A008D | 32 |  |
| 0x48 | REG_0x48 | R/W | 0xFFF0000 | 32 |  |
| 0x 4c | REG_0x4c | R/W | 0xFFF0000 | 32 |  |
| 0x50 | REG_0x50 | R/W | 0xFFF0000 | 32 |  |
| 0x54 | REG_0x54 | R/W | 0x00 | 32 |  |
| 0x58 | REG_0x58 | R/W | 0x00 | 32 |  |
| 0x60 | REG_0x60 | R/W | 0x8CA0898 | 32 |  |
| 0x64 | REG_0x64 | R/W | 0xA002C | 32 |  |
| 0x68 | REG_0x68 | R/W | 0x5200C0 | 32 |  |
| 0x6c | REG_0x6c | R/W | 0x8700780 | 32 |  |
| 0x70 | REG_0x70 | R/W | 0x6400095 | 32 |  |
| 0x74 | REG_0x74 | R/W | 0x118008C | 32 |  |
| 0x78 | REG_0x78 | R/W | 0xFFF0000 | 32 |  |
| 0x7c | REG_0x7c | R/W | 0xFFF0000 | 32 |  |
| 0x80 | REG_0x80 | R/W | 0xFFF0000 | 32 |  |
| 0x84 | REG_0x84 | R/W | 0x00 | 32 |  |
| 0x88 | REG_0x88 | R/W | 0x00 | 32 |  |
| 0x90 | REG_0x90 | R/W | 0x8CA0898 | 32 |  |
| 0x94 | REG_0x94 | R/W | 0x10000 | 32 |  |
| 0x98 | REG_0x98 | R/W | 0x00 | 32 |  |
| 0x9c | REG_0x9c | R/W | 0x8700780 | 32 |  |
| 0xa0 | REG_0xa0 | R/W | 0x6400095 | 32 |  |
| 0xa4 | REG_0xa4 | R/W | 0x118008C | 32 |  |
| 0xa8 | REG_0xa8 | R/W | 0xFFF0000 | 32 |  |
| 0xac | REG_0xac | R/W | 0xFFF0000 | 32 |  |
| 0xb0 | REG_0xb0 | R/W | 0xFFF0000 | 32 |  |
| 0xb4 | REG_0xb4 | R/W | 0x00 | 32 |  |
| 0xb8 | REG_0xb8 | R/W | 0x00 | 32 |  |

3 Detailed definition of registers

REG_0x00

- Size：32bits

- Addr：0x400c 0000

- Read/Write access：read/write

- Reset Value: 0x8CA0960

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [27:16] | FRMV_SIZE_CCHG | RW | 0x8CA |  |
| [12:0] | FRMH_SIZE_CCHG | RW | 0x960 |  |

REG_0x04

- Size：32bits

- Addr：0x400c 0004

- Read/Write access：read/write

- Reset Value: 0x30000

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [22,16] | V_OFFSET_CCHG | RW | 0x03 |  |
| [3,0] | H_OFFSET_CCHG | RW | 0x00 |  |

REG_0x08

- Size：32bits

- Addr：0x400c 0008

- Read/Write access：read/write

- Reset Value: 0x00

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [22,16] | V_OFFSET2_CCHG | RW | 0x00 |  |

REG_0x0c

- Size：32bits

- Addr：0x400c 000c

- Read/Write access：read/write

- Reset Value: 0x880078C

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [27,16] | TPV_SIZE_CCHG | RW | 0x880 |  |
| [11,0] | TPH_SIZE_CCHG | RW | 0x78C |  |

REG_0x10

- Size：32bits

- Addr：0x400c 0010

- Read/Write access：read/write

- Reset Value: 0x64C0096

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [26,16] | GSV_SIZE_CCHG | RW | 0x64C |  |
| [7,0] | GSH_SIZE_CCHG | RW | 0x96 |  |

REG_0x14

- Size：32bits

- Addr：0x400c 0014

- Read/Write access：read/write

- Reset Value: 0x11A008D

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [24,16] | GSV_START_CCHG | RO | 0x11A |  |
| [7,0] | GSH_START_CCHG | RW | 0x8D |  |

REG_0x18

- Size：32bits

- Addr：0x400c 0018

- Read/Write access：read/write

- Reset Value: 0xFFF0000

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [27,16] | CG_R_E_CCHG | RW | 0xFFF |  |
| [11,0] | CG_R_S_CCHG | RW | 0x0 |  |

REG_0x1c

- Size：32bits

- Addr：0x400c 001c

- Read/Write access：read/write

- Reset Value: 0xFFF0000

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [27,16] | CG_G_E_CCHG | RW | 0xFFF |  |
| [11,0] | CG_G_S_CCHG | RW | 0x0 |  |

REG_0x20

- Size：32bits

- Addr：0x400c 0020

- Read/Write access：read/write

- Reset Value: 0xFFF0000

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [27,16] | CG_B_E_CCHG | RW | 0xFFF |  |
| [11,0] | CG_B_S_CCHG | RW | 0x0 |  |

REG_0x24

- Size：32bits

- Addr：0x400c 0024

- Read/Write access：read/write

- Reset Value: 0x00

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [17,16] | PH_CCHG | RW | 0x00 |  |
| [1,0] | PT_CCHG | RW | 0x00 |  |

REG_0x28

- Size：32bits

- Addr：0x400c 0028

- Read/Write access：read/write

- Reset Value: 0x00

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [0] | ENABLE_CCHG | RW | 0x00 |  |

REG_0x30

- Size：32bits

- Addr：0x400c 0030

- Read/Write access：read/write

- Reset Value: 0x8CA0898

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [27:16] | FRMV_SIZE_ISP | RW | 0x8CA |  |
| [12:0] | FRMH_SIZE_ISP | RW | 0x898 |  |

REG_0x34

- Size：32bits

- Addr：0x400c 0034

- Read/Write access：read/write

- Reset Value: 0x30000

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [22,16] | V_OFFSET_ISP | RW | 0x03 |  |
| [3,0] | H_OFFSET_ISP | RW | 0x00 |  |

REG_0x38

- Size：32bits

- Addr：0x400c 0038

- Read/Write access：read/write

- Reset Value: 0x00

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [22,16] | V_OFFSET2_ISP | RW | 0x00 |  |

REG_0x3c

- Size：32bits

- Addr：0x400c 003c

- Read/Write access：read/write

- Reset Value: 0x880078C

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [27,16] | TPV_SIZE_ISP | RW | 0x880 |  |
| [11,0] | TPH_SIZE_ISP | RW | 0x78C |  |

REG_0x40

- Size：32bits

- Addr：0x400c 0040

- Read/Write access：read/write

- Reset Value: 0x64C0096

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [26,16] | GSV_SIZE_ISP | RW | 0x64C |  |
| [7,0] | GSH_SIZE_ISP | RW | 0x96 |  |

REG_0x44

- Size：32bits

- Addr：0x400c 0044

- Read/Write access：read/write

- Reset Value: 0x11A008D

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [24,16] | GSV_START_ISP | RO | 0x11A |  |
| [7,0] | GSH_START_ISP | RW | 0x8D |  |

REG_0x48

- Size：32bits

- Addr：0x400c 0048

- Read/Write access：read/write

- Reset Value: 0xFFF0000

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [27,16] | CG_R_E_ISP | RW | 0xFFF |  |
| [11,0] | CG_R_S_ISP | RW | 0x0 |  |

REG_0x4c

- Size：32bits

- Addr：0x400c 004c

- Read/Write access：read/write

- Reset Value: 0xFFF0000

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [27,16] | CG_G_E_ISP | RW | 0xFFF |  |
| [11,0] | CG_G_S_ISP | RW | 0x0 |  |

REG_0x50

- Size：32bits

- Addr：0x400c 0050

- Read/Write access：read/write

- Reset Value: 0xFFF0000

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [27,16] | CG_B_E_ISP | RW | 0xFFF |  |
| [11,0] | CG_B_S_ISP | RW | 0x0 |  |

REG_0x54

- Size：32bits

- Addr：0x400c 0054

- Read/Write access：read/write

- Reset Value: 0x00

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [17,16] | PH_ISP | RW | 0x00 |  |
| [1,0] | PT_ISP | RW | 0x00 |  |

REG_0x58

- Size：32bits

- Addr：0x400c 0058

- Read/Write access：read/write

- Reset Value: 0x00

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [0] | ENABLE_ISP | RW | 0x00 |  |

REG_0x60

- Size：32bits

- Addr：0x400c 0060

- Read/Write access：read/write

- Reset Value: 0x8CA0898

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [27:16] | FRMV_SIZE_LISR | RW | 0x8CA |  |
| [12:0] | FRMH_SIZE_LISR | RW | 0x898 |  |

REG_0x64

- Size：32bits

- Addr：0x400c 0064

- Read/Write access：read/write

- Reset Value: 0xA002C

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [19,16] | VSW_LISR | RW | 0xA |  |
| [5,0] | HSW_LISR | RW | 0x2C |  |

REG_0x68

- Size：32bits

- Addr：0x400c 0068

- Read/Write access：read/write

- Reset Value: 0x5200C0

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [22,16] | VFRBLK_LISR | RW | 0x52 |  |
| [7,0] | HFRBLK_LISR | RW | 0xC0 |  |

REG_0x6c

- Size：32bits

- Addr：0x400c 006c

- Read/Write access：read/write

- Reset Value: 0x8700780

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [27,16] | TPV_SIZE_LISR | RW | 0x870 |  |
| [11,0] | TPH_SIZE_LISR | RW | 0x780 |  |

REG_0x70

- Size：32bits

- Addr：0x400c 0070

- Read/Write access：read/write

- Reset Value: 0x6400095

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [26,16] | GSV_SIZE_LISR | RW | 0x640 |  |
| [7,0] | GSH_SIZE_LISR | RW | 0x95 |  |

REG_0x74

- Size：32bits

- Addr：0x400c 0074

- Read/Write access：read/write

- Reset Value: 0x118008C

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [24,16] | GSV_START_LISR | RO | 0x118 |  |
| [7,0] | GSH_START_LISR | RW | 0x8C |  |

REG_0x78

- Size：32bits

- Addr：0x400c 0078

- Read/Write access：read/write

- Reset Value: 0xFFF0000

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [27,16] | CG_R_E_LISR | RW | 0xFFF |  |
| [11,0] | CG_R_S_LISR | RW | 0x0 |  |

REG_0x7c

- Size：32bits

- Addr：0x400c 007c

- Read/Write access：read/write

- Reset Value: 0xFFF0000

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [27,16] | CG_G_E_LISR | RW | 0xFFF |  |
| [11,0] | CG_G_S_LISR | RW | 0x0 |  |

REG_0x80

- Size：32bits

- Addr：0x400c 0080

- Read/Write access：read/write

- Reset Value: 0xFFF0000

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [27,16] | CG_B_E_LISR | RW | 0xFFF |  |
| [11,0] | CG_B_S_LISR | RW | 0x0 |  |

REG_0x84

- Size：32bits

- Addr：0x400c 0084

- Read/Write access：read/write

- Reset Value: 0x00

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [17,16] | PH_LISR | RW | 0x00 |  |
| [1,0] | PT_LISR | RW | 0x00 |  |

REG_0x88

- Size：32bits

- Addr：0x400c 0088

- Read/Write access：read/write

- Reset Value: 0x00

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [0] | ENABLE_LISR | RW | 0x00 |  |

REG_0x90

- Size：32bits

- Addr：0x400c 0090

- Read/Write access：read/write

- Reset Value: 0x8CA0898

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [27:16] | FRMV_SIZE_MIPITX | RW | 0x8CA |  |
| [12:0] | FRMH_SIZE_MIPITX | RW | 0x898 |  |

REG_0x94

- Size：32bits

- Addr：0x400c 0094

- Read/Write access：read/write

- Reset Value: 0x10000

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [22,16] | V_OFFSET_MIPITX | RW | 0x01 |  |
| [3,0] | H_OFFSET_MIPITX | RW | 0x00 |  |

REG_0x98

- Size：32bits

- Addr：0x400c 0098

- Read/Write access：read/write

- Reset Value: 0x00

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [22,16] | V_OFFSET2_MIPITX | RW | 0x00 |  |

REG_0x9c

- Size：32bits

- Addr：0x400c 009c

- Read/Write access：read/write

- Reset Value: 0x8700780

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [27,16] | TPV_SIZE_MIPITX | RW | 0x870 |  |
| [11,0] | TPH_SIZE_MIPITX | RW | 0x780 |  |

REG_0xa0

- Size：32bits

- Addr：0x400c 00a0

- Read/Write access：read/write

- Reset Value: 0x6400095

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [26,16] | GSV_SIZE_MIPITX | RW | 0x640 |  |
| [7,0] | GSH_SIZE_MIPITX | RW | 0x95 |  |

REG_0xa4

- Size：32bits

- Addr：0x400c 00a4

- Read/Write access：read/write

- Reset Value: 0x118008C

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [24,16] | GSV_START_MIPITX | RO | 0x118 |  |
| [7,0] | GSH_START_MIPITX | RW | 0x8C |  |

REG_0xa8

- Size：32bits

- Addr：0x400c 00a8

- Read/Write access：read/write

- Reset Value: 0xFFF0000

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [27,16] | CG_R_E_MIPITX | RW | 0xFFF |  |
| [11,0] | CG_R_S_MIPITX | RW | 0x0 |  |

REG_0xac

- Size：32bits

- Addr：0x400c 00ac

- Read/Write access：read/write

- Reset Value: 0xFFF0000

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [27,16] | CG_G_E_MIPITX | RW | 0xFFF |  |
| [11,0] | CG_G_S_MIPITX | RW | 0x0 |  |

REG_0xb0

- Size：32bits

- Addr：0x400c 00b0

- Read/Write access：read/write

- Reset Value: 0xFFF0000

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [27,16] | CG_B_E_MIPITX | RW | 0xFFF |  |
| [11,0] | CG_B_S_MIPITX | RW | 0x0 |  |

REG_0xb4

- Size：32bits

- Addr：0x400c 00b4

- Read/Write access：read/write

- Reset Value: 0x00

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [17,16] | PH_MIPITX | RW | 0x00 |  |
| [1,0] | PT_MIPITX | RW | 0x00 |  |

REG_0xb8

- Size：32bits

- Addr：0x400c 00b8

- Read/Write access：read/write

- Reset Value: 0x00

| Bits | Name | Access | Reset | Definition |
| --- | --- | --- | --- | --- |
| [0] | ENABLE_MIPITX | RW | 0x00 |  |